3GPP TSG-RAN WG2 Meeting #122	R2-2305687

Incheon, Korea, May 22-26, 2023	(Rev. of R2-2303233)

Agenda item:		7.15.4

Source:		Lenovo

Title:		Other remaining issue for NR SL-U

Document for:		Discussion and Decision

- Introduction

This paper will discuss the other remaining issue for NR SL-U, which includes leftovers of CAPC for RRC inactive/idle/ooc UE, autonomous retransmission for CG, and SL DRX issues. The corresponding agreement [2] in last meeting is as following

Agreement on SL CAPC:

For ‘best-match’ issue, UE may determine it based on closest PDB, and capture it in stage-2 spec only. Detailed wording can be discussed in running CR phase. FFS on whether to consider default priority as well.

Agreement on SL CG

Confirm the working assumption: Not to support CG retransmission timer in SL-U.

- Remaining issues for CAPC

In last meeting, it was agreed UE in RRC inactive/idle state or out-of-coverage may determine CAPC for non-standardized PQI based on closest PDB of standardized PQI. For a non-standardized PQI, UE will find a standard PQI which has the closest PDB with the non-standardized PQI, and using the CAPC of this standard PQI for the non-standardized PQI. With this scheme, all UEs will determine the same CAPC for the same non-standardized PQI no matter the UE is IC/OOC, or in which RRC state.

However, when UE determines the standardized PQI with closest PDB, there may be multiple standardized PQIs with the same closest PDB, having different CAPC. For example, as analysed in [3], when PDB of non-standardized PQI is 300ms, PQIs with closest PDB are PQI 25, 26 and 61, which has CAPC of 2/1/3. In this case, if left to UE implementation to select CAPC, there still happens unaligned CAPC among UEs. So it is better to specify a simple rule, e.g. select PQI with minimum CAPC value if multiple standardized PQIs are determined as the same closest PDB.

Proposal 1: For ‘best match’ issue, if multiple standardized PQIs with same closest PDB are determined, specify a simple rule to select CAPC among these standardized PQIs e.g. select PQI with minimum/maximum CAPC value

Another issue is whether to consider the default priority when determine ‘best matches’ PQIs. When determining the mapping table for standardized PQI, there considered services with high priority e.g. PQIs of mission critical services were all mapped to CAPC#1 as for legacy NR-U. So it is better to align with the CAPC mapping rule for standardized PQI, that default priority is also considered when determining the ‘best match’ PQI.

Proposal 2: For ‘best match’ issue, consider default priority as well.

Then how to consider default priority for ‘best match’ issue could check and reuse the case of mission critical services. According to PQI table in Annex, it is observed that mission critical services have a default priority 1 or 2. But on the other hand, not all PQIs with default priority 2 are mission critical services. So, if consider default priority as well for ‘best match’ issue, those non-standardized PQI with default priority 1 can be mapped to CAPC 1.

Proposal 3: For ‘best match’ issue, those non-standardized PQI with default priority 1 can be mapped to CAPC#1.

Another remaining issue is the CAPC determination for SL bearer without configured with CAPC, as proposed in [4]. The case is that for a (default) SL bearer of a SL UE in RRC inactive/idle state or out-of-coverage, if the CAPC value if not configured in SIB/pre-configuration, how to determine the CAPC of the (default) SL bearer according to the CAPC of associated QoS flow. Firstly we think this might be a corner case since this could be easily solved by network implementation. If UE based solution needs to be specified, we slightly prefer to align the UE’s behaviour e.g. the CAPC of the SLRB will be determined by the lowest CAPC priority among the CAPC of all QoS flow(s) in the SLRB.

Proposal 4: if UE based solution is needed for a SLRB without configured with CAPC, the CAPC of the SLRB will be determined by the lowest CAPC priority among the CAPC of all QoS flow(s) in the SLRB

- CAPC indication by gNB

In NR-U the CAPC value is either selected by the gNB, e.g. for dynamically scheduled PUSCH transmissions, or selected by the UE autonomously, i.e. for CG PUSCH transmissions. It needs to be discussed how the CAPC value is selected for SL transmissions when operating in shared spectrum channel access considering that SL resource allocation can either done by the gNB (mode 1) or by the Tx UE autonomously (mode 2).

For sidelink UE in mode 1 transmission, one may argue that the case is very similar as for legacy NR-U, and hence the CAPC for a SL TB transmission can be configured by gNB directly. For SL transmission scheduled by gNB, gNB can indicate CAPC for SL transmission in DCI for SL grant. However, the assumption for NR-U was that gNB is aware of which data (e.g. which logical channels/MAC CEs) is multiplexed in a UL grant. This assumption however doesn’t hold for sidelink, e.g. gNB cannot really predict which data Tx UE multiplexes on a SL grant since gNB cannot know which transmission parameters the UE selects. Even though there are differences between NR-U and SL-U with respect to Mode 1 CAPC handling, we think reusing the NR-U principles would be sufficient. It seems anyway that this is the assumption. Therefore, we have the following proposal:

Proposal 5: RAN2 to confirm that SL DCI allocating SL resources to a UE indicates a CAPC index/value within the DCI which is to be used for when performing a LBT for the transmission of the TB generated according to the allocated SL resources

- CG impact

In NR-U, CG is enhanced considering the DL feedback/retransmit scheduling might be missed because of LBT failure. Autonomous retransmission is introduced to avoid UL retransmission of CG waiting too long. Further, autonomous retransmission of CG can utilize other CG configuration with same TB size, thus HARQ process id indication in CG-UCI is introduced, and NR-U UE can select HARQ process id by itself according to the retransmission on CG resource. For SL-U, it is necessary to discuss whether NR-U CG feature e.g. autonomous retransmission and UE selection HARQ process id needs to be introduced or not.

For autonomous retransmission, which purpose is to handling feedback missing or retransmission schedule missing because of LBT failure. When considering SL-U, for SL-U UE in mode 1 resource allocation, if PSSCH transmission can’t be performed due to LBT failure, no HARQ feedback will be received for cases that PSFCH is configured. According to the current specified procedure UE will not deliver a NACK to the corresponding sidelink HARQ entity. Only for cases when the PSSCH transmission was performed and no feedback is no acknowledgement is obtained from the physical layer, UE will deliver a NACK.

The MAC entity shall for each PSSCH transmission: 1> if an acknowledgement corresponding to the PSSCH transmission in clause 5.22.1.3.1a is obtained from the physical layer: 2> deliver the acknowledgement to the corresponding Sidelink HARQ entity for the Sidelink process; 1> else: 2> deliver a negative acknowledgement to the corresponding Sidelink HARQ entity for the Sidelink process; <omit>

We think that UE should treat a LBT failure for a PSSCH transmission as if no HARQ feedback is received for the corresponding Sidelink HARQ process. We see some benefit if UE triggers an autonomous retransmission for this case. So we have the following proposal

Proposal 6: Support autonomous retransmission if SL CG transmission is dropped due to LBT failure

The feature of HARQ process ID and RV selection for CG transmission is linked to the configuration of the CGRT timer, e.g. support of autonomous retransmissions due to LBT failures. Since in NR-U, there might not be a resource for autonomous retransmission, or the resource is in next period and very late for retransmission, UE can also perform resources of another CG configuration as long as the TB sizes match,, i.e. . If cg-RetransmissionTimer is configured, retransmissions with the same HARQ process may be performed on any configured grant configuration if the configured grant configurations have the same TBS. The selected HARQ process ID is indicated in the CG-UCI. In NR SL, for one SL CG configuration, multiple resource can be reserved for both initial transmission and retransmission, whether it is necessary to use resource of other CG configuration needs to be further discussed. And consider the multiple consecutive PSSCH transmission is still studied in RAN1, whether to introduce the feature of UE autonomous select HARQ process id for SL CG transmission can wait for RAN1 progress.

Proposal 7: The discussion of necessity to introduce the feature of UE autonomous select HARQ process id for SL CG transmission can wait for RAN1 progress.

In legacy NR SL, for SL CG transmission, to determine when to flush HARQ buffer, Tx UE is configured with the parameter sl-MaxTransNum, and Tx UE will flush HARQ buffer when the transmission number during one SL CG period reached sl-MaxTransNum. In SL-U, if the transmission is failure because of LBT failure, the transmission number of MAC PDU should not be increased, otherwise sl-MaxTransNum will be easily reached and HARQ buffer will be flushed.

Proposal 8: For UE configured with mode 1 and CG, a PSSCH transmission dropped by LBT failure does not count as a transmission and increase the transmission number for sl-MaxTransNum comparison

- Remaining issue for SL-DRX

In the last meeting, whether to confirm the working assumption for SL-DRX i.e. Not define shared COT as SL DRX active time has no conclusion. It was commented by company that this relates to the additional id that was specified in RAN1. In last RAN1 meeting, additional id issue is still FFS as following

Agreement At least the following information should be used as part of COT sharing information from the COT initiator UE. - CAPC used for initiating the COT - Existing / legacy R16/17 L1 source and destination IDs - FFS additional ID(s) - Time domain information of the shared COT - FFS: starting offset, number of slots, [remaining or total] COT duration, or a combination of them - Frequency domain information of the shared COT - FFS applicable RB set(s), FRIV, and any other(s) - FFS: how each of the above is indicated. - Note, other information is not precluded.

So it is suggested to still postpone to confirm the working assumption for SL-DRX.

Proposal 9: RAN2 should wait for RAN1 decision on additional id in COT sharing before deciding whether to confirm WA or not for active time for COT duration.

- Conclusion

In this contribution, the following observations and proposals are made:

Proposal 1: For ‘best match’ issue, if multiple standardized PQIs with same closest PDB are determined, specify a simple rule to select CAPC among these standardized PQIs e.g. select PQI with minimum/maximum CAPC value

Proposal 2: For ‘best match’ issue, consider default priority as well.

Proposal 3: For ‘best match’ issue, those non-standardized PQI with default priority 1 can be mapped to CAPC#1.

Proposal 4: if UE based solution is needed for a SLRB without configured with CAPC, the CAPC of the SLRB will be determined by the lowest CAPC priority among the CAPC of all QoS flow(s) in the SLRB

Proposal 5: RAN2 to confirm that SL DCI allocating SL resources to a UE indicates a CAPC index/value within the DCI which is to be used for when performing a LBT for the transmission of the TB generated according to the allocated SL resources

Proposal 6: Support autonomous retransmission if SL CG transmission is dropped due to LBT failure

Proposal 7: The discussion of necessity to introduce the feature of UE autonomous select HARQ process id for SL CG transmission can wait for RAN1 progress.

Proposal 8: For UE configured with mode 1 and CG, a PSSCH transmission dropped by LBT failure does not count as a transmission and increase the transmission number for sl-MaxTransNum comparison

Proposal 9: Whether confirm WA or not for active time for COT duration is wait for RAN1 decision of additional id in COT sharing

- References

RP-230077, WID revision: NR sidelink evolution

RAN2#121bis-e meeting chairman minutes

R2-2303914	Discussion on CAPC for non-standardized PQI to decide 'best match', vivo, Lenovo, InterDigital, ASUSTeK, Huawei, HiSilicon

R2-2302622, Consideration on CAPC and LBT Impacts, CATT

- Annex: PQI table

Table 5.4.4-1: Standardized PQI to QoS characteristics mapping [5]

| PQI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 21 | GBR | 3 | 20 ms | 10-4 | N/A | 2000 ms | Platooning between UEs – Higher degree of automation; Platooning between UE and RSU – Higher degree of automation |
| 22 | (NOTE 1) | 4 | 50 ms | 10-2 | N/A | 2000 ms | Sensor sharing – higher degree of automation |
| 23 |  | 3 | 100 ms | 10-4 | N/A | 2000 ms | Information sharing for automated driving – between UEs or UE and RSU - higher degree of automation |
| 55 | Non-GBR | 3 | 10 ms | 10-4 | N/A | N/A | Cooperative lane change – higher degree of automation |
| 56 |  | 6 | 20 ms | 10-1 | N/A | N/A | Platooning informative exchange – low degree of automation; Platooning – information sharing with RSU |
| 57 |  | 5 | 25 ms | 10-1 | N/A | N/A | Cooperative lane change – lower degree of automation |
| 58 |  | 4 | 100 ms | 10-2 | N/A | N/A | Sensor information sharing – lower degree of automation |
| 59 |  | 6 | 500 ms | 10-1 | N/A | N/A | Platooning – reporting to an RSU |
| 90 | Delay Critical GBR | 3 | 10 ms | 10-4 | 2000 bytes | 2000 ms | Cooperative collision avoidance; Sensor sharing – Higher degree of automation; Video sharing – higher degree of automation |
| 91 | (NOTE 1) | 2 | 3 ms | 10-5 | 2000 bytes | 2000 ms | Emergency trajectory alignment; Sensor sharing – Higher degree of automation |
| NOTE 1: GBR and Delay Critical GBR PQIs can only be used for unicast PC5 communications. |  |  |  |  |  |  |  |

- NOTE 1:	For Standardized PQI to QoS characteristics mapping, the table will be extended/updated to support service requirements for other identified V2X services.

- NOTE 2:	The PQIs may be used for other services than V2X.

- NOTE 3:	A PQI may be used together with an application indicated priority, which overrides the Default Priority Level of the PQI.

Table 5.6.1-1: Standardized PQI values that are additionally defined to QoS characteristics mapping [8]

| PQI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | GBR (NOTE 1) | 1 | 150 ms | 10-2 | N/A | 2000 ms | Mission Critical user plane Push To Talk voice (e.g. MCPTT) |
| 25 |  | 2 | 200 ms | 10-2 | N/A | 2000 ms | Non-Mission-Critical user plane Push To Talk voice |
| 26 |  | 2 | 200 ms | 10-3 | N/A | 2000 ms | Mission Critical Video user plane |
| 60 | Non-GBR | 1 | 120 ms | 10-6 | N/A | N/A | Mission Critical delay sensitive signalling (e.g. MC-PTT signalling) |
| 61 |  | 6 | 400 ms | 10-6 | N/A | N/A | Mission Critical Data (e.g. example services are the same as 5QI 6/8/9 as specified in TS 23.501 [4]) |
| 92 | Delay Critical GBR (NOTE 1) | 5 | 5ms | 10-4 | 20000 bytes | 2000 ms | Interactive service - consume VR content with high compression rate via tethered VR headset (See TS 22.261 [6]) |
| 93 |  | 6 | 10ms | 10-4 | 20000 bytes | 2000 ms | interactive service - consume VR content with low compression rate via tethered VR headset; Gaming or Interactive Data Exchanging (See TS 22.261 [6]) |
| NOTE 1: GBR and Delay Critical GBR PQIs can only be used for unicast PC5 communications. |  |  |  |  |  |  |  |