3GPP TSG-RAN WG2 Meeting #122	R2-2305695

Incheon, Korea, May 22-26, 2023

Agenda item:		7.4.2.2

Source:		Lenovo

Title:		Compliance check for LTM configuration

Document for:		Discussion and Decision

- Introduction

The following agreements related to Candidate and Reference Configuration are obtained [1].

RAN2#121:

In the RRC procedures, the candidate delta configuration is applied on top of the reference configuration to form a complete candidate configuration when the UE receives the LTM configuration (before the LTM cell switch). UE implementation can postpone that step to the reception of the LTM cell switch command. FFS Discuss early vs late compliance check.

In the RRC procedures, the complete candidate configuration is applied and replacing the current UE configuration (at the time of reconfiguration execution/cell switch), by a RRC reconfiguration procedure that makes replacements of configuration but doesn’t necessarily reset MAC, RLC or PDCP. FFS whether we can rely on a modified version of the reconfiguration procedure with fullconfig flag set. FFS how to make sure the procedures work in case the LTM candidate configuration is a complete configuration.

RAN2#121bis:

Discuss terminology for the TS in the RRC stage-3 discussions when/if needed (not at current meeting).

Whether the Reference configuration is a complete configuration or not is up to the network implementation.

Reference configuration + LTM candidate configuration (in combination) has to be a complete configuration.

The reference configuration is always explicitly signalled (not automatically derived from any other config, e.g. current).

Confirm that only the replacement procedure (the “full config without L2 reset”) is supported for Execution of LTM cell switch.

The UE may perform early decoding and early validity check. FFS whether Early validity check triggers early re-establishment. FFS the possible timing, FFS subset of cells, FFS if need to specify anything or just up to UE impl, FFS if other signalling to notify network is needed.

In this contribution, we discuss above highlighted issues for LTM configuration.

- Discussion

It was agreed in last metting that UE may perform early decoding and early validity check. However, what content should be checked is not clear. According to the RAN2 agreement, the reference configuration is managed separately and always explicitly signaled. Candidate cell configuration can be provided as delta configurations on top of a reference configuration. In the RRC procedures, the candidate delta configuration is applied on top of the reference configuration to form a complete candidate configuration. A UE stores the reference configuration as a separate configuration before complete candidate configuration is formed. The time point to form complete candidate configuration can be upon the reception of the LTM configuration or the reception of cell switch command.

In addition, it was agreed in RAN2#121bis that RRC RACH configuration for early TA acquisition is signaled separately from the candidate cell configuration. RACH resource for early TA acquisition should be applied when receiving PDCCH order (i.e. before receiving cell switching command). Therefore, compliance check for RACH configuration for early TA acquisition should be performed upon the reception of RACH configuration or the reception of PDCCH order to trigger early TA acquisition.

Proposal 1: Compliance check includes reference configuration, candidate delta configuration and RACH configuration for early TA acquisition.

Proposal 2: Compliance check for reference configuration and candidate delta configuration can be performed upon the reception of the configuration or the reception of cell switch command.

Proposal 3: Compliance check for RACH configuration for early TA acquisition should be performed upon the reception of RACH configuration for early TA acquisition or the reception of PDCCH order.

In traditional handover or condition-based handover, UE will perform recovery procedure when compliance check failure happens. Regarding LTM, UE also can consider reconfiguration failure and perform recovery procedure once the failure of compliance check happens. More specification, if candidate cell is PCell, UE will perform re-establishment procedure. If candidate cell is PSCell, UE will perform SCG failure information procedure. On the contrary, UE also can report compliance check failure to source serving cell instead of triggering recovery procedure. Then, the source cell may reconfigure/release the corresponding candidate cell.

Proposal 4: RAN2 to discuss which option should be supported upon compliance check failure for the case that candidate cell is PCell or PSCell.

- Option 1: UE performs recovery procedure upon compliance check failure. Re-establishment or SCGfailureinformation will be triggered for candidate PCell or candidate PSCell, respectively.

- Option 2: UE reports compliance check failure to source serving cell. Then, the source cell may reconfigure/release the corresponding candidate cell.

- Conclusion

In this contribution, we have the following proposals:

Proposal 1: Compliance check includes reference configuration, candidate delta configuration and RACH configuration for early TA acquisition.

Proposal 2: Compliance check for reference configuration and candidate delta configuration can be performed upon the reception of the configuration or the reception of cell switch command.

Proposal 3: Compliance check for RACH configuration for early TA acquisition should be performed upon the reception of RACH configuration for early TA acquisition or the reception of PDCCH order.

Proposal 4: RAN2 to discuss which option should be supported upon compliance check failure for the case that candidate cell is PCell or PSCell.

- Option 1: UE performs recovery procedure upon compliance check failure. Re-establishment or SCGfailureinformation will be triggered for candidate PCell or candidate PSCell, respectively.

- Option 2: UE reports compliance check failure to source serving cell. Then, the source cell may reconfigure/release the corresponding candidate cell.

References

RAN2 chairman notes for RAN2#121.

RAN2 chairman notes for RAN2#121bis.