3GPP TSG-RAN WG2 Meeting #122	R2-2305697

Incheon, Korea, May 22-26, 2023

Agenda item:		7.9.2

Source:		Lenovo

Title:		Discussion on L2 U2U relay

Document for:		Discussion and Decision

- Introduction

This document discusses aspects related to relay selection, relay reselection, fallback from relay operation to direct PC5 link and end-to-end PC5 connection in L2 U2U relay case.

- Discussion

2.1 Relay (Re)selection

In legacy sidelink, the discovery configuration can be provided to UE via dedicated signalling. In U2U case, we don’t see any additional configuration is needed comparing to the legacy. Therefore, we can follow the legacy even we agreed to reduce the impact on gNB in U2U case.

Proposal 1: In U2U relay, the remote/relay UE in RRC_CONNECTED can acquire discovery configuration via dedicated signaling.

UE-to-UE relay selection can be triggered based on upper layer or the PC5 RSRP of the direct link falling below a threshold. The remote UE needs to find the suitable relay UE which can reach the peer UE. According to TS23.304, U2U Relay will announce remote UE IDs in proximity. Since RAN2 has agreed to have a threshold for determining whether relay (re)selection should be performed. Therefore, the remote UE list included in Model A announcement message should have a good PC5 link between relay UE and remote UE.

Proposal 2: In Model A, the UE list in a discovery message announced by the relay UE contains UEs for which the quality of PC5 link between the relay UE and the UE is above a configured threshold.

Relay selection triggers include at least 1) Upper layer trigger; 2) PC5 signal strength conditions. As usual, the UE may detect the sidelink RLF of the PC5 direct link between two remote UEs. Once the RLF on the direct PC5 link is detected, the remote UE capable of U2U relay operation should perform relay selection. In addition, it is possible that the threshold of triggering relay selection is not configured to remote UE. If so, RLF on the direct PC5 link between two remote UEs can be used to trigger relay selection.

Proposal 3: RLF on the direct PC5 link between two remote UEs can be used to trigger relay selection.

Fig.1 U2U relay operation

This section aims to address the FFS if/how the second hop between the relay UE and the peer UE is considered. It was agreed by RAN2 that UE-to-UE relay reselection can be triggered based on the PC5 RSRP (FFS SL-RSRP or SD-RSRP) between a remote UE and the relay UE falling below a threshold. In this way, one threshold will be configured to the remote UE e.g UE2 in Fig.1. UE2 determines the PC5 link between the relay UE and UE2 is less than the threshold associated with relay reselection, the remote UE2 indicates to the relay UE. Then, the relay UE transmits the indication such as PC5 unicast link release to the remote UE1. The remote UE1 will perform relay reselection as well.

Proposal 4: Once the second (receiving) remote UE detects the PC5 link between the relay UE and the second(receiving) remote UE is less than the threshold associated with relay reselection, the second(receiving) remote UE indicates to the relay UE. Then, the relay UE transmits the indication to the first (transmitting) remote UE.

If the threshold associated with relay reselection can be (pre)configured to the relay UE, it can reduce the interruption time. Otherwise, the relay UE may continue receiving the data from the first remote UE when the second remote UE is aware of that the second hop is less than the threshold. In this way, the relay UE can transmit the indication of relay reselection to the first remote UE in the first hop once the relay UE detects the PC5 link between the relay UE and the second remote UE is less than the threshold associated with relay reselection.

Proposal 5: A threshold of triggering relay reselection can be configured to the relay UE.

Proposal 6: If the threshold for triggering relay reselection can be (pre)configured to the relay UE, the relay UE can transmit the indication of relay reselection to the first (transmitting) remote UE once the relay UE detects the PC5 link between the relay UE and the second (receiving) remote UE is less than the threshold.

When a remote UE communicates another remote UE via a U2U relay UE, the direct PC5 link may become better. In this case, the remote UE may fall back to the direct PC5 link between two remote UEs. Once the quality of the PC5 direct link is better than the configured threshold, the remote UE can fall back to the direct PC5 link. Namely, the remote UE establishes the PC5 link to the peer remote UE and release the link between the remote UE and the relay UE.

Proposal 7: Remote UE can switch back from the U2U relay operation to direct PC5 link if PC5 signal strength condition of direct PC5 link between two remote UEs is better than a threshold.

2.2 End-to-end PC5 connection

Fig1. RRC reconfiguration for end-to-end PC5 link, successful

It was agreed in RAN2 that E2E PC5-RRC connection is considered to be established after a corresponding E2E PC5 unicast link is established. In legacy direct PC5 link, SL-SRB3 is used to transmit PC5-RRC signalling e.g RRCReconfigurationSidelink message after the PC5-S security has been established. Similarly, we can have a RRC Reconfiguration message for end-to-end PC5 link for configuaration exchange between source remote UE and target remote UE. One way is to reuse legacy RRCReconfigurationSidelink message for end-to-end PC5 RRC connection.

Proposal 8: Legacy RRCReconfigurationSidelink message is used for end-to-end PC5 RRC connection.

In legacy direct PC5 RRC connection, UE consider sidelink RLF to be detected for this destination upon indication from sidelink RLC entity that the maximum number of retransmissions, T400 expiry, indication from MAC entity that the maximum number of consecutive HARQ DTX, or integrity check failure indication. Therefore, we need to discuss whether to discuss whether to specify maintenance for end-to-end PC5 RRC connection. As usual, UE can declare failure of end-to-end PC5 RRC connection upon sidelink RLF on the first or second hop, timer for end-to-end sidelink RRC reconfiguration for expiry.

Proposal 9: Maintenance for end-to-end PC5 RRC connection should be supported.

Proposal 10: The remote UE declares failure for end-to-end PC5 RRC connection upon detection of sidelink RLF on the first, reception of sidelink RLF on the second hop from relay UE, timer for end-to-end sidelink RRC reconfiguration for expiry.

It was concluded by SA2 that splitting end-to-end QoS into each hop can be handled by RAN. There are two options for the issue which node is responsible for splitting the e2e QoS. If relay UE is responsible, remote UE needs to provide the QoS information to relay UE since the relay UE is not aware of QoS information. If remote UE is responsible for QoS splitting, the channel quality of the second hop should be indicated to the remote UE. In legacy U2N relay UE, gNB will split the QoS into each hop. Furthermore, SA2 point out in LS that the source remote UEs signals the end-to-end QoS to the UE-to-UE Relay, and the Relay determines the individual hop’s QoS based on the end-to-end QoS. Therefore, RAN2 can confirm it.

Proposal 11: RAN2 confirms that splitting end-to-end QoS should be handled by RAN.

Proposal 12: Relay UE is responsible for QoS splitting in L2 U2U relay.

Proposal 13: Remote UE transmits the QoS information to relay UE for QoS splitting purpose.

- Conclusion

In this contribution, we have the following proposals:

Relay (Re)selection

Proposal 1: In U2U relay, the remote/relay UE in RRC_CONNECTED can acquire discovery configuration via dedicated signaling.

Proposal 2: In Model A, the UE list in a discovery message announced by the relay UE contains UEs for which the quality of PC5 link between the relay UE and the UE is above a configured threshold.

Proposal 3: RLF on the direct PC5 link between two remote UEs can be used to trigger relay selection.

Proposal 4: Once the second (receiving) remote UE detects the PC5 link between the relay UE and the second(receiving) remote UE is less than the threshold associated with relay reselection, the second(receiving) remote UE indicates to the relay UE. Then, the relay UE transmits the indication to the first (transmitting) remote UE.

Proposal 5: A threshold of triggering relay reselection can be configured to the relay UE.

Proposal 6: If the threshold for triggering relay reselection can be (pre)configured to the relay UE, the relay UE can transmit the indication of relay reselection to the first (transmitting) remote UE once the relay UE detects the PC5 link between the relay UE and the second (receiving) remote UE is less than the threshold.

Proposal 7: Remote UE can switch back from the U2U relay operation to direct PC5 link if PC5 signal strength condition of direct PC5 link between two remote UEs is better than a threshold.

End-to-end PC5 connection

Proposal 8: Legacy RRCReconfigurationSidelink message is used for end-to-end PC5 RRC connection.

Proposal 9: Maintenance for end-to-end PC5 RRC connection should be supported.

Proposal 10: The remote UE declares failure for end-to-end PC5 RRC connection upon detection of sidelink RLF on the first, reception of sidelink RLF on the second hop from relay UE, timer for end-to-end sidelink RRC reconfiguration for expiry.

Proposal 11: RAN2 confirms that splitting end-to-end QoS should be handled by RAN.

Proposal 12: Relay UE is responsible for QoS splitting in L2 U2U relay.

Proposal 13: Remote UE transmits the QoS information to relay UE for QoS splitting purpose.

References

Chairman notes for RAN2#121bis meeting.