3GPP TSG-RAN WG2 Meeting #122	R2-2305703

Incheon, KR, 22 – 26 May, 2023

Agenda item:	 7.13.2

Source:		Lenovo

Title:		MRO for inter-system handover for voice fallback

Document for:		Discussion and Decision

1	Introduction

RAN2#119-e meeting discussed MRO enhancement for inter-system handover for voice fallback and achieved some agreements [1]:

Agreements:

1	RAN2 to include an indication regarding voice fallback in the RLF report.

FFS: implicit or explicit flag and other details.

2	RAN2 discuss the following scenarios:

Suitable EUTRA cell found after MobilityFromNR failure

No suitable EUTRA cell found after MobilityFromNR failure

Some agreements were achieved in RAN2#119bis-e meeting [2]:

Agreements:

1	An explicit indication is included in RLF-report when mobility from NR fails and the corresponding MobilityFromNRCommand includes voiceFallbackIndication

2	The below content is included in RLF-report when reestablishment procedure is initiated due to mobility From NR failure.

a. reestablishmentCellID

In RAN2#121bis-e meeting [3], further agreements were achieved:

Agreements:

1	RAN2 to support the scenario of “after RLF occurs shortly after successful HO from NR to E-UTRAN for voice fallback, a suitable E-UTRA cell is selected, and the UE tries RRC connection setup procedure for the voice service in the E-UTRA cell, which is agreed in RAN3”.

2	FFS: Introduce an indication for the scenario of RLF after successful voice fallback HO in the LTE RLF report regarding voice fallback.

3	UE logs the agreed indication regarding voice fallback in the NR RLF report.

4	FFS: RAN2 agree to differentiate an acceptable E-UTRA cell from a suitable E-UTRA cell in the RLF report in case of voiceFallback HOF. FFS explicit or implicit indications.

In this paper, we would further discuss MRO for inter-system handover for voice fallback.

2	Discussion

In RAN3#117-e meeting, Case 1 and Case 2 are agreed for MRO consideration for inter-system inter-RAT handover for voice fallback, and the two cases were also confirmed in previous RAN2 meetings:

-	Case 1: after failure (HOF/RLF) of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, a suitable E-UTRAN cell is selected, and the UE tries RRC connection setup procedure for the voice service in the E-UTRAN cell.

-	Case 2: after failure (HOF) of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, none suitable E-UTRAN cell can be selected, the UE reverts back to the configuration of the source PCell and initiates RRC re-establishment procedure in NR.

Based on agreements, Case 1 can be split into two sub-cases:

-	Case 1a: a suitable E-UTRAN cell2 is selected after HOF of inter-system inter-RAT handover from source NR cell1 to target E-UTRAN cell1;

-	Case 1b: a suitable E-TURAN cell2 is selected, after a RLF occurs shortly after a successful handover from source NR cell1 to target E-UTRAN cell1. (it was also agreed to be considered in RAN2#121bis-e meeting that RAN2 to support the scenario of “after RLF occurs shortly after successful HO from NR to E-UTRAN for voice fallback, a suitable E-UTRA cell is selected, and the UE tries RRC connection setup procedure for the voice service in the E-UTRA cell, which is agreed in RAN3”.)

For Case 1a, there are two potential options for RLF reporting:

- Option i: UE reports a NR RLF report (which is enhanced to include an indication concerning that the last failed inter-system inter-RAT HO was triggered due to voice fallback as RAN2#121bis-e meeting agreed that UE logs the agreed indication regarding voice fallback in the NR RLF report) and source NR cell1 ID out of the NR RLF report to E-UTRAN cell2, E-UTRAN cell2 would not decode the NR RLF report, but based on the source NR cell1 ID, E-UTRAN cell2 would forward the NR RLF report to source NR cell1.

- Option ii: if UE is back to a NR cell3, UE reports a NR RLF report (which is enhanced to include an indication concerning that the last failed inter-system inter-RAT HO was triggered due to voice fallback as RAN2#121bis-e meeting agreed that UE logs the agreed indication regarding voice fallback in the NR RLF report) to NR cell3, NR cell3 forwards the NR RLF report to source NR cell1.

Compared with Option i, Option ii is preferred since it has less spec impact.

Proposal 1: For the case that a suitable E-UTRAN cell is selected after HOF of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, the UE may send a NR RLF report including an indication concerning that the last failed inter-system inter-RAT HO was triggered due to voice fallback to a NG-RAN node in which the UE is back to.

It is FFS in RAN2#121bis-e meeting that whether to introduce an indication for Case 1b (the scenario of RLF after successful voice fallback HO) in the LTE RLF report regarding voice fallback. The indication is benefical for the network to distinguish legacy inter-system handover from inter-system handover due to voice fallback. The UE may report LTE RLF report which includes an indication concerning that the last failed inter-system inter-RAT HO was triggered due to voice fallback to the re-connected E-UTRAN node (e.g. E-UTRAN cell2), the re-connected E-UTRAN node may forward the LTE RLF report to target E-UTRAN cell1, then target E-UTRAN cell1 decodes the LTE RLF report and explicitly transfers failure related information which is decoded from the LTE RLF report to source NR cell1, in such a way, corresponding optimisation can be performed in network side.

Proposal 2: For the case that a suitable E-UTRAN cell is selected after a RLF occurs shortly after a successful inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, the UE may send the LTE RLF report which is enhanced to include an indication concerning that the last failed inter-system inter-RAT HO was triggered due to voice fallback to the re-connected E-UTRAN node.

3	Conclusion

In this contribution, we discussed MRO for inter-system inter-RAT handover for voice fallback, the following proposals are given:

Proposal 1: For the case that a suitable E-UTRAN cell is selected after HOF of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, the UE may send a NR RLF report including an indication concerning that the last failed inter-system inter-RAT HO was triggered due to voice fallback to a NG-RAN node in which the UE is back to.

Proposal 2: For the case that a suitable E-UTRAN cell is selected after a RLF occurs shortly after a successful inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, the UE may send the LTE RLF report which is enhanced to include an indication concerning that the last failed inter-system inter-RAT HO was triggered due to voice fallback to the re-connected E-UTRAN node.

Reference

[1] Chairman notes for RAN2#119-e meeting

[2] Chairman notes for RAN2#119bis-e meeting

[3] Chairman notes for RAN2#121bis-e meeting