3GPP TSG-RAN WG2 Meeting #122	R2-2305711

Incheon, Korea, May 22-26, 2023

Agenda item:		7.6.2.2

Source:		Lenovo

Title:		Further considerations on GNSS operations in RRC_CONNECTED for IoT NTN

Document for:		Discussion

- Introduction

The latest RAN2#121bis meeting made the following agreements regarding the GNSS operation in IoT NTN:

- There is no need for UE to provide GNSS position fix time duration in Msg3. - RLM is suspended during the GNSS measurement gap while the UE is measuring GNSS. - UE can stay in RRC_CONNECTED state when current GNSS position becomes out-of-date if the UE enters a GNSS measurement gap. FFS whether the new GNSS measurement shall be started before, upon or after the current GNSS validity duration expiry. - For a UEs that cannot acquire system information and GNSS position at the same time, acquisition of SIB31 may be postponed until GNSS measurement is completed if the UE cannot complete acquisition of SIB31 before the start of GNSS measurement gap. - For the NB-IoT CP solution, UE will report the GNSS validity duration by using a MAC CE - RAN2 can discuss UE autonomously reacquire GNSS during inactive state of C-DRX based on RAN1’s input in the next RAN2 meeting. Working Assumption: GNSS validity duration UE reported after GNSS measurement is the remaining validity duration

And the agreements from RAN1#112bis meeting:

- For the GNSS measurement gap aperiodically triggered with MAC CE, the duration for the GNSS measurement gap can be configured by eNB. - The gap duration is equal to the latest reported GNSS position fix time duration for measurement when the duration for GNSS measurement gap is not included in the configuration by eNB. - On when the aperiodic GNSS measurement gap starts, which is aperiodically triggered by eNB with MAC CE, the start time should be at n+X, where n is the end of MAC CE receiving subframe/slot - FFS: details of X, e.g. predefined value or configured value, considering HARQ feedback for the MAC CE, etc - UE reports one GNSS position fix time duration for GNSS measurement via a N-bit field at least including [1,2] seconds as component values. - FFS: value of N, other component value(s) of GNSS position fix time duration (e.g. N=3, with value in [3,7,13,19,25, X] seconds, and X is FFS). - FFS: whether RAN4 input is needed.

In this contribution we further discuss the open issues of supporting GNSS operation in RRC_CONNECTED based on the above agreements.

- Discussion

## GNSS measurement gap and measurement start

For UE’s RRC state regarding GNSS measurement, it has been agreed thatUE can stay in RRC_CONNECTED state when current GNSS position becomes out-of-date if the UE enters a GNSS measurement gap. Based on RAN1’s agreement, the duration for the GNSS measurement gap can be configured by eNB or equal to the latest reported GNSS position fix time duration if not configured by eNB. As a result, the 2 following cases are possible:

- Case A: The end of the duration for the GNSS measurement gap is late than the end of the latest reported GNSS position fix time duration.

- Case B: The end of the duration for the GNSS measurement gap is NO late than the end of the latest reported GNSS position fix time duration.

From our understanding Case A is only possible when the duration for the GNSS measurement gap is configured by eNB, while Case B is possible when the duration for the GNSS measurement gap is configured by eNB and is inevitable when the duration is not configured by eNB.

Observation 1: Based on current agreements, UE cannot stay in RRC_CONNECTED state when current GNSS position becomes out-of-date if the duration for the GNSS measurement gap is NOT configured by eNB.

For Case A, the answer to the corresponding FFS (whether the new GNSS measurement shall be started before, upon or after the current GNSS validity duration expiry) can be no timing restriction, i.e., the new GNSS measurement can be started regardless of the current GNSS validity duration expiry, as long as that UE can complete within the gap.

Observation 2: If the end of the duration for the GNSS measurement gap is late than the end of the latest reported GNSS position fix time duration, the new GNSS measurement can be started before, upon or after the current GNSS validity duration expiry.

For Case B, one possible understanding is that eNB triggers GNSS measurement before the end of the latest reported GNSS position fix time duration and assumes that UE can complete before it as well, which is not always guaranteed considering the length of GNSS measurement. Another understanding is that eNB does not consider it necessary to let UE stay in RRC_CONNECTED state when current GNSS position becomes out-of-date. For either understanding, the answer to the corresponding FFS is the new GNSS measurement shall be started BEFORE the current GNSS validity duration expiry.

Observation 3: If the end of the duration for the GNSS measurement gap is NO late than the end of the latest reported GNSS position fix time duration, the new GNSS measurement shall be started BEFORE the current GNSS validity duration expiry.

Based on the above analysis we propose for the FFS regarding GNSS measurement start:

Proposal 1: The start timing of the new GNSS measurement in a GNSS measurement gap depends on the duration of the GNSS measurement gap:

- if the end of the GNSS measurement gap duration is late than the current GNSS validity duration expiry, the new GNSS measurement can be started before, upon or after the current GNSS validity duration expiry;

- else the new GNSS measurement shall be started BEFORE the current GNSS validity duration expiry.

## Postponing acquisition of SIB31

In our previous contribution R2-2303251, two possible options were discussed for the issue of possible ephemeris expiry during GNSS measurement:

- Option 1: Before a GNSS measurement start, UE may re-acquire SIB31 to ensure a valid ephemeris by implementation. This does not mean to postpone the GNSS measurement procedure to acquire SIB31.

- Option 2: UE may postpone acquisition of SIB31 until GNSS measurement is completed.

After email discussion, agreement based on Option 2 was made following majority’s understanding. In our opinion this is feasible, and the wording “may” does not forbid UE to re-acquire SIB31 before a GNSS measurement start by implementation as in Option 1.

Observation 4: The agreement “For a UEs that cannot acquire system information and GNSS position at the same time, acquisition of SIB31 may be postponed until GNSS measurement is completed if the UE cannot complete acquisition of SIB31 before the start of GNSS measurement gap” does not forbid UE to re-acquire SIB31 by implementation before the start of GNSS measurement gap.

Therefore, we think it could be better to clarify this to avoid possible confusion:

Proposal 2: UE can re-acquire SIB31 by implementation if UE can complete acquisition of SIB31 before the start of GNSS measurement gap.

## Reacquire GNSS during inactive state of C-DRX

In the previous meeting this issue is postponed waiting for RAN1’s input, and RAN1 has concluded on this as:

Conclusion From RAN1 perspective, UE is not forbidden to autonomously re-acquire GNSS position fix during inactive state of Connected DRX. - Note: The configured DL/UL transmissions during inactive state of Connected DRX should not be impacted - Note: details are up to RAN2 Send an LS to RAN2 for the conclusion.

Therefore RAN2 can confirm this as well.

Proposal 3: UE can autonomously reacquire GNSS during inactive state of C-DRX.

## Report of GNSS validity duration

In the previous meetings there was a discussion on the content of UE reported GNSS validity duration after GNSS measurement. A working assumption was made indicating that it is the remaining validity duration. After checking on RAN1’s agreement as below, we found that it may need further discussion.

- UE reports one GNSS position fix time duration for GNSS measurement via a N-bit field at least including [1,2] seconds as component values. - FFS: value of N, other component value(s) of GNSS position fix time duration (e.g. N=3, with value in [3,7,13,19,25, X] seconds, and X is FFS). - FFS: whether RAN4 input is needed.

The following cases need to be considered:

- Case 1: The remaining validity duration when UE reports is not an integer in second, or not equal to any value defined by RAN1, e.g., 5.5s or 5s.

- Case 2: The remaining validity duration when UE reports is smaller/larger than the minimum/maximum value defined by RAN1, e.g., 0.5s or X+5s.

Observation 5: The remaining validity duration when UE reports may not match the values of GNSS position fix time duration defined by RAN1.

Proposal 4: RAN2 to discuss how to report remaining validity duration when it does not match the values of GNSS position fix time duration defined by RAN1 based on the current working assumption, or reconsider the working assumption with further input or progress from RAN1.

## CONNECTED neighbour cell measurement

To avoid unnecessary power consumption and RLF due to GNSS measurement, it was agreed that RLM is suspended during the GNSS measurement gap while the UE is measuring GNSS. However similar issues may occur in other aspects including neighbour cell measurement triggering.

In Rel-17, the criteria of triggering neighbour cell measurement in RRC_CONNECTED for NB-IoT is a combination of serving cell RSRP and its variance. That is, if the measured serving cell RSRP is lower than a threshold and its variance compared to a reference RSRP is higher than a threshold, the UE performs neighbour cell measurements.

During the time duration of GNSS measurement, NB-IoT UE will not receive any signal from network including the reference signal for RSRP measurement, and as a result neighbour cell measurement could be triggered with corresponding RF switch. The similar issue could also occur for eMTC UEs configured with measurement objects including events regarding serving cell RSRP are configured.

Observation 6: During the time duration of GNSS measurement, unnecessary neighbour cell measurement in RRC_CONNECTED could be triggered after the beginning of the gap.

To avoid unnecessary neighbour cell measurement or report during the time duration of GNSS measurement, it may be also necessary to disable the measurements to the serving cell or the triggering of neighbour cell measurement (for NB-IoT) or measurement report (for eMTC) at the beginning the GNSS measurement.

Proposal 5: RAN2 to consider solutions to avoid unnecessary neighbour cell measurement during the time duration of GNSS measurement.

- Conclusion

In this contribution we further discuss the open issues of supporting GNSS operation in RRC_CONNECTED based on the latest RAN2 and RAN1 agreements. The following observation are given:

Observation 1: Based on current agreements, UE cannot stay in RRC_CONNECTED state when current GNSS position becomes out-of-date if the duration for the GNSS measurement gap is NOT configured by eNB.

Observation 2: If the end of the duration for the GNSS measurement gap is late than the end of the latest reported GNSS position fix time duration, the new GNSS measurement can be started before, upon or after the current GNSS validity duration expiry.

Observation 3: If the end of the duration for the GNSS measurement gap is NO late than the end of the latest reported GNSS position fix time duration, the new GNSS measurement shall be started BEFORE the current GNSS validity duration expiry.

Observation 4: The agreement “For a UEs that cannot acquire system information and GNSS position at the same time, acquisition of SIB31 may be postponed until GNSS measurement is completed if the UE cannot complete acquisition of SIB31 before the start of GNSS measurement gap” does not forbid UE to re-acquire SIB31 by implementation before the start of GNSS measurement gap.

Observation 5: The remaining validity duration when UE reports may not match the values of GNSS position fix time duration defined by RAN1.

Observation 6: During the time duration of GNSS measurement, unnecessary neighbour cell measurement in RRC_CONNECTED could be triggered after the beginning of the gap.

And we propose:

Proposal 1: The start timing of the new GNSS measurement in a GNSS measurement gap depends on the duration of the GNSS measurement gap:

- if the end of the GNSS measurement gap duration is late than the current GNSS validity duration expiry, the new GNSS measurement can be started before, upon or after the current GNSS validity duration expiry;

- else the new GNSS measurement shall be started BEFORE the current GNSS validity duration expiry.

Proposal 2: UE can re-acquire SIB31 by implementation if UE can complete acquisition of SIB31 before the start of GNSS measurement gap.

Proposal 3: UE can autonomously reacquire GNSS during inactive state of C-DRX.

Proposal 4: RAN2 to discuss how to report remaining validity duration when it does not match the values of GNSS position fix time duration defined by RAN1 based on the current working assumption, or reconsider the working assumption with further input or progress from RAN1.

Proposal 5: RAN2 to consider solutions to avoid unnecessary neighbour cell measurement during the time duration of GNSS measurement.