3GPP TSG-RAN WG2 Meeting #121-bis electronic	R2-2305716

April 17-26, 2023                                           Revision of R2-2303254

Agenda item:		7.7.4.1.2

Source:		Lenovo

Title:		Neighbour cell measurement triggering in NTN moving cells

Document for:		Discussion

- Introduction

The latest RAN2 meetings made the following agreements regarding IDLE/INACTIVE mobility for moving cells in NR NTN:

RAN2#121 agreements - In R18, for earth-moving system, satellite with steerable beam is not considered as part of mobility enhancement in NTN. - A serving cell reference location and a distance threshold/radius will be broadcast for earth-moving cell. FFS on whether the R17 IEs are reused or not. FFS on whether additional information needs to be broadcast to inform the UE how the reference location moves over time or if this can be derived from other information (e.g. Epoch time and ephemeris). - For cell selection/reselection, location-based measurement initiation is supported in earth-moving cell - For earth-moving cell, the location-based cell measurement rules of quasi-fixed cell is reused, i.e., for cell reselection in earth-moving cell, UE initiates measurements when its location to serving cell reference location is larger than the configured distance threshold. RAN2#121bis agreements - RAN2 understands that for earth-moving cell reselection, the UE can derive the trajectory of serving cell with rough accuracy based on serving satellite ephemeris and epochTime, with the assumption that the serving cell reference location broadcast by the network is the one at Epoch time (FFS whether a new epochTime IE is needed). RAN2 understanding is that both PVT and orbital parameters can be used for this. FFS if additional information is needed to allow more accurate measurements. - For earth-moving cell, new IE is introduced to indicate the reference location of serving cell. - For cell (re)selection in earth-moving system, a distance threshold is introduced for location-based measurement initiation, which reuses distanceThresh in SIB19. - For cell (re)selection in earth-moving system, time-based measurement initiation is used to address feeder-link switch case. - Time-based cell reselection criteria is not pursued in R18.

In this contribution we further discuss solutions for neighbour cell measurement triggering in moving cells based on above agreements.

- Discussion

In Rel-17 NR NTN, reference location referenceLocation of serving cell is broadcast in SIB19, which is fixed for a quasi-earth-fixed cell during before its stop serving time. For an earth-moving cell with varying referenceLocation over time, frequent UE re-acquiring (e.g., triggered by SI change) will cause more power consumption. It could likely be similar to the ephemeris handling in Rel-17 NTN wherein no SI change is triggered by ephemeris change. Therefore an issue arises as how to indicate the moving referenceLocation of an earth-moving cell to the UE.

In the discussion one agreement is that the trajectory of serving cell reference location can be derived by UE with rough accuracy solely based on satellite’s ephemeris and epoch time. In our understanding, such accuracy is uncertain and meanwhile more calculation needs to be done at UE, see in Figure 1. Firstly, the trajectory of satellite represented by ephemeris is an ellipse which is not parallel to the earth, meaning that the velocity of satellite does not equal to the velocity of its projection (e.g., cell reference location) on earth. Secondly, even the projection is derived, it only represents the nadir point of the satellite which could be the reference location of one cell, while the reference locations of the other cells generated by the same satellite could not be represented or derived, e.g., the elevation angle between the reference location of a cell and the satellite, which is not part of ephemeris, also affects the velocity of the cell reference location.

Figure 1. The trajectory of satellite and cell reference location

Observation 1: Deriving the reference location of an Earth-moving cell only by the ephemeris data at epoch time has uncertainty in accuracy and introduces more power consumption for UE’s calculation.

Meanwhile, it has also been agreed that a new IE is introduced to indicate the reference location of serving cell, which allows more space in content design to improve the accuracy and ease UE’s power consumption for calculation. Therefore, the following options can be considered:

- Option 1A: Multiple reference locations. Each reference location could be associated to a time duration, an area or elevation angle. And UE could apply one reference location based on the current time, its area or elevation angle.

- Option 1B: Multiple offsets to referenceLocation. Each offset could be associated to a time duration, an area or elevation angle. And UE could apply one offset based on the current time, its area or elevation angle.

- Option 1C: Additional change information, e.g., drift or velocity vector of referenceLocation. UE estimates real-time reference location based on the change information.

Proposal 1: For earth-moving cells, RAN2 to consider the following options to indicate the changing referenceLocation efficiently.

- Option 1A: Multiple reference locations.

- Option 1B: Multiple offsets to referenceLocation.

- Option 1C: Additional change information, e.g., drift or velocity vector of referenceLocation.

For an earth-moving cell with different serving duration for different locations, its stop serving time is location-specific and thus UE-specific (i.e. when the serving cell stops providing coverage at the present UE location). As a result broadcasting a single cell-specific t-Service is inaccurate and is difficult to use. Therefore an issue arises as how to indicate the UE/location-specific cell stop serving time of an earth-moving cell to the UE. Similar to the derivation of reference location, the same issue also exists for deriving the UE-specific cell stop serving time.

Observation 2: Deriving the cell stop serving time of an Earth-moving cell at the present UE location only by the ephemeris data at epoch time has uncertainty in accuracy and introduces more power consumption for UE’s calculation.

Therefore, the following options can be considered for UE to derive when the serving cell stops providing coverage at the present UE location:

- Option 2A: Multiple stop serving times similar to t-Service. Each stop serving time could be associated to an area or elevation angle. And UE could apply one stop serving time based on its area or elevation angle.

- Option 2B: Multiple offsets to t-Service. Each offset could be associated to an area or elevation angle. And UE could apply one offset based on its area or elevation angle.

- Option 2C: UE estimates stop serving time based on cell coverage information (e.g., ellipse radius, minimum angle). Network could indicate UE that the stop serving time is location/UE-specific and thus needs to be estimated.

Proposal 2: For earth-moving cells, RAN2 to consider the following options to indicate UE to estimate when the serving cell stops providing coverage at the present UE location.

- Option 2A: Multiple stop serving times similar to t-Service.

- Option 2B: Multiple offsets to a reference t-Service.

- Option 2C: Cell coverage information (e.g., ellipse radius, minimum angle).

- Conclusion

In this contribution, we provide further issue analysis of IDLE/INACTIVE mobility in NTN considering moving cells and TN area. The following observation are given:

Observation 1: Deriving the reference location of an Earth-moving cell only by the ephemeris data at epoch time has uncertainty in accuracy and introduces more power consumption for UE’s calculation.

Observation 2: Deriving the cell stop serving time of an Earth-moving cell at the present UE location only by the ephemeris data at epoch time has uncertainty in accuracy and introduces more power consumption for UE’s calculation.

And we propose:

Proposal 1: For earth-moving cells, RAN2 to consider the following options to indicate the changing referenceLocation efficiently.

- Option 1A: Multiple reference locations.

- Option 1B: Multiple offsets to referenceLocation.

- Option 1C: Additional change information, e.g., drift or velocity vector of referenceLocation.

Proposal 2: For earth-moving cells, RAN2 to consider the following options to indicate UE to estimate when the serving cell stops providing coverage at the present UE location.

- Option 2A: Multiple stop serving times similar to t-Service.

- Option 2B: Multiple offsets to a reference t-Service.

- Option 2C: Cell coverage information (e.g., ellipse radius, minimum angle).