3GPP TSG-RAN WG2 Meeting #122                             	R2-2305737

Incheon, Korea, May 22–26, 2023

Agenda item:	6.2.2

Source:	Samsung

Title:	DRX Timers for PTP Retransmission

Document for:	Discussion & Decision

Introduction

In RAN2#121bis-e meeting, RAN2 discussed how to modify the starting condition of the unicast HARQ RTT timer (i.e. drx-HARQ-RTT-TimerDL) but did not reach the conclusion as follows:

P6 from R2-2303967 is postponed (companies should check the current status in RAN1 and in RAN1 specifications)

This document discusses this leftover issue on DRX.

Discussion

The relevant RAN1 issue is multicast HARQ feedback conversion from NACK-only feedback to ACK-NACK feedback. If the gNB receives a converted NACK, gNB may need to perform a PTP retransmission by using C-RNTI or CS-RNTI. The related RAN2 issue is whether the unicast DRX Active Time should be extended in case that NACK-only feedback is converted to ACK-NACK feedback.

In RAN2#121bis-e meeting, no conclusion was made because RAN1 did not have a conclusion for their issue, i.e. whether to support PTP retransmission for the converted feedback. However, no relevant correction was agreed in RAN1#112 and RAN1#112bis-e meetings, as follows:

< RAN1#112 > [1] Issue 1-5 PTP retransmission R1-2300076 Draft CR on retransmission schemes for MBS HARQ-ACK feedback to TS38.213 Huawei, HiSilicon, CBN R1-2301672 Draft CR to 38.213 for PTP retransmissions of PTM Ericsson R1-2301923 Draft CR on retransmission schemes for MBS Moderator (Huawei) Conclusion No correction is needed for issue 1-5 in Rel-17. < RAN1#112bis-e > [2] Issue (1-17) PTP retransmission R1-2303546 Draft CR to 38.213 for PTM retransmissions of SPS multicast Ericsson Conclusion Not an essential correction. This issue is not pursued in Rel-17.

The reason of no correction in RAN1 is that PTP retransmission for NACK-only feedback conversion is already possible at gNB, so no additional correction is needed. Even without HARQ feedback, gNB is able to perform a retransmission. Hence, it is not a new RAN1 issue.

Observation 1. PTP retransmission for NACK-only feedback conversion is already possible at gNB and not a new issue.

Then RAN2 needs to focus on the necessity of modification of C-DRX to support PTP retransmission for NACK-only feedback. Currently, unicast DRX Active Time is not extended for multicast reception with NACK-only feedback regardless of feedback conversion.

However, the current text does not have a critical problem. In NACK-only feedback, multicast DRX Active Time can be extended by starting multicast timers (i.e. drx-HARQ-RTT-TimerDL-PTM and drx-InactivityTimerPTM). PTM retransmission is still possible. Also, PTP retransmission can be performed during unicast DRX Active Time by drx-onDurationTimer or other HARQ processes. Although it is not fully optimized, it does not give a big problem.

Observation 2. Although the current MAC procedure is not fully optimized, it does not block retransmission for NACK-only feedback and feedback conversion.

In RAN2#121bis-e meeting, many companies were in favour of change and text proposal was discussed. The case which requires the change is that HARQ feedback is reported as an ACK-NACK feedback. Thus, if RAN2 agrees to optimize the case, we see “if the first HARQ-ACK reporting mode (i.e. ack-nack) is configured as specified in TS 38.213 [6]” can be changed to “if the first HARQ-ACK reporting mode (i.e. ack-nack) is used as specified in TS 38.213 [6]” as discussed in [3].

Proposal. RAN2 to select one of the following options:

- Option 1. No specification change

- Option 2. In 5.7b of TS 38.321, “if the first HARQ-ACK reporting mode (i.e. ack-nack) is configured as specified in TS 38.213 [6]” is modified to “if the first HARQ-ACK reporting mode (i.e. ack-nack) is used as specified in TS 38.213 [6]”

Conclusion

RAN2 is requested to discuss and agree to the following observations and proposal:

Observation 1. PTP retransmission for NACK-only feedback conversion is already possible at gNB and not a new issue.

Observation 2. Although the current MAC procedure is not fully optimized, it does not block retransmission for NACK-only feedback and feedback conversion.

Proposal. RAN2 to select one of the following options:

- Option 1. No specification change

- Option 2. In 5.7b of TS 38.321, “if the first HARQ-ACK reporting mode (i.e. ack-nack) is configured as specified in TS 38.213 [6]” is modified to “if the first HARQ-ACK reporting mode (i.e. ack-nack) is used as specified in TS 38.213 [6]”

Reference

[1] Chair’s notes RAN1#112

[2] Chair’s note RAN1#112bis-e

[3] R2-2304325, [AT121bis-e][602][MBS-R17] Stage-2 and UP issues (Nokia), Nokia (Rapporteur)