3GPP TSG-RAN WG2 Meeting #122	R2-2305750

# Incheon, South Korea, 22– 26 May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | Num | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of MT-SDT in Stage-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NR_MT_SDT-Core |  |  |  |  |  | Date: |  |  | 2023-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of MT-SDT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implemented RAN2#120 agreements: - For RAN paging, MT-SDT indication (at least one bit) is explicitly included per UE via a paging message. FFS if more information for MT-SDT are needed FFS what the indication will be called. FFS signalling details - Rel-18 MT-SDT after the MT-SDT paging trigger is detected, RA-SDT and CG SDT solutions/procedures specified in Rel-17 is re-used as a baseline. The detailed triggers will be discussed on case by case. FFS on resources used for access - UE can use non-SDT random access resources for accessing the network for an MT-SDT transfer. The UE can also use the configured grant resources and/or MO-RA resources. - The network should be able to differentiate why the UL access was triggered, i.e. implicit or explicit indication by the UE. - MT-SDT is data that belongs to bearers that are configured for SDT. FFS whether the configuration is MO-SDT or MT-SDT specific. The network can only trigger MT-SDT if the data belongs to those bearers. - New Resume cause in RRC resume will be introduced, one code point MT-SDT indication Implemented RAN2#121 agreements: - Include a one-bit indication in paging to trigger MT-SDT. We will ensure that the CCCH message can be transmitted over CG. - Indication is per UE. FFS on signalling. - In case condition for paging triggered MT-SDT is not fulfilled the UE initiates RRC Resume procedure. Resume cause FFS - Upon receiving MT-SDT trigger, the UE shall initiate SDT procedure if the following checks are satisfied (all these same as Rel-17) - FFS a: Check for DVT (if UL data becomes available in UL) - b: Check for SDT RSRP threshold - c: Check for TA validation before selecting CG (if applicable) - d: Check for SSB level RSRP threshold for CG resource (if applicable) - RBs configured for SDT are common for MO-SDT and MT-SDT - To confirm that when SDT is initiated due to MT-SDT, UE can exchange subsequent DL/UL SDT data on the resumed RBs. This clarifies the RB behaviour of related RAN2#120 agreement. - Specify a RRC procedure for RRCResume for MT-SDT initiation without checking for availability of UL data (i.e. if MT-SDT is initiated first the resume cause will be set to MT-SDT) Impact analysis Impacted functionality: Small data transmission. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MT-SDT feature cannot be exploited. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 18.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AGC	Automatic Gain Control

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCCH	Broadcast Control Channel

BCH	Broadcast Channel

BFD	Beam Failure Detection

BH	Backhaul

BL	Bandwidth reduced Low complexity

BPSK	Binary Phase Shift Keying

C-RNTI	Cell RNTI

CAG	Closed Access Group

CAPC	Channel Access Priority Class

CBRA	Contention Based Random Access

CCE	Control Channel Element

CD-SSB	Cell Defining SSB

CFR	Common Frequency Resource

CFRA	Contention Free Random Access

CG	Configured Grant

CHO	Conditional Handover

CIoT	Cellular Internet of Things

CLI	Cross Link interference

CMAS	Commercial Mobile Alert Service

CORESET	Control Resource Set

CP	Cyclic Prefix

CPA	Conditional PSCell Addition

CPC	Conditional PSCell Change

DAG	Directed Acyclic Graph

DAPS	Dual Active Protocol Stack

DFT	Discrete Fourier Transform

DCI	Downlink Control Information

DCP	DCI with CRC scrambled by PS-RNTI

DL-AoD	Downlink Angle-of-Departure

DL-SCH	Downlink Shared Channel

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

E-CID	Enhanced Cell-ID (positioning method)

EHC	Ethernet Header Compression

ePWS	enhancements of Public Warning System

ETWS	Earthquake and Tsunami Warning System

FS	Feature Set

FSA ID	Frequency Selection Area Identity

G-CS-RNTI	Group Configured Scheduling RNTI

G-RNTI	Group RNTI

GFBR	Guaranteed Flow Bit Rate

GIN	Group ID for Network selection

GNSS	Global Navigation Satellite System

GSO	Geosynchronous Orbit

H-SFN	Hyper System Frame Number

HAPS	High Altitude Platform Station

HRNN	Human-Readable Network Name

IAB	Integrated Access and Backhaul

IFRI	Intra Frequency Reselection Indication

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

KPAS	Korean Public Alarm System

L2	Layer-2

L3	Layer-3

LDPC	Low Density Parity Check

LEO	Low Earth Orbit

MBS	Multicast/Broadcast Services

MCE	Measurement Collection Entity

MCCH	MBS Control Channel

MDBV	Maximum Data Burst Volume

MEO	Medium Earth Orbit

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MO-SDT	Mobile Originated SDT

MPE	Maximum Permissible Exposure

MRB	MBS Radio Bearer

MT	Mobile Termination

MT-SDT	Mobile Terminated SDT

MTCH	MBS Traffic Channel

MTSI	Multimedia Telephony Service for IMS

MU-MIMO	Multi User MIMO

Multi-RTT	Multi-Round Trip Time

MUSIM	Multi-Universal Subscriber Identity Module

NB-IoT	Narrow Band Internet of Things

NCD-SSB	Non Cell Defining SSB

NCGI	NR Cell Global Identifier

NCL	Neighbour Cell List

NCR	Neighbour Cell Relation

NCRT	Neighbour Cell Relation Table

NGAP	NG Application Protocol

NGSO	Non-Geosynchronous Orbit

NID	Network Identifier

NPN	Non-Public Network

NR	NR Radio Access

NSAG	Network Slice AS Group

NTN	Non-Terrestrial Network

P-MPR	Power Management Maximum Power Reduction

P-RNTI	Paging RNTI

PCH	Paging Channel

PCI	Physical Cell Identifier

PDC	Propagation Delay Compensation

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PEI	Paging Early Indication

PH	Paging Hyperframe

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated NPN

PO	Paging Occasion

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PRG	Precoding Resource block Group

PRS	Positioning Reference Signal

PS-RNTI	Power Saving RNTI

PSS	Primary Synchronisation Signal

PTM	Point to Multipoint

PTP	Point to Point

PTW	Paging Time Window

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QFI	QoS Flow ID

QMC	QoE Measurement Collection

QoE	Quality of Experience

QPSK	Quadrature Phase Shift Keying

RA	Random Access

RA-RNTI	Random Access RNTI

RACH	Random Access Channel

RANAC	RAN-based Notification Area Code

REG	Resource Element Group

RIM	Remote Interference Management

RLM	Radio Link Monitoring

RMSI	Remaining Minimum SI

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNTI	Radio Network Temporary Identifier

RQA	Reflective QoS Attribute

RQoS	Reflective Quality of Service

RS	Reference Signal

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

RTT	Round Trip Time

SCS	SubCarrier Spacing

SD	Slice Differentiator

SDAP	Service Data Adaptation Protocol

SDT	Small Data Transmission

SFI-RNTI	Slot Format Indication RNTI

SHR	Successful Handover Report

SIB	System Information Block

SI-RNTI	System Information RNTI

SLA	Service Level Agreement

SMC	Security Mode Command

SMF	Session Management Function

SMTC	SS/PBCH block Measurement Timing Configuration

S-NSSAI	Single Network Slice Selection Assistance Information

SNPN	Stand-alone Non-Public Network

SNPN ID	Stand-alone Non-Public Network Identity

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRAP	Sidelink Relay Adaptation Protocol

SRS	Sounding Reference Signal

SRVCC	Single Radio Voice Call Continuity

SS	Synchronization Signal

SSB	SS/PBCH block

SSS	Secondary Synchronisation Signal

SSSG	Search Space Set Group

SST	Slice/Service Type

SU-MIMO	Single User MIMO

SUL	Supplementary Uplink

TA	Timing Advance

TB	Transport Block

TCE	Trace Collection Entity

TNL	Transport Network Layer

TPC	Transmit Power Control

TRP	Transmit/Receive Point

TRS	Tracking Reference Signal

U2N	UE-to-Network

UCI	Uplink Control Information

UDC	Uplink Data Compression

UE-Slice-MBR	UE Slice Maximum Bit Rate

UL-AoA	Uplink Angles of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SCH	Uplink Shared Channel

UPF	User Plane Function

URLLC	Ultra-Reliable and Low Latency Communications

VR	Virtual Reality

V2X	Vehicle-to-Everything

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

XnAP	Xn Application Protocol

Next Modified Subclause

# 18	Small Data Transmission

## 18.0	General

Small Data Transmission (SDT) is a procedure allowing data and/or signalling transmission while remaining in RRC_INACTIVE state (i.e. without transitioning to RRC_CONNECTED state). SDT is enabled on a radio bearer basis and can be initiated either by the UE for MO-SDT (Mobile Originated SDT) or by the network for MT-SDT (Mobile Terminated SDT). SDT for MO-SDT is initiated by the UE only if less than a configured amount of UL data awaits transmission across all radio bearers for which MO-SDT is enabled, the DL RSRP is above a configured threshold, and a valid SDT resource is available as specified in clause 5.27.1 of TS 38.321 [6]. SDT for MT-SDT is initiated by the network with an indication to the UE in a paging message when DL data awaits transmission for MT-SDT radio bearers; based on the indication, the UE initiates the SDT for MT-SDT only if the DL RSRP is above a configured threshold as specified in clause 5.27.X of TS 38.321 [6]. When SDT is initiated for MT-SDT by the UE, a resume cause indicating MT-SDT is included in the RRCResumeRequest/RRCResumeRequest1. Maximum duration the SDT procedure can last is dictated by a SDT failure detection timer that is configured by the network (see clause 6.2.2 of TS 38.331 [12]).

SDT procedure is initiated with either a transmission over RACH (configured via system information) or over Type 1 CG resources (configured via dedicated signalling in RRCRelease). The SDT resources can be configured on initial BWP for both RACH and CG. RACH and CG resources for SDT can be configured on either or both of NUL and SUL carriers. The CG resources for SDT are valid only within the PCell of the UE when the RRCRelease with suspend indication is received. CG resources are associated with one or multiple SSB(s). For RACH, the network can configure 2-step and/or 4-step RA resources for SDT. When both 2-step and 4-step RA resources for SDT are configured, the UE selects the RA type according to clause 9.2.6. If SDT procedure is initiated for MT-SDT, the RACH not configured for SDT can be used by the UE. When SDT resources are not configured for RACH nor CG in the serving cell, only SDT procedure for MT-SDT can be supported. CFRA is not supported for SDT over RACH.

Once initiated, the SDT procedure is either:

-	successfully completed after the UE is directed to RRC_IDLE (via RRCRelease) or to continue in RRC_INACTIVE (via RRCRelease or RRCReject) or to RRC_CONNECTED (via RRCResume or RRCSetup); or

-	unsuccessfully completed upon cell re-selection, expiry of the SDT failure detection timer, a MAC entity reaching a configured maximum PRACH preamble transmission threshold, an RLC entity reaching a configured maximum retransmission threshold, or expiry of SDT-specific timing alignment timer while SDT procedure is ongoing over CG and the UE has not received a response from the network after the initial PUSCH transmission.

Upon unsuccessful completion of the SDT procedure, the UE transitions to RRC_IDLE.

For SDT, network should not send RRCReject in response to RRCResumeRequest/RRCResumeRequest1 if DL data over any radio bearer configured for SDT is transmitted.

The initial PUSCH transmission during the SDT procedure includes at least the CCCH message. When using CG resources for initial SDT transmission, the UE can perform autonomous retransmission of the initial transmission if the UE does not receive confirmation from the network (dynamic UL grant or DL assignment) before a configured timer expires as specified in clause 5.4.1 of TS 38.321 [6]. After the initial PUSCH transmission, subsequent transmissions are handled differently depending on the type of resource used to initiate the SDT procedure:

-	When using CG resources, the network can schedule subsequent UL transmissions using dynamic grants or they can take place on the following CG resource occasions. The DL transmissions are scheduled using dynamic assignments. The UE can initiate subsequent UL transmission only after reception of confirmation (dynamic UL grant or DL assignment) for the initial PUSCH transmission from the network. For subsequent UL transmission, the UE cannot initiate re-transmission over a CG resource.

-	When using RACH resources, the network can schedule subsequent UL and DL transmissions using dynamic UL grants and DL assignments, respectively, after the completion of the RA procedure.

While the SDT procedure is ongoing, if data appears in a buffer of any radio bearer not enabled for SDT, the UE initiates a transmission of a non-SDT data arrival indication using UEAssistanceInformation message to the network and, if available, includes the resume cause.

SDT procedure over CG resources can only be initiated with valid UL timing alignment. The UL timing alignment is maintained by the UE based on a SDT-specific timing alignment timer configured by the network via dedicated signalling and, for initial CG-SDT transmission, also by DL RSRP of configured number of highest ranked SSBs which are above a configured RSRP threshold. Upon expiry of the SDT-specific timing alignment timer, the CG resources are released while maintaining the CG resource configuration.

Logical channel restrictions configured by the network while in RRC_CONNECTED state and/or in RRCRelease message for radio bearers enabled for SDT, if any, are applied by the UE during SDT procedure.

The network may configure UE to apply ROHC continuity for SDT either when the UE initiates SDT in the PCell of the UE when the RRCRelease with suspend indication was received or when the UE initiates SDT in a cell of its RNA.

End of Changes

# Annex (not part of the specification): RAN2 agreements:

## RAN2#120

1. For RAN paging, MT-SDT indication (at least one bit) is explicitly included per UE via a paging message. FFS if more information for MT-SDT are needed FFS what the indication will be called. FFS signalling details 2. Rel-18 MT-SDT after the MT-SDT paging trigger is detected, RA-SDT and CG SDT solutions/procedures specified in Rel-17 is re-used as a baseline. The detailed triggers will be discussed on case by case. FFS on resources used for access 3. UE can use non-SDT random access resources for accessing the network for an MT-SDT transfer. The UE can also use the configured grant resources and/or MO-RA resources. 4. The network should be able to differentiate why the UL access was triggered, i.e. implicit or explicit indication by the UE. 5. MT-SDT is data that belongs to bearers that are configured for SDT. FFS whether the configuration is MO-SDT or MT-SDT specific. The network can only trigger MT-SDT if the data belongs to those bearers. 6. It is possible for the network to configure only MT-SDT without MO-SDT RA resources and/or CG-SDT. Subsequent UL/DL data belonging to SDT bearers while in INACTIVE is allowed like MO-SDT procedure. FFS stage 3 details 7. New Resume cause in RRC resume will be introduced, one code point MT-SDT indication

## RAN2#121

1. Include a one-bit indication in paging to trigger MT-SDT. We will ensure that the CCCH message can be transmitted over CG. 2. Indication is per UE. FFS on signalling. 3. In case condition for paging triggered MT-SDT is not fulfilled the UE initiates RRC Resume procedure. Resume cause FFS 4. Upon receiving MT-SDT trigger, the UE shall initiate SDT procedure if the following checks are satisfied (all these same as Rel-17) - FFS a: Check for DVT (if UL data becomes available in UL) - b: Check for SDT RSRP threshold - c: Check for TA validation before selecting CG (if applicable) - d: Check for SSB level RSRP threshold for CG resource (if applicable) 5. When UE resumes for MT-SDT, UE resumes all RBs configured for SDT 6. RBs configured for SDT are common for MO-SDT and MT-SDT 7. If there is valid CG-SDT resources, the UE should use CG-SDT to transmit the response. FFS on whether we need to optimize for case when CG periodicity is too long 8. To confirm that when SDT is initiated due to MT-SDT, UE can exchange subsequent DL/UL SDT data on the resumed RBs. This clarifies the RB behaviour of related RAN2#120 agreement. 9. Specify a RRC procedure for RRCResume for MT-SDT initiation without checking for availability of UL data (i.e. if MT-SDT is initiated first the resume cause will be set to MT-SDT) 10. UE is allowed to initiate either MO-SDT based resume or non-SDT based resume at any point (before initation RRCResumeRequest for MT-SDT) using separate procedures 11. If MT-SDT procedure is initiated, for RACH during subsequent data transfer (i.e. RACH triggered due to SR), UE uses only the non-SDT RACH resources (i.e. like legacy)