3GPP TSG-RAN WG2 Meeting #122	R2-2305752

Incheon, South Korea, 22– 26 May 2023	Revision of R2-2203691

Agenda item:	7.20.2

Source:	Nokia, Nokia Shanghai Bell

Title:	RA procedure while SpCell is configured with 2 TAGs

WID/SID:	NR_MIMO_evo_DL_UL-Core – Release 18

Document for:	Discussion and Decision

# Introduction

Work for NR MIMO evolution (RP-223276) has been ongoing in RAN1 and they sent an LS to RAN2 in R2-2300021:

- Overall Description In Rel-18 discussions on MIMO Evolution for Downlink and Uplink, RAN1 has agreed to support multi-DCI multi-TRP operation with two Tas. The following has been agreed/concluded in RAN1: Agreement For multi-DCI based multi-TRP operation with two Tas, support configuring two TAGs belonging to a serving cell. Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, it is up to RAN2 to decide whether there is a need to enhance CBRA procedure to support per TRP UE-initiated RACH procedure. 2. Actions To RAN WG2 ACTION: RAN1 would like to kindly ask RAN2 to take the above agreements/conclusions into account in their future work related to two Tas for multi-DCI multi-TRP operation, and notify RAN1 of any RAN2 decisions on per TRP UE-initiated RACH procedure that require RAN1 involvement.

In RAN2#121-bis meeting, it was agreed that no UE based RA triggering conditions specific for the two TA enhancement is introduced:

From RAN2 perspective, per TRP UE-initiated RACH procedure is not supported.

In this Tdoc, RAN2 issues wrt. CBRA are discussed when SpCell is configured with multiple TAGs.

# 2	Discussion

While it was meaningless to introduce any new UE based RA triggering conditions specific for the two TAs enhancement, UE can trigger CBRA procedure for other reasons like for SR (no SR resources or SR failure), BFR, or consistent LBT failure detection. Furthermore, while CBRA is triggered, there could be multiple situation that needs to be accounted with the new mTRP scenario: whether the TATs associated with the two TAGs are both running, only one TAT is running, or none of the TATs is running.

Observation 1: CBRA can be triggered by various means while the SpCell is configured with two TAGs and the running statuses of the associated TATs may vary.

Regardless of the running statuses of the TATs associated with the TAGs within a serving cell, the UE may require information for which TAG the TAC received in the RAR or fallbackRAR applies to so that appropriate TA can be utilized for the Msg3 and subsequent UL transmissions in the RA procedure – this is because the UE transmits the Msg3 and subsequent UL transmissions in the RA procedure towards the TRP that is associated with the transmitted preamble (via preamble to SSB mapping). This naturally applies only for the intra-cell scenario where different SSBs apply to different TRPs while the inter-cell scenario is clear – UE would not perform CBRA towards the other TRP.

Observation 2: For the intra-cell scenario where different SSBs apply to different TRPs, the UE needs to be made aware which TAG the TAC in RAR/fallbackRAR applies to while for the inter-cell scenario this is not needed – UE would not perform CBRA towards the other TRP in inter-cell scenario.

Proposal 1: When CBRA is performed, UE needs to be made aware to which TAG the TAC in RAR/fallbackRAR applies to transmit Msg3 (and subsequent UL) appropriately.

RAN1 was discussing in their RAN1#112-bis-e meeting how this informing of the UE about the TAG could be achieved and down scoped into the following three alternatives:

Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, down-select one of the following alternatives: - Alt 1: indicate TAG ID as part of TA command in RAR - Alt 2: indicate TAG ID as part of PDCCH order - Alt 3: divide SSBs into two groups, one for each TRP. If a SSB associated to a RACH procedure belongs to the nth group (n=1, 2), then the TA obtained via the RACH procedure corresponds to the nth TRP.

It should be noted that the Alt 2 only works CFRA case while the information is also required for CBRA case. RAN2 should advise RAN1 the Alt 2 would not work for CBRA case and, hence, from RAN2 POV, only Alt 1 or Alt 3 should be considered.

Proposal 2: Inform RAN1 that the alternative where TAG ID is provided to the UE as part of PDCCH order does not work in case of CBRA procedure.

On the other hand, when one of the TATs is not running while the CBRA is performed, it should be discussed whether the UE should apply TAC and start the TAT associated with the TAG in case, e.g., the RAR/fallbackRAR would indicate the TAG ID associated with the not running TAT. This would seem beneficial since it would follow the logic as applied currently in the RA procedure when TAT is not running for pTAG. While if the TAT associated with the TAG for which the TAC is indicated in the RAR/fallbackRAR is running, the UE can ignore the TAC (as in legacy).

Proposal 3: If the TAT associated with a TAG ID for which TAC is indicated in RAR/fallbackRAR is not running, the TAT is started.

Proposal 4: If the TAT associated with a TAG ID for which TAC is indicated in RAR/fallbackRAR is running, the TAC is ignored (as in legacy).

# 3	Conclusion

Observation 1: CBRA can be triggered by various means while the SpCell is configured with two TAGs and the running statuses of the associated TATs may vary.

Observation 2: For the intra-cell scenario where different SSBs apply to different TRPs, the UE needs to be made aware which TAG the TAC in RAR/fallbackRAR applies to while for the inter-cell scenario this is not needed – UE would not perform CBRA towards the other TRP in inter-cell scenario.

Proposal 1: When CBRA is performed, UE needs to be made aware to which TAG the TAC in RAR/fallbackRAR applies to transmit Msg3 (and subsequent UL) appropriately.

Proposal 2: Inform RAN1 that the alternative where TAG ID is provided to the UE as part of PDCCH order does not work in case of CBRA procedure.

Proposal 3: If the TAT associated with a TAG ID for which TAC is indicated in RAR/fallbackRAR is not running, the TAT is started.

Proposal 4: If the TAT associated with a TAG ID for which TAC is indicated in RAR/fallbackRAR is running, the TAC is ignored (as in legacy).