3GPP TSG-RAN WG2 Meeting #122	R2-2305755

Incheon, South Korea, 22– 26 May 2023

Agenda item:	7.14.2

Source:	Nokia, Nokia Shanghai Bell

Title:	QoE measurements support in RRC IDLE and INACTIVE

WID/SID:	NR_QoE_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In Rel-18, the QoE measurement (WID RP-223488) will apply for UE in Idle/Inactive mode, e.g., to support Application Layer measurement Collection functionality for MBS broadcast service received in RRC Idle/Inactive mode.

- Specify for QoE measurement configuration and collection in RRC_INACTIVE and RRC_IDLE states for MBS, at least for broadcast service [RAN3, RAN2].

- Specify the mechanism to support the alignment of the existing radio related measurement and QoE reporting.

It is expected that the UE would be able to make QoE measurements during Idle/Inactive RRC states and report QoE measurements back to the network. Based on the progress made in both RAN2 and RAN3, in this contribution we elaborate more on the potential way forward for extending the QoE framework to RRC Idle and RRC Inactive for MBS broadcast service.

# 2	Discussion

## 2.1	QoE Configuration for MBS

For MBS broadcast communication service, the same service and the same specific content data are provided simultaneously to all UEs in a geographical area. A broadcast communication service is delivered to the UEs using a broadcast session. And a UE can receive a broadcast communication service in RRC IDLE, RRC INACTIVE and RRC_CONNECTED state. To support QoE for the broadcast service, NR QoE framework needs to enable data collection for the service that can be continued through all RRC states.

In previous RAN2 meetings, it was agreed that the QoE configuration for MBS broadcast service will be provided via dedicated signalling such as RRCReconfiguration and RRCResume message but not RRCRelease message. The UE will then store the configuration and perform QoE measurements accordingly when it transfers to RRC IDLE/INACTIVE. As a UE may be configured with a list of application layer measurement configurations for MBS, when the UE is transferred from CONNECTED to RRC IDLE/INACTIVE state the gNB should indicate which QoE configuration is to be continued. The open part is whether the “continuity” flag is explicit or implicit, and in which RRC message to deliver the flag to UE.

Agreement: The QoE configuration indicates the applicable states (i.e. that the QoE measurements for CONNECTED are supposed to be gathered also in RRC_IDLE/INACTIVE). FFS whether this is explicit or implicit.

There are several options on how the gNB informs the continuity of the QoE collection configuration for MBS to UE. One option is to indicate the continuity information in RRCReconfiguration and RRCResume message. The other option is that the gNB can indicate the continuity of the application layer configuration in RRCRelease messages. If the gNB already know the QoE configuration applicable state for RRC_IDLE/INACTIVE when gNB configure or resume the QoE configurations, gNB may indicate the applicable state per QoE configuration in RRCReconfiguration and RRCResume message. However, normally RRCRelease message will be used to switch the UE from RRC Connected state to idle/inactive, it is a simpler way-forward to include the QoE configuration applicable states in RRCRelease message. This is also helpful to handle the case where the OAM change (e.g., release) some of the QoE configurations for MBS during the connection to avoid additional while unnecessary RRCReconfiguration before gNB triggers the RRCRelease.

Proposal 1: RRCReconfiguration, RRCResume and RRCRelease can be used to inform UE the applicable state of QoE configuration(s) for UEs being in RRC idle and RRC inactive state.

To indicate the applicable state of QoE configurations, for RRCReconfiguration and RRCResume, a new optional “flag” can be added in AppLayerMeasConfig for each QoE configuration. For RRCRelease message, the gNB can either indicate the RRC IDs for the QoE collections to be released or the RRC IDs for the QoE collections to be maintained/continued in RRCRelease message. In Rel-17, if the UE enters RRC idle state the UE should release all application layer measurement configurations. Following this principle, to continue the QoE collection for MBS when UE is in RRC idle, the gNB should indicate the RRC IDs for the QoE collections to be continued in RRCRelease message. Hence the UE should only release the application layer measurement configurations which are not indicated to be maintained/continued when it enters RRC idle.

Proposal 2a: In RRCReconfiguration and RRCResume message, the gNB can indicate whether a QoE collection is to be continued in RRC idle/inactive via an optional flag in each QoE configuration.

Proposal 2b: In RRCRelease message, the gNB can indicate the RRC ID(s) for the QoE collection to be continued in RRC idle/inactive.

## 2.2	Area Scope handling

On the aspect of QoE measurement area scope handling for MBS broadcast services, RAN3 agreed that in case QoE measurements are collected by the UE in RRC IDLE/INACTIVE state, area scope is checked by the UE. For UE in RRC CONNECTED state, it is not clear the area scope should be checked by UE or network as discussed in RAN2-121bis meeting.

For MBS broadcast services: - Area scope is checked by the UE when the UE is in RRC IDLE/INACTIVE state. - FFS whether area scope is checked by the network or by the UE when the UE is in RRC CONNECTED state for MBS broadcast services.

In Rel-17, for UE in RRC CONNECTED state, it is the network who is responsible for keeping track of whether the UE is inside or outside the Area Scope. The network can check the area scope for each serving cell (e.g. after Handover). If the UE leaves the Area Scope, the network can indicate to the UE to release the QoE configuration. In our understanding, the current mechanism can be reused by RRC Connected UE with MBS broadcast services without specification change. Instead, if the area scope is checked by UE for the MBS broadcast services, it needs further specification change (in both stage-2 and stage-3) and further network implementation to differentiate handling for various QoE service type. It will be even worse when the UE is out of the Area Scope while the QoE collection continues since UE may only check the area scope at the start of a QoE session instead of checking the scope for each serving cell.

Proposal 3: For UE in RRC CONNECTED state with MBS broadcast services, network is responsible for area scope checking (following Rel-17 principle without specification impact).

On how UE should check the area scope when the UE is in RRC IDLE/INACTIVE state with MBS broadcast service, the first issue is how UE can get the area scope information.

In the response LS S4-230369, SA4 confirmed that, the area scope of a QoE configuration can be provided within the QoE configuration container and it can be indicated via the Location Filter, which can be a list of cell IDs. Since the area scope can be provided to UE’s application layer based on current SA4 specification and the UE should keep the QoE configuration when it transfers from RRC Connected to RRC idle/inactive, the UE can exactly know the area scope to be checked when it moves to RRC idle/inactive. To minimize the specification impact, we think it is a reasonable way-forward to reuse the area scope information in the application layer for area scope checking when the UE moves to RRC idle/inactive.

Proposal 4: When UE moves to RRC idle/inactive, the UE should perform the area scope check for MBS broadcast service based on the area scope information provided in Application layer.

Based on the area scope information provided in Application layer, the next issue is the checking should be performed in UE application layer or AS layer.

In our view, the area scope checking in UE’s application layer is the most straight-forward way. However, in RAN2-121bis email discussion [R2-2304529], there are concerns that AS layer has to inform the application layer the RRC state of the UE, otherwise the UE doesn’t know when it should handle the area scope checking. Actually, we understand the AS layer should also inform the application layer the cell id after each cell change to facilitate the area scope checking in application layer. But it will burden the AS layer to deliver the infromation after each cell change to different applications, considering every application needs to implement the area handling. The concern can be addressed if it is the AS layer to perform the area scope checking because AS layer itself know everything about RRC state change and cell change.

Observation 1: When UE is in RRC idle/inactive state, checking the area scope in UE’s AS layer can avoid implementation efforts to provide the cell id to different applications after every cell change.

Furthermore, in LS S4-230369, SA4 mentioned the UE Application layer only check the area scope at the start of a QoE session. If the the area scope checking is implemented in AS layer, the UE can check the area scope after each cell change.

Observation 2: When UE is in RRC idle/inactive state, checking the area scope in UE’s AS layer enables the area scope checking after every cell change instead of only at the start of the QoE session.

Based on above observations, we think it is worth supporting the area scope checking in UE’s AS layer based on the area scope information provided from Application layer.

Proposal 5: For UE in RRC IDLE/INACTIVE state with MBS broadcast services, UE AS layer is responsible for area scope checking based on the area scope information provided from Application layer.

## 2.3	QoE data buffer handling for UE in RRC idle/inactive

When the UE is in RRC idle or inactive state, it is expected that the UE needs to continue collecting QoE data for MBS. Therefore, QoE data shall be stored when the UE is in RRC idle or RRC inactive state until the UE establishes a connection with the network and QoE data can be transmitted to the network. Since the UE needs to buffer the QoE data for as long as it stays in RRC idle or inactive, it means the stored data may impact the UE memory more seriously.

RAN2 agreed that UE’s AS layer should discard the QoE data if the AS layer buffer is full. As default behaviour, the UE should discard older reports to make room for the new one.

Agreement: As a default behavior, when the UE’s buffer for storing QoE reports is full and a new report arrives, the UE should discard older report(s) to make room for the new one. FFS whether it is possible to provide information (e.g. priority, service type, etc.) to UE about buffering for the UE to decide which reports to discard in case the UE’s QoE buffer becomes full.

As indicated in the SA5 LS (S5-232760) , in case of the UE’s QoE buffer is full, different consumers may have different selection policies. Therefore, the gNB may need to provide assistance information to UE to decide which reports should be discarded. Most importantly, SA5 indicated that if network wants to change the QoE reports selection policy, all the UEs should have a consistent behaviour.

SA5 has the following responses to RAN2's questions: Question 4: In case of limited storage space for QoE reports at the UE, is there any preference from the OAM side on which QoE reports should be reported and which should be discarded, e.g. is there a principle that newer or older reports are more useful for the network? SA5: From SA5’s point of view, some selection policies from consumers could be possible, but there are no use cases for it yet. But as a consistent behavior is wanted from all UEs, a default behavior should be to prioritize new data.

Observation 3: All the UEs should be configured to have a consistent behaviour on how to discard QoE reports when the AS buffer becomes full.

To indicate the consistent behaviour for all UEs, the assistance information to be provided to UE should be delivered in a broadcast instead of unicast way.

Proposal 6: gNB can provide assistance information to UE on how to discard QoE reports when the AS buffer becomes full. The assistance information should be provided to UE in a broadcast way .

In RAN3 discussion, as indicated in LS R3-232047, most companies preferred to introduce a priority per QoE configuration that can be used as the assistance information. However, considering there may have multiple types of consumers for receiving the QoE reports, RAN3 is still asking SA5 to confirm whether the OAM can provide the explict reporting priority per QoE configuration to gNB, or provide  the “characteristics of reporting” for different “type of consumer” to gNB.

Q1: Can there be multiple types of consumers for receiving the QoE reports (pursuing the role of the MCE)? If yes, what are those potential consumers as supported by the current specifications? Q2: If the answer to the first part of Q1 is “yes”, and if different consumers can have different priorities in receiving the QoE reports, is the OAM able to compare and rank by priority the preferences of different consumers or rank the consumers themselves? Can the OAM make the final decision regarding setting the reporting priorities? Can the OAM coordinate with the consumers and inform the consumers if the intended priority is not met? Q3: Is there any other issue(s) related to sending such an explicit priority per QoE configuration as assistance information to the NG-RAN node? Q4: Can the OAM indicate the “type of consumer” (as in Q1) or “characteristics of reporting” (e.g., the loop cycle, reporting periodicity, expected number of reports or amount of data to be reported etc.) as assistance to the NG-RAN node in the QoE configuration?

Given the above details, RAN2 can wait for SA5 and RAN3 to decide the exact assistance information to differentiate the priority of each QoE configuration.

Proposal 7: RAN2 to wait for SA5 and RAN3 to decide the exact content of QoE assistance information to differentiate the priority of each QoE configuration.

## 2.4	UE selection for QoE collection

Given the MBS broadcast service characteristics it can be provided simultaneously to all UEs in a geographical area (i.e., all UEs in the broadcast service area are authorized to receive the data). To ensure controlled data collection through all RRC states of the UEs, RAN2#119 identified an FFS to discuss the way how the gNB selects the MBS users:

Agreement: FFS how does gNB determine which UEs can be configured with MBS QoE measurements

In contrary to Rel-17, Rel-18 QoE for MBS broadcast service might result in a massive reporting and long sessions. To save the Uu interface signalling overhead, there is no need to configure all the UEs in an area to collect QoE measurements. In fact, from network point of view, it is sufficient to configure only a limited number of UEs in an area for QoE collection to benchmark the QoE of the MBS service. In other words, to ensure the constrains with buffering data and generating higher load, the QoE configuration should support the option to choose UEs in a selective manner.

The gNB may select UEs based on the requirements on how to benchmark MBS service quality. For example, network may select the UEs in a randomized way in which the QoE reports from the selected UEs can represent the overall MBS service quality of a dedicated area. Another way is that network may select UEs who have potential to have poor MBS service experience. It is gNB’s strategy to decide which UEs should be selected to configure the QoE measurements.

Proposal 8: The gNB can select MBS UEs for QoE measurement either in a randomized way or only select UEs that experience poor MBS service experience.

## 2.5	QoE reporting for MBS

According to the agreements in RAN2#120 meeting, as a baseline, UE does not trigger RRC connection setup or resume just for the sake of reporting QoE measurement results. It is FFS whether there are cases where UE can trigger RRC connection Setup for the sake of reporting QoE.

Agreement: As a baseline, UE does not tigger RRC Resume – RRC Setup just for the sake of reporting QoE. FFS whether there are cases where we deviate from this baseline.

As discussed in section 2.3, since the UE needs to buffer the QoE data for as long as it stays in RRC idle or inactive, it means the stored data may impact the UE memory seriously. When the UE’s AS layer buffer is full, the UE has to discard the old QoE reports and store the new one received from Application layer, or vice versa.  For the MBS transmission observability, e.g., to compare reception of the service in idle/inactive UEs, we believe Rel-18 solution should provide certain means to overcome discarding QoE data at a high rate. One possibility is to extend UE memory requirement, but it can only mitigate the issue instead of resolving the issue. From network point of view, if the system load is low enough, it makes no sense to forbid UE to trigger the RRC connection for the QoE data report before the UE discards it. Therefore, we would like to suggest that gNB could control whether the UE is allowed to resume or setup the RRC connection to report the QoE data which is to be discarded.

Proposal 9: When UE’s QoE buffer is full, the UE is allowed to trigger RRC Resume or Setup to report the QoE data if it is allowed by network.

# 3	Conclusion

This document has made the following observations:

Observation 1: When UE is in RRC idle/inactive state, checking the area scope in UE’s AS layer can avoid implementation efforts to provide the cell id to different applications after every cell change.

Observation 2: When UE is in RRC idle/inactive state, checking the area scope in UE’s AS layer enables the area scope checking after every cell change instead of only at the start of the QoE session.

Observation 3: All the UEs should be configured to have a consistent behaviour on how to discard QoE reports when the AS buffer becomes full.

And proposed the following:

Proposal 1: RRCReconfiguration, RRCResume and RRCRelease can be used to inform UE the applicable state of QoE configuration(s) for UEs being in RRC idle and RRC inactive state.

Proposal 2a: In RRCReconfiguration and RRCResume message, the gNB can indicate whether a QoE collection is to be continued in RRC idle/inactive via an optional flag in each QoE configuration.

Proposal 2b: In RRCRelease message, the gNB can indicate the RRC ID(s) for the QoE collection to be continued in RRC idle/inactive.

Proposal 3: For UE in RRC CONNECTED state with MBS broadcast services, network is responsible for area scope checking (following Rel-17 principle without specification impact).

Proposal 4: When UE moves to RRC idle/inactive, the UE should perform the area scope check for MBS broadcast service based on the area scope information provided in Application layer.

Proposal 5: For UE in RRC IDLE/INACTIVE state with MBS broadcast services, UE AS layer is responsible for area scope checking based on the area scope information provided from Application layer.

Proposal 6: gNB can provide assistance information to UE on how to discard QoE reports when the AS buffer becomes full. The assistance information should be provided to UE in a broadcast way .

Proposal 7: RAN2 to wait for SA5 and RAN3 to decide the exact content of QoE assistance information to differentiate the priority of each QoE configuration.

Proposal 8: The gNB can select MBS UEs for QoE measurement either in a randomized way or only select UEs that experience poor MBS service experience.

Proposal 9: When UE’s QoE buffer is full, the UE is allowed to trigger RRC Resume or Setup to report the QoE data if it is allowed by network.