3GPP TSG-RAN WG2 Meeting #122	R2-2305758

Incheon, South Korea, 22– 26 May 2023

Agenda item:	7.6.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	On HARQ enhancements for IoT NTN

WID/SID:	IoT_NTN_enh - Release 18

Document for:	Discussion and Decision

# 1	Introduction

At RAN2#119bis and RAN2#120 meetings, RAN2 discussed the HARQ enhancements on aspects including the signalling to support HARQ feedback enabling/disabling and HARQ mode A/B, the DRX impact for NB-IoT with single HARQ process, as well as LCP impact for eMTC NTN etc.

At RAN2#121bis meeting, the relevant issues were further discussed and most HARQ enhancements depend on RAN1 progress, therefore one LS R2-2304274 were sent to RAN1. In addition, the following issues are left to be further addressed by RAN2 :

- For the UL HARQ mode signalling, whether both RRC configuration and DCI-based solution should be supported.

- Whether UL transmission using PUR can be configured with HARQ mode B

- For eMTC NTN, whether it can be left to eNB’s implementation to configure either HARQ mode A or HARQ mode B for all HARQ process (or no HARQ mode) if mpdcch-UL-HARQ-ACK-FeedbackConfig is configured

- For eMTC NTN, whether to enhance the TAR MAC CE transmission to avoid or mitigate the outdated TA and Koffset impact in Rel-18.

In this contribution, we further discuss above open issues.

# 2	Discussions

## 2.1	Signalling for UL HARQ mode A/B

Only DL HARQ feedback disabling was discussed in RAN1 meetings while the signalling to support HARQ mode A and HARQ mode B in UL has not been discussed yet. For DL HARQ operation, the motivation to support DL HARQ feedback disabling is that it can avoid HARQ stalling to improve DL throughput while HARQ feedback enabling can not only achieve high reliability but also provide HARQ feedback for some important MAC CE activation.  Disabling HARQ process feedback for the “important” PDSCH carrying the MAC CE may impact the system operation since NW has no idea whether the MAC CE was transmitted successfully or not. In the single HARQ process case for NB-IoT, if only RRC based solution is supported to switch the HARQ feedback disabling/enabling via RRC reconfiguration message, it will result in not only significant signalling overhead and latency but also the reduced the system performance. Therefore, in RAN1 agreement, DCI-based HARQ feedback enabling/disabling is introduced on top of RRC-based solution for fast HARQ feedback enabling/disabling switch/indication for NB-IoT and eMTC CE mode B in which only support small number of HARQ processes.

For UL, NW may configure HARQ mode B for data transmission to avoid HARQ stalling while configure HARQ mode A for high reliability transmission. To minimize the efforts on the discussion and standardization impact, one unified solution on the signalling of HARQ mode A/B for UL and HARQ feedback enabling/disabling for DL is preferred. However, there are some differences between DL and UL for IoT NTN.

For UL, it is up to the NW implementation to schedule the UL grant with new transmission or retransmission before/after decoding previous uplink transmissions, and the UE can only follow the scheduling decision of the NW. For NB-IoT, due to LCP is not supported, no matter  HARQ mode A or B is configured for a HARQ process, the transmission reliability for a LCH or MAC CE maybe same no matter the NW schedule the blind retx (as if in HARQ mode B) or the new transmission /retransmission based on the UL decoding results (as if in HARQ modeA). Additionally, the NW may not know whether there is MAC CE for transmission in this uplink HARQ process, thus can’t determine whether to schedule HARQ mode A in this HARQ process. Therefore, for NB-IoT NTN, it seems RRC-based HARQ mode A/B indication is enough.

Proposal 1: For NB-IoT NTN, support RRC-based UL HARQ mode A/B indication is sufficient.

For eMTC NTN, it was agreed in RAN2#119e that HARQ mode-based LCP is supported.

Agreements: The solutions of LCP restriction on allowed HARQ mode in NR NTN can be reused for eMTC NTN

With HARQ mode related LCP restriction, the NW can configure the HARQ mode (e.g., HARQ mode A or HARQ mode B) to a UL HARQ process, additionally the NW could configure the preferred HARQ mode to a logical channel. In the data multiplexing, the LCH data is mapped only to a HARQ process configured with the same HARQ mode. Due to at most 2 HARQ processes are supported for eMTC CE mode B if the UE is not configured with higher layer parameter ce-PUSCH-MultiTB-Config, the RRC-based HARQ mode indication may bring some restriction on the scheduling flexibility. For example, assuming UE is configured with one HARQ process with HARQ mode B to avoid HARQ stalling and one HARQ process is configured with HARQ mode A for reliability,if new data arrived for transmission from one LCH configured with HARQ mode A but the HARQ processes configured with HARQ mode A is occupied for data transmission and the HARQ process configured with HARQ mode B is in idle,  the NW can’t schedule this idle HARQ processes for data transmission due to the unmatched HARQ mode between the HARQ processes and the LCH. In this case, DCI-based HARQ mode indication can bring more scheduling flexibility via changing the HARQ mode directly in DCI grant instead of changing the HARQ mode via RRC first.

So RAN2 can have further discussion on whether DCI-based HARQ mode indication is supported for eMTC CE mode B for uplink.

Observation 1: For eMTC CE Mode B, fast UL HARQ mode indication/switch via DCI can provide more scheduling flexibility than RRC signaling.

Proposal 2: RAN2 to further discuss whether DCI-based UL HARQ mode indication is supported for eMTC NTN CE mode B.

## 2.2	UL HARQ mode for the transmission using PUR

At RAN2#121bis meeting, it was open whether UL HARQ mode B should be supported for PUR transmission. The argument to support HARQ mode B for PUR is that it can support blind retransmission by adjusting the start of pur-ResponseWindowTimer without delay of UE-eNB RTT.

UE specific PUR is introduced to allow one uplink transmission from RRC_IDLE using a preconfigured uplink resource without performing the random-access procedure. In our understanding, PUR is targeted for small packet transmission and the preconfigured uplink resource such as the TBS and periodicity is configured based on UE request. Hence HARQ stalling is not a critical issue.  In addition, preconfigured uplink resource is configured via RRCRelease when the UE moves from RRC Connected to RRC_IDLE. This means the network can know more channel information of the UE in RRC Connected state before configuring the PUR resource for the UE, thus it can configure a reasonable repetition number for the PUR transmission. Moreover, the eNB can also further adjust the repetition number though the PDCCH scrambled with PUR-RNTI. Therefore, blind retransmission seems not that necessary for PUR transmission which is different from blind retransmission for Msg3.

Observation 2: For PUR, HARQ stalling is not a critical issue and blind retransmission support is not necessary since reasonable repetition number can be used for reliability.

Proposal 3:  No need to support UL HARQ Mode B for PUR transmission.

## 2.3	UL HARQ mode for PUSCH repetition early termination

The early termination function is used to early termination of PUSCH transmission in the case that repetition number is large. This function can be configured through mpdcch-UL-HARQ-ACK-FeedbackCronfig as specified in 36.331:

mpdcch-UL-HARQ-ACK-FeedbackConfig TRUE indicates E-UTRAN may send UL HARQ-ACK feedback or UL grant corresponding to a new transmission for early termination of PUSCH transmission, or positive acknowledgement of completed PUSCH transmissions as specified in TS 36.321 [6] and TS 36.212 [22]. In case of acknowledgement of RRC Connection Release, MPDCCH monitoring is terminated.

For UEs configured with mpdcch-UL-HARQ-ACK-FeedbackConfig, repetitions within a bundle are stopped if an UL HARQ-ACK feedback or an uplink grant corresponding to a new transmission of the bundle is received on PDCCH during the bundle transmission. At RAN2#121bis meeting, it was discussed but not agreed on the UL HARQ mode configurations if early termination is configured. E.g., For eMTC NTN, whether it can be left to eNB’s implementation to configure either HARQ mode A or HARQ mode B for all HARQ process (or no HARQ mode) if mpdcch-UL-HARQ-ACK-FeedbackConfig is configured. In our understanding, early termination is to terminate the unnecessary repetitions and is workable for either HARQ mode A or HARQ mode B for all HARQ process or even no HARQ mode configured at all. The reason is that it is NW’s implementation to decide whether to send UL HARQ-ACK feedback/UL grant in DL PDCCH (based on UL decoding result), in order to early terminate the PUSCH repetitions regardless of the HARQ mode associated with the repetitions to be early terminated.

In addition, 36.321 defined the DRX operation when mpdcch-UL-HARQ-ACK-FeedbackConfig is configured as below:

When a DRX cycle is configured, the Active Time includes the time while: - …… - mpdcch-UL-HARQ-ACK-FeedbackConfig is configured and repetitions within a bundle are being transmitted according to UL_REPETITION_NUMBER. If this Serving Cell is part of a non-terrestrial network, the Active Time starts after the first repetition within the bundle plus the UE-eNB RTT when repetitions within the bundle are being transmitted When DRX is configured, the MAC entity shall for each subframe: …… if mpdcch-UL-HARQ-ACK-FeedbackConfig is configured and an UL HARQ-ACK feedback has not been received on PDCCH until the last repetition of the corresponding PUSCH transmission: - start or restart the drx-ULRetransmissionTimer for the corresponding HARQ process in the subframe containing the last repetition of the corresponding PUSCH transmission; if the PDCCH indicates an UL HARQ-ACK feedback for an asynchronous UL HARQ process for a UE configured with mpdcch-UL-HARQ-ACK-FeedbackConfig: - if the lower layer had indicated scheduling of transmission of multiple TBs: - stop drx-ULRetransmissionTimer for the corresponding UL HARQ process(es). - else if the PUSCH transmission is completed: - stop drx-ULRetransmissionTimer for all UL HARQ processes

According to current specification, the DRX Active Time starts after the first repetition within the bundle plus the UE-eNB RTT when repetitions within the bundle are being transmitted. It makes sense since the gNB can only determine to early terminate the repetitions when it decodes the uplink transmission correctly. So when the PDCCH indicates an UL HARQ-ACK feedback,  the drx-ULRetransmissionTimer is stopped. If an UL HARQ-ACK feedback has not been received on PDCCH until the last repetition of the corresponding PUSCH transmission which means the retransmisison of the PUSCH may be scheduled by eNB,   the configured HARQ mode for this PUSCH repetition  can be HARQ mode A or  HARQ mode B. Therefore, current DRX behavior is workable for both HARQ mode A and HARQ mode B when mpdcch-UL-HARQ-ACK-FeedbackConfig is configured.

Proposal 4: It can be left to eNB’s implementation to early terminate the PUSCH repetitions when mpdcch-UL-HARQ-ACK-FeedbackConfig is configured, regardless of the HARQ mode configured for the PUSCH repetitions.

## 2.4	Unreliable Timing Advance Report MAC CE transmission

For eMTC NTN, whether to enhance the TAR MAC CE transmission to avoid or mitigate the outdated TA and Koffset impact in Rel-18 was briefly discussed in email discussion [AT121bis-e][103] with summary in report R2-2304254. Since the companies have divergent views (4 companies agree, 4 companies disagree and 5 companies with no strong view), it was decided to be further discussed in RAN2#122 meeting.

As explained in paper R2-2303644, the eNB may maintain an outdated Timing Advance information if the TAR MAC CE was not transmitted to eNB successfully, especially when the MAC CE was transmitted in UL HARQ Mode B.

The typical case is that, when the UE’s TA is increased due to the increase of UE-Satellite-eNB distance, the UE may trigger a TA report to update the latest TA to NW. However, if the TAR MAC CE was transmitted in UL HARQ mode B, the eNB may not decode the MAC PDU correctly since HARQ mode B best supports no UL retransmission and/or blind UL retransmission (where the retransmission is not based on UL decoding result). Different from data transmission where the RLC retransmission mechanism may be triggered for transmission robustness, there is no upper layer retransmission for MAC CE. Due to the unsuccessful TAR MAC CE transmission, the eNB would not know TA is updated/increased in UE hence the outdated TA is maintained in eNB. Therefore, the eNB has no way to update/increase its Koffset based on the outdated TA. The procedure can be well illustrated in below Figure1.

Figure 1 An example of UE using outdated UE Koffset due to unsuccessful TA report

Observation 3: The eNB may configure an outdated Koffset based the outdated Timing Advance, when the TAR MAC CE was transmitted in UL HARQ Mode B.

One may argue that the NW can configure a larger Koffset to accommodate possible the TA change hence the outdate Koffset is not an issue. However, we would like to emphasize the motivation to introduce the TA Report MAC CE is to configure updated Koffset (e.g., configure UE specific Koffset based on the UE reported TA), hence data transmission latency can be reduced for UE located in the cell centre. In short, configuring a larger Koffset to mitigate the issue is not a reasonable solution since NW has to configure a Koffset which can cover the largest TA of all the UEs under the cell. While this solution will kill the motivation to support TA report which implies the TA report is not useful at all.

Observation 4: The outdated Koffset issue cannot be resolved by configuring a larger Koffset since it will kill the motivation and benefit of introducing the TA report MAC CE mechanism.

Furthermore, based on current specification, we believe the outdated Koffset issue will happen frequently since the TA change is a common behaviour in NTN due to flying characteristics of satellites, no matter in earth fixed cell or earth moving cell.

Observation 5: Based on current specification, the outdated Koffset issue will happen frequently in NTN.

In the end, with the increase of UE’s TA, when the UE’s actual TA is larger than (UE's outdated Koffset + 4 - UE's UL processing delay) the UE cannot perform UL transmission at all due to not enough processing time in UE in between PDCCH grant and the corresponding PUSCH transmission. (i.e., the gap between slot n in UE’s DL timing and the scheduled UL transmission slot m in the UE’s UL timing is smaller than a threshold (e.g., the UL processing delay) after UE applied it TA adjustment.)

Observation 6: The outdated Koffset may cause PUSCH transmission failure in eMTC NTN.

Figure 2: UL transmission timing for eMTC NTN

To handle the outdated Koffset thus PUSCH transmission failure due to unreliable TAR MAC CE transmission, there may have two directions for the solutions: one way is to improve the TAR MAC CE transmission robustness to avoid outdated TA at network. The other way is to mitigate the PUSCH transmission failure impact caused by the outdated TA and Koffset. In our view, the enhancement on TAR MAC CE transmission reliability should be supported to avoid the PUSCH transmission failure as much as possible. Furthermore, if UE detected PUSCH transmission failure which was caused by inappropriate Koffset, the UE should inform NW to stop further UL grant hence NW can reconfigure the Koffset for normal PUSCH transmission.

Observation 7: The PUSCH transmission failure issue caused by unreliable TAR MAC CE transmission should be addressed for eMTC NTN.

Proposal 5: The TAR MAC CE transmission reliability should be enhanced to avoid PUSCH transmission failure caused by outdated TA.

Proposal 6: UE should report the PUSCH transmission failure to NW if it is caused by inappropriate Koffset configured by NW.

# 3	Conclusion

This document has made the following observations:

Observation 1: For eMTC CE Mode B, fast UL HARQ mode indication/switch via DCI can provide more scheduling flexibility than RRC signaling.

Observation 2: For PUR, HARQ stalling is not a critical issue and blind retransmission support is not necessary since reasonable repetition number can be used for reliability.

Observation 3: The eNB may configure an outdated Koffset based the outdated Timing Advance, when the TAR MAC CE was transmitted in UL HARQ Mode B.

Observation 4: The outdated Koffset issue cannot be resolved by configuring a larger Koffset since it will kill the motivation and benefit of introducing the TA report MAC CE mechanism.

Observation 5: Based on current specification, the outdated Koffset issue will happen frequently in NTN.

Observation 6: The outdated Koffset may cause PUSCH transmission failure in eMTC NTN.

Observation 7: The PUSCH transmission failure issue caused by unreliable TAR MAC CE transmission should be addressed for eMTC NTN.

And proposed the following:

Proposal 1: For NB-IoT NTN, support RRC-based UL HARQ mode A/B indication is sufficient.

Proposal 2: RAN2 to further discuss whether DCI-based UL HARQ mode indication is supported for eMTC NTN CE mode B.

Proposal 3:  No need to support UL HARQ Mode B for PUR transmission.

Proposal 4: It can be left to eNB’s implementation to early terminate the PUSCH repetitions when mpdcch-UL-HARQ-ACK-FeedbackConfig is configured, regardless of the HARQ mode configured for the PUSCH repetitions.

Proposal 5: The TAR MAC CE transmission reliability should be enhanced to avoid PUSCH transmission failure caused by outdated TA.

Proposal 6: UE should report the PUSCH transmission failure to NW if it is caused by inappropriate Koffset configured by NW.