3GPP TSG-RAN WG2 Meeting #122	R2-2305759

Incheon, South Korea, 22– 26 May 2023

Agenda item:	7.6.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	GNSS operation enhancements for IoT NTN

WID/SID:	IoT_NTN_enh - Release 18

Document for:	Discussion and Decision

# 1	Introduction

The release 18 work item [1] on IoT support of non-terrestrial networks includes the following objective

Study and specify needed improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1, RAN2]

In release 17, the work was limited to short and sporadic connection and therefore it was feasible that a UE upon GNSS validity timer expiry could move to RRC Idle mode. However, the release 18 work will address the broader use case of long connection time. Therefore, it is important to consider how the UE can obtain a new GNSS-based position fix during the long connection. It is also good to note that the simultaneous use of GNSS and IoT operation is not assumed as was also the case in release 17.

On top of the agreements reached in RAN1 and RAN2, in this contribution, we provide our view on the improved GNSS operations for IoT NTN from RAN2 point of view.

# 2	Discussion

## 2.1	GNSS assistance information report

At RAN1 #110 it was agreed that the UE reports GNSS assistance information to the eNB. The information includes the GNSS validity duration and the GNSS position fix time duration for measurement.

Agreement GNSS assistance information that UE reports to eNB at least consists of: - GNSS position fix time duration for measurement - GNSS validity duration

For GNSS position fix time duration,  as discussed in RAN2-112bis meeting, one open issue is that it is not clear whether the UE needs to report GNSS position fix time duration in RRC connected state. Some companies think the fix time duration value does not change during the connected mode because the IoT device is stationary, therefore the UE does not need to report GNSS fix time in Connected state. However, the assumption is not true because the IoT NTN device is not always stationary.

In the NTN IoT WID [RP-211601], it lists key target industries including transportation (maritime, road, rail, air), logistics, farming, and mining. In all those industries a certain degree of UE movement is to be expected. Furthermore, the UE movement is also confirmed in NTN IoT Rel-18 WID [RP-223519] since a NOTE was added as below:

NOTE: It is expected a UE can move while under discontinuous coverage.

Observation 1: According to WID description, UE movement is expected to be supported for IoT NTN.

Due to UE movement, the GNSS position fix time duration maybe dynamically changed during the long connection if the UE starts moving and the (GNSS) propagation conditions changed a lot (e.g, shadowed when UE moves from outdoor to indoor).

Observation 2: For UE in RRC Connected mode, the UE may move and the propagation conditions may change and therefore cause a change in the required GNSS position fix time duration.

At RAN1-112bis meeting, it was agreed the GNSS measurement gap length is equal to the latest UE reported GNSS position fix time duration if eNB does not provide the gap length in the configuration.

Agreement For the GNSS measurement gap aperiodically triggered with MAC CE, the duration for the GNSS measurement gap can be configured by eNB. - The gap duration is equal to the latest reported GNSS position fix time duration for measurement when the duration for GNSS measurement gap is not included in the configuration by eNB.

If the UE observed the GNSS position fix time duration is changed while the UE does not report the latest value to eNB, the eNB and UE would assume a GNSS measurement gap based on the outdated GNSS position fix time value. This may cause the UE cannot complete the GNSS re-acquisition during the gap when the assumed gap length is less than the required GNSS fix time. Finally, the UE may go to RRC idle due to GNSS re-acquisition failure during the GNSS measurement gap.

Observation 3: If the latest GNSS position fix time cannot be reported to eNB, the RRC connected UE may go to RRC idle state incorrectly because the eNB and UE assume an incorrect GNSS measurement gap duration.

From RAN2 point of view, if the GNSS position fix time changes, it is no harm to let the UE report the latest GNSS position fix time duration in RRC Connected state. Therefore, RAN2 can take a working assumption that the UE should report GNSS position fix time duration in RRC connected state if the value is changed.

In addition, RAN1 is discussing the supported values for GNSS position fix time duration. It is FFS how many component values should be supported and FFS whether RAN4 input is needed. To make better progress, we think RAN2 can also send LS to RAN1 and RAN4 to confirm RAN2’s working assumption.

Third Round Proposal 4-1c: UE reports one GNSS position fix time duration for GNSS measurement via a N-bit field at least including with [1,2] seconds as candidate component values. - FFS: value of N, other component value(s) of candidate GNSS position fix time duration (e.g. N=3, with value in [1,2,3,7,13,19,25, X] seconds, and X is FFS). FFS: whether RAN4 input is needed.

Proposal 1: RAN2 assumes the UE should report GNSS position fix time duration in RRC connected state if the value changes. This can be revised based on RAN1 and RAN4 further input if necessary.

Proposal 2: Send LS to RAN1 and RAN4 to confirm the working assumption in Proposal1.

At RAN2#121 meeting, it was agreed that UE reports GNSS position fix time duration for measurement during the initial access stage in some Msg5 messages. But it is FFS whether the messages RRCreestablishmentComplete, RRCConnectionReconfigurationComplete can be used for the information report.

Agreements: 1. For UE to report GNSS position fix time duration for measurement during the initial access, at least the following Msg5 message can be used: - RRCConnectionSetupComplete, RRCConnectionSetupComplete-NB, - RRCConnectionResumeComplete, RRCConnectionResumeComplete-NB, - FFS for RRCreestablishmentComplete and RRCConnectionReconfigurationComplete.

On the FFS for RRCreestablishmentComplete message, it is true that the target eNB may retrieve the UE context from the source eNB after an RRC reestablishment. However, as the GNSS position fix time duration maybe dynamically changed during the long connection, it is not reasonable to add a dynamic changed parameter as part of UE context at least for RRC reestablishment scenario. Furthermore, in Rel-17, GNSS validity duration was agreed to be reported in  RRCReestablishmentComplete(-NB) and RRCConnectionReconfigurationComplete messages. We don’t see the motivation to differentiate the behaviour for GNSS position fix duration hence introduce more complexity. To keep the solution simple, we prefer to have a unified solution to report the GNSS position fix time duration in RRC response messages in both RRC reestablishment and handover cases.

Proposal 3: UE can report GNSS position fix time duration for measurement in RRCreestablishmentComplete and RRCConnectionReconfigurationComplete messages.

After GNSS re-acquisition for UE in RRC Connected state, UE may need to report GNSS validation duration to eNB as agreed by RAN1. However, there are several FFS identified on how UE should report the GNSS validity duration to eNB from RAN2 point of view.

RAN2-121 Agreements: 1. UE reports GNSS validity duration after GNSS measurement. FFS whether the UE reports every time or only if the validity duration changes. FFS if the duration is the remaining validity duration or the whole duration RAN2-121bis Working Assumption: 1. GNSS validity duration UE reported after GNSS measurement is the remaining validity duration

To ensure timely reporting of the GNSS validity duration it is necessary to define triggers for the UE. A simple approach is to define the UE reports the GNSS validity duration after every GNSS measurement or alternatively only if the current GNSS validity duration is different from the previously reported value. In principle, it is not needed for the UE to report the new remaining GNSS validity duration if it does not change.

Proposal 4: When UE is in RRC Connected mode, the UE should report GNSS validity duration to eNB only if the current remaining GNSS validity duration and the previously reported value is different.

After every GNSS re-acquisition, it is important for the eNB to know whether the UE made a successful GNSS measurement to ensure the common understanding between the UE and the eNB about the GNSS-related status of the UE. Otherwise, the UE may lose UL synchronization while eNB is not aware of that, which may cause unexpected UL/DL grant as well as UL transmission/HARQ feedback failure. Therefore, after GNSS re-acquisition, even the new duration is same as the previously reported duration it needs to further consider whether the UE can indicate “successful GNSS measurement” without providing the GNSS validity duration.

Proposal 5: After GNSS re-acquisition, the UE should indicate success of the GNSS measurement by an explicit indication if there is no need to provide the GNSS validity duration to eNB.

After the successful GNSS measurement gap, an additional important aspect is how the UE performs uplink transmission since the UE will apply the new GNSS information instead of the previous one for TA pre-compensation. One option, with high signalling overhead, is to require the UE always performs the Random Access procedure to obtain a new Timing Advance N_TA. Alternatively, RAN2 can discuss whether the UE can reuse or reset the previous N_TA such that the UE can skip the Random Access procedure and instead transmit a Scheduling Request or directly transmit in a preconfigured uplink resource.

Proposal 6: RAN2 to discuss whether the UE after a successful GNSS measurement gap either performs the Random Access procedure or reuses/resets the previous N_TA.

## 2.2	GNSS measurement gap configuration

At RAN1 #111 meeting, RAN1 agreed UE can re-acquire GNSS position fix with a gap and the GNSS measurement gap can be aperodically triggered by eNB via MAC CE.

RAN1#111 Agreement For GNSS measurement in RRC connected, if eNB aperiodically triggers connected UE to make GNSS measurement, UE can re-acquire GNSS position fix with a gap - FFS details of gap configuration RAN1#112bis Agreement For the GNSS measurement gap aperiodically triggered with MAC CE, the duration for the GNSS measurement gap can be configured by eNB. - The gap duration is equal to the latest reported GNSS position fix time duration for measurement when the duration for GNSS measurement gap is not included in the configuration by eNB.

Similar to legacy Measurement Gap configuration, the UE and eNB should have a common understanding of when the UE is unavailable for communication due to the UE performing a GNSS measurements. This is of key importance to ensure the UE is not scheduled, when it is unavailable. For this aperiodic gap, the gap configurations shall include at least the start time of the gap and the gap duration.

On the gap duration, it is configured by eNB and it is eNB implementation to decide the duration length. We assume the eNB should determine the gap length based on the UE reported GNSS position fix time duration and eNB should guarantee the gap length is long enough to accommodate the UE required GNSS position fix time duration.

Observation 4: The length of GNSS measurement gap depends on the UE required GNSS position fix time duration.

Regarding when the gap should be configured, which is to be configured by eNB, there was no conclusion reached in both RAN1 and RAN2. At RAN2-112bis, it was agreed UE can stay in RRC_CONNECTED state when current GNSS position becomes out-of-date if the UE enters a GNSS measurement gap and it is FFS when the GNSS measurement (gap) should be started.

Agreement: 1. UE can stay in RRC_CONNECTED state when current GNSS position becomes out-of-date if the UE enters a GNSS measurement gap. FFS whether the new GNSS measurement shall be started before, upon or after the current GNSS validity duration expiry

The GNSS measurement gap can be started before, upon or after the current GNSS validity duration expiry. If the GNSS measurement (gap) starts after the timer expiry, there will have a time gap between the timer expiry and the start of GNSS measurement gap. During this time gap, the UL sync is lost hence UL transmission should not be performed. Therefore, the UE behaviour during the time gap should be specified/studied which will complex the specification. In our understanding, the new GNSS measurement gap configured after the current GNSS validity duration expiry is not necessary.

Proposal 7: For the GNSS measurement gap aperiodically triggered by eNB, the start time for the GNSS measurement gap should be configured before or upon the current GNSS validity duration expiry.

On the signalling for aperiodic gap configuration, RAN2 agreed “For the NB-IoT CP solution, UE will report the GNSS validity duration by using a MAC CE”. To have a unified solution for both eMTC and NB-IoT, a common way-forward seems to be eNB configure multiple gap configuration via RRC messages, which could in principle provide multiple configurations for the UE with index for different configurations (e.g., for the various gap length). Based on the UE reported GNSS position fix time duration, the eNB can indicate the index of the gap configuration to activate the gap via MAC CE.

Proposal 8: Support eNB to configure multiple GNSS measurement gap configurations in RRC message, hence eNB can activate the gap configuration via MAC CE based on the UE reported GNSS position fix time duration.

At RAN1 #111, RAN1 agreed the GNSS measurement in the other way, e.g., the GNSS autonomous re-acquisition if the UE does not receive the eNB triggered GNSS measurement gap. Like the aperodically MG triggered by eNB, the open issues on the timing for the autonomous measurement as well as the gap length need further discussions.

Agreement The UE may re-acquire GNSS autonomously (when configured by the network) if UE does not receive eNB trigger to make GNSS measurement - FFS based on configured timing

First, the UE autonomous GNSS measurement shall rely on the GNSS measurement gap configuration in terms of the gap duration, such that the eNB knows how long the UE is not available. Second, the eNB and UE must have a common understanding of when the UE starts the autonomous measurement. For example, the measurement may impact the eNB expectation on when the UE is to monitor the PDCCH. It seems logical to define the GNSS measurement to start around the time where the remaining GNSS validity duration expires, because the UE will not be uplink synchronized after this expiry. However, it is RAN1 who should decide the exact timing definition of when the UE autonomously can start the measurement and also whether there are any conditions, which would allow the UE to shift such measurement in the time domain.

Observation 5: UE and eNB must have a common understanding of when the UE is performing the autonomous GNSS measurement.

Proposal 9: Postpone the discussion in RAN2 on the configuration for autonomously GNSS measurement.

## 2.3	GNSS measurements in C-DRX inactive time

RAN1 has discussed GNSS position fix during inactive state of Connected DRX for improved GNSS operations and agreed the following conclusion as indicated in the LS [R1-2304126].

Conclusion From RAN1 perspective, UE is not forbidden to autonomously re-acquire GNSS position fix during inactive state of Connected DRX. - Note: The configured DL/UL transmissions during inactive state of Connected DRX should not be impacted - Note: details are up to RAN2 Send an LS to RAN2 for the conclusion.

RAN1 concluded UE is not forbidden to autonomously re-acquire GNSS position fix in the C-DRX inactive time, while the details are up to RAN2 on the feasibility and impact. First of all, there is no guarantee that the “next” inactive state of C-DRX will happen, because the Active Time may be extended if there is new data incoming or retransmissions occur. Even if the inactive state were to happen it may be shorter than expected due to the extension of the Active Time.

Observation 6: Inactive state of C-DRX may not occur at all or be shorter than expected due to extension of the active state because of new data and retransmissions.

For UE in DRX inactive Time, the UE is still required to monitor PDCCH for P-RNTI and SI-RNTI. For example, the NTN UE may have to acquire serving cell satellite assistance information (SIB31) during the inactive state of C-DRX. This will be necessary if the validity timer T317 for SIB31 is about to expire/has expired. Therefore, the UE will not be able to perform the GNSS measurement during the part of the inactive state, which collides with the reception of the SIB31. The UE may also need to monitor PDCCH for Paging message during the inactive state of C-DRX. In addition, the UE may be configured with measurement gaps, e.g. for inter-frequency and inter-RAT measurements. Such gaps are configured independently of the C-DRX and therefore the gap and the corresponding RRM measurement may take place during the inactive state of C-DRX. This would mean the UE cannot perform the GNSS measurement in order to perform the configured RRM measurement.

Observation 7: UE may need to perform multiple different events (e.g., SIB reading, Paging monitoring, RRM measurement etc.) during the inactive state of Connected DRX and is therefore not able to perform the GNSS measurement.

During the C-DRX inactive time, if UE is not required to monitor the multiple different events, it is beneficial for UE to re-acquire GNSS position fix to avoid cellular communication interruption during the connection. The GNSS position fix time duration is expected to be in the order of 1-2 seconds, and the inactive state would need to be at least this long to accommodate the GNSS reacquisition. However, due to the dynamically changed C-DRX inactive time and possible events on SIB reading/paging monitoring, the C-DRX inactive time which can be used for GNSS measurement may be shorter or longer than the GNSS position fix time duration.

Observation 8: The C-DRX inactive time which can be used for GNSS measurement may be shorter or longer than the required GNSS position fix time duration.

If the C-DRX inactive time which can be used for GNSS measurement is long enough to accommodate the GNSS reacquisition, the UE may complete the GNSS measurement within the inactive time. To support this operation, firstly, the UE needs to decide when the GNSS measurement should be started within the C-DRX inactive time. Since the eNB does not know whether UE will perform some events such as SIB reading, the simplest option is that it is up to UE implementation to decide when to start the GNSS measurement within the inactive time. Secondly, if the UE measured the GNSS successfully, the UE should inform the eNB to skip the next configured GNSS measurement gap  to avoid the unnecessary cellular service interruption. On when the notification should be sent to eNB, to save UE’s power consumption, it seems not needed to send the notification right after the GNSS measurement. Instead, the UE may send the notification to eNB at the start of the next C-DRX active time or right before the current validity timer expiry.

Proposal 10a: If the C-DRX inactive time which can be used for GNSS measurement is longer than the required GNSS position fix time duration, it is up to UE implementation to decide whether and when to initiate GNSS measurement within the inactive time.

Proposal 10b: The UE should notify eNB for the successful autonomous GNSS measurement in the C-DRX inactive time. RAN2 to discuss when the notification should be sent.

Proposal 10c: After successful autonomous GNSS measurement in the C-DRX inactive time, the next configured GNSS measurement gap can be skipped.

If the C-DRX inactive time which can be used for GNSS measurement is shorter than the required GNSS position fix time duration, the UE needs to perform the GNSS measurement with a GNSS measurement gap since at least part of the measurement will overlap with the C-DRX active time. As a baseline, the UE should follow the GNSS measurement gap configured by eNB. In some cases, if the UE finds the C-DRX inactive time contains part of the GNSS measurement gap, the UE may decide whether to initiate the GNSS measurement gap in the inactive time before the scheduled GNSS MG in order to utilize the inactive time as much as possible and thus reduce the gap overlap with C-DRX active time.

Proposal 11: If the C-DRX inactive time which can be used for GNSS measurement is less than the required GNSS position fix time duration, UE can initiate GNSS measurement gap in the inactive time before the scheduled GNSS measurement gap if it detects the gap partly overlapped with C-DRX inactive time.

## 2.4	GNSS measurements in configured PDCCH search space gap (natural gap)

In NB-IoT, connected mode measurements was introduced in Rel-17. There is no explicit scheduling gap created for this purpose. UE is expected to make use of gaps already configured for PDCCH search-space and C-DRX offtime for this purpose.

Details on the search space configuration (for NB-IoT) are given below. Note how the start offset (alpha_offset * R_max * G) in 36.213 defines the offset between PDDCH monitoring indications can become very long depending on the configuration of npdcch-NumRepetitions, npdcch-StartSF-USS and npdcch-Offset-USS in 36.331.

36.331: NPDCCH-ConfigDedicated-NB information element -- ASN1START NPDCCH-ConfigDedicated-NB-r13 ::= SEQUENCE { npdcch-NumRepetitions-r13 ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048, spare4, spare3, spare2, spare1}, npdcch-StartSF-USS-r13 ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64}, npdcch-Offset-USS-r13 ENUMERATED {zero, oneEighth, oneFourth, threeEighth} } NPDCCH-ConfigDedicated-NB-v1530 ::= SEQUENCE { npdcch-StartSF-USS-v1530 ENUMERATED {v96, v128} } -- ASN1STOP NPDCCH-ConfigDedicated-NB field descriptions. npdcch-NumRepetitions Maximum number of repetitions for NPDCCH UE specific search space (USS), see TS 36.213 [23], clause 16.6. UE monitors one set of values (consisting of aggregation level, number of repetitions and number of blind decodes) according to the configured maximum number of repetitions.. npdcch-Offset-USS Fractional period offset of starting subframe for NPDCCH UE specific search space (USS), see TS 36.213 [23], clause 16.6.. npdcch-StartSF-USS Starting subframe configuration for an NPDCCH UE-specific search space, see TS 36.213 [23], clause 16.6. Value v1dot5 corresponds to 1.5, value 2 corresponds to 2 and so on. E-UTRAN may configure values v1dot5 and v2 only in FDD mode and values v96 and v128 only in TDD mode. The UE shall use the value signalled in npdcch-StartSF-USS-v1530, if present, and ignore the value signalled in npdcch-StartSF-USS-r13.. 36.213: ## 16.6 Narrowband physical downlink control channel related procedures The locations of starting subframe are given by where is the th consecutive NB-IoT DL subframe from subframe , excluding subframes used for transmission of SI messages, and , and , and where - subframe is a subframe satisfying the condition , where , T≥4. - for NPDCCH UE-specific search space, - is given by the higher layer parameter npdcch-StartSF-USS, except for NPDCCH candidates associated with PUR-RNTI in which case it is given by higher layer parameter npdcch-StartSF-USS in PUR-Config-NB, - is given by the higher layer parameter npdcch-Offset-USS, except for NPDCCH candidates associated with PUR-RNTI in which case it is given by higher layer parameter npdcch-Offset-USS in PUR-Config-NB,

The configured gap between PDCCH search space can be illustrated in below figure.

Figure 1: Configured Gap between PDCCH Search Space

Similar to C-DRX inactive time, the UE is not required to monitor PDCCH during the configured gap between PDCCH search space. Therefore, the configured search space gap (natural gap) can be used for GNSS measurement.

Proposal 12: Similar to C-DRX inactive time, UE can use configured gap between PDCCH search space for GNSS measurement to avoid cellular communication interruption during the connection.

Depending on the parameters configured for PDCCH search space and whether the UE is scheduled in the previous PDCCH search space, the natural gap may be shorter or longer than the required GNSS position fix time duration. Hence, as a baseline, the solutions agreed for C-DRX inactive time can be used for natural gap as well.

Proposal 13: As a baseline, the solutions agreed for GNSS measurement in C-DRX inactive time can be used in configured gap between PDCCH search space.

## 2.5	Conflict between RLF and GNSS measurement

When the UE is experiencing radio link problem, UE should trigger the RLF procedure in which UE will evaluate the number of  “in-sync” and “out-of-sync” indications to decide whether UE should declare radio link failure. However, if the eNB configure a measurement gap for GNSS measurement with the gap length up to 1 or 2 seconds, the UE will tune to GNSS measurement during the gap which may interrupt the ongoing RLF procedure. In this case, the UE may have no chance to evaluate “in-sync” and “out-of-sync” for RLF hence declare a fake/false radio problem if T310 expired during the gap. Therefore, RAN2 agreed that RLM should be suspended during the GNSS measurement gap.

Agreement: RLM is suspended during the GNSS measurement gap while the UE is measuring GNSS.

There are two options to implement the RLM suspension during the GNSS measurement gap. The first option is that the UE pauses the T310 when the GNSS measurement gap starts and continue with the remainder of the T310 when the GNSS measurement is finished. The second option is that the UE extends the T310 when the GNSS measurement gap starts such that the T310 does not expire during the GNSS measurement gap. The extension value can be the duration of the GNSS measurement gap. In our understanding, both options are workable. RAN2 can select either option.

Proposal 14: To suspend the RLM during the GNSS measurement gap, RAN2 to select one of below options:

- Option1: UE pauses the T310 when the GNSS measurement gap starts and continue with the remainder of the T310 when the GNSS measurement is finished.

- Option2: UE extends the T310 value with the duration of the GNSS measurement gap.

# 3	Conclusion

This document has made the following observations:

Observation 1: According to WID description, UE movement is expected to be supported for IoT NTN.

Observation 2: For UE in RRC Connected mode, the UE may move and the propagation conditions may change and therefore cause a change in the required GNSS position fix time duration.

Observation 3: If the latest GNSS position fix time cannot be reported to eNB, the RRC connected UE may go to RRC idle state incorrectly because the eNB and UE assume an incorrect GNSS measurement gap duration.

Observation 4: The length of GNSS measurement gap depends on the UE required GNSS position fix time duration.

Observation 5: UE and eNB must have a common understanding of when the UE is performing the autonomous GNSS measurement.

Observation 6: Inactive state of C-DRX may not occur at all or be shorter than expected due to extension of the active state because of new data and retransmissions.

Observation 7: UE may need to perform multiple different events (e.g., SIB reading, Paging monitoring, RRM measurement etc.) during the inactive state of Connected DRX and is therefore not able to perform the GNSS measurement.

Observation 8: The C-DRX inactive time which can be used for GNSS measurement may be shorter or longer than the required GNSS position fix time duration.

And proposed the following:

Proposal 1: RAN2 assumes the UE should report GNSS position fix time duration in RRC connected state if the value changes. This can be revised based on RAN1 and RAN4 further input if necessary.

Proposal 2: Send LS to RAN1 and RAN4 to confirm the working assumption in Proposal1.

Proposal 3: UE can report GNSS position fix time duration for measurement in RRCreestablishmentComplete and RRCConnectionReconfigurationComplete messages.

Proposal 4: When UE is in RRC Connected mode, the UE should report GNSS validity duration to eNB only if the current remaining GNSS validity duration and the previously reported value is different.

Proposal 5: After GNSS re-acquisition, the UE should indicate success of the GNSS measurement by an explicit indication if there is no need to provide the GNSS validity duration to eNB.

Proposal 6: RAN2 to discuss whether the UE after a successful GNSS measurement gap either performs the Random Access procedure or reuses/resets the previous N_TA.

Proposal 7: For the GNSS measurement gap aperiodically triggered by eNB, the start time for the GNSS measurement gap should be configured before or upon the current GNSS validity duration expiry.

Proposal 8: Support eNB to configure multiple GNSS measurement gap configurations in RRC message, hence eNB can activate the gap configuration via MAC CE based on the UE reported GNSS position fix time duration.

Proposal 9: Postpone the discussion in RAN2 on the configuration for autonomously GNSS measurement.

Proposal 10a: If the C-DRX inactive time which can be used for GNSS measurement is longer than the required GNSS position fix time duration, it is up to UE implementation to decide whether and when to initiate GNSS measurement within the inactive time.

Proposal 10b: The UE should notify eNB for the successful autonomous GNSS measurement in the C-DRX inactive time. RAN2 to discuss when the notification should be sent.

Proposal 10c: After successful autonomous GNSS measurement in the C-DRX inactive time, the next configured GNSS measurement gap can be skipped.

Proposal 11: If the C-DRX inactive time which can be used for GNSS measurement is less than the required GNSS position fix time duration, UE can initiate GNSS measurement gap in the inactive time before the scheduled GNSS measurement gap if it detects the gap partly overlapped with C-DRX inactive time.

Proposal 12: Similar to C-DRX inactive time, UE can use configured gap between PDCCH search space for GNSS measurement to avoid cellular communication interruption during the connection.

Proposal 13: As a baseline, the solutions agreed for GNSS measurement in C-DRX inactive time can be used in configured gap between PDCCH search space.

Proposal 14: To suspend the RLM during the GNSS measurement gap, RAN2 to select one of below options:

- Option1: UE pauses the T310 when the GNSS measurement gap starts and continue with the remainder of the T310 when the GNSS measurement is finished.

- Option2: UE extends the T310 value with the duration of the GNSS measurement gap.