3GPP TSG-RAN WG2 Meeting #122	R2-2305761

Incheon, Korea, May 22-26, 2023

Agenda item:		7.9.3

Source:		Lenovo

Title:		Lossless Inter-gNB path switching

Document for:		Discussion and Decision

- Introduction

RAN2 achieved the following agreement for service continuity in last meeting [1].

Agreements:

For uplink lossless data delivery for path switch, continue considering solutions U3 and U5 from R2-2304305.  Other solutions are not pursued.

For downlink lossless data delivery for path switch, Solution-D4 is taken as the baseline solution and keep Solution-D3/D5 on the table for further decision at the next meeting.

In this contribution, we would like to discuss the U5 for lossless path switching in L2 U2N relay case.

- Discussion

The following description for Solution-U5 is captured in [2].

The data loss could happen in the case that the Remote UE’s Uu configuration is released before the UL data are totally transmitted from the Relay UE to the source gNB. One possible way to address this issue is to keep source Relay UE’s Uu configuration for the Remote UE and allow the source Relay UE to continue to transmit the Remote UE’s UL packets. And the source gNB should forward received UL packets to the target gNB. It can leave source gNB implementation (e.g. setting a longer release timer or does not release Remote UE Uu context in the Relay UE, etc) or target gNB implementation (the target gNB will know the UL packets are totally received and request to release the Remote UE context on source part and UL forwarding tunnel.

Evaluation

This solution attempts to reuse the existing Uu procedure but requires that the source gNB still keeps the Remote UE/Relay UE context even after the Remote UE’s handover. Meanwhile, it is unclear how long the target gNB should wait for such data forwarding.

Based on the above description, when the remote UE receives path switching command, some UL data will be pending at the relay UE side. In this situation, it is possible that some PDCP discarded by the remote UE but source gNB will not transfer it to target gNB since source gNB does not receive it at all. For the remote UE’s path switching, source gNB will reconfigure the relay UE to release PC5 link. To avoid data loss, the source gNB can configure the relay UE to continue transferring the buffered data. Once relay UE transfers all the buffered data to source gNB, relay UE can indicate it to the serving gNB and release the configuration related to that remote UE.

Observation 1: The concern for U5 is that it is unclear how long the target gNB should wait for such data forwarding.

Proposal 1: Regarding U5, gNB can configure the relay UE to release the configuration related to that remote UE until receiving all the UL data buffered in the relay UE.

Proposal 2: Once relay UE transfers all the buffered data to source gNB, relay UE can indicate the end of transferring to the serving gNB and release the configuration related to the remote UE.

- Conclusion

In this contribution, we have the following proposals:

Observation 1: The concern for U5 is that it is unclear how long the target gNB should wait for such data forwarding.

Proposal 1: Regarding U5, gNB can configure the relay UE to release the configuration related to that remote UE until receiving all the UL data buffered in the relay UE.

Proposal 2: Once relay UE transfers all the buffered data to source gNB, relay UE can indicate the end of transferring to the serving gNB and release the configuration related to the remote UE.

References

Chairman notes for RAN2#121bis meeting.

R2-2304305