3GPP TSG-RAN WG2 Meeting #122                                                        R2-2305764

Incheon, Korea, 22 May – 26 May, 2023

Source:	Qualcomm Incorporated, OPPO, Xiaomi

Title:	Evaluation and proposals on U3 and U5

Agenda Item:	7.9 Enhanced NR Sidelink Relay

Document for:	Discussion and Decision

# Introduction & Background

In last RAN2 meeting, potential solutions to support lossless delivery were discussed, and for UL, solution U3 and U5 are captured on the table for further discussion.

Agreements:

For uplink lossless data delivery for path switch, continue considering solutions U3 and U5 from R2-2304305.  Other solutions are not pursued.

For downlink lossless data delivery for path switch, Solution-D4 is taken as the baseline solution and keep Solution-D3/D5 on the table for further decision at the next meeting.

This contribution evaluates U3 and U5, and provides proposals.

# Discussion

## 2.1 General solution description

In solution U3, UE PDCP layer retransmits PDCP packets based on the PDCP status report instead of lower layer acknowledgement. The UE needs to buffer all the PDCP packets regardless they are acknowledged by lower layer or not and determines whether to retransmit the PDCP packets based on received PDCP status report.

In solution U5, after path switching the source Relay UE will continue to transmit UL packets and the source gNB forwards the UL packets to target gNB.

## 2.2 Evaluation on solution U3

Existing PDCP status report is used for PDCP packet discard, i.e. used to discard those PDCP packets which are received successful in the gNB but the ACK are lost during re-establishment or HO. Based on the PDCP status report, UE will not retransmit the those PDCP packets and discard them.

Currently, PDCP layer retransmission only happens during e.g.re-establishment, data recovery. PDCP layer retransmission is based lower layer acknowledgement, once one PDCP packet is acknowledged by lower layer, then PDCP layer can discard the packet because it is already transmitted successfully and is useless to buffer this PDCP packet. For those PDCP packets not acknowledged by lower layer, PDCP layer will retransmit them when re-establishment or data recovery happens. That means currently UE is only required to buffer those PDCP packets not acknowledged by lower layer.

Observation 1: Currently, PDCP status report is used for discarding those PDCP packets which are received successfully in the gNB but the RLC ACK are lost during RRC re-establishment or HO.

As U3 solution description, in order to achieve retransmission based on PDCP status report, UE is required to buffer all the transmitted PDCP packets (regardless of they are transmitted successfully or not) till receiving PDCP status report. But PDCP status report only happens in some special scenarios, e.g. re-establishment, data recovery. The buffered transmitted PDCP packets will occupy the PDCP layer memory and there is no or less memory for newly incoming packets buffering and processing. UE may delay UL transmission for the new packets or even discard the new packets when buffer is full. This will impact UL packet transmission latency, packets loss rate and UL throughput. This system performance impact will definitely happen for normal UL transmission, while path switching may never happen.

Some companies commented in the last meeting the discard timer can be used to discard the buffer packets. However, the discard timer should be set to large value or infinity to avoid discard gNB unacknowledged data. If the discard timer is set to “infinity” (it is observed this is typical setting in practical deployment), it is not implementable for UE to buffer all the transmitted packets forever as proposed by U3. If the discard timer is set to a value other than “infinity” e.g. 2s, it will introduce data loss for all the PDCP packets including new packets and transmitted packets when the discard timer expires. And also the buffered transmitted PDCP packets will occupy PDCP memory quickly. Taking the example of UL 20Mbps throughput, every 2s period, the additional buffered data is 40Mbps; and if taking 5G UL capability of 100Mbps, every 2s period, the additional buffered data is 200Mbps. However, most of the packets will not be retransmitted and the buffering will be wasted.

Observation 2: In U3, UE is required to buffer all the transmitted PDCP packets (regardless of they are transmitted successfully or not) till receiving PDCP status report or discard timer expires. The buffered transmitted PDCP packets will occupy the PDCP layer memory quickly and there is no or less memory for newly incoming packets buffering and processing.

Observation 3: The inappropriate discard timer setting will lead to data loss for new packets and the already transmitted packets.

Another issue in U3 is, BSR calculation will be impacted. Currently, BSR calculation takes the all the packets buffered in PDCP layer into account. But in U3, it needs to figure out which data to be considered when calculating BSR. So BSR calculation has to be changed.

Summary of U3: Solution U3 has the following disadvantages and is not an implementable solution.

- Has system performance impact on latency, packet loss rate, throughput

- QoS requirements will not be guaranteed due to inappropriate discard timer setting and less PDCP memory space used for new packets processing.

- UE has to buffer more data even if the data has been received by NW.

- BSR calculation has to be changed.

## 2.3 Evaluation on solution U5

Solution U5 reuses the existing mechanism defined for intra-gNB path switching and UL data forwarding between the source and target gNB. UL data forwarding is existing typical mechanism to address UL data loss.

In the last RAN2 meeting, companies raised some comments on solution U5, this part explains and addresses the comments

Companies comment 1: U5 cannot address Relay UE Uu failure case

Relay UE Uu failure is not inter-gNB specific issue and can happen in intra-gNB path switch case and non-path switch case. Then the optimization for Uu failure case is out of Rel-18 scope.

Remote UE path switching is triggered by the SL quality deterioration instead of Relay Uu coverage deterioration. If Relay Uu link is failure, then RRC re-establishment should be triggered. If path switching instead of RRC re-establishment is triggered, Relay UE Uu coverage should be good. Then it is very rare case then Relay UE Uu failure in case of path switching.

Observation 4: Relay UE Uu failure during path switching is rare case.

Observation 5: Relay UE Uu failure is not inter-gNB specific issue and can happen in intra-gNB path switch case and non-path switch case. Then the optimization for Uu failure case is out of Rel-18 scope.

Companies comment 2: Source gNB should keep the Remote UE/Relay UE context even after the Remote UE’s handover

This is existing gNB behavior used for intra-gNB path switching in which the gNB should keep Remote UE context in Relay UE for a while to let the Relay UE continue to transmit UL data. Also, gNB may keep the remote UE context to prepare potential reestablishment in case of path switch/handover failure. There are no new things to be introduced for inter-gNB case.

Observation 6: It is existing gNB behavior used for intra-gNB path switching in which the gNB should keep Remote UE context in Relay UE for a while to let the Relay UE continue to transmit UL data. Also, gNB may keep the remote UE context to prepare potential reestablishment in existing path switch/handover failure.

Companies comment 3: It is unclear how long the target gNB should wait for such data forwarding

In existing HO procedure, it is not specified when UL data forwarding tunnel should be released, normally, it is left to gNB implementation. In our target scenario, it is can also based on target gNB implementation to release the UL forwarding tunnel, e.g., the target gNB can set a tunnel release timer by implementation (e.g., normally 5s is enough due to latency requirements) to release UL data forwarding tunnel; or, when all the missed PDCP SDUs are received, the target gNB can release the UL data forwarding tunnel.

Observation 7: Currently, it is not specified when UL data forwarding tunnel should be released, then it should be possible to release the UL data forwarding tunnel by target gNB implementation e.g. setting a release timer by implementation.

Companies comment 4: U5 is not PDCP SR based solution

U5 still requires the target gNB send PDCP SR to the UE after path switching, and the UE will discard the PDCP SDU already received successfully as today. So solution U5 is PDCP SR based solution.

Summary of U5: The gap between intra-gNB and inter-gNB path switching is UL data forwarding. Solution U5 reuses existing mechanism of intra-gNB path switching and UL data forwarding, can achieve UL lossless without specification impact.

## 2.4 Proposals

Based on the discussion in previous clauses, solution U5 is easy solution without much specification impact, while solution U3 has large impact on UE side and system performance. It is proposed to adopt solution U5 to address UL data loss.

Proposal 1: Solution U3 is not pursued.

Proposal 2: Solution U5 is adopted to address UL data loss.

# Conclusion

This contribution evaluates solution U3 and U5 for lossless delivery during path switching.

Evaluation on solution U3,

Observation 1: Currently, PDCP status report is used for discarding those PDCP packets which are received successfully in the gNB but the RLC ACK are lost during RRC re-establishment or HO.

Observation 2: In U3, UE is required to buffer all the transmitted PDCP packets (regardless of they are transmitted successfully or not) till receiving PDCP status report or discard timer expires. The buffered transmitted PDCP packets will occupy the PDCP layer memory quickly and there is no or less memory for newly incoming packets buffering and processing.

Observation 3: The inappropriate discard timer setting will lead to data loss for new packets and the already transmitted packets.

Summary of U3: Solution U3 has the following disadvantages and is not an implementable solution.

- Has system performance impact on latency, packet loss rate, throughput

- QoS requirements will not be guaranteed due to inappropriate discard timer setting and less PDCP memory space used for new packets processing.

- UE has to buffer more data even if the data has been received by NW.

- BSR calculation has to be changed.

Evaluation on solution U5,

Observation 4: Relay UE Uu failure during path switching is rare case

Observation 5: Relay UE Uu failure is not inter-gNB specific issue and can happen in intra-gNB path switch case and non-path switch case. Then the optimization for Uu failure case is out of Rel-18 scope.

Observation 6: It is existing gNB behavior used for intra-gNB path switching in which the gNB should keep Remote UE context in Relay UE for a while to let the Relay UE continue to transmit UL data. Also, gNB may keep the remote UE context to prepare potential reestablishment in existing path switch/handover failure.

Observation 7: Currently, it is not specified when UL data forwarding tunnel should be released, then it should be possible to release the UL data forwarding tunnel by target gNB implementation e.g. setting a release timer by implementation.

Summary of U5: The gap between intra-gNB and inter-gNB path switching is UL data forwarding. Solution U5 reuses existing mechanism of intra-gNB path switching and UL data forwarding, can achieve UL lossless without specification impact.

Proposals,

Proposal 1: Solution U3 is not pursued.

Proposal 2: Solution U5 is adopted to address UL data loss.

# Reference

[1]    RP-221262: New WID on NR sidelink relay enhancements

[2]    TS 38.300 V17.1.0, NR; NR and NG-RAN Overall Description

[3]    R2-2304305  Summary of [AT121bis-e][432]Candidate solutions for lossless delivery	NEC	discussion