3GPP TSG-RAN WG2 Meeting #122                                                     R2-2305765

Incheon, Korea, May 22 – May 26, 2023

Source:	Qualcomm Incorporated

Title:	Address controversial issues on multi-path relay

Agenda Item:	7.9 Enhanced NR Sidelink Relay

Document for:	Discussion and Decision

# Introduction & Background

According to WID [1] and RAN2 agenda for REl-18 Relay enhancement, and this contribution discusses the open issues for scenario 1 and scenario 2.

- Study the benefit and potential solutions for multi-path support to enhance reliability and throughput (e.g., by switching among or utilizing the multiple paths simultaneously) in the following scenarios [RAN2, RAN3]:

  - A UE is connected to the same gNB using one direct path and one indirect path via 1) Layer-2 UE-to-Network relay, or 2) via another UE (where the UE-UE inter-connection is assumed to be ideal), where the solutions for 1) are to be reused for 2) without precluding the possibility of excluding a part of the solutions which is unnecessary for the operation for 2).

Note 3A: Study on the benefit and potential solutions are to be completed in RAN#98 which will decide whether/how to start the normative work.

Note 3B: UE-to-Network relay in scenario 1 reuses the Rel-17 solution as the baseline.

Note 3C: Support of Layer-3 UE-to-Network relay in multi-path scenario is assumed to have no RAN impact and the work and solutions are subject to SA2 to progress.

# Discussion

## 2.1  SRB1 and SRB2 path configuration

It was agreed that PCell is only on direct path and following existing DCCA mechanism, the SRB1 and SRB2 are configured in the same path as PCell. it should be taken as baseline; otherwise, RAN2 needs to spend time to review whether all the existing procedures can work well.

Uu coverage is more larger than PC5 coverage, it is more possible that UE moves outside or inside of PC5 coverage frequently, and then indirect path may lost coverage more frequently. There are two hops on indirect path, any hop failure will lead to indirect path failure. So direct path is more reliable than indirect path.

Observation 1: Direct path is more reliable than indirect path, due to 1) Uu coverage is much larger than SL coverage, 2) any hop failure on the indirect path will lead to indirect path failure.

Observation 2: UE implementation complexity will be introduced if allowing various configurations in specification.

Observation 3: Both SRB1 and SRB2 are configured on the path of PCell currently, RAN2 needs to spend time to review whether existing procedures will be broken if deviating from existing design principle.

Then it is proposed to reuse the existing mechanism.

Proposal 1: SRB1 and SRB2 are configured on direct path.

## 2.2  Path failure reporting

In the last RAN2 meeting, RAN2 is requested to discuss whether the RRC message is the existing message e.g. MCGFailureInformation or a new message to be used for direct path failure report. Since MCG is always configured in direct path, then when the direct path fails, existing MCGFailureInformation message could be reused.

Proposal 2: In case of Uu-RLF, the existing message i.e MCGFailureInformation is used to report direct path failure over indirect path.

Similarly, when the indirect path failure, including PC5-RLF and Relay Uu failure, Remote UE should report indirect failure indication to the gNB over direct path. One possible way is to reuse existing SCGFailureInformation message with adding indirect path failure as a new failure type

Proposal 3: In case indirect path failure, including PC5-RLF or Relay UE Uu failure, the existing message i.e. SCGFailureInformation is used to report indirect path failure over direct path with adding indirect path failure as a new failure type.

2.4 Relay UE identity for Scenario 2

And it is not reasonable to request the Relay UE entering CONNECTED state before it is added to MP relay, in case that the gNB does not configure MP relay to the Remote UE. For example, when gNB is overload and will not configure MP to Remote UE after receiving report from the Remote UE. But the Remote UE will trigger the Relay UE entering CONNECTED state again and again in order to request MP relay operation. It is a huge burden for Relay UE to enter CONNECTED state every time the Remote UE reports the Relay UE ID to the gNB.

Observation 4: If Relay UE is required to be in CONNECTED state when the Remote UE reports the Relay UE ID to the gNB,  in case that gNB dos not configure MP relay due to overload, Remote UE will trigger the Relay UE entering CONNECTED state again and again in order to request MP relay operation. It is a huge burden for Relay UE to enter CONNECTED state every time the Remote UE reports the Relay UE ID to the gNB .

When Relay UE enters CONNECTED state from IDLE state, Relay UE will indicates PLMN ID (MCC, MNC), GUAMI(AMF Region ID, an AMF Set ID and an AMF Pointer), and 5G-S-TMSI as shown in following message.

RRCSetupComplete-IEs ::=            SEQUENCE {

selectedPLMN-Identity               INTEGER (1..maxPLMN),

registeredAMF                       RegisteredAMF                                   OPTIONAL,

guami-Type                          ENUMERATED {native, mapped}                     OPTIONAL,

s-NSSAI-List                        SEQUENCE (SIZE (1..maxNrofS-NSSAI)) OF S-NSSAI  OPTIONAL,

dedicatedNAS-Message                DedicatedNAS-Message,

ng-5G-S-TMSI-Value                  CHOICE {

ng-5G-S-TMSI                        NG-5G-S-TMSI,

ng-5G-S-TMSI-Part2                  BIT STRING (SIZE (9))

}                                                                                   OPTIONAL,

lateNonCriticalExtension            OCTET STRING                                    OPTIONAL,

nonCriticalExtension                RRCSetupComplete-v1610-IEs                      OPTIONAL

}

RegisteredAMF ::=                   SEQUENCE {

plmn-Identity                       PLMN-Identity                                   OPTIONAL,

amf-Identifier                      AMF-Identifier

}

–	AMF-Identifier

The IE AMF-Identifier (AMFI) comprises of an AMF Region ID, an AMF Set ID and an AMF Pointer as specified in TS 23.003 [21], clause 2.10.1.

AMF-Identifier information element

-- ASN1START

-- TAG-AMF-IDENTIFIER-START

AMF-Identifier ::=                      BIT STRING (SIZE (24))

-- TAG-AMF-IDENTIFIER-STOP

-- ASN1STOP

Currently, the 5G-GUTI and 5-S-TMSI definition is as following in TS23.003,

<5G-GUTI>:= <MCC> <MNC><AMF Region ID> <AMF Set ID> <AMF Pointer><5G-TMSI>

<5G-S-TMSI> := <AMF Set ID> <AMF Pointer> <5G-TMSI>

Then when Remote UE reports Relay UE’s 5G-GUTI, the gNB should be able to identify the Relay UE based on the information received from the Relay UE.

Observation 5: Today, gNB can obtain Relay UE 5G-GUTI during RRC setup procedure.

For Inactive state Relay UE, the I-RNTI can be used to identify the Relay UE.

Proposal 4: RAN2 confirms target Relay UE can be in IDLE and Inactive state.

Proposal 5: Using the following ID to identify the candidate Relay UE

- If the Relay UE in IDLE state, Remote UE reports candidate Relay UE’s 5G-GUTI to     identify the Relay UE;

- If the Relay UE is in Inactive state, Remote UE reports candidate Relay UE’s I-RNTI to identify the Relay UE;

- If the Relay UE is in CONNECTED state, Remote UE reports candidate Relay UE’s C-RNTI to identify the Relay UE.

2.5 Relay UE configuration for Scenario 2

In last RAN2 meeting, it was agree that gNB provides bearer mapping information to relay UE through dedicated signalling, one further issue is which message should be used. One possible way is the existing sl-L2RelayUE-Config-r17 can be reused as following. The gNB only needs to configure the yellow highlight IEs; for the unnecessary mandatory IE in green highlight, the Relay UE can ignore it.

–	SL-L2RelayUE-Config

The IE SL-L2RelayUE-Config is used to configure L2 U2N relay operation related configurations used by L2 U2N Relay UE, e.g. SRAP-Config.

SL-L2RelayUE-Config information element

-- ASN1START

-- TAG-SL-L2RELAYUE-CONFIG-START

SL-L2RelayUE-Config-r17 ::=        SEQUENCE {

sl-RemoteUE-ToAddModList-r17       SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-RemoteUE-ToAddMod-r17    OPTIONAL,    -- Need N

sl-RemoteUE-ToReleaseList-r17      SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-DestinationIdentity-r16  OPTIONAL,    -- Need N

...

}

SL-RemoteUE-ToAddMod-r17 ::=       SEQUENCE {

sl-L2IdentityRemote-r17            SL-DestinationIdentity-r16,

sl-SRAP-ConfigRelay-r17           SL-SRAP-Config-r17                                                    OPTIONAL,    -- Need M

...

}

-- TAG-SL-L2RELAYUE-CONFIG-STOP

-- ASN1STOP

–	SL-SRAP-Config

The IE SL-SRAP-Config is used to set the configurable SRAP parameters used by L2 U2N Relay UE and L2 U2N Remote UE as specified in TS 38.351 [66].

SL-SRAP-Config information element

-- ASN1START

-- TAG-SL-SRAP-CONFIG-START

SL-SRAP-Config-r17 ::=                  SEQUENCE {

sl-LocalIdentity-r17                    INTEGER (0..255)                                                 OPTIONAL, -- Need M

sl-MappingToAddModList-r17              SEQUENCE (SIZE (1..maxLC-ID)) OF SL-MappingToAddMod-r17          OPTIONAL, -- Need N

sl-MappingToReleaseList-r17             SEQUENCE (SIZE (1..maxLC-ID)) OF SL-RemoteUE-RB-Identity-r17     OPTIONAL, -- Need N

...

}

SL-MappingToAddMod-r17 ::=              SEQUENCE {

sl-RemoteUE-RB-Identity-r17             SL-RemoteUE-RB-Identity-r17,

sl-EgressRLC-ChannelUu-r17              Uu-RelayRLC-ChannelID-r17                                        OPTIONAL, -- Cond L2RelayUE

sl-EgressRLC-ChannelPC5-r17             SL-RLC-ChannelID-r17                                             OPTIONAL, -- Need N

...

}

SL-RemoteUE-RB-Identity-r17 ::=         CHOICE {

srb-Identity-r17                        INTEGER (0..3),

drb-Identity-r17                        DRB-Identity,

...

}

-- TAG-SL-SRAP-CONFIG-STOP

-- ASN1STOP

Proposal 6: Reuse existing IE sl-L2RelayUE-Config-r17 to configure the mapping of E2E bearer and Uu RLC Channel for Relay UE, with the following restrictions:

- gNB only provides the IE configuration: sl-RemoteUE-RB-Identity-r17, Uu-RelayRLC-ChannelID-r17

- Relay UE ignores the IE sl-L2IdentityRemote-r17 if received

# Conclusion

This contribution discusses the aspects of architecture, bearer management, path management for multi-path operation, and provides the proposals.

SRB1 and SRB2 path configuration,

Observation 1: Direct path is more reliable than indirect path, due to 1) Uu coverage is much larger than SL coverage, 2) any hop failure on the indirect path will lead to indirect path failure.

Observation 2: UE implementation complexity will be introduced if allowing various configurations in specification.

Observation 3: Both SRB1 and SRB2 are configured on the path of PCell currently, RAN2 needs to spend time to review whether existing procedures will be broken if deviating from existing design principle.

Proposal 1: SRB1 and SRB2 are configured on direct path.

Path failure reporting,

Proposal 2: In case of Uu-RLF, the existing message i.e MCGFailureInformation is used to report direct path failure over indirect path.

Proposal 3: In case indirect path failure, including PC5-RLF or Relay UE Uu failure, the existing message i.e. SCGFailureInformation is used to report indirect path failure over direct path with adding indirect path failure as a new failure type.

Relay UE identity for Scenario 2,

Observation 4: It is a huge burden for Relay UE to enter CONNECTED state every time the Remote UE reports the Relay UE ID to the gNB in case MP is not configured due to e.g. gNB overload.

Observation 5: Today, gNB can obtain Relay UE 5G-GUTI during RRC setup procedure.

Proposal 4: RAN2 confirms target Relay UE can be in IDLE and Inactive state.

Proposal 5: Using the following ID to identify the candidate Relay UE

- If the Relay UE in IDLE state, Remote UE reports candidate Relay UE’s 5G-GUTI to     identify the Relay UE;

- If the Relay UE is in Inactive state, Remote UE reports candidate Relay UE’s I-RNTI to identify the Relay UE;

- If the Relay UE is in CONNECTED state, Remote UE reports candidate Relay UE’s C-RNTI to identify the Relay UE.

Relay UE configuration for Scenario 2,

Proposal 6: Reuse existing IE sl-L2RelayUE-Config-r17 to configure the mapping of E2E bearer and Uu RLC Channel for Relay UE, with the following restrictions:

- gNB only provides the IE configuration: sl-RemoteUE-RB-Identity-r17, Uu-RelayRLC-ChannelID-r17

- Relay UE ignores the IE sl-L2IdentityRemote-r17 if received

# Reference

[1]    RP-221262  New WID on NR sidelink relay enhancements

[2]    R2-2211101 RAN2#121 Meeting Report	MCC