3GPP TSG RAN WG2 Meeting #122                                      	R2-2305789

Incheon, Korea, May 22-26, 2023

Agenda item:	7.16.2.4

Source:	Samsung

Title:	Indication of supported AI/ML models and functionalities

Document for:	Discussion and Decision

# 1. Introduction

In RAN2#121bis-e, RAN2 further discussed AI/ML capability and concluded that [1]:

FFS if For UE capability for AIML methods we use the UE capability mechanisms as defined for RRC reported and LPP reported capabilities.

In this contribution, we discuss aspects of model identification, functionality identification, indication of supported models and functionalities, and reporting of the UE and network AI/ML capability.

# 2. Discussion

2.1 Indication of supported AI/ML models and functionalities

In RAN1#112 meeting, RAN1 discussed the UE reporting (indication) of supported AI/ML functionalities/functionality and AI/ML models for the UE-side models and UE-part of two-sided models [2]:

Agreement For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Reuse legacy 3GPP framework of Features as a starting point for discussion. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models.

Based on RAN1 agreement, for UE-side and UE-part of two-sided models, the UE can indicate its supported models and/or functionalities using the following two options:

Option 1 (supported functionalities):

- The UE indicates to the network the functionality/functionalities (or use cases) supported by this UE. For example, a list of functionalities and information related to each functionality.

- The UE includes the list and/or information on supported functionalities (or use cases) in the AI/ML capability indication to the network.

Option 2 (supported models):

- The UE indicates to the network the models supported by (or available at) this UE. For example, a list of models supported (or available), models IDs, and information related to the models, etc.

- The UE includes the list and/or information on supported (or available) models in the AI/ML capability indication to the network.

Observation 1: For UE-side model and UE-part of two-sided models, there are two options to indicate the UE supported AI/ML models and functionalities to the network:

- Option 1: For AI/ML functionality identification, the UE includes a list of supported functionalities (or use cases) in the UE capability report to the network.

- Option 2: For AI/ML model identification, the UE may include the list of supported (or available) models in the UE capability report to the network

One question related to RAN1 agreement, is whether the UE can use both options (together)? Or need to select one option. In our understanding this could depend on the AI/ML deployment scenario. For example, assuming that the UE supports multiple AI/ML models for the same functionality (or a given use case), then the UE can use both options (together) to indicate the supported AI/ML models for that given functionality (or use case).

Observation 2: the UE may use Option 1, Option 2, or both to indicate its supported (or available) models and functionalities to the network.

Moreover, in RAN1#112bis-e, RAN1 agreed the following UE capability indication [3]:

Agreement - For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models: - Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. - … - For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models: - model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side. - …

Observation 3: both functionality and model may be associated with specific configuration(s) or conditions (e.g. scenarios, sites, etc.) that is (are) indicated by (or associated with) UE capability.

Based on the above, the AI/ML capability report of the UE and the network may include the following information:

- Generic AI/ML capability, i.e. support for AI/ML operations and/or,

- Specific AI/ML capability, i.e. support for a functionality, use case, scenario, configuration, or service, etc. For example, the UE may indicate its capability to support the use case “positioning accuracy”.

Proposal 1: RAN2 to agree that reported AI/ML capability can indicate a generic AI/ML capability support and/or specific AI/ML capability (e.g. a functionality, use-case, scenario, and configuration).

2.2 Indication of AI/ML UE Measurement Capability

In our view, the UE measurement capabilities play a significant role in the management of data in AI/ML models’ LCM procedures. The quality of radio measurements (e.g., RSRQ) the UE can provide as data to an AI/ML model can vary significantly depending on factors such as, the UE model or UE category (e.g., Commercial device, REDCAP device or IoT device) etc. The measurement quality will directly influence the subsequent performance of the AI/ML model.

Observation 4: It is necessary to define UE Measurement capability reporting between the UE and network for some AI/ML models.

For AI/ML models that depend on training data collected by UEs (or Mobile Terminals (MT) or devices), some form of calibration of the data, depending on the UE/MT/device measurement capability will be required at data collection and processing. Thus UE measurement capability reporting between the UE and the network should be considered for these AI/ML models. This reporting could be based on a coarse categorisation of UE providing the AI/ML data (1 to 10, for example) or a much finer categorisation, depending on the requirements and UE reported data dependency of the AI/ML model. For example, this could be categorisation into a few broad categories (like high, mid or low accuracy), or it could be a more accurate numerical categorization, for example for the RSRQ measurements reported by the UE. This could contain a noise or error level of the RSRQ value, which could be indicated by a % error.

Proposal 2: RAN2 to introduce UE measurement capability reporting between the UE and network.

Proposal 3: RAN2 to discuss whether UE measurement capability is reported as a coarse categorisation or a finer categorisation or have both options to be selected, as per the requirements of the AI/ML model.

# 3. Conclusion

In this contribution, we discussed the indication of supported functionalities (or use cases) and models as part of the UE and network reporting of AI/ML capability. Additionally, we presented the reporting of UE measurement capability to the network. This capability should be reported from the UE to the network, in a granularity that corresponds to the requirements of the AI/ML model (or AI/ML functionality or use case). The following are the observations and proposals in this document:

Observation 1: For UE-side model and UE-part of two-sided models, there are two options to indicate the UE supported AI/ML models and functionalities to the network:

- Option 1: For AI/ML functionality identification, the UE includes a list of supported functionalities (or use cases) in the UE capability report to the network.

- Option 2: For AI/ML model identification, the UE may include the list of supported (or available) models in the UE capability report to the network.

Observation 2: the UE may use Option 1, Option 2, or both to indicate its supported (or available) models and functionalities to the network.

Observation 3: both functionality and model may be associated with specific configuration(s) or conditions (e.g. scenarios, sites, etc.) that is (are) indicated by (or associated with) UE capability.

Observation 4: It is necessary to define UE Measurement capability reporting between the UE and network for some AI/ML models.

Proposal 1: RAN2 to agree that reported AI/ML capability can indicate a generic AI/ML capability support and/or specific AI/ML capability (e.g. a functionality, use-case, scenario, and configuration).

Proposal 2: RAN2 to introduce UE measurement capability reporting between the UE and network.

Proposal 3: RAN2 to discuss whether UE measurement capability is reported as a coarse categorisation or a finer categorisation or have both options to be selected, as per the requirements of the AI/ML model.

# 4. Reference

RAN2#121bis-e Chair Notes.

Chair’s notes RAN1#112.

Chair’s notes RAN1#112bis-e.

# 5. Annex A (RAN1#112: selected agreements on AI/ML)

Agreement For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Reuse legacy 3GPP framework of Features as a starting point for discussion. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. - In functionality-based LCM - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI). - Models may not be identified at the Network, and UE may perform model-level LCM. - Study whether and how much awareness/interaction NW should have about model-level LCM - In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. FFS: Relationship between functionality identification and model identification FFS: Performance monitoring and RAN4 impact FFS: detailed understanding on model

# 6. Annex B (RAN1#112bis-e: selected agreements on AI/ML)

Agreement - For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models: - Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. - Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. - FFS: Signaling to support functionality-based LCM operations, e.g., to activate/deactivate/fallback/switch AI/ML functionalities - FFS: Whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level - FFS: Other aspects that may constitute Functionality - FFS: which aspects should be specified as conditions of a Feature/FG available for functionality will be discussed in each sub-use-case agenda. - For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models: - model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side. - FFS: Which aspects should be considered as additional conditions, and how to include them into model description information during model identification will be discussed in each sub-use-case agenda. - FFS: Relationship between functionality and model, e.g., whether a model may be identified referring to functionality(s). - FFS: relationship between functionality-based LCM and model-ID-based LCM - Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion. Conclusion From RAN1 perspective, it is clarified that an AI/ML model identified by a model ID may be logical, and how it maps to physical AI/ML model(s) may be up to implementation. - When distinction is necessary for discussion purposes, companies may use the term a logical AI/ML model to refer to a model that is identified and assigned a model ID, and physical AI/ML model(s) to refer to an actual implementation of such a model. Agreement - Study necessity, mechanisms, after functionality identification, for UE to report updates on applicable functionality(es) among [configured/identified] functionality(es), where the applicable functionalities may be a subset of all [configured/identified] functionalities. - Study necessity, mechanisms, after model identification, for UE to report updates on applicable UE part/UE-side model(s), where the applicable models may be a subset of all identified models.