3GPP TSG-RAN2 Meeting #122	R2-2305791

Incheon, Korea, 22nd - 26th May 2023

Agenda item:	7.18.2

Source:	Qualcomm Incorporated

Title:	Control plane aspects of MT-SDT

WID/SID:	NR_MT_SDT-Core – Release 18

Document for:	Discussion and Decision

# Introduction

The RAN2 #121 meeting has made several agreements regarding the MT-SDT.

Agreement:

- Include a one-bit indication in paging to trigger MT-SDT.   We will ensure that the CCCH message can be transmitted over CG.

- Indication is per UE.  FFS on signalling.

- In case condition for paging triggered MT-SDT is not fulfilled the UE initiates RRC Resume procedure. Resume cause FFS

- Upon receiving MT-SDT trigger, the UE shall initiate SDT procedure if the following checks are satisfied (all these same as Rel-17)

-   FFS 3a: Check for DVT (if UL data becomes available in UL)

-   3b: Check for SDT RSRP threshold

-   3c: Check for TA validation before selecting CG (if applicable)

-   3d: Check for SSB level RSRP threshold for CG resource (if applicable)

5.  When UE resumes for MT-SDT, UE resumes all RBs configured for SDT

6. 	RBs configured for SDT are common for MO-SDT and MT-SDT

7.	If there is valid CG-SDT resources, the UE should use CG-SDT to transmit the response.   FFS on whether we need to optimize for case when CG periodicity is too long

8.  To confirm that when SDT is initiated due to MT-SDT, UE can exchange subsequent DL/UL SDT data on the resumed RBs. This clarifies the RB behaviour of related RAN2#120 agreement.

Agreements

- Specify a RRC procedure for RRCResume for MT-SDT initiation without checking for availability of UL data (i.e. if MT-SDT is initiated first the resume cause will be set to MT-SDT)

- UE is allowed to initiate either MO-SDT based resume or non-SDT based resume at any point (before initation RRCResumeRequest for MT-SDT) using separate procedures

- If MT-SDT procedure is initiated, for RACH during subsequent data transfer (i.e. RACH triggered due to SR), UE uses only the non-SDT RACH resources (i.e. like legacy)

In the RAN2 #120 meeting, the following agreements were made.

Agreements

- For RAN paging, MT-SDT indication (at least one bit) is explicitly included per UE via a paging message.  FFS if more information for MT-SDT are needed FFS what the indication will be called.  FFS signalling details

- Rel-18 MT-SDT after the MT-SDT paging trigger is detected, RA-SDT and CG SDT solutions/procedures specified in Rel-17 is re-used as a baseline.  The detailed triggers will be discussed on case by case.  FFS on resources used for access

- UE can use non-SDT random access resources for accessing the network for an MT-SDT transfer.  The UE can also use the configured grant resources and/or MO-RA resources.

- The network should be able to differentiate why the UL access was triggered, i.e. implicit or explicit indication by the UE.

- MT-SDT is data that belongs to bearers that are configured for SDT.    FFS whether the configuration is MO-SDT or MT-SDT specific.  The network can only trigger MT-SDT if the data belongs to those bearers.

- It is possible for the network to configure only MT-SDT without MO-SDT RA resources and/or CG-SDT.  Subsequent UL/DL data belonging to SDT bearers while in INACTIVE is allowed like MO-SDT procedure.  FFS stage 3 details

- New Resume cause in RRC resume will be introduced, one code point MT-SDT indication

In this paper, we would like to focus on the control plane aspects of MT-SDT and provide our views on more details of MT-SDT.

# Discussion

General MT-SDT

RAN2 has agreed that UE is allowed to initiate either MO-SDT based resume or non-SDT normal resume using separate procedure before initiation RRCResumeRequest for MT-SDT, and RAN2 agreed to specify a RRC procedure for RRC resume for MT-SDT initiation without checking the UL data availability.

Based on RAN2 agreements, we believe that UE can be allowed to select either MT-SDT or MO-SDT to handle the coming DL/UL data. If UE determines to respond the paging message with MT-SDT indication, (up to UE check availability of UL data), UE selects to perform MT-SDT and set the MT-SDT resume cause in the CCCH message. If UL data arrives at UE and meanwhile UE receives the paging message with MT-SDT indication (though this is a rare case), UE can either trigger a MT-SDT procedure with MT-SDT resume cause or select Rel-17 MO-SDT procedure without mixing with the MT-SDT. Network can send the DL data at the subsequent phase of MO-SDT. Thus, we have the following proposal.

Proposal 1: It is up to UE to select either MT-SDT or MO-SDT procedure. If UE determines to respond to the paging message with MT-SDT indication, UE selects MT-SDT and set MT-SDT resume cause in the RRCResumeRequest message. If UL data is arrived at UE before UE triggers any RRC resume procedure, UE could determine to perform Rel-17 MO-SDT.

When UE receives the paging message with MT-SDT indication, UE may decide to trigger MT-SDT for the DL data reception. Upon MT-SDT trigger, the MT-SDT conditions should be checked. Only if the MT-SDT conditions are all satisfied, UE is allowed to trigger MT-SDT. From RRC layer perspective, almost all legacy conditions for Rel-17 MO-SDT can be reused except whether the pending UL data mapped to the radio bearers configured for SDT. Because checking UL data and data volume is not the must for MT-SDT. It is up to UE to check the UL data and UL data volume. If UE to do so and the data volume is not above the threshold, UE may trigger MO-SDT like the Rel-17 SDT behavior. From MAC player perspective, the MO-SDT conditions for RA resource and CG resource can be mostly reused for MT-SDT conditions.

Proposal 2: The Rel-17 SDT conditions can be reused for MT-SDT conditions except checking the pending UL data whether is mapped to the radio bearers for SDT.

Proposal 3: It is up to UE to check the UL data volume when UE receives the paging message with MT-SDT indication.

If UE selects MT-SDT, UE is allowed to use the Rel-17 RA-SDT resource if configured. Since it is possible that a MT-SDT user may also have UL data to be handled in Msg3, it is beneficial that network can identify the SDT user earlier among the RACH users if RA-SDT resource is selected during UE performing MT-SDT. If RA-SDT resource is not configured, UE should use normal RACH resource for MT-SDT.

Proposal 4: UE is allowed to select RA-SDT resource (if configured) when performing MT-SDT.

Proposal 5: If RA-SDT resource is not configured, the normal RACH resource is used for MT-SDT.

Regarding the CG-SDT resource or RA-SDT resource, as long as they are configured to the UE, similar to Rel-17, UE is allowed to use either CG or RA resource to perform MT-SDT. The rule of selection between CG and RA resource could be the same as Rel-17 SDT.

Proposal 6: Upon initiation of MT-SDT procedure, the rule of selection between CG and RA resource could be the same as Rel-17 SDT.

Paging message enhancement

It has been agreed that the paging message is enhanced to carry the MT-SDT indication so that UE can be informed that DL data is arrived at network. It is up to network configuration on what resources for UE to perform SDT. Since network has less information on the UE radio conditions, which resources the UE chooses for MT-SDT could be up to UE evaluation. Considering the consistent UE behavior on MO-SDT and MT-SDT, it is preferable to allow UE to select either RA resource or CG resource (if configured) for MT-SDT. Thus, we think which resource (RA or CG) is selected for MT-SDT is not indicated in paging message.

Proposal 7: Which resource (RA or CG) is selected for MT-SDT is not indicated in paging message.

In Rel-17 SDT, separate RACH resource can be configured for SDT users. The MO-SDT UE is allowed to use the separate RACH resource to perform SDT and the separate RO or separate preamble RACH resource serves as the implicit SDT indication to network so that network can assign the appropriate resource for UL/DL data transmission.

For the RACH based MT-SDT, since the MT-SDT is triggered by paging message, there are benefits that network directly indicates the dedicated resource for the target UE. The similar design principle can be observed in CG-SDT that the dedicated CG resource is configured via the RRC release message.

Using dedicated resource for RACH based MT-SDT not only reduces the latency of UE performing RRC resume but also improves the reliability of the resume procedure. The dedicated resource improves the success possibility of the procedure. It is also good for UE power saving. Therefore, we have following proposal.

Proposal 8: A dedicated preamble for MT-SDT can be carried in the paging message for the target UE for RACH based MT-SDT.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: It is up to UE to select either MT-SDT or MO-SDT procedure. If UE determines to respond to the paging message with MT-SDT indication, UE selects MT-SDT and set MT-SDT resume cause in the RRCResumeRequest message. If UL data is arrived at UE before UE triggers any RRC resume procedure, UE could determine to perform Rel-17 MO-SDT.

Proposal 2: The Rel-17 SDT conditions can be reused for MT-SDT conditions except checking the pending UL data whether is mapped to the radio bearers for SDT.

Proposal 3: It is up to UE to check the UL data volume when UE receives the paging message with MT-SDT indication.

Proposal 4: UE is allowed to select RA-SDT resource (if configured) when performing MT-SDT.

Proposal 5: If RA-SDT resource is not configured, the normal RACH resource is used for MT-SDT.

Proposal 6: Upon initiation of MT-SDT procedure, the rule of selection between CG and RA resource could be the same as Rel-17 SDT.

Proposal 7: Which resource (RA or CG) is selected for MT-SDT is not indicated in paging message.

Proposal 8: A dedicated preamble for MT-SDT can be carried in the paging message for the target UE for RACH based MT-SDT.

# References

# Annex