3GPP TSG RAN WG2 Meeting #122                                      	R2-2305792

Incheon, Korea, 22-26 May, 2023

Agenda item:	7.16.2.2

Source:	Samsung

Title:	Enhancements for RRM/MDT to support AI/ML data collection

Document for:	Discussion and Decision

# 1. Introduction

In RAN2#121bis-e meeting, RAN2 discussed aspects of data collection for AI/ML LCM purposes/functions. The following is RAN2 conclusion on whether to support enhancement of existing MDT/RRM methods to be used for data collection for different LCM purposes/functions [1]:

Observation: RAN2 may need to consider enhancements for AIML to existing functionality for data collection, e.g. for timing control (e.g. for MDT/RRM).

In this contribution, we discuss possible enhancements of existing MDT/RRM mechanisms to support data collection for AI/ML model LCM purposes.

# 2. Discussion

### 2.1 Existing RRM/MDT mechanisms

The existing (legacy) RRM measurements configured by RRC include e.g. serving cell and neighbouring cells RSRP and RSRQ which are used to support UE mobility procedures, for cell handover (network controlled mobility), for the UE in the RRC_Connected state, and cell reselection (UE controlled mobility based on the network configuration) for the UE in RRC_IDLE and RRC_INACTIVE states.

Beside mobility procedures, RRM measurements are also used by other RAN mechanisms, e.g. MDT. The main difference in RRM cells measurements in RRC_CONNECTED and RRC_IDLE/RRC_INACTIVE states, is the measurements periodicity, which is significantly lowered in the case of RRC_IDLE/RRC_INACTIVE state, in order to reduce the UE’s energy consumption in those states.

Moreover, RRM measurements configuration requirements are linked to the used RRC state in the UE. As a result, RRM measurements configuration must be updated (or restarted) every time the RRC state is changed. This may be suboptimal as RRC state transition requires time and introduces latency, e.g. to establish or release dedicated RRC signalling connection, between the UE and NG-RAN (when the UE moves to or leaves RRC_CONNECTED state), reconfigure the UE radio protocol stack, receive new measurements control information from NG-RAN in the UE, etc.

In a typical UE implementation, RRC state transition causes discontinuity in the RRM measurements, i.e. old measurements configuration and actual cell measurements get deleted and stopped until the new configuration is received in the UE from NG-RAN and the serving and neighbour cells measurements could be restarted.

Observation 1: RRC state transition causes discontinuity in the RRM/MDT measurements.

### 2.2 Enhancement of RRM/MDT for AI/ML

In RAN3#119bis-e meetings, RAN3 discussed whether existing MDT mechanisms can be used for data collection from the UE and agreed the following [2]:

The existing MDT framework is used as baseline for data collection from the UE.

Observation 2: RAN3 agreed to use existing MDT as baseline for data collection from the UE.

Additionally, RAN3 discussed the problem of discontinuity of MDT data collection, due to the UE transition across different RRC states, and how it could impact the performance of an AI/ML model. For example, if a model is trained on non-consecutive data samples, i.e. non-continuous time series of data for model training/re-training purpose, the model training procedure may not be useful and could result in a poor model performance (e.g. inaccurate prediction or inference). To address this issue, RAN3 agreed to enable continuous collection of MDT data from the same UE across different RRC states for training in OAM [2]:

Continuous collection of MDT traces is beneficial only for AI/ML training in OAM. Continuous MDT collection is to enable the continuous collection of MDT data from the same UE across RRC state changes (RRC_Connected, RRC_Idle, RRC_Inactive).

Observation 3: continuous collection of MDT data from the same UE across RRC states should ensure no interruptions in data collected for AI/ML training in OAM.

However, the following issues may need to be addressed in relation to the new concept of continuous data collection:

- Whether continuous data collection in time (across RRC states) may impact the UE.

- Whether to enhance existing MDT mechanism in order to support continuous data collection.

In RAN3#121bis-e, RAN3 listed a set of potential solutions to support continuous MDT collection [2]:

FFS on determining the need for continuous MDT support for the Rel.18 use cases. FFS on the solutions to support continuous MDT collection: - Solution 1: existing MDT mechanism without enhancement. - Solution 2: Introduce an indication in the logged MDT measurement configuration to indicate UE whether to collect continuous location information (e.g., historical cell information, latitude, longitude, altitude, velocity, etc.) across various RRC states (connected, inactive, idle). - Solution 3: introduce a “Continuous MDT” flag in the management based logged MDT measurement configuration, and enable the UE to report a “Continuous MDT” flag as soon as the UE establishes an RRC connection from Idle. Introduce a continuous MDT indication in the Handover Request message to enable the target NG-RAN to configure the UE with a management-based MDT configuration - Solution 4: down-select the solution from option 1 to 3 to configure multiple signaling based MDTs (e.g., immediate MDT and log MDT) to a UE

Observation 4: RAN3 has been discussing a set of potential solutions to support continuous MDT collection.

Similar to RAN3, RAN2 is also considering enhancements for MDT/RRM mechanisms to support AI/ML data collection [1]:

Observation: RAN2 may need to consider enhancements for AIML to existing functionality for data collection, e.g. for timing control (e.g. for MDT/RRM).

To this end, similar to RAN3, RAN2 should also start looking into potential solutions for enhancements of existing MDT/RRM mechanisms in order to support the different data collection requirements in AI/ML.

Proposal 1: RAN2 to study enhancements for existing RRM/MDT mechanisms to support data collection in AI/ML.

# 4. Conclusion

In this contribution, we discussed existing MDT/RRM and potential enhancements for MDT/RRM mechanisms to support data collection for AI/ML. The following are the observations and proposal in this document:

Observation 1: RRC state transition causes discontinuity in the RRM/MDT measurements.

Observation 2: RAN3 agreed to use existing MDT as baseline for data collection from the UE.

Observation 3: continuous collection of MDT data from the same UE across RRC states should ensure no interruptions in data collected for AI/ML training in OAM.

Observation 4: RAN3 has been discussing a set of potential solutions to support continuous MDT collection.

Proposal 1: RAN2 to study enhancements for existing RRM/MDT mechanisms to support data collection in AI/ML.

# 5. Reference

- RAN2#121bis-e Chair Notes

- RAN3#119bis meeting report

- R3-230861 Summary of Discussion for CB: # 20_AIRAN4_MDT, Ericsson

- R3-230612 Discussion on MDT enhancement for AI/ML for NG-RAN, Samsung