3GPP TSG-RAN2 Meeting #122	R2-2305793

Incheon, Korea, 22nd - 26th May 2023

Agenda item:	7.18.3

Source:	Qualcomm Incorporated

Title:	User plane aspects of MT-SDT

WID/SID:	NR_MT_SDT-Core – Release 18

Document for:	Discussion and Decision

# Introduction

The RAN2 #121 meeting has made several agreements regarding the MT-SDT.

Agreement:

- Include a one-bit indication in paging to trigger MT-SDT.   We will ensure that the CCCH message can be transmitted over CG.

- Indication is per UE.  FFS on signalling.

- In case condition for paging triggered MT-SDT is not fulfilled the UE initiates RRC Resume procedure. Resume cause FFS

- Upon receiving MT-SDT trigger, the UE shall initiate SDT procedure if the following checks are satisfied (all these same as Rel-17)

-   FFS 3a: Check for DVT (if UL data becomes available in UL)

-   3b: Check for SDT RSRP threshold

-   3c: Check for TA validation before selecting CG (if applicable)

-   3d: Check for SSB level RSRP threshold for CG resource (if applicable)

5.  When UE resumes for MT-SDT, UE resumes all RBs configured for SDT

6. 	RBs configured for SDT are common for MO-SDT and MT-SDT

7.	If there is valid CG-SDT resources, the UE should use CG-SDT to transmit the response.   FFS on whether we need to optimize for case when CG periodicity is too long

8.  To confirm that when SDT is initiated due to MT-SDT, UE can exchange subsequent DL/UL SDT data on the resumed RBs. This clarifies the RB behaviour of related RAN2#120 agreement.

Agreements

- Specify a RRC procedure for RRCResume for MT-SDT initiation without checking for availability of UL data (i.e. if MT-SDT is initiated first the resume cause will be set to MT-SDT)

- UE is allowed to initiate either MO-SDT based resume or non-SDT based resume at any point (before initation RRCResumeRequest for MT-SDT) using separate procedures

- If MT-SDT procedure is initiated, for RACH during subsequent data transfer (i.e. RACH triggered due to SR), UE uses only the non-SDT RACH resources (i.e. like legacy)

In the RAN2 #120 meeting, the following agreements were made.

Agreements

- For RAN paging, MT-SDT indication (at least one bit) is explicitly included per UE via a paging message.  FFS if more information for MT-SDT are needed FFS what the indication will be called.  FFS signalling details

- Rel-18 MT-SDT after the MT-SDT paging trigger is detected, RA-SDT and CG SDT solutions/procedures specified in Rel-17 is re-used as a baseline.  The detailed triggers will be discussed on case by case.  FFS on resources used for access

- UE can use non-SDT random access resources for accessing the network for an MT-SDT transfer.  The UE can also use the configured grant resources and/or MO-RA resources.

- The network should be able to differentiate why the UL access was triggered, i.e. implicit or explicit indication by the UE.

- MT-SDT is data that belongs to bearers that are configured for SDT.    FFS whether the configuration is MO-SDT or MT-SDT specific.  The network can only trigger MT-SDT if the data belongs to those bearers.

- It is possible for the network to configure only MT-SDT without MO-SDT RA resources and/or CG-SDT.  Subsequent UL/DL data belonging to SDT bearers while in INACTIVE is allowed like MO-SDT procedure.  FFS stage 3 details

- New Resume cause in RRC resume will be introduced, one code point MT-SDT indication

In this paper, we would like to focus on the user plane aspects of MT-SDT and provide our views on the details of MT-SDT.

# Discussion

The initial DL data reception in MT-SDT

For the initial DL data reception of MT-SDT, network should send the (initial) DL data after network has known UE is ready for DL data reception. There were extensive discussions related to MT data handling in Rel-16 MT-EDT [1], and several solutions were studied. In the MT-EDT discussions, it seems that the Msg2 based MT-EDT solution is complicated in terms of the specification impacts and needs more time for study. In addition, the DL data transmission within paging message has been excluded from the NR MT-SDT WI scoping.

Therefore, for RACH based MT-SDT, the simplest solution with the least specification impacts for the initial DL reception is Msg4/MSGB based MT-SDT solution. It means that the initial DL data could be received in or after Msg4/MSGB message together with the contention resolution in RA-SDT. For CG-SDT, UE is ready for initial DL reception after UE has received the network acknowledgement as the response to the RRCResumeRequest/1 message. Thus, we have following proposals.

Proposal 1: For RACH based MT-SDT, the initial DL data delivered in or after Msg4/MSGB should be supported.

Proposal 2: For CG based MT-SDT, the initial DL data delivered after UE has received the network acknowledgement in response to the RRCResumeRequest/1 message should be supported.

Subsequent DL data reception

If Rel-17 MO SDT resource is not configured, the legacy RACH resource could be selected for MT-SDT. Then how to handle the UL data if legacy RACH resource is selected needs more discussion.

In MT-SDT, the UL data in response to the DL data may be generated after contention resolution in a legacy RACH. In the subsequent phase of MT-SDT, if no UL resource is available, UE has to trigger another RACH to request the UL resource for UL data handling which is obviously inefficient. In this case, if SR resource is configured, it will be quite helpful to use SR resource to request more UL grant for UL data handling in the subsequent MT-SDT. Having SR resource configuration for MT-SDT can also avoid excessively triggering RACH just for small application layer UL data feedback in the subsequent phase of MT-SDT.

Observation 1: If no SR resource is configured, UE has to trigger another RACH if UL SDT data is arrived during MT-SDT.

Proposal 3: RAN2 should study to have SR resource configuration for supporting UL data handling in MT-SDT procedure.

UE power saving in SDT

In Rel-17 SDT study, the power consumption of SDT UE has less consideration. There is no window/timer control for PDCCH monitoring after each uplink transmission. UE has to keep monitoring the network response or potential DL/UL scheduling and wait for network sending the RRC message to terminate the whole SDT procedure even though UE only plans to send one-shot UL data. In a worse case, UE has to monitor PDCCH for several seconds till the T319a expiry.

If we consider Rel-17/18 SDT for different UE device types in different use cases, for example, for a RedCap wearable device, it may require much more power saving SDT procedure considering its reduced capability. However, we think there is no power saving mechanism defined in Rel-17 SDT study.

In Rel-18 MT-SDT, UE still needs to monitor PDCCH for network scheduling the DL data in the subsequent data phase. Since the DL traffic profile and scheduling gaps are handled and controlled by network, network has full knowledges on when to send DL data to UE. It can be up to network to configure proper DRX to adapt to the DL traffic pattern so that UE can wake up for reception only when DL data is arrived to save UE power. Especially for the case that there could be round-trip delay at application layer for the DL data, it is bad for such as the RedCap device to monitor the network response during a long gap which obviously wastes UE power and is not power efficiency. If DRX is configured, UE can take time to sleep during the time of network fetching data and then UE wakes up in the configured active window for DL data reception. It should be noted that for UE entering connected state, DRX mechanism can be enabled by network for UE normal DL/UL data exchanges. There is no reason that SDT UE has worse power consumption in RRC inactive state than the UE performing normal data communication in RRC connected state. In that sense, it is important to consider the power saving mechanism for Rel-18 MT-SDT, and DRX mechanism could be one of candidate schemes.

Proposal 4: RAN2 should study power saving mechanism such as to support DRX for MT-SDT.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Observation 1: If no SR resource is configured, UE has to trigger another RACH if UL SDT data is arrived during MT-SDT.

Proposal 1: For RACH based MT-SDT, the initial DL data delivered in or after Msg4/MSGB should be supported.

Proposal 2: For CG based MT-SDT, the initial DL data delivered after UE has received the network acknowledgement in response to the RRCResumeRequest/1 message should be supported.

Proposal 3: RAN2 should study to have SR resource configuration for supporting UL data handling in MT-SDT procedure.

Proposal 4: RAN2 should study power saving mechanism such as to support DRX for MT-SDT.

# References

- TS 36.300 v17.2.0, E-UTRA and E-UTRAN overall description, stage 2, (Release 17), 3GPP.

# Annex