3GPP RAN WG2 Meeting #122	R2-2305812

Incheon, Korea, 22nd – 26th May 2023

Agenda Item:	7.4.3

Source:	InterDigital Inc.

Title:	Subsequent change of SCGs and selective activation

Document for:	Discussion, Decision

# Introduction

The topic of selective activation of cell groups was discussed in RAN2#121bis-e and the following were agreed [1]:

For the reference configuration for SCG Selective Activation, aim at following similar design as LTM.

For inter-SN SCG Selective Activation, the RRC reconfiguration message containing the Rel-18 CPC configurations provided to the UE is in MN format.

For MN initiated inter-SN SCG selective activation, source MN generates the execution conditions for the initial CPAC.

- FFS on the following options for subsequent CPC:

- Option 1: Source MN generates the execution conditions for all subsequent CPC.

- Option 2: Candidate SN may generate execution conditions for subsequent CPC.

For SN initiated inter-SN SCG selective activation, source SN generates the execution conditions for the initial CPC. 
FFS if Candidate SN may generate/modify execution conditions for subsequent CPC

Assume for now that there is only one reference configuration.

The following may be included in the initial RRC reconfiguration message containing the Rel-18 CPC configurations:

- Reference SCG configuration (Optionality FFS). Assume as for LTM Reference configuration may be empty.

- FFS whether MCG configuration is included.

- FFS RRC model for the reference configuration.

- Initial List of candidate target PSCells (this list can be updated by the network, e.g., cells may be added or removed) with associated target SCG configurations. FFS whether the MCG configurations associated with the target SCG configurations are included.

- 3. The execution conditions associated with each candidate target PSCell.

- a.	For MN initiated procedure, execution conditions based on event A4 are supported. FFS whether A3/A5 are supported.

- b.	For SN initiated procedure, execution conditions based on events A3/A5 are supported.

UE will keep R18 CPC configurations after CPC execution. It should be possible to release a CPC candidate explicitly by RRC reconfiguration procedure.

This contribution addresses some of the open issues regarding subsequent and selective activation of SCGs.

# Discussion

As has been agreed already in in RAN2, subsequent secondary cell group change is supported without the need for RRC reconfiguration, which requires maintaining of CPC configurations after executing a certain CPC configuration. However, it is still not clear if the UE could maintain all the CPC configurations. Our understanding is that it is not necessary to maintain all CPAC configurations as some of them may not be relevant after the execution of a certain CPAC. Thus, it should be possible for the network to configure/indicate which CPC configurations are kept and which are released after the execution of a CPAC.

It has already been agreed in RAN2 that it should be possible to release a CPC candidate/configuration explicitly via RRC reconfiguration procedure, which in our understanding is also possible in rel-17 CPC. Requiring the network to explicitly release some CPC configuration is in a way contradictory to the subsequent CPC objectives, which is avoiding the need to reconfigure the UE every time after a CPC.

- Observation 1: Requiring the explicit release of the CPC configurations that are not relevant anymore after a certain CPC is executed is contradictory to the objectives of subsequent CPAC (i.e., performing subsequent CPACs without the need for RRC reconfigurations).

Thus, it should be possible to pre-configure information in the UE regarding which CPAC configurations can be maintained or should be released after a certain CPAC configuration is executed.

- Proposal 1:	The network can pre-configure the UE with information regarding the handling of the remaining CPAC configurations when one CPAC configuration is executed (e.g., which CPAC configurations are released, which are kept, etc.,). The exact signalling of this relationship between the CPAC configurations is FFS.

Regarding the support for delta configuration, both for the selective SCG activation as well as the L1/L2 PCell switching, RAN2 has made the agreement that there will be a reference configuration and a delta configuration will be performed on top of the reference configuration.

If there was no SCG configured, it will be natural to signal the reference SCG configuration separately. However, there is no reason why this reference configuration cannot be one of the CPA configurations. Not doing so actually could lead to redundant signalling. The other CPA configurations can be provided as a delta to this reference configuration. If the triggering conditions of this reference configuration are fulfilled, then the UE just executes the reference configuration. If the triggering condition to any of the other CPA configurations is fulfilled, then the UE will execute the reference configuration followed by the detla configuration included in the CPA whose conditions just got fulfilled.

- Proposal 2:	If the UE was not configured with SCG, it can be configured with several CPA configurations, where one of them is indicated to be a reference configuration and the others are a delta from this reference configuration.

On the other hand, if SCG was already configured by the time the configuration regarding the configuration for selective activation was received, either the current SCG configuration can be considered as the reference configuration, or a separate reference configuration can be provided.

- Proposal 3:	The UE can be configured to use the current SCG configuration as a reference configuration for subsequent SCG changes.

- Proposal 4:	The UE can be configured with a separate explicit SCG reference configuration (full SCG configuration).

- Proposal 5:	The UE can be configured with a delta configuration that can be applied to the current SCG configuration to generate the reference SCG configuration.

Keeping the network resources reserved all the time for all the CPAC configurations may not be feasible or desirable. Also, currently, there is a limitation of a maximum of 8 PCell/PSCell candidates that can be configured at the same time. A more flexible solution to reduce the signalling is to enable CPC configurations that are not always active. When the CPC configuration is not active, the UE does not monitor the associated triggering conditions and network does not have to reserve the resources for the UE at the target. That way, many CPC configurations can be provided to the UE, and network can ensure not more than 8 of them are active at the same time (or not more than what the UE can support if it is less than 8).

- Proposal 6:	A UE can be configured with multiple CPAC configurations that can be active or inactive.

- Proposal 7:	When a CPAC configuration is inactive, the UE does not monitor the triggering conditions associated with it.

- Proposal 8:	The network can explicitly activate/deactivate a CPAC configuration. FFS if the execution of a certain CPAC configuration could trigger the dynamic activation/deactivation of other CPAC configurations.

The above discussion only considers the case of CPAC. However, the network may prefer not to use conditional reconfigurations, or the UE may not support it. Thus, it is desirable for the network to be able to configure several SCG configurations and activate one of them at a time. This, for example, can be integrated with the L1/L2 mobility objective of the WI, where the L1/L2 indication is used not only for the PCell change but also for PSCell change or SCG activation/deactivation.

- Proposal 9:	A UE can be configured with multiple SCG configurations, where only one of them can be enabled/activated at a time. How the activation/deactivation of the SCGs is triggered (e.g., via L1/L2 signalling, UE triggered, etc.,) is FFS.

# Conclusion

In this contribution, the selective activation of cell groups is discussed, and the following observations and proposals are made:

- Observation 1: Requiring the explicit release of the CPC configurations that are not relevant anymore after a certain CPC is executed is contradictory to the objectives of subsequent CPAC (i.e., performing subsequent CPACs without the need for RRC reconfigurations).

- Proposal 1:	The network can pre-configure the UE with information regarding the handling of the remaining CPAC configurations when one CPAC configuration is executed (e.g., which CPAC configurations are released, which are kept, etc.,). The exact signalling of this relationship between the CPAC configurations is FFS.

- Proposal 2:	If the UE was not configured with SCG, it can be configured with several CPA configurations, where one of them is indicated to be a reference configuration and the others are a delta from this reference configuration.

- Proposal 3:	The UE can be configured to use the current SCG configuration as a reference configuration for subsequent SCG changes.

- Proposal 4:	The UE can be configured with a separate explicit SCG reference configuration (full SCG configuration).

- Proposal 5:	The UE can be configured with a delta configuration that can be applied to the current SCG configuration to generate the reference SCG configuration.

- Proposal 6:	A UE can be configured with multiple CPAC configurations that can be active or inactive.

- Proposal 7:	When a CPAC configuration is inactive, the UE does not monitor the triggering conditions associated with it.

- Proposal 8:	The network can explicitly activate/deactivate a CPAC configuration. FFS if the execution of a certain CPAC configuration could trigger the dynamic activation/deactivation of other CPAC configurations.

- Proposal 9:	A UE can be configured with multiple SCG configurations, where only one of them can be enabled/activated at a time. How the activation/deactivation of the SCGs is triggered (e.g., via L1/L2 signalling, UE triggered, etc.,) is FFS.

# References

RAN2 #121bis-e Chairman Notes, April 2023