3GPP TSG-RAN WG2 Meeting #122	R2-2305823

Incheon, KR, May 22 – 26, 2023

Agenda item:	7.2.3

Source: 	Qualcomm Incorporated

Title: 	Integrity of NR Positioning Technologies

Document for:		Discussion and Decision

# 1.	Introduction

The Work Item on "Expanded and Improved NR Positioning" includes the following objective for "RAT-dependent Integrity" [1]:

- Specify the error modelling parameters, signalling, and procedures to support UE-based and LMF-based integrity of RAT-dependent positioning methods [RAN2, RAN3].

During the Study Item Phase, RAN2 made the general agreement to re-use the GNSS 'Integrity Principle of Operation' also for the NR positioning technologies [2]:

RAN2 to confirm the integrity principle of operation defined in the section 8.1.1a of TS38.305, including integrity definition (e.g., Error, Bound, Time to Alert, DNU, Residual Risk, irMinimum, irMaximum and Correlation Times; FFS if all parameters are needed in the RAT-dependent case), Equations for the GNSS integrity are reused for RAT dependent positioning methods. RAN2 may add the mapping between Integrity definition/Fields (Integrity Alerts, error bounds (mean, StdDev), Residual Risks, Integrity correlation times) and Error sources/assistance data for RAT-dependent positioning methods later once RAN1 identifies new error sources. Use DNU flag for RAT-dependent integrity, with the meaning that the concerned assistance data cannot be used for integrity calculation but may be usable for positioning. Signalling details and relation to error sources can be determined in normative work. FFS which positioning methods are affected based on the progress in RAN1.

At RAN2#121 and RAN2#121bis-e, the following agreements were made [3],[4]:

Agreements:

RAN2 anticipate that the error sources are overbounded by a Gaussian distribution.

LS to RAN1 to check this view and ask about the parameters for the overbound distributions.

Agreements:

TRP related error source bounds can be provided to UE via dedicated LPP providing assistance message or posSIB.

Any interaction between the LMF and NG-RAN to support determination of error sources is in RAN3 scope.  Other aspects of determining the TRP error sources are left to deployment and implementation.

For UE-based RAT-dependent integrity, the PL and/or its corresponding TIR are provided to LMF as legacy, using the existing common LPP signalling from Rel-17.

Agreement:

LS to RAN1 to include a request for confirmation that the beam-related information (Beam Bore-Sight Direction and Beam Antenna Information) are error sources for DL-AoD positioning.

Agreement:

LS to RAN1 to include the question of whether RAN1 identify a need for a DNU flag for measurements.

Agreement:

For RAT-dependent integrity, the PL calculation is performed by the entity which also performs the position calculation for a location process.

Agreements:

For UE-based integrity, the integrity parameters of error sources for RAT-dependent integrity are included in assistance data.

LPP Request/Provide Assistance Data are reused for retrieving the integrity parameters to the UE from the LMF.  The request is per positioning method (as in legacy operation) and the provided integrity parameters are as appropriate for the selected positioning method.

Use of posSIBs for integrity parameters is not excluded.

Working assumption:

For LMF-based integrity, no integrity KPI (TTA, TIR, and AL) and integrity results transfer in LPP message.

Working assumption:

It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results from UE and/or NG-RAN.

Agreement:

Indicate the WA above in the LS to RAN1 to allow them to register any concern.

Agreement:

Capture the stage 2 impact for RAT-dependent integrity in section 7 of 38.305.  Initial running CR to be seen at next meeting, using R2-2302504 and R2-2303682 as baseline.

Therefore, there is good progress on the Stage 2 aspects for the Integrity of the NR positioning technologies. The following items require further RAN1 confirmation:

-	Gaussian overbounding;

-	Error sources for DL-AoD;

- 	DNU flags for measurements;

-	feedback on RAN2 WA on UE/TRP measurement error bounds.

On the first item (Gaussian overbounding), RAN1 discussed the issue at RAN1#112bis-e and provided the following response [5]:

1. Overall Description: RAN1 would like to thank RAN2 for the LS R1-2302282/ R2-2302271 on error source distributions. RAN1 would like to provide the answers to the following questions: Q1: RAN2 would like to confirm with RAN1 on the agreement “RAN2 anticipate that the error sources are overbounded by a Gaussian distribution.” regarding the distribution of error sources Reply to Question 1: From RAN1’s perspective, the RAN2 agreement “the error sources are overbounded by a Gaussian distribution” can be confirmed for the error sources listed in Table 6.1.1-2 in TR 38.859 if the error source follows a Gaussian distribution. Q2: RAN2 respectfully ask RAN1 to provide the parameters (e.g. mean and standard deviation) for the overbound Gaussian distribution. Reply to Question 2: Parameters for the overbound Gaussian distribution can be mean and standard deviation. From RAN1’s perspective, zero is a valid possible option for the mean value for the overbound Gaussian distribution for the error sources listed in Table 6.1.1-2 in TR 38.859. From RAN1 perspective, the value ranges of existing fields corresponding to quality information (e.g., nr-TimingQuality, rtd-Quality-r16) and uncertainty information (e.g., LocationUncertainty-r16) can be reused as a reference to derive the value ranges for the parameters (e.g., standard deviation) for the overbound Gaussian distribution for the error sources listed in Table 6.1.1-2 in TR 38.859.

In this contribution, we discuss some remaining aspects of integrity support for the NR Positioning Technologies.

# 2.	Discussion

##### Bounding Model

According to the RAN1 response LS [5], RAN1 confirms:

-	the error sources can be overbounded by a Gaussian distribution for the error sources listed in Table 6.1.1-2 in TR 38.859 if the error source follows a Gaussian distribution.

-	parameters for the overbounded Gaussian distribution can be mean and standard deviation.

-	zero is a valid possible option for the mean value.

The above is in general agreement with the GNSS Integrity Principle of Operation ([6], section 8.1.1a); in particular:

-	The bound formula describes a bounding model including a mean and standard deviation (e.g., paired over-bounding Gaussian). Paired Gaussian overbounding enables tighter bounding of the error distributions (to better reflect their real-world error properties), leading to smaller PLs (in particular given the wide range of TIRs identified in TR 38.859). The paired Gaussian overbounding applies generally to any error distribution, be it non-zero mean, asymmetric, or multimodal [7].

-	The mean value could be zero (in which case the paired overbounding may collapse into the tail-overbounded Gaussian distribution). Hence, paired overbounding leads to greater flexibility in the choice of implementation by sending both mean and standard deviation in the assistance data (where the mean could be zero when appropriate).

Proposal 1:	RAN2 confirm the GNSS bounding model, including a mean and standard deviation (e.g., paired over-bounding Gaussian), is also used for integrity for NR Positioning Methods.

##### Rate Error bounds

The GNSS integrity bounds also include the error changes over time as a bound of the errors mean and standard deviation's rate of change [6], [8]. Since the same behaviour is expected for the NR positioning methods – e.g., inter-TRP synchronization error may change over time (drift) – the mean and standard deviation for the rate error bounds should optionally also be provided for the NR positioning error sources.

Proposal 2:	Analogous to GNSS, the mean and standard deviation of rate error bounds can optionally also be provided for the NR positioning error sources.

##### DNU Flags

The GNSS integrity principle of operation is contingent on 'DNU flags'. I.e., the probability that an error exceeds the bound for longer than TTA under the condition 'no DNU' must be less than a specified threshold. One reason for providing these 'DNU flags' is that the UE must fail safe even in the event of a loss of connectivity [9]. This means that the UE must receive an affirmative signal that the bounds are still valid within the TTA, otherwise it must assume that a feared event may have occurred and the notification was not delivered. If the network detects a feared event in the window between regular assistance data updates the DNU flags are set to 'true'. The DNU flags must be provided more frequently (according to the Time-to-Alert (TTA) requirements) so that the assistance data (e.g., obtained from broadcast or pre-configured) can be invalidated quickly.

For GNSS, the granularity of the DNU flags is currently as follows [8]:

-	per GNSS,

-	per SV,

-	per Signal,

- 	per ionospheric correction,

-	per tropospheric correction.

Employing "do not use" flags for GNSS ensure users drop satellites/assistance data that may be unhealthy or performing poorly. For example, orbit and clock data may be computed separately from atmospheric errors such that the UE can fall back to PPP with integrity in the case the PPP-RTK chain is unavailable, etc..  The same argumentation applies to NR positioning technologies: If e.g., beam antenna information is not available (DNU), the UE can fall back to bore-sight direction information; or if RTD information is not available (DNU), the UE may fall back to DL-AoD, etc.

NOTE: 	Pending RAN1 confirmation whether Beam Bore-Sight Direction and Beam Antenna Information are error sources for DL-AoD positioning.

The 'DNU Flags' for GNSS are provided in the IEs GNSS-RealTimeIntegrity and GNSS-Integrity-ServiceAlert. For NR positioning, a new assistance data element is required for providing 'DNU Flags'; e.g., IE NR-Integrity-ServiceAlert.

Proposal 3:	The DNU flags are provided per TRP and per error contribution (e.g., TRP location, RTD, beam information, etc.) in a new IE NR-Integrity-ServiceAlert.

##### Correlation Times

UEs/LMFs which may make use of  time-based estimation techniques such as Kalman Filtering must also be provided with information about the time correlation of the errors. For GNSS, the assistance data may include the 'correlation times', defined as the minimum time interval beyond which two sets of assistance data parameters for a given error can be considered to be independent from one another.

An implementation typically uses the same integrity algorithm/principles for all supported positioning technologies, in particular if the position calculation is hybrid (e.g., GNSS+Sensors+NR, etc.). Therefore, a UE may use a sequential integrity monitoring algorithm also for the NR positioning technologies.

Proposal 4:	The 'Integrity Correlation Times', defining the minimum time interval beyond which two sets of assistance data parameters for a given error can be considered to be independent from one another, can optionally be provided for the integrity assistance data.

##### LMF-based PL calculation

To determine the PL by an LMF for UE-assisted mode or network-based positioning methods, the LMF requires the overbounding standard deviation of the UE and TRP measurement error (e.g., RSTD, RTOA, Rx-Tx, AoA, DL-PRS RSRP, DL-PRS RSRPP), which provides an upper bound on the standard deviation of the nominal measurement error σx,i.  That is, under fault-free condition, the standard deviation of the measurement error is no greater than σx,i.

The measurement error is not observable and need to be approximated/estimated via e.g., standard error of the mean which estimates the variability across multiple samples of a population:

| σ=(xi-x)2N | (1) |
| --- | --- |

This standard deviation may serve as a measure of uncertainty and requires a number N of samples. Therefore, periodic measurements are required, where either the UE/TRP determines the sample statistics or the LMF uses the periodic reports from the UE/TRP to determine sample statistics.

The sample statistics may be derived or used by an e.g., Kalman Filter whose details depends on LMF implementation/algorithms used. Instead of deriving the measurement statistics at the UE/TRP and reporting to an LMF, it seems more flexible and implementation-independent if an LMF determines any statistics from the (already specified) UE/TRP measurement report directly. I.e., a LMF could request periodic reporting from UEs and TRPs and could derive any required statistics from the  measurement reports.

Observation 1:	For LMF-based PL calculation ("LMF based integrity" in the Work Item objective), there are two general options:
Option 1:  The UE/TRP determines the sample statistics/error bounds and reports them to the LMF, or 
Option 2:  the LMF determines the sample statistics/error bounds using (existing) periodic UE/TRP measurement reporting.

Both Options in Observation 1 seem equivalent and would allow the determination of measurement error bounds (e.g., using equation (1)). However, Option 2 would not require specifying a particular method/equation for determining the error statistics/bounds and therefore, could be up to LMF implementation. For example, the LMF could decide which measurement items are used to determine the bounds (e.g., multiple paths, measurement quality, LOS/NLOS Information, etc.), or how many samples are used or needed (e.g., N in equation (1), etc.). Therefore, Option 2 in Observation 1 could be implementation defined and would be most flexible (i.e., no "hard coding" of the method/definition would be needed).

Proposal 5:	From RAN2 perspective, the local errors/threats associated with the UE and TRP measurements are implementation dependent. For UE-based mode, local UE errors/threats may be addressed by the UE when determining protection levels. 
For UE-assisted mode and network-based positioning, an LMF may address the UE and TRP local errors from UE and/or TRP measurement results. A specific method for determining local UE and TRP errors/threats is not specified as this is implementation defined.

Proposal 6: 	From RAN2 perspective, the Working Assumption can be confirmed: "It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results from UE and/or NG-RAN."

# 3.	Summary

In this contribution, we discussed some remaining aspects of integrity support for the NR Positioning Technologies. The following proposals are made.

Proposal 1:	RAN2 confirm the GNSS bounding model, including a mean and standard deviation (e.g., paired over-bounding Gaussian), is also used for integrity for NR Positioning Methods.

Proposal 2:	Analogous to GNSS, the mean and standard deviation of rate error bounds can optionally also be provided for the NR positioning error sources.

Proposal 3:	The DNU flags are provided per TRP and per error contribution (e.g., TRP location, RTD, beam information, etc.) in a new IE NR-Integrity-ServiceAlert.

Proposal 4:	The 'Integrity Correlation Times', defining the minimum time interval beyond which two sets of assistance data parameters for a given error can be considered to be independent from one another, can optionally be provided for the integrity assistance data.

Proposal 5:	From RAN2 perspective, the local errors/threats associated with the UE and TRP measurements are implementation dependent. For UE-based mode, local UE errors/threats may be addressed by the UE when determining protection levels. 
For UE-assisted mode and network-based positioning, an LMF may address the UE and TRP local errors from UE and/or TRP measurement results. A specific method for determining local UE and TRP errors/threats is not specified as this is implementation defined.

Proposal 6: 	From RAN2 perspective, the Working Assumption can be confirmed: "It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results from UE and/or NG-RAN."

# References

[1]		RP-230328, "Revised WID on Expanded and Improved NR Positioning".

[2]		RP-222982, "Status Report to TSG", RAN#98e.

[3]		R2-2301904, "Report from session on positioning and sidelink relay", Session Chair (MediaTek) 						(RAN2#121).

[4]		R2-2304204, "Report from session on positioning and sidelink relay", Session Chair (MediaTek) 						(RAN2#121bis-e).

[5]		R2-2304614 (R1-2304147), "Reply LS to RAN2 on error source distributions", RAN1.

[6]		3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[7]		J. Rife, S. Pullen, B. Pervan, and P. Enge, "Paired Overbounding and Application to GPS Augmentation", 			PLANS 2004. Position Location and Navigation Symposium.

[8]		3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[9]		R2-2110141, "Discussion on GNSS Integrity Assistance Data", Swift Navigation, Mitsubishi Electric 					Corporation, Intel Corporation, Ericsson.

[10]	R2-2303682, "Integrity of NR Positioning Technologies", Qualcomm Incorporated.