3GPP TSG-RAN WG2 #122	R2-2305830

May 22 – 26, 2023

Agenda Item:	7.5.3

Source:	Ericsson

Title:	Discussion on XR-specific power saving

Document for:	Discussion, Decision

# 1	Introduction

In this paper we discuss DRX enhancements for the following power saving objective in Rel-18 XR WID [1]:

Specify the enhancements related to power saving: - DRX support of XR frame rates corresponding to non-integer periodicities (through at least semi-static mechanisms e.g. RRC signalling) (RAN2).

# 2	Discussion on DRX Enhancements for XR

## 2.1 Align DRX to non-integer traffic periodicity

RAN2 #121-bis-e [5] made the following agreement:

RAN2 will not consider solution 3, i.e. multiple active DRX configurations as a solution to the non-integer periodicity for XR traffic, i.e. UE would have only one DRX configuration.

According to this agreement, this paper proposes to match the DRX cycle with the traffic period based on adding a integer time offset after a given number of cycles. The network provides these two parameters to the UE via RRC signalling:

- traffic_time_offset: indicates a fixed time shift for the start of drx-onDurationTimer;

- drx_offset: a number of cycles after which the new shift traffic_time_offset should be added.

With this solution, the DRX cycle length would be approximated to the next lower supported cycle value w.r.t. the non-integer traffic periodicity. The MAC formula to start the drx-onDurationTimer for Long DRX would then be modified to:

[(SFN × 10) + subframe number] modulo (drx-LongCycle) =

(drx-StartOffset + n × traffic_time_offset) modulo (drx-LongCycle).

Additionally, the following steps describe how to apply the formula above:

- initialize the following variables when DRX starts the first time: tmp_cycle=0 and drx_cycle=-1;

- if the condition in the MAC formula is true:

  - if (tmp_cycle+1) modulo drx_offset=0, then increment n and tmp_cycle;

  - otherwise, start drx-onDurationTimer and increment drx_cycle and assign tmp_cycle=drx_cycle.

The steps above ensure the following aspects:

- Counter n keeps track of how many times traffic_time_offset is accumulated in time over multiple cycles.

- Counters tmp_cycle and drx_cycle ensure that, if the accumulated traffic_time_offset (represented by n×traffic_time_offset) becomes larger than drx-LongCyle, this is properly dealt with in the modified formula.

The required changes to the 3GPP MAC specifications for this solution are shown in Annex A. Furthermore, although the pseudo-code of how to apply the modified DRX formula is shown above, this does not need to be introduced in the main procedural text of the specifications. Instead, the pseudo-code can be included in an annex. Moreover, we emphasize that pseudo-code for simple operations, e.g. incrementing counters, was omitted in the specifications also for other existing features, e.g. formula for CG Type 1 with a counter N that is incremented in every CG cycle [3].

The MAC formula to start the drx-onDurationTimer for Short DRX can be modified in the same way as for Long DRX above and it would have its own set of parameters. With this solution we do not foresee any impact on RAN1/4 specifications.

The proposed DRX alignment solution does not have any impact on RAN1/4 specifications.

The proposed solution to enhance the formula for DRX cycles has several advantages:

- This solution has a small impact in terms of specification changes and is close to the existing DRX behaviour. Namely, the proposed solution can be easily configured to work as current DRX with integer cycle lengths, by simply setting traffic_time_offset=0.

- The two new parameters traffic_time_offset and drx_offset are configured at RRC and have values in integers of milliseconds, thus having a low signalling overhead.

- This solution is flexible and supports many DRX alignments with the combination of these two new parameters (traffic_time_offset and drx_offset with integer values), even if only few values are specified for these parameters. Thus, it can achieve DRX alignments with currently known and, additionally, future XR non-integer periodicities (which may not yet be known).

- Can be applied in the same way for Long and Short DRX.

In contrast to the proposed alignment solution, other alignment solutions have the following disadvantages:

- Modifying the DRX formula by using cadences instead of cycle lengths: This solution deviates most from the legacy DRX behaviour. Specifically, configuring legacy DRX cycles would have to be treated differently than cadences. Thus, different procedures and additional signalling would be needed to indicate to the UE which formula/procedure to use to determine the start of the drx-onDurationTimer (legacy or new). Furthermore, with this solution, the exact traffic cadence value needs to be supported at RRC, which may currently be difficult to predict for future applications. Thus, the list of supported values would have to be updated in different future releases. This may cause issues regarding the UE capabilities according to different releases.

- Modifying the DRX formula by adding rational cycle values: This solution introduces rational cycle values, which are signalled from the network to the UE via two integer parameters (dividend and divisor). If this solution is adopted, the range of supported values at RRC for the dividend and divisor has to be large enough to cover currently known and potential future XR traffic periodicities.

Additionally, the UE and network implementation should ensure that the rounding to an integer in milliseconds, due to the floor or modulo operation on a rational value, is performed correctly in the DRX formula. More specifically, in [7] it was suggested to modify the DRX formula as:

floor [n – [drx-LongCycle × floor (n / drx-LongCycle) ] ] = drx-StartOffset,

where n = [(SFN × 10) + subframe number].

For the example of 60 fps, the following configuration is applied {dividend=1000, divisor=60}. Also, let’s assume n=1050. In MATLAB we observe different results for the following two cases, which should in fact be equivalent:

floor(1050 -(dividend/divisor*floor(1050/(dividend/divisor)))) = 16  rounding problem

floor(1050 -(dividend/divisor*floor(1050/dividend*divisor))) = 0  correct.

Furthermore, if n=2050, in MATLAB we observe:

floor(2050 -(dividend/divisor*floor(2050/(dividend/divisor)))) = 16  rounding problem

floor(2050 -(dividend/divisor*floor(2050/dividend*divisor))) = 16  rounding problem.

If another DRX formula variant is used as proposed in [8], namely floor(n modulo drx-LongCycle) = drx-StartOffset, in MATLAB we observe:

floor(2050 modulo (dividend/divisor)) = 0  correct.

Thus, it should be ensured that the formula and its implementation and precision of the variables are carefully chosen to avoid rounding errors.

- Introducing DRX cycle patterns or DRX offset patterns: These solutions introduce more significant changes in the way that DRX works. Namely, their behaviour is equivalent to employing two levels of nested cycles. This may have a broader specification impact, potentially affecting features that are configured based on Long DRX cycle length and offsets. Examples of such features are Short DRX and wake-up signal based on DCI format 2-6. Additionally, for some values of fps, long cycle or offset patterns may be needed.

Enhance Long and Short DRX formulas to match non-integer XR traffic periods as described in this section, by adding two new parameters: (i) a fixed time shift for the start of drx-onDurationTimer; and (ii) a number of DRX cycles after which the new shift should be added.

The DRX alignment solution proposed above works with legacy Long/Short DRX cycle lengths. However, to obtain very tight alignment of DRX cycles with the traffic periodicity, it is useful to introduce integer cycle lengths that are close to expected non-integer traffic periodicities. For instance, for the frame generation rates of {15, 30, 45, 60, 72, 90, 120} fps it is useful for the specs to support the DRX cycle values of {8, 9, 11, 13, 16, 22, 33, 66} ms. We note that some of these values are already supported for Short DRX. To obtain tight alignments also with future traffic periodicities that are not yet known, it would be useful to support the entire range of integer values from, e.g., 8 to 70 ms.

New integer values in ms for DRX cycle lengths (e.g. {8, 9, 11, …, 13…, 16, 22, …33, …, 66} ms) are introduced.

## 2.2 SFN wrap-around enhancement for C-DRX

RAN2 #121-bis-e [5] made the following agreement:

To address SFN wrap around, it is proposed to adopt option with a counter in DRX formula that increments at every SFN wrap around and an DRX reference SFN signalled by network. FFS if this is based on H-SFN, E-SFN or a generic counter.

This section addresses this agreement and proposes an SFN wrap-around solution. Namely, a simple modification of the DRX formula can be made, by introducing a generic counter m:

[((1024×m + SFN) × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset,

where m=0 when DRX is activated and is incremented every time SFN=0.

This SFN wrap-around solution requires only minimal MAC specification changes (see Annex B) and can be easily integrated into the proposed DRX alignment solution in Section 2.1, or another DRX alignment solution based on rational cycle lengths, or patterns of start offsets. With this solution, no impact is foreseen on RAN1/4 specifications.

The proposed SFN wrap-around solution does not have any impact on RAN1/4 specifications.

Let us now discuss why it is useful to use a generic counter m, instead of H-SFN or E-SFN.

- If m is a generic counter, it can be initialized when DRX is activated and can be increased for a long time, ensuring that no interruption occurs during the XR session. This can be ensured by, e.g. using a sufficient number of bits for m. For instance, if m is represented on 16 bits, m wraps around 10240*65536 ms = 186.4 hours after DRX was configured, most likely covering the duration of an XR session. Moreover, a similar approach with a counter N that keeps increasing has also been specified for CG Type 1 in [3], so this overall approach of an increasing counter is known in 3GPP specifications:

After an uplink grant is configured for a configured grant Type 1, the MAC entity shall consider sequentially that the Nth (N >= 0) uplink grant occurs in the symbol for which: [(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] = (timeReferenceSFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

- If H-SFN (hyper SFN) is used in the DRX formula instead of the generic counter m, there is a risk of a DRX misalignment as in the original SFN wrap-around problem. This could happen since H-SFN is a parameter that is sent by the network in the SIB1 message and has a range from 0 to 1023 (10 bits) [6]. If DRX is configured close to the H-SFN wrap-around, a wrap-around misalignment may occur. Moreover, H-SFN was specified as an optional parameter in Rel-17 (hyperSFN-r17) for eDRX (used for paging in idle/inactive RRC modes), so it is not always sent by the network.

- Instead of the generic counter m, some companies proposed to introduce a counter E-SFN. Although E-SFN is also a type of generic counter, the difference is that it was proposed that E-SFN has a configurable maximum value, e.g. 9, after which it wraps around. If such a feature is adopted, the maximum value of E-SFN needs to be selected based on the DRX cycle length (and implicitly on the traffic periodicity), to ensure there is no E-SFN wrap-around problem. Namely, the time after which E-SFN wraps around needs to be a multiple of the DRX cycle length. Thus, using E-SFN is more complicated and requires more careful configuration based on traffic characteristics, compared to the simpler solution of a generic counter m that keeps increasing, as proposed above.

Solve the SFN wrap-around problem in the DRX formula, by introducing a generic counter which increments every time that SFN wraps around.

In order to solve ambiguities about when to initialize m=0, the network could indicate to the UE via RRC a specific reference SFM in which m should be initialized. For this, RRC specification changes are required to introduce this new parameter (specific reference SFN), while in the MAC specifications some short procedural text is sufficient and no further changes are required in the DRX formula. Namely, the reference SFN does not need to be introduced in the DRX formula, unlike the solution for Rel-16 IIoT CG Type 1 above. Nonetheless, the definition of the reference SFN for DRX can be similar to that of the reference SFN for CG Type 1 [3]: “timeReferenceSFN: SFN used for determination of the offset of a resource in time domain. The UE uses the closest SFN with the indicated number preceding the reception of the configured grant configuration.”

Initialize the generic counter for SFN wrap-around solution based on a specific reference SFN value sent from network to UE via RRC.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The proposed DRX alignment solution does not have any impact on RAN1/4 specifications.

Observation 2	The proposed SFN wrap-around solution does not have any impact on RAN1/4 specifications.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Enhance Long and Short DRX formulas to match non-integer XR traffic periods as described in this section, by adding two new parameters: (i) a fixed time shift for the start of drx-onDurationTimer; and (ii) a number of DRX cycles after which the new shift should be added.

Proposal 2	New integer values in ms for DRX cycle lengths (e.g. {8, 9, 11, …, 13…, 16, 22, …33, …, 66} ms) are introduced.

Proposal 3	Solve the SFN wrap-around problem in the DRX formula, by introducing a generic counter which increments every time that SFN wraps around.

Proposal 4	Initialize the generic counter for SFN wrap-around solution based on a specific reference SFN value sent from network to UE via RRC.

# References

3GPP RP-230786, “Updated WID on XR Enhancements for NR”, Mar. 2023.

3GPP TR 38.835, “Study on XR enhancements for NR”, V1.0.0, Dec. 2022.

3GPP TS 38.321, “Medium Access Control (MAC) protocol specification”, V17.0.0, Mar. 2022.

R2-1903001, “Report of 3GPP TSG RAN WG2 meeting #105 Athens, Greece, 25 February - 1 March, 2019”.

R2-2304202, “Report on LTE legacy, XR, QoE and MUSIM”.

3GPP TS 38.331

R2-2301372, “C-DRX enhancements for XR”, MediaTek Inc., Feb-Mar. 2023.

R2-2302514, “DRX enhancements for XR”, Qualcomm Incorporated, MediaTek, CATT, vivo, NEC, Apr. 2023.

# Annex A: Proposed modifications to integrate DRX alignment solution in 3GPP specifications

We show how to modify the 3GPP MAC specifications in [3] to capture the proposed DRX alignment solution in Section 2.1, as follows. The proposed modifications are marked in red.

1> if the Long DRX cycle is used for a DRX group, and [SFN × 10 + subframe number] modulo (drx-LongCycle) = (drx-StartOffset + n × traffic_time_offset) modulo (drx-LongCycle) (See Annex … for details on how to use the formula.)

# Annex B: Proposed modifications to solve SFN wrap-around problem in 3GPP specifications

We show how to modify the 3GPP MAC specifications in [3] to capture the SFN wrap-around solution in Section 2.2, as follows. The proposed modifications are marked in red, and m is initialized to zero.

1> if SFN=0

2> m = m+1

1> if the Long DRX cycle is used for a DRX group, and [(1024×m + SFN) × 10 + subframe number] modulo (drx-LongCycle) = drx-StartOffset

Below we show how to integrate the proposed SFN wrap-around solution into the proposed DRX alignment solution in Annex A. The proposed modifications are marked in red.

1> if SFN=0

2> m = m+1

1> if the Long DRX cycle is used for a DRX group, and [(1024×m + SFN) × 10 + subframe number] modulo (drx-LongCycle) = (drx-StartOffset + n × traffic_time_offset) modulo (drx-LongCycle) (See Annex … for details on how to use the formula.)