3GPP TSG-RAN WG2 Meeting #122	R2-2305836

# Incheon, South Korea, May 22 – 26, 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0917 | rev | - | Current version: | 15.20.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on SRS Tx switching capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | 15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN2#121-bis-e, the following was agreed: RAN2 confirms the following behaviour for the parameters txSwitchImpactToRx and txSwitchWithAnotherBand in srs-TxSwitch: - - Bands with UL that impact each other define a group (i.e. SRS TX switching on any of the cells will impact UL on all the cells in the group). All the band entries in the group will signal the same group identifier in txSwitchWithAnotherBand. The first-listed band entry number in the group shall be used as identifier for the group. An UL group with only one band entry is not signaled in txSwitchWithAnotherBand. - - For bands where the DL is impacted by an UL group with a single band entry, txSwitchImpactToRx shall indicate the band entry number of that UL band. For bands where the DL is impacted by an UL group with more than one band entry, txSwitchImpactToRx shall point to the UL group using the group identifier number (as defined by txSwitchWithAnotherBand). However, this is currently not clear from the field descriptions in 38.306, since no clear mentioning to grouping is made. Hence a clarification to the grouping as agreed in RAN2#121-bis-e is added to the field descriptions of txSwitchImpactToRx and txSwitchWithAnotherBand in srs-TxSwitch. Impact analysis Impacted 5G architecture options: NR SA, NR-DC Impacted functionality: UE radio capability Inter-operability: - If the UE is implemented according to the CR and the network is not, the network may misjudge the impacted bands, which may lead to performance degradation. - If the network is implemented according to the CR and the UE is not, the UE will improperly set the impacted bands, which may lead to performance degradation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 4.2.7.1 BandCombinationList parameters - Clarified the grouping as agreed in RAN2#121-bis-e in the field descriptions of txSwitchImpactToRx and txSwitchWithAnotherBand. - Clarified that a band with value ‘notSupported’ indicated in supportedSRS-TxPortSwitch could still be impacted by another band using SRS TX swtiching and thus can include the parameters txSwitchImpactToRx / txSwitchWithAnotherBand. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There may be different interpretations on how to set the fields txSwitchImpactToRx and txSwitchWithAnotherBand in srs-TxSwitch. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

| srs-TxSwitch Defines whether UE supports SRS for DL CSI acquisition as defined in clause 6.2.1.2 of TS 38.214 [12]. The capability signalling comprises of the following parameters: - supportedSRS-TxPortSwitch indicates SRS Tx port switching pattern supported by the UE. The indicated UE antenna switching capability of ′xTyR′ corresponds to a UE, capable of SRS transmission on ′x′ antenna ports over total of ′y′ antennas, where ′y′ corresponds to all or subset of UE receive antennas, where 2T4R is two pairs of antennas; - txSwitchImpactToRx indicates the identifier of the UL group (see txSwitchWithAnotherBand) that impacts the DL of this band entry. For bands where the DL is impacted by an UL group with a single band entry, txSwitchImpactToRx shall indicate the band entry number of that UL band. For bands where the DL is impacted by an UL group with more than one band entry, txSwitchImpactToRx shall indicate the UL group using the group identifier number (as defined by txSwitchWithAnotherBand). - txSwitchWithAnotherBand indicates the identifier of the UL group, which is defined as band entries with UL (see NOTE) that impact each other’s UL (i.e. SRS TX port switching on any of the cells in the group will impact UL on all the cells in the group). All the band entries in the band combination belonging to the same UL group will include txSwitchWithAnotherBand set to the same identifier number. The lowest band entry number in the group shall be used as identifier for the group. This parameter is absent if an UL group contains only one band entry. For txSwitchImpactToRx and txSwitchWithAnotherBand, value 1 means first entry, value 2 means second entry and so on. The UE may include txSwitchImpactToRx and txSwitchWithAnotherBand for a band even if supportedSRS-TxPortSwitch is set to ‘notSupported’ for that band. All DL and UL that switch together indicate the same entry number. The entry number is the band entry number in a band combination. The UE is restricted not to include fallback band combinations for the purpose of indicating different SRS antenna switching capabilities. NOTE: The band with UL includes a band associated with FeatureSetUplinkId set to 0 corresponding to the support of SRS-SwitchingTimeNR. | BC | Yes | N/A | N/A |
| --- | --- | --- | --- | --- |

END OF CHANGE