3GPP TSG-RAN WG2 #122	Tdoc R2-2305839

Incheon, South Korea, May 22 – 26, 2023

Agenda Item:	5.1.3.2

Source:	Ericsson

Title:	Handling of per-UE parameters for NR-DC

Document for:	Discussion, Decision

# 1	Introduction

For parameters defined per band combination, 38.331 has the following clarification on how to handle what the UE supports for NR-DC:

CA-ParametersNRDC field descriptions

ca-ParametersNR-forDC (with and without suffix) If this field is present for a band combination, it reports the UE capabilities when NR-DC is configured with the band combination. If a version of this field (i.e., with or without suffix) is absent for a band combination, the corresponding ca-ParametersNR field version in BandCombination is applicable to the UE configured with NR-DC for the band combination. If a version of this field (i.e., with or without suffix) is present for a band combination but does not contain any parameters, the UE does not support the corresponding field version when configured with NR-DC for the band combination.

However, for per-UE capabilities, it is ambiguous how to interpret absence or presence of parameters and hence it remains ambiguous which features a UE supports when configured with NR-DC. This contribution further discusses this issue and proposes a way forward.

# 2	Discussion

According to 38.331, the parameters applicable only to NR-DC are contained in the NRDC-Parameters IE, as described below.

#### –	NRDC-Parameters

The IE NRDC-Parameters contains parameters specific to NR-DC, i.e., which are not applicable to NR SA.

NRDC-Parameters information element

-- ASN1START

-- TAG-NRDC-PARAMETERS-START

NRDC-Parameters ::=                 SEQUENCE {

measAndMobParametersNRDC            MeasAndMobParametersMRDC                    OPTIONAL,

generalParametersNRDC               GeneralParametersMRDC-XDD-Diff              OPTIONAL,

fdd-Add-UE-NRDC-Capabilities        UE-MRDC-CapabilityAddXDD-Mode               OPTIONAL,

tdd-Add-UE-NRDC-Capabilities        UE-MRDC-CapabilityAddXDD-Mode               OPTIONAL,

fr1-Add-UE-NRDC-Capabilities        UE-MRDC-CapabilityAddFRX-Mode               OPTIONAL,

fr2-Add-UE-NRDC-Capabilities        UE-MRDC-CapabilityAddFRX-Mode               OPTIONAL,

dummy2                              OCTET STRING                                OPTIONAL,

dummy                               SEQUENCE {}                                 OPTIONAL

}

The highlighted description above does not clarify whether any of the fields signaled for NR SA are also applicable for NR-DC, or whether the IE above overrides any capability signaled for NR SA. For instance, the field independentGapConfig is present in both

UE-NR-Capability->measAndMobParameters -> independentGapConfig

and in

UE-NR-Capability->nrdc-Parameters->measAndMobParametersNRDC -> independentGapConfig

It is not clear if the same logic as for ca-ParametersNR-forDC should be applied (see highlighted part in field description in Introduction section). In other words, it is unclear whether a UE that supports NR-DC and that sets “UE-NR-Capability->measAndMobParameters -> independentGapConfig” but omits the entire “UE-NR-Capability->nrdc-Parameters->measAndMobParametersNRDC” supports independent gaps when running NR-DC. According to the rules for “ca-ParametersNR-forDC” it would have to support it. But we did not find any statement in the specification confirming this.

It should be noted that the fields “UE-NR-Capability->measAndMobParameters” and for UE-NR-Capability->nrdc-Parameters->measAndMobParametersNRDC” do not use the same ASN.1 types. The latter contains only the fields for which a “per-DC” distinction is allowed. The same applies to other branches of the UE-NR-Capability and the NR-DC-Parameters. Naturally, the capabilities that occur only in the main branch are true “per-UE capabilities” whereas the UE may differentiate for the ones in NR-DC-Parameters whether or not it supports them in NR-DC, too.

In the following we discuss only the handling of the capability bits that occur both inside and outside the NR-DC-Parameters (not the true per-UE capabilities).

For fields that can be signaled both inside and outside NRDC-Parameters IE (e.g. as independentGapConfig), a rule should be defined on how to derive the UE support for these features when the UE is configured with NR-DC. There could be two interpretations for capability parameter that exist both inside and outside of NR-DC-Parameters:

- The capability parameter outside NR-DC-Parameters determines whether the UE supports the feature when not configured with NR-DC, and
the capability parameter inside NR-DC-Parameters determines whether (or not) the UE supports the feature when configured with NR-DC
.

- The same logic as for ca-ParametersNR-forDC is used: If a field is present in NRDC-Parameters but does not contain any parameters, the UE does not support the corresponding field when configured with NR-DC.
If a sequence of capability parameters (e.g. measAndMobParameters) is present but the corresponding sequence in NR-DC-Parameters (e.g. measAndMobParametersNRDC) is not, the capability parameters in the former determine the feature support for both configurations with and without NR-DC, 
else, If a sequence of capability parameters (e.g. measAndMobParameters) is present and the corresponding sequence in NR-DC-Parameters (e.g. measAndMobParametersNRDC) is present, too, the capability parameters in the former determine the feature support for configurations without NR-DC whereas the capability parameters in the latter determine the feature support for configurations with NR-DC.

As the outlined rules show, the first interpretation is simpler. It also avoids ambiguities that could arise with the second interpretation since the “parent IEs” in the two branches are not of the same IE type and that it is hence not obvious absence of which “container” results in implicit support according to the per-UE capability bit. Most importantly, though,  inter-operability issues are more likely and more severe with interpretation 2: if the network adopts interpretation 2 and the UE adopted interpretation 1, the network may attempt to configure the UE in NR-DC with a feature that the UE does not support in NR-DC, leading to configuration failure. If the network adopts interpretation 1 and the UE adopted interpretation 2, the network may unnecessarily refrain from configuring the UE in NR-DC with a feature that the UE would have support. Obviously, the latter is far less severe.

Clarify in 38.306 that capability parameters that exist both inside and outside of NR-DC-Parameters:
the capability parameter outside NR-DC-Parameters determines whether (or not) the UE supports the feature when not configured with NR-DC, and
the capability parameter inside NR-DC-Parameters determines whether (or not) the UE supports the feature when configured with NR-DC.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Clarify in 38.306 that capability parameters that exist both inside and outside of NR-DC-Parameters: the capability parameter outside NR-DC-Parameters determines whether (or not) the UE supports the feature when not configured with NR-DC, and the capability parameter inside NR-DC-Parameters determines whether (or not) the UE supports the feature when configured with NR-DC.