3GPP TSG-RAN WG2 #122	Tdoc R2-2305843

Incheon, South Korea, May 22 – 26, 2023

Agenda Item:	7.25.1

Source:	Ericsson

Title:	Support of lower MSD capability

Document for:	Discussion, Decision

# 1	Introduction

An LS was received from RAN4 (R4-2306594) on lower maximum sensitivity degradation (MSD) capability, i.e. certain group of bands for which the UE can support lower degradation than the values defined in TS 38.101-1/3 clause 7 for CA/DC respectively. This contribution discusses initial signaling aspects and which feedback RAN2 should give to RAN4.

# 2	Discussion

- A UE should be allowed to report the low MSD capability for any MSD mechanism and order that have been defined in the 3GPP specifications for a given band combination.

- No dynamic signaling scheme will be introduced for lower MSD report in Rel-18

- Define one set of absolute multiple thresholds for lower MSD

  - Respective MSD indication of corresponding threshold(s) are reported with one of the thresholds listed in the set

- Lower MSD capability for higher order combination is inherited from lower order fallback combinations

  - For 2-bands combination, the MSD indication of corresponding threshold (or capability class) are supposed to be reported separately as per victim band per MSD type per band combination

  - For 3-bands combination, the MSD indication of corresponding threshold (or capability class) are only reported for IMD of dual UL falling into the third band DL whose UL band is not configured with, other kinds of Lower MSD capability (harmonic/ harmonic mixing/cross band isolation/IMD due to dual UL falling into own DL) could inherit from 2-band combinations with the same power class.

  - For combination with more than 3 bands, no need to report the Lower MSD capability any more, the capability could inherit from the fallback combinations with the same power class.

- UE is allowed to report lower MSD capability for different power classes

The RAN4 LS states the following:

However, it should be noted that the UE capability signaling is designed such that a feature cannot be derived from multiple band combinations, i.e. a feature that requires per band combination signaling is indicated separately for each band combination.

RAN2 also confirmed this behaviour in RAN2#113-e and have sent an LS to RAN4 by that time (R2-2102495).

In addition, RAN2 concluded that the UE capability signalling does not account for the indication of support of a feature that needs to be derived from multiple band combinations. RAN2 kindly asks RAN4 to inform RAN2 if there is any further cases need to be covered from RAN4 perspective.

RAN2 already concluded that the UE capability signalling does not account for the indication of support of a feature that needs to be derived from multiple band combinations.

Hence, it would be worth to remind RAN4 of the RAN2 conclusion, also pointing out that the scheme related to “inheritance of a lower order band combination” would not be in line with the RAN2 signaling and thus explicit signaling may also be required for the case with more than 3 bands.

Reply to RAN4 that “inheritance of a lower order band combination” would not be in line with the RAN2 signaling and thus explicit signaling may also be required for the case with more than 3 bands.

Furthermore, the current LS seems to focus on cases of 2 bands and 3 bands, i.e. an UL band (or two UL bands) that affect a certain DL band, where the UE may want to indicate support of lower MSD capability. Despite this focus on a small set of bands, it is beneficial to design the signaling such that it would not require the UE to include an explicit fallback band combination to indicate support of this feature. As RAN4 points out, there are many factors and group of bands that could result in different MSD values, which could easily fill in the UE capability size if fallback band combinations would be explicitly reported for the sake of indicating different lower MSD capabilities.

Lower MSD capability design should not imply in the inclusion of fallback band combinations for the purpose of indicating different MSD capability.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Reply to RAN4 that “inheritance of a lower order band combination” would not be in line with the RAN2 signaling and thus explicit signaling may also be required for the case with more than 3 bands.

Proposal 2	Lower MSD capability design should not imply in the inclusion of fallback band combinations for the purpose of indicating different MSD capability.