3GPP TSG-RAN WG2 Meeting #122	R2-2305846

Incheon, Korea, May 22-26, 2023

Agenda item:		6.3.1

Source:		Lenovo

Title:		Reception of PC5 release message during re-establishment

Document for:		Discussion and Decision

- Introduction

In this contribution, we would like to discuss one issue related to the reception of PC5 release message with re-establishment in Rel-17 L2 U2N relay case.

- Discussion

In Rel-17 L2 U2N relay, The NotificationMessageSidelink is triggered due to Uu RLF of relay UE, HO of relay UE, cell Reselection of relay UE, or RRC resume Failure. The events of Uu RLF and HO of relay UE are related to connected state. The U2N Remote UE shall initiate the RRC connection re-establishment procedure upon receiving the NotificationMessageSidelink if T301 is not running.

5.8.9.10.4	Actions related to reception of NotificationMessageSidelink message

Upon receiving the NotificationMessageSidelink, the U2N Remote UE shall:

1>	if the indicationType is included:

2>	if the UE is L2 U2N Remote UE in RRC_CONNECTED:

3>	if T301 is not running, initiate the RRC connection re-establishment procedure as specified in 5.3.7;

2>	else (the UE is L3 U2N Remote UE, or L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE):

3>	if the PC5-RRC connection with the U2N Relay UE is determined to be released:

4>	indicate upper layers to trigger PC5 unicast link release;

3>	else (i.e., maintain the PC5 RRC connection):

4>	if the UE is L2 U2N Remote UE and the indicationType is relayUE-HO or relayUE-CellReselection:

5>	consider cell re-selection occurs;

Observation 1: Based on 5.8.9.10.4, the L2 U2N Remote UE in RRC_CONNECTED shall initiate the RRC connection re-establishment procedure upon receiving the NotificationMessageSidelink if T301 is not running.

5.3.7.7	T301 expiry or selected cell/L2 U2N Relay UE no longer suitable

The UE shall:

1>	if timer T301 expires; or

1>	if the selected cell becomes no longer suitable according to the cell selection criteria as specified in TS 38.304 [20]; or

1>	if the (re)selected L2 U2N Relay UE becomes unsuitable; or

1>	upon receiption of NotificationMessageSidelink indicating relayUE-HO or relayUE-CellReselection:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

Observation 2: Based on 5.3.7.7 of TS38.331, the L2 U2N Remote UE shall perform the actions upon going to RRC_IDLE upon receiption of NotificationMessageSidelink indicating relayUE-HO or relayUE-CellReselection if T301 is running.

According to Rel-17 L2 U2N relay, relay UE can decide to transmit NotificationMessageSidelink or PC5 unicast link release. In current specification, UE shall initiate the re-establishment procedure upon PC5 unicast link release indicated by upper layer at L2 U2N Remote UE in RRC_CONNECTED. Therefore, the condition of T301 is missing for the case of the reception of PC5 unicast link release from relay UE. Upon PC5 unicast link release indicated by upper layer at L2 U2N Remote UE in RRC_CONNECTED, UE shall initiate the re-establishment procedure if T301 is not running. L2 U2N Remote UE shall enter RRC_IDLE upon PC5 unicast link release indicated by upper layer if T301 is running.

Proposal 1: Upon PC5 unicast link release indicated by upper layer, L2 U2N Remote UE shall initiate the re-establishment procedure if T301 is not running.

Proposal 2: L2 U2N Remote UE shall enter RRC_IDLE upon PC5 unicast link release indicated by upper layer if T301 is running.

- Conclusion

In this contribution, we have the following observations and proposals:

Observation 1: Based on 5.8.9.10.4, the L2 U2N Remote UE in RRC_CONNECTED shall initiate the RRC connection re-establishment procedure upon receiving the NotificationMessageSidelink if T301 is not running.

Observation 2: Based on 5.3.7.7 of TS38.331, the L2 U2N Remote UE shall perform the actions upon going to RRC_IDLE upon receiption of NotificationMessageSidelink indicating relayUE-HO or relayUE-CellReselection if T301 is running.

Proposal 1: Upon PC5 unicast link release indicated by upper layer, L2 U2N Remote UE shall initiate the re-establishment procedure if T301 is not running.

Proposal 2: L2 U2N Remote UE shall enter RRC_IDLE upon PC5 unicast link release indicated by upper layer if T301 is running.

References

TS38.331 NR Radio Resource Control (RRC) protocol specification