3GPP TSG-RAN WG2#122	R2-2305865

Incheon, Korea, 22-26 May 2023

Agenda Item:	7.24.2

Source: 	MediaTek Inc.

Title:  	Downlink positioning performance results for remote UEs out of coverage

Document for:	Information

# 1	Introduction

This document presents some performance results showing that downlink positioning is not greatly degraded when used with a remote UE out of direct Uu coverage.  It is intended as a companion paper providing background information for the evaluation of the proposals in [1].

# 2	Discussion

## 2.1	Assumptions

The results below are derived from the well-known 19-cell layout used for simulations in RAN1.  The assumed environment is shown in figure 1.

Figure 1: Diagram of simulation setting

We compare a UE in direct service (located at site 0: blue in the figure) with a UE served by site 0 but located at site 2 (green in the figure), using the same assistance data.  The assistance data provide configurations for the 19 cells shown, but the UE at site 2 is offset from the centre of the diagram and experiences a different radio environment for DL-PRS reception compared to the UE at the nominal centre.

The UEs perform DL-TDOA positioning, with no sync error assumed between base stations and with the LOS link selected for position calculation.  The ISD in figure 1 is normalised to 1; the results in the next section assume a street canyon scenario with an ISD of 200 m.

## 2.2	Results

The simulation results are shown in figure 2.

Figure 2: Results of DL-TDOA positioning

With no compensation for the choice of assistance data, the remote UE experiences a degradation of positioning on the order of 0.5 m at the 90% level.

The results above do not assume an actual coverage hole at site 2, which could distort the performance of positioning especially for the remote UE.  However, disabling DL-PRS transmission at site 2 makes only a very small difference in the positioning accuracy (0.04 m at the 90% level, and less than a 2% increase in the chance of experiencing an error of 4 m or above), as shown in figure 3.

Figure 3: Remote UE positioning performance with and without coverage at site 2

Note that, with enhancements to the assistance data, these performance results could be expected to improve.  If the LMF provides assistance data for additional cells “below” (in the diagram orientation) the ones shown in figure 1, the remote UE can have a significantly increased population of sites from which to measure DL-PRS; this is feasible in case the LMF is provided with a remote UE indication.  Similarly, a network that intended to cater for UEs in relaying environments could expand the number of cells for which it provides assistance data in the posSIBs; this is a deployment choice, allowing the network to balance the tradeoff between assistance data size and positioning performance for remote UEs.

# 3	Conclusion

This document does not need to be evaluated for a conclusion; it provides background information indicating that downlink positioning performance is not badly degraded for remote UEs out of coverage, even without assuming enhancements to the assistance data.

# 4	References

[1]	R2-235850, “Positioning and posSIB forwarding for remote UEs”, MediaTek Inc. / CATT / Huawei / HiSilicon / Qualcomm Incorporated / Xiaomi / Intel Corporation / vivo, RAN2#122