3GPP TSG-RAN WG2 Meeting #122                                                      R2-2305872

Incheon, KR, May 22-26, 2023

Source:             CATT, Turkcell

Title:	             CHO enhancement for NES

Agenda Item:	7.3.5

Document for:	Discussion and Decision

Introduction

This contribution will discuss CHO enhancement based on below agreements and three questions in RAN2#121bis.

Agreements

-	RAN2 agree to make enhancement in CHO procedure based on that the source cell entering “NES mode”.  FFS further details

-	For source cell CHO framework, RAN2 assumes a reference scenario where the UE has already performed CHO conditions evaluation by the time the source cell starts some “NES-mode”

-	As a baseline, UE initiates CHO evaluation upon receiving the CHO configuration.  FFS what trigger is used for execution of CHO

Question

Proposal 7: [21/ 25] Legacy measurement events A3, A4 and A5 are considered as NES CHO conditions.

Proposal 8: RAN2 to discuss following two options to ensure that the UE can find a suitable target cell:

-	Option 1: [11/ 25] NW implementation to reconfigure candidate cells, i.e., no spec impact

-	Option 2: [9/ 25] Network provides additional prioritization for candidate cells that UE can take into consideration.

Proposal 9: [13/ 25] RAN2 further discuss if legacy connection re-establishment is an appropriate way to handle the case when no suitable candidate it found, and the source cell is about to sleep.

Discussion

  - Enhancement for source cell

According to the agreements, the CHO procedure should be:

- UE initiates CHO evaluation upon receiving the CHO configuration;

- UE has already performed CHO conditions evaluation by the time the source cell starts some “NES-mode”;

- UE performs CHO execution as soon as the CHO conditions are met, or when some additional specific trigger is received.

Basically, we support the “Proposal 7: [21/ 25] Legacy measurement events A3, A4 and A5 are considered as NES CHO conditions.” in [1]. However, CondEvent A3, A5 cannot be applied directly for cell DTX/DRX and cell switch off because the channel quality of the source cell is not the reason for handing over the UEs. Then gNB would hand over the UEs by explicit handover command or CHO procedure with CondEvent A4. Note that CondEvent A4 is used for non-terrestrial network only in current specification [2]. So there are two options: define a new conditional event for the UE or reuse CondEvent A4. There is no obvious issues that prevent CondEvent A4 to be used for a CHO preceding a cell entering Cell DTX/DRX or switching off.

Observation 1: For both Cell switch-off and Cell DTX/DRX, the specific aspect of this CHO is that the UEs to be hand-off’ed are in good channel conditions, hence CondEvent A3, A5 cannot be applied directly.

Observation 2: No obvious issues prevent CondEvent A4 to be used for a CHO preceding a cell entering Cell DTX/DRX or switching off.

Proposal 1: CondEvent A4 should be applied in terrestrial network as the CHO enhancement for NES.

Now the remaining issue is “FFS what trigger is used for execution of CHO”.

For cell switch-off, if gNB decides to switch-off a cell, it must be light-load with small number of connected UEs. So gNB can hand over the UEs before switching off the cell. L1/L2 triggered CHO was mentioned for turning off the cell faster. This method is not really usable because gNB must confirm all UEs are handed over before switch off the cell. Otherwise, the UEs’ performance will be severely impaired.

Observation 3: For the Cell switch-off scenario, those UEs that have not yet hand-off’ed to another cell will be severely impaired when the cell switches off.

For Cell DTX/DRX, gNB should configure Cell DTX/DRX by dedicated RRC signaling to the UEs which C-DRX on-durations are aligned with the Cell DTX/DRX active period (suitable UEs). And simultaneously, CHO can be configured to the UEs which DRX on-durations are not aligned with the Cell DTX/DRX or which are not Cell DTX/DRX capable (unsuitable UEs). It was proposed that the L1/L2 signaling for Cell DTX/DRX activation can be used to trigger CHO simultaneously. However, this would mean that the unsuitable UEs are still in the cell (executing the CHO command) while Cell DTX/DRX is activated. Obviously, the transmission performance of such unsuitable UEs will be impaired similar to the use case of cell switch off.

Observation 4: Similarly, the unsuitable UEs that have not yet hand-off’ed to another cell will be severely impaired when the cell enters Cell DTX/DRX mode

Hence we get below observation.

Observation 5: Similar to the CHO conditions evaluation, the CHO execution should be performed before the cell enters Cell DTX/DRX or switches off.

Proposal 2: As legacy, the CHO execution should be triggered by the successful CHO condition evaluation, and no additional trigger is required.

  - Enhancement for target cell

The enhancement for target cell was discussed in [1] as below.

Proposal 8: RAN2 to discuss following two options to ensure that the UE can find a suitable target cell:

-	Option 1: [11/ 25] NW implementation to reconfigure candidate cells, i.e., no spec impact

-	Option 2: [9/ 25] Network provides additional prioritization for candidate cells that UE can take into consideration.

The target cell should meet the requirements: 1) a cell that can satisfy the UE’s transmission requirements; and 2) the entering UE has minimal impact on the cell’s energy saving state.

Considering the NES techniques in the NES WID:

- SSB-less SCell operation: The SSB-less SCell should not be a target PCell as now. It is not related to the CHO enhancement.

- Cell DTX/DRX: There are two options: the cell with Cell DTX/DRX configuration should not be a target cell; the cell with Cell DTX/DRX configuration can accommodate the UEs for which the UE DRX can be aligned with Cell DTX/DRX of the target cell. In any case, UE cannot get the Cell DTX/DRX state of the candidate target cell timely. RAN2 agreed “It is up to NW whether legacy UEs can access cells with Cell DTX/DRX” in [3]. And RAN3 agreed that Cell DTX/DRX configuration can be transmitted between neighbor cells. Since source gNB can get the Cell DTX/DRX configuration of neighbor cells, it should be decided by gNB implementation whether to set a neighbor cell with Cell DTX/DRX configuration as a candidate cell.

- NES in spatial/power domain: If a UE connects to a cell with finer beam/power management, the cell needs to provide beam/power for the new UE. It will increase the cell’s energy consumption. The source cell should avoid setting such cell as a candidate cell. Even if source cell has to consider it as a candidate cell (such as no other cell can be used), it should be decided by NW implementation.

Proposal 3: It is up to NW implementation whether to set a NES cell as a candidate cell. No specification change is needed.

  - Failure case

Failure case was mentioned in [1] as below.

Proposal 9: [13/ 25] RAN2 further discuss if legacy connection re-establishment is an appropriate way to handle the case when no suitable candidate it found, and the source cell is about to sleep.

There can be two cases where the UEs cannot hand-off:

- The CHO execution condition is never met, in such case, the UE stays in the cell, the network is aware of it and can decide whether to switch-off/turn-on Cell DTX/DRX anyways or to keep the cell “on” for that UE or to update the CHO condition.

- The CHO execution condition is met, so the HO is executed but fails. At HO failure, the UE performs re-establishment, that is, cell selection and if the selected cell is a CHO candidate then the UE applies the CHO configuration of the cell.

How to handle the UE in Case 1) can be fully left to gNB implementation and nothing more is needed. For case 2, NES techniques should not impact UEs’ performance, so gNB should avoid HO failure. If the gNB is aware the served UEs cannot be switched to other cells, it should stay in non-NES mode.

Observation 6: NES techniques should not impair UEs’ performance: it is left to NW implementation to configure the CHO conditions stringent enough to avoid UEs’ handover failures.

Two optimizations were proposed and summarized in [1],

- Report the situation to source cell

- Choose the best among worst candidate cells

For 1), as discussed above, source cell knows the NES configuration of candidate cells and UE’s situations. If the trigger condition is never met, the gNB will be aware by not receiving the successful HO feedback. If UE’s HO fails and UE re-establishes to the source cell, UE doesn’t need to report the situation either. If UE re-establishes to other cell, whether to introduce new reestablishmentCause should be considered in SON/MDT scope.

For 2), it could force a UE to HO in a bad cell and then a new RLF and re-establishment will be triggered. Compared with current re-establishment procedure in which UE applies CHO configuration if a cell within CHO configuration is selected, the optimization increases the re-establishment latency without any benefit.

Since the HO failure is rare case as legacy HO, and the optimization cannot provide any performance enhancement, the enhancement for HO failure is not needed.

Proposal 4: Considering the rare cases of HO failures, legacy connection re-establishment is an appropriate way to handle the case when HO fails and the source cell is about to enter NES mode.

Conclusion

According to the analysis in section 2, we provides below observations and proposals.

Observations:

Observation 1: For both Cell switch-off and Cell DTX/DRX, the specific aspect of this CHO is that the UEs to be hand-off’ed are in good channel conditions, hence CondEvent A3, A5 cannot be applied directly.

Observation 2: No obvious issues prevent CondEvent A4 to be used for a CHO preceding a cell entering Cell DTX/DRX or switching off.

Observation 3: For the Cell switch-off scenario, those UEs that have not yet hand-off’ed to another cell will be severely impaired when the cell switches off.

Observation 4: Similarly, the unsuitable UEs that have not yet hand-off’ed to another cell will be severely impaired when the cell enters Cell DTX/DRX mode

Observation 5: Similar to the CHO conditions evaluation, the CHO execution should be performed before the cell enters Cell DTX/DRX or switches off.

Observation 6: NES techniques should not impair UEs’ performance: it is left to NW implementation to configure the CHO conditions stringent enough to avoid UEs’ handover failures.

Proposals:

Proposal 1: CondEvent A4 should be applied in terrestrial network as the CHO enhancement for NES.

Proposal 2: As legacy, the CHO execution should be triggered by the successful CHO condition evaluation, and no additional trigger is required.

Proposal 3: It is up to NW implementation whether to set a NES cell as a candidate cell. No specification change is needed.

Proposal 4: Considering the rare cases of HO failures, legacy connection re-establishment is an appropriate way to handle the case when HO fails and the source cell is about to enter NES mode.

References

- R2-2304353	Summary of EMAIL DISC [121bis#xxx] on AI 7.3.5 Connected Mode Mobility Lenovo

- TS 38.331

- TR38.864, Study on network energy savings for NR, V18.1.0