3GPP TSG-RAN WG2 Meeting #122	R2-2305888

Incheon, South Korea, 22– 26 May 2023

Agenda item:	7.8.5

Source:	Nokia, Nokia Shanghai Bell

Title:	On How To Ensure QoS for PC5-based BRID and DAA

WID/SID:	NR_UAV-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

At RAN2#121bis (April 2023) the following decisions on BRID and DAA have been made [1]:

Agreements: 1. DAA can be supported using the same framework as used for BRID transmission over the LTE and NR PC5 interface, without any specific enhancements. 2. LTE PC5 Mode-4 resource allocation is supported, and LTE PC5 Mode-3 is not supported 3. NR PC5 mode-1 is not supported 4. For LTE PC5, we will follow the NR PC5 framework agreements, unless explicitly identified e.g. a strong technical reason 5. RAN2 assumes that BRID and DAA services will be delivered on a frequency designated by regulators 6. As a baseline, we will use the existing V2X QoS framework. FFS whether different resource pools are needed for UAV services 7. No further enhancement on PC5 range for A2X broadcast services will be pursued in this release 8. We will not investigate interference for BRID and DAA broadcast 9. Send an LS to SA2 to: a. inform them as a result of RAN Plenary decision to re-use BRID RAN2 will only support PC5 broadcast for deconfliction in RAN in release 18. b. ask SA2 whether BRID and DAA broadcast over LTE and NR PC5 requires new QoS requirements and parameters not supported within the ranges supported for V2X

It remained unclear if the existing QoS requirements, defined for PC5-based V2X in both LTE and NR, can be reused for BRID and DAA. Thus, the LS has been sent to SA2 [2], wherein RAN2 seeks for the answer to these doubts. In this paper we attempt to shed some light on the topic.

# 2	Discussion

## 2.1	PQIs and PPPP/PPPR

The QoS requirements for Sidelink/PC5-based V2X have been defined in SA2 specifications: for LTE in [3], while for NR in [4]. In NR the quality requirements are described using PQI which stands for PC5 5QI (5G Quality Indicator). Such parameter is typically assigned per service type, i.e. so far it was meant to represent the quality requirements for V2X, now it is supposed to also indicate what requirements shall be ensured for BRID or DAA.

Observation 1: For NR the QoS requirements for PC5 are represented using service specific PQIs, defined in TS 23.287.

In LTE the QoS handling for PC5 was implemented via PPPP (ProSe Per Packet Priority) and PPPR (ProSe Per Packet Reliability). It is the application layer’s responsibility to assign PPPP and PPPR to each V2X message when passing it to lower layers for transmission. The detailed principles are described in TS 23.285.

Observation 2: For LTE the QoS requirements for PC5 are represented using PPPP and PPPR values (defined in TS 23.285), assigned by the application layer to each message sent to lower layers for transmission.

The PQI values for NR PC5 are standardized in [4]. Each PQI value has a corresponding resource type (e.g. GBR or non-GBR), priority level, packet delay budget or exemplary services. The details are captured in Table I below (excerpt from [4]).

The requirements concerning BRID messages are defined in [5]. Based on this source of information, it can be concluded BRID does not require too tight QoS requirements – i.e. several hundreds of bytes transmitted roughly once per second.

Observation 3: The minimum periodicity of BRID is 1 s while the expected message size is up to 250 bytes.

Thus, it can be observed the QoS requirements for BRID are more relaxed than for most of the PQIs listed in Table I [4].

Observation 4: QoS requirements for BRID are more relaxed than for most of the PQIs listed in Table I.

However, it remains to be decided in SA2 whether any of the values listed in Table I is suitable also for BRID (and for DAA, as DAA shall follow the same principles as defined for BRID) or a new dedicated PQI shall be defined.

Observation 5: SA2 should decide whether any of the NR PQI values listed in TS 23.287 is suitable also for BRID and DAA or a new dedicated PQI shall be defined for this purpose.

In case of LTE, the PPPP principles are described in [6]. However, the application layer selects PPPPs based on criteria that are outside of the 3GPP. Thus, we cannot provide a similar table for LTE.

Observation 6: PPPP values for LTE PC5 are described in TS 23.303. However, the application layer selects PPPPs based on criteria that are outside of the 3GPP, so a similar table to Table I cannot be provided.

Table I. Standardized PQIs [4]

| PQI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 21 | GBR | 3 | 20 ms | 10-4 | N/A | 2000 ms | Platooning between UEs – Higher degree of automation; Platooning between UE and RSU – Higher degree of automation |
| 22 | (NOTE 1) | 4 | 50 ms | 10-2 | N/A | 2000 ms | Sensor sharing – higher degree of automation |
| 23 |  | 3 | 100 ms | 10-4 | N/A | 2000 ms | Information sharing for automated driving – between UEs or UE and RSU - higher degree of automation |
| 55 | Non-GBR | 3 | 10 ms | 10-4 | N/A | N/A | Cooperative lane change – higher degree of automation |
| 56 |  | 6 | 20 ms | 10-1 | N/A | N/A | Platooning informative exchange – low degree of automation; Platooning – information sharing with RSU |
| 57 |  | 5 | 25 ms | 10-1 | N/A | N/A | Cooperative lane change – lower degree of automation |
| 58 |  | 4 | 100 ms | 10-2 | N/A | N/A | Sensor information sharing – lower degree of automation |
| 59 |  | 6 | 500 ms | 10-1 | N/A | N/A | Platooning – reporting to an RSU |
| 90 | Delay Critical GBR | 3 | 10 ms | 10-4 | 2000 bytes | 2000 ms | Cooperative collision avoidance; Sensor sharing – Higher degree of automation; Video sharing – higher degree of automation |
| 91 | (NOTE 1) | 2 | 3 ms | 10-5 | 2000 bytes | 2000 ms | Emergency trajectory alignment; Sensor sharing – Higher degree of automation |
| NOTE 1: GBR and Delay Critical GBR PQIs can only be used for unicast PC5 communications. |  |  |  |  |  |  |  |

## 2.2	Resource Pools for BRID and DAA

One of the open points noted during RAN2#121bis [1] was whether separate resource pools are needed for UAVs:

FFS whether different resource pools are needed for UAV services

For NR PC5 resource pools are configured within BWP-specific configuration. For NR PC5 Mode 2 the following parameter would apply: sl-TxPoolSelectedNormal-r16. There can be up to maxNrofTXPool-r16 of such pools configured per BWP and the maximum value defined in NR RRC is 8. Related ASN.1 excerpt is provided below:

SL-BWP-PoolConfig information element -- ASN1START -- TAG-SL-BWP-POOLCONFIG-START SL-BWP-PoolConfig-r16 ::= SEQUENCE { sl-RxPool-r16 SEQUENCE (SIZE (1..maxNrofRXPool-r16)) OF SL-ResourcePool-r16 OPTIONAL, -- Cond HO sl-TxPoolSelectedNormal-r16 SL-TxPoolDedicated-r16 OPTIONAL, -- Need M sl-TxPoolScheduling-r16 SL-TxPoolDedicated-r16 OPTIONAL, -- Need N sl-TxPoolExceptional-r16 SL-ResourcePoolConfig-r16 OPTIONAL -- Need M } SL-TxPoolDedicated-r16 ::= SEQUENCE { sl-PoolToReleaseList-r16 SEQUENCE (SIZE (1..maxNrofTXPool-r16)) OF SL-ResourcePoolID-r16 OPTIONAL, -- Need N sl-PoolToAddModList-r16 SEQUENCE (SIZE (1..maxNrofTXPool-r16)) OF SL-ResourcePoolConfig-r16 OPTIONAL -- Need N } SL-ResourcePoolConfig-r16 ::= SEQUENCE { sl-ResourcePoolID-r16 SL-ResourcePoolID-r16, sl-ResourcePool-r16 SL-ResourcePool-r16 OPTIONAL -- Need M } SL-ResourcePoolID-r16 ::= INTEGER (1..maxNrofPoolID-r16) -- TAG-SL-BWP-POOLCONFIG-STOP -- ASN1STOP

Observation 7: For NR PC5 Mode 2 the UE can be configured with up to 8 Tx resource pools per BWP.

Thus, it could be up to the NW’s configuration whether to use a single separate pool specifically for UAV purposes (e.g. for BRID and/or DAA), especially if such recommendation is provided by SA2. However, there is likely no need for pursuing specification changes and such configuration is already feasible.

Proposal 1: As per NR RRC, NW is able to configure one or more PC5 Tx pool for UAV purposes (e.g. BRID or DAA), so RAN2 does not pursue further specification changes to enable separate PC5 pools for UAV services.

# 3	Conclusion

In this document the following observations and proposals have been made:

Observation 1: For NR the QoS requirements for PC5 are represented using service specific PQIs, defined in TS 23.287.

Observation 2: For LTE the QoS requirements for PC5 are represented using PPPP and PPPR values (defined in TS 23.285), assigned by the application layer to each message sent to lower layers for transmission.

Observation 3: The minimum periodicity of BRID is 1 s while the expected message size is up to 250 bytes.

Observation 4: QoS requirements for BRID are more relaxed than for most of the PQIs listed in Table I.

Observation 5: SA2 should decide whether any of the NR PQI values listed in TS 23.287 is suitable also for BRID and DAA or a new dedicated PQI shall be defined for this purpose.

Observation 6: PPPP values for LTE PC5 are described in TS 23.303. However, the application layer selects PPPPs based on criteria that are outside of the 3GPP, so a similar table to Table I cannot be provided.

Observation 7: For NR PC5 Mode 2 the UE can be configured with up to 8 Tx resource pools per BWP.

Proposal 1: As per NR RRC, NW is able to configure one or more PC5 Tx pool for UAV purposes (e.g. BRID or DAA), so RAN2 does not pursue further specification changes to enable separate PC5 pools for UAV services.

# References

- R2-2304203 Report from UP, Small data, URLLC/IIoT, RACH indication, NWES and UAV 3GPP TSG-RAN WG2 Meeting #121-bis electronic April 17-26, 2023

- R2-2304564 LS on BRID and DAA broadcast over LTE and NR PC5 3GPP TSG-RAN WG2 Meeting #121-bis electronic April 17-26, 2023

- 3GPP TS 23.285 Architecture enhancements for V2X services

- 3GPP TS 23.287 Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services

- F3411-19 Standard Specification for Remote ID and Tracking, May 25, 2022

- 3GPP TS 23.303 Proximity-based services (ProSe); Stage 2