3GPP TSG-RAN WG2 Meeting #122	R2-2305899

Incheon, Korea, 22nd – 26th April, 2023	Revision of R2-2303303

Agenda Item:	7.5.4.2

Source:	MediaTek Inc.

Title:	Further aspects of PDU discard

Document for:	Discussion and decision

# 1 Introduction

- PDU set discard is modelled using the existing PDCP discard timer for the uplink. The timer is in network control.

We reached the following agreement at the last RAN2 meeting.

In this document, we go into further detail on how the PSDB based discard could be implemented. We also discuss how PDU Set importance can be used for data discard procedures to improve capacity.

# 2 Discussion

## 2.1 PSDB based discard

Given that we’re reusing the PDCP discard timer to model PSDB based discard, the first step is to check whether the current range of values of the timer that can be configured by the network are sufficient. From the RRC specification [1], we have the following range of values that can be configured for the discard timer:

discardTimer            ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200,

ms250, ms300, ms500, ms750, ms1500, infinity}

DiscardTimerExt-r16 ::= ENUMERATED {ms0dot5, ms1, ms2, ms4, ms6, ms8, spare2, spare1}

The existing range of discard timer values maps well to the PDB values as defined in the TR 38.838 [2]. Therefore we can continue to use the existing PDCP discard timer range as is for PSDB based discard.

Proposal 1: The current range of the PDCP discard timer is sufficient to cover PSDB based discard for XR.

We had a brief discussion on the transmitter entity notifying the receiver entity of discarded PDUs to avoid window stalls. This is important because when data is discarded, it may result in ‘holes’ in the sequence number space at the receiver. If receiver is unaware of the existence of such ‘holes’, it can result in a window stall at the receiver (reassembly/reordering timers) as it waits for this missing data to arrive. Such a window stall at the receiver is undesirable as it results in unnecessary delays to delay-sensitive XR traffic, and should be avoided. Therefore, when PDU discard takes place at the transmitter, it is important that the receiver is made aware of this operation to avoid unnecessary costly delays to the processing of XR traffic.

Proposal 2: The receiver entity is notified of any PDUs that are discarded by the transmitter entity to avoid window stalls.

Figure 1 PDCP control PDU format for PDCP status report

A straightforward way to do so is to reuse the structure of the PDCP status report. It’s structure is shown in Figure 1 for reference. FMC indicates the count of the first packet that is missing (in our case, dropped by the transmitter) which is then followed by a bitmap of subsequent missing packets. This structure can be used as is, as it is suitable for our purpose, i.e. for the transmitter to notify the receiver of those packets that are dropped by the transmitter.

Proposal 3: The notification from the transmitter entity of discarded packets reuses the structure of the PDCP status report.

## 2.2 PSI based discard

Due to a lack of time, we did not have a discussion on differential encoded video at the previous RAN2 meeting. This section of the document therefore remains unchanged from the previous version of this document.

In differentially encoded video, the encoder looks at what has changed from a previous scene (e.g. motion on a screen) and provides only that information to the decoder. The decoder applies this differential information to the current scene to generate the next scene. We use the Group of Pictures (GOP) example in Figure 2 below as a simple illustration of how this works.

Figure 2: Illustrative example of a GOP approach to video

A GOP consists of a set of interdependent frames, consisting of a set of I and P frames, that form part of a video stream. The I frame contains information that is independent of a previous frame (e.g. the entire image to be displayed on the screen). The set of P frames that follow are differentially encoded and consist of that information in the current scene which is different from the preceding frame. In this context, the I frame can be described as an independent frame, while the P frame can be described as a dependent frame. As can be seen above, even if the delay budget associated with presenting the video frame to the end user has been exceeded, transmission of frames are still needed due to their dependencies on earlier frames.

Observation 1: In case of differential encoded video, PDB/PSDB expiry cannot be used as a trigger for PDU discard as the information is still needed to decode future frames.

In case of real time streams, it is also equally important to present the latest information to the end user where possible. This is possible when independently decodable frames such as I-frames arrive. If the RAN is aware of an independent frame (using the PDU Set importance indicator of the PDU set), the transmission of this information to the receiver can be expedited. Given that all existing data in the transmission buffer correspond to older video frames that are now irrelevant to the real time video stream, there is no point in transmitting them any longer and can be dropped as illustrated in Figure 3 below. This will help ease congestion in the network while improving the end-user experience of the real time video stream.

Figure 3: Prioritisation of an independent frame along with data discard to ease congestion

Observation 2: In cases where PSDB expiry based PDU discard cannot be used (e.g. differentially encoded information), it is useful to drop all earlier PDU sets in the transmission buffer of the transmitter when a PDU set of high importance arrives, to ease congestion and to ensure that newly arriving independent video frames can be provided to the end-user in a timely manner.

Proposal 4: Arrival of a PDU set with high importance can trigger the discard of all PDU sets that arrived earlier.

# 3 Conclusion

In this contribution we have the following observations and proposals:

On PSDB based discard

Proposal 1: The current range of the PDCP discard timer is sufficient to cover PSDB based discard for XR.

Proposal 2: The receiver entity is notified of any PDUs that are discarded by the transmitter entity to avoid window stalls.

Proposal 3: The notification from the transmitter entity of discarded packets reuses the structure of the PDCP status report.

On PSI based discard

Observation 1: In case of differential encoded video, PDB/PSDB expiry cannot be used as a trigger for PDU discard as the information is still needed to decode future frames.

Observation 2: In cases where PSDB expiry based PDU discard cannot be used (e.g. differentially encoded information), it is useful to drop all earlier PDU sets in the transmission buffer of the transmitter when a PDU set of high importance arrives, to ease congestion and to ensure that newly arriving independent video frames can be provided to the end-user in a timely manner.

Proposal 4: Arrival of a PDU set with high importance can trigger the discard of all PDU sets that arrived earlier.

# 4 References

- TS 38.331 v17.4.0

- TR 38.838 v17.0.0