3GPP TSG-RAN WG2 Meeting #122	R2-2305901

Incheon, Korea, 22nd – 26th May, 2023

Agenda Item:	7.19.3

Source:	MediaTek Inc.

Title:	Open aspects of initial access for eRedCap UEs

Document for:	Discussion and decision

# 1 Introduction

- Working assumption: Use two new LCID values to support Msg3 early identification for eRedCap UE (can be revised and discussed together with other R18 WIs, if R18 WIs may occupy relatively many LCIDs).

We now have an initial direction on early identification for eRedCap devices from the last RAN2 meeting.

Confirm the following working assumption by assuming that Msg3 indication is available

Working Assumption

- For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot.

  - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

RAN1 would like to inform RAN2 about the following case, to consider, if needed, the UE behavior in the RAN2 specifications, and ask RAN2 for feedback if any:

- For UE BB complexity reduction, the case when the UE detects a DCI scheduling a Msg4 PDSCH transmission with a larger bandwidth than it can receive or process

We also received an LS from RAN1 [1] on Msg4 aspects of the RACH procedure, with some relevant excerpts from the document highlighted below.

This document progresses further on related open aspects of the initial access procedure for eRedCap devices.

# 2 Discussion

In Rel-17, we had chosen to always provide Msg3 based early indication for RedCap UEs. This decision was based on two factors:

- There is no additional signalling overhead associated with Msg3 based early identification as Msg3 size is unchanged

- If Msg3 based early identification was to be configurable, additional broadcast signalling would have been necessary

With our working assumption that we use two new LCIDs (and not eLCIDs) for eRedCap Msg3-based early identification, the same arguments hold true for Rel-18. Therefore, we propose to follow the same approach for eRedCap as Rel-17, i.e. Msg3/MsgA based early identification is always enabled for eRedCap UEs. By doing so, we also confirm RAN1’s agreement as it is currently based on the assumption that Msg3 based early identification is available, as highlighted in yellow in the excerpt above.

Proposal 1: Msg3/MsgA based early identification is always enabled for eRedCap UEs.

The other aspect that RAN1 requested feedback on (highlighted in green above), was whether we need to define UE behaviour when the UE detects a Msg4 PDSCH transmission with a larger bandwidth that it can receive/process. As they have already agreed that the UE is not required to process this Msg4, the question that remains open is what happens to the contention resolution procedure following reception of the Msg4.

The only time when the NW would provide a PDSCH that exceeds the eRedCap UE’s capability could be during the initial access procedure. If RACH occasions are shared between eRedCap and non eRedCap UEs, the Msg4 that exceeds the eRedCap UE’s capability would be directed to a non eRedCap UE that is in contention with the eRedCap UE. It could not have been directed to the eRedCap UE as the NW would have already been aware of the eRedCap UE’s constraints from Msg3 early indication (as per Proposal 1).

Figure 1 Example illustration where Msg4 > 5MHz can be received by an eRedCap UE

At this point in time, the eRedCap UE is monitoring the same Temporary C-RNTI (T-CRNTI) as the non-eRedCap UE as illustrated in Figure 1. Given that the Msg4 is addressed to the non-eRedCap UE, the T-CRNTI would be promoted to C-RNTI for that UE. All downlink and uplink transmissions thereafter for that non-eRedCap UE would be scheduled using this C-RNTI. There is no point requiring the eRedCap UE to continue monitoring this T-CRNTI, as it has been allocated to a different UE and would be in use for that UE’s data scheduling. It is only natural therefore for the eRedCap UE to consider contention resolution to be unsuccessful and to reattempt RACH, as the eRedCap UE has pending data for transmission. Therefore, we propose that if the eRedCap UE receives a Msg4 PDSCH allocation that exceeds its capabilities, it considers contention resolution to have failed.

Proposal 2: If an eRedCap UE receives a Msg4 PDSCH allocation that exceeds its capabilities, the UE considers contention resolution to be unsuccessful.

Proposal 3: Respond to RAN1 with the following:

- RAN2 confirms RAN1 assumption on Msg3 indication availability, i.e. Msg3/MsgA based early indication is always available for eRedCap UEs.

- RAN2 agrees that if an eRedCap UE receives a Msg4 PDSCH allocation that exceeds its capabilities, the UE considers contention resolution to be unsuccessful

We do not need to consider any changes to the 2-step RACH procedure in case the PDSCH addressed to MsgB-RNTI exceeds the RedCap UE’s capabilities as multiple MsgBs can be sent by the NW. Unlike T-CRNTI which is meant to only address one UE, MsgB-RNTI is meant to be shared by all UEs that attempted initial access using a RACH occasion. The eRedCap UE should continue to monitor MsgB-RNTI for further potential responses from the NW for this UE, until the msgB-ResponseWindow expires.

Observation 1: No change is needed to the 2 step RACH procedure for eRedCap UEs in case a MsgB PDSCH allocation exceeds the UE’s capabilities.

# 3 Conclusion

In this contribution we have the following observations and proposals:

Proposal 1: Msg3/MsgA based early identification is always enabled for eRedCap UEs.

Proposal 2: If an eRedCap UE receives a Msg4 PDSCH allocation that exceeds its capabilities, the UE considers contention resolution to be unsuccessful.

Proposal 3: Respond to RAN1 with the following:

- RAN2 confirms RAN1 assumption on Msg3 indication availability, i.e. Msg3/MsgA based early indication is always available for eRedCap UEs.

- RAN2 agrees that if an eRedCap UE receives a Msg4 PDSCH allocation that exceeds its capabilities, the UE considers contention resolution to be unsuccessful

Observation 1: No change is needed to the 2 step RACH procedure for eRedCap UEs in case a MsgB PDSCH allocation exceeds the UE’s capabilities.

# 4 References

- R2-2304619 - LS on Msg4 PDSCH transmission to Rel-18 eRedCap UEs (Ericsson)