3GPP TSG-RAN WG2 Meeting #122	R2-2305908

Incheon, South Korea, 22– 26 May 2023

Agenda item:	7.4.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion On RRC Reconfiguration Aspects

WID/SID:	NR_Mob_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

According to the agreements from RAN2#120 [1]:

- The RadioBearerConfig IE can be optionally supported in an LTM candidate configuration - The LTM candidate cell configuration should be designed as a To AddMod/ToRelease structure. - FFS how the UE determine the BWPs (for DL and UL) to be used upon the execution of L1/L2 inter-cell mobility

According to the agreements from RAN2#121 [2]:

- To determine if to reset L2 or not is based on RRC configuration (e.g. set of cells. FFS if separate for RLC, MAC, PDCP).

According to the agreements from RAN3#119bis [3]:

- The gNB-CU may modify or release L1/2 Triggered Mobility (LTM) candidate cells in the gNB-DU. - The (candidate) gNB-DU may cancel already configured L1/2 Triggered Mobility (LTM) candidate cells and notify to the CU. - The gNB-CU may use the UE Context Modification procedure to modify or release the prepared resources of candidate cells in the (candidate) gNB-DU and use the UE Context Release procedure to release the UE context in the (candidate) gNB-DU.

The above topics have not been discussed in the R2#121bis but they should be handled to facilitate cell change and switch relevant to RRC Reconfiguration. Additionally, RAN3 (in RAN3#119bis) has discussed topics relevant to the discussions RAN2 had in R2#120.

This document discusses additional configuration aspects of the RRC Reconfiguration on top of the RRC Reconfiguration aspects relevant to RRC Reconfiguration modelling. Specifically, it discusses the way to release candidate cells as well as the handling of the BWP configuration of the target cell.

# 2	Discussion on additional RRC Configuration Aspects

## 2.1	Discussion on release of candidate cell configuration

In the WID [2] the description of the LTM is:

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction:

- Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3]

- Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1]

- […]

The above description contains two parts, namely, the configuration and maintenance of the configuration to be applied to enable mobility with reduced interruption time, and mechanisms to enable dynamic switch among the serving and target cells. To our understanding, this means that LTM has two operation modes, one where the configurations are released upon the execution of the LTM, and one where these configurations are maintained for dynamic switching. Whether the former or the latter is applied depends on the NW-originated configuration.

Observation 1: LTM is expected to have two operation modes, one where the configurations are released upon the execution of the LTM, and one where these are maintained for dynamic switching.

Observation 2: Whether the UE releases the configurations upon the execution of LTM or maintains them for further dynamic switching is a matter of configuration from the network.

Specifically, if the UE is moving towards new candidate cells, the new candidate cells should be added for LTM mobility and at the same time some of the earlier cells of LTM may not be needed for further mobility. Additionally, some parts of the configuration may become obsolete after some time (e.g., RACH preambles in case of CFRA). In such scenarios, CU can decide to release the candidate cell and update the UE configuration via RRC signalling.  But if the radio condition of one of the prepared candidate LTM cells are not suitable over long duration for dynamic switching, it may be preferred that source DU decides on the release of the candidate cell as it has the knowledge of radio condition of candidate cells based on L1 measurements. One further alternative could be to allow the UE to release the configuration considering various factors, e.g., time-based or depending on the recently accessed cell, etc.

Even though the delay in releasing the candidate cell is not time critical, the additional signalling to CU and RRC signalling procedure to release the candidate cell can be avoided if the DU can trigger the same actions, or if the releasing criteria are included as a part of the initial RRC configuration.

Proposal 1: Whether candidate LTM configuration is maintained for dynamic switching is controlled by the network via RRC configuration.

According to the RAN3 agreement the CU or the DU can decide about the release of the target cells. The actual release can be further triggered via MAC CE or an RRC Reconfiguration message. However, since RAN2 decided to follow Model 1 for LTM, it should be the CU that releases of the prepared cells using RRC Reconfiguration message.

Proposal 2: Use RRC Reconfiguration for the de-configuration of prepared cells.

## 2.2	Discussion on BWP handling

One point raised in RAN2#119bis [3] was the BWP handling in LTM. Specifically, it was discussed how the UE should be made aware of which BWP to use in the DL and the UL upon the LTM triggering. For this, two options are available:

- Option 1: indicated directly within the candidate cell configuration for L1/L2 inter-cell mobility (e.g., by means of firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id)

- Option 2: indicated in the LTM triggering.

Option 1 means that the BWPs are provided in the RRC Configuration of the target cell, and this is provided in advance. In Option 2 the source cell decides the BWP to be used and indicates it to the target cell.

To our understanding Option 1 is more suitable since the BWP to be used in the target cell upon LTM is part of the target cell RRC configuration already. Option 2 can also be used since it has benefits; if the same BWP is used in the target and the source at the time of LTM, the interruption time for LTM cell-change is reduced since BWP change is not needed in this case. In such case, the change of BWP in source node needs to be provided, at least for the RACH-less case to the target node.

Proposal 3: BWP to be used upon the LTM execution is either indicated directly within the candidate cell configuration for L1/L2 inter-cell mobility using RRC Configuration message or indicated in the LTM triggering using MAC CE.

Observation 3: Usage of the same BWP between the source and the target reduces the interruption time, but in case the BWP changes in the source, it requires the target cell to be notified, at least for the RACH-less scenario.

Proposal 4: If Option 2 is followed, the notification from the source to the target about the BWP used before the LTM is FFS.

## 2.3	Discussion on LTM Configuration for action-related parameters

During mobility, the actions related to PDCP and RLC layers related to re-establishment and recovery are configured as one-time parameters without need to maintain in the UE RRC configuration. This action depends on whether there is change of RLC instance or PDCP anchor point during mobility.  The action is decided based on whether source and target cells belong to the same anchor point or RLC functionality continuity between these cells. In case of LTM, the switching is decided by DU dynamically across cells. In this scenario configuring static values related to these parameters will not work as the actions required at target cell depends on source cell as well as the status of activity at lower layers.  This issue can be avoided if candidate cell configuration (LTM configuration) includes multiple instances of parameters related to these action parameters as additional configuration and indices for these configurations are given as part of LTM configuration.  During LTM switching depending on the required actions for these parameters the configuration index from this list chosen and provided to UE. NW can combine the candidate-configuration-index and the dynamic-config-index as single number.

Proposal 5: Action-related parameters for RLC and PDCP (i.e., PDCP-Recovery, RLC-Recovery) are selected at the time of switching and indicated through additional configuration index in the LTM command

# 3	Conclusion

This document has made the following observations:

Observation 1: LTM is expected to have two operation modes, one where the configurations are released upon the execution of the LTM, and one where these are maintained for dynamic switching.

Observation 2: Whether the UE releases the configurations upon the execution of LTM or maintains them for further dynamic switching is a matter of configuration from the network.

Observation 3: Usage of the same BWP between the source and the target reduces the interruption time, but in case the BWP changes in the source, it requires the target cell to be notified, at least for the RACH-less scenario.

This document has proposed the following:

Proposal 1: Whether candidate LTM configuration is maintained for dynamic switching is controlled by the network via RRC configuration.

Proposal 2: Use RRC Reconfiguration for the de-configuration of prepared cells.

Proposal 3: BWP to be used upon the LTM execution is either indicated directly within the candidate cell configuration for L1/L2 inter-cell mobility using RRC Configuration message or indicated in the LTM triggering using MAC CE.

Proposal 4: If Option 2 is followed, the notification from the source to the target about the BWP used before the LTM is FFS.

Proposal 5: Action-related parameters for RLC and PDCP (i.e., PDCP-Recovery, RLC-Recovery) are selected at the time of switching and indicated through additional configuration index in the LTM command

# References

[1] R2-2300001	RAN2#120 Meeting Report 3GPP TSG-RAN WG2 Meeting #121	Athens, Greece, 27 February – 03 March 2023

[2] R2_121 Chair Notes EOM.docx

[3] RAN3_119bis-e_agenda_20230426_EOM1