3GPP TSG-RAN WG2 Meeting #122	R2-2305909

Incheon, South Korea, 22– 26 May 2023

Agenda item:	7.4.2.3

Source:	Nokia, Nokia Shanghai Bell

Title:	On the cell switch in LTM

WID/SID:	NR_Mob_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

According to the agreements from the previous meetings [1]:

R2-121bis Agreements: - Following behaviors of LTM supervisor timer are agreed: - 1: The UE starts the LTM supervisor timer, upon reception of the LTM cell switch MAC CE; - 2: The UE stops the LTM supervisor timer, upon successful completion of LTM cell switch; - 3: If the LTM supervisor timer for MCG expires, as baseline, the UE considers LTM failure and initiates RRC re-establishment. (SCG switch case FFS) - LTM supervisor timer is RRC layer timer. - At RLF or LTM execution failure (for MCG), RAN2 intend to support fast recovery to a candidate cell by LTM execution.

The open issues as captured in the R2-120 meeting report [2] could be summarized as follows (excluding the ones handled by the above agreements, i.e., procedure of LTM cell switch to be supervised by a timer, how to indicate UE arrival in the target cell):

- Security concerns for LTM when using L1/L2 signalling in L1 measurement report or LTM trigger command

In this paper the above issues are discussed. Specifically, how the LTM timer operates in order to identify potential failures, how the UE should operate in case of a failure in order to reduce interruption time, and how the cell switch can be triggered without exposing the target cell PCI.

## 2.1	Cell Switch Failure Monitoring

Following the latest agreements in R2-120 the LTM procedure can be captured in the following MSC [3].

Figure 1: Running CR - Signalling procedure for LTM

According to that and following the relevant LTM agreements of R2#121bis:

- The UE sends a MeasurementReport message to the gNB. The gNB decides to use LTM and initiates LTM candidate preparation.

- The gNB transmits an RRCReconfiguration message to the UE including the configuration of one or multiple LTM candidate target cells.

- The UE stores the configuration of LTM candidate target cell(s) and transmits an RRCReconfigurationComplete message to the gNB.

- The UE may perform DL synchronization and TA acquisition with candidate target cell(s) before receiving the LTM cell switch command.

- The UE performs L1 measurements on the configured LTM candidate target cell(s) and transmits lower-layer measurement reports to the gNB.

- The gNB decides to execute LTM cell switch to a target cell and transmits a MAC CE triggering LTM cell switch by including the candidate configuration index of the target cell. The UE initiates the LTM supervisor timer and switches to the configuration of the LTM candidate target cell.

- The UE performs a random access procedure towards the target cell, if cell switch command indicates performing random access procedure.

- The UE indicates successful completion of the LTM cell switch towards target cell, using an RRC reconfiguration complete (as agreed in R2#121bis).

For LTM the timer will be part of the RRC configuration and it will start at the reception of the cell switch command. In other words, the timer will start once the condition to trigger the timer will be fulfilled; in this case the condition is the reception of the cell switch command. However, this is the case for T304 and every other reconfigurationWithSync that it is triggered.

Observation 1: LTM supervisor timer has the same functionality with T304.

LTM is expected to have different timescale compared to other mobility procedures (and reconfigurationWithSynch processes). This raises the question whether the LTM supervisor timer functionality can be performed by T304, or a new timer is needed to be introduced.

Proposal 1: Use T304 timer to supervise LTM procedure.

## 2.2	LTM Failure Handling

If a failure occurs, and under the assumption that the same paradigm is followed as in Baseline HO, the UE will have to perform re-establishment including cell reselection and decoding system information in order to identify the proper cell to re-establish to. This will increase the interruption time and signalling overhead, even though the UE may have already the configurations of cells that it could recover to. On the other hand, the RRC Configuration of the prepared cells is already available in the UE and the UE can take advantage of this to reduce the interruption time of the re-establishment of the connection.

Observation 2: Early provision of RRC configurations to the UE can enable the reduction of the interruption time caused by re-establishment procedure in case of failures.

For example, in case an RLF or a HOF occurs, the UE may apply one of the available target cell configurations to reduce the interruption time, similar to Conditional Handover Recovery.

Proposal 2: If the UE has stored LTM command when RLF occurs, the UE can be configured to apply that first without triggering re-establishment procedure.

A similar procedure could be used for LTM to allow faster recovery from PCell re-establishment procedure. In this case the UE will perform cell selection procedure and if the selected cell (as per cell selection procedure) is an LTM candidate cell, the UE should execute LTM to the selected target cell.

Proposal 3: In LTM Recovery the UE will perform cell selection procedure and if the selected cell (as per cell selection procedure) is an LTM candidate cell, the UE should execute LTM to the selected target cell.

Proposal 4: Whether RACH or RACHless access to the target cell for recovery should be followed is FFS.

## 2.3	Target Cell indication in MAC CE triggering LTM

In the running CR of the TS 38.300 ([3]) the following has been mentioned in Figure 1 describing the LTM procedure:

6.	The gNB decides to execute LTM cell switch to a target cell and transmits a MAC CE triggering LTM cell switch by including the candidate configuration index of the target cell. The UE switches to the configuration of the LTM candidate target cell.

The way in which this indication of the target cell configuration will take place is not yet defined. For this, two aspects should be taken into consideration:

- Protection of the target cell indication (i.e., security aspect)

- Signaling overhead in the MAC CE (i.e., efficiency aspect)

Overhead is not expected to be the biggest concern (since there are both large MAC CEs already, and the MAC CE size would in any case be likely much smaller than the RRCReconfiguration size). However, the protection of the LTM command is a relevant aspect to consider: The security aspect arises because MAC CEs are neither ciphered nor integrity protected: therefore, allowing for the sending transmission of plaintext MAC CE that changes the serving cell of the UE could be subject to denial-of-service (DoS) attack, wherein such commands are sent to UEs randomly, potentially causing RLFs or UP interruption. Therefore, it is necessary to avoid at least indicating details of target cell configuration within the MAC CE. Having, and some form of protection would help to avoid obvious attacks.

There are a few ways to address such problems (which are not mutually exclusive):

Providing ciphering or integrity protection for the MAC CE: If the MAC CE were ciphered, this might resolve several problems. However, it may not be desirable to duplicate PDCP functionalities within MAC. One way to resolve this would be for the CU to provide cell-pair tokens that are integrity-protected and/or ciphered by PDCP within the MAC CE – then the UE would pass those to PDCP for validation, and if that is successful, it would trigger the LTM procedure.

Observation 3: It is not desirable to duplicate ciphering or integrity protection in MAC. However, reusing ciphered or integrity-protected PDCP PDUs as part of LTM MAC CE could help to resolve any security issues.

Proposal 5: LTM MAC CE can contain ciphered or integrity-protected PDCP PDU that UE PDCP verifies before triggering LTM procedure.

Figure . LTM procedure with PDCP PDU within MAC CE as security check

Using indirect indication (e.g., cell index within a configuration that does not tell the exact PCI) to the target cell – this protects from attacker knowing the exact configuration and provides similar security as SCell activation MAC CE. However, this does not protect from UE potentially receiving false MAC CEs.

Proposal 6: The LTM MAC CE uses indirect indication of the target cell (e.g., an index to a part of the target cell configuration in RRC).

For deciding the cell index three options exist:

- Source DU determines the cell index: In this option, the Source DU associates the PCI of the prepared target cell with the cell index, and it provides the association between the cell index and PCI to CU which in turn send it to the UE (and the other DUs, in case of Dynamic Switching).

- CU determines the cell index: In this option the CU associates the PCI of the prepared target cell with the cell index, and it provides the association between the cell index and PCI to Source DU (and the other DUs, in case of Dynamic Switching) and to the UE

- UE uses the ID of the target cell configuration (i.e., RRC configuration for L1/2 mobility) as cell index: In this option the CU informs the (source) DU about the ID of the target cell configuration.

Proposal 7: RAN2 to decide which of the options 1, 2, and 3 should be followed for indicating the target cell index in the LTM MAC CE.

# 3	Conclusion

This document has made the following observations:

Observation 1: LTM supervisor timer has the same functionality with T304.

Observation 2: Early provision of RRC configurations to the UE can enable the reduction of the interruption time caused by re-establishment procedure in case of failures.

Observation 3: It is not desirable to duplicate ciphering or integrity protection in MAC. However, reusing ciphered or integrity-protected PDCP PDUs as part of LTM MAC CE could help to resolve any security issues.

The document has made the following proposals:

Proposal 1: Use T304 timer to supervise LTM procedure.

Proposal 2: If the UE has stored LTM command when RLF occurs, the UE can be configured to apply that first without triggering re-establishment procedure.

Proposal 3: In LTM Recovery the UE will perform cell selection procedure and if the selected cell (as per cell selection procedure) is an LTM candidate cell, the UE should execute LTM to the selected target cell.

Proposal 4: Whether RACH or RACHless access to the target cell for recovery should be followed is FFS.

Proposal 5: LTM MAC CE can contain ciphered or integrity-protected PDCP PDU that UE PDCP verifies before triggering LTM procedure.

Proposal 6: The LTM MAC CE uses indirect indication of the target cell (e.g., an index to a part of the target cell configuration in RRC).

Proposal 7: RAN2 to decide which of the options 1, 2, and 3 should be followed for indicating the target cell index in the LTM MAC CE.

# References

[1] R2_121bis Chair Notes 2023-04-26 EOM

[2] R2_120 Chair Notes 2022-11-18 EOM

[3] R2-2302039– 38.300 running CR for introduction of NR further mobility enhancements 3GPP TSG-RAN WG2 Meeting #121 Athens, Greece, February 27-3 of March 2023