3GPP TSG-RAN2 Meeting #122	R2-2305915

Incheon, South Korea, 22 - 26 May 2023

Agenda Item:	6.2.2 CP corrections

Source: 	Ericsson

Title:  	MBS broadcast on SCell using plmn-Index

Document for:	Discussion and Decision

# Introduction

MBS broadcast reception on SCell using plmn-Index is discussed further in this contribution.

# Discussion

During RAN2#121bis-e it was discussed that there is a problem when the UE wants to receive an MBS broadcast session on SCell when the plmn-Index is used for the corresponding TMGI on the MCCH on the SCell frequency, see proposal 4 in [1, 2]. The UE does not acquire/receive SIB1 on the SCell frequency, i.e. when the UE acquires the MCCH on SCell frequency and plmn-Index is used than the UE cannot identify the broadcast session correctly:

Observation: When the UE acquires the MCCH on SCell frequency and plmn-Index is used for the PLMN/NPN in the TMGI then the UE cannot receive the broadcast session on SCell.

It is beneficial to use the plmn-Index (4 bits) instead of full PLMN ID (24 bits) especially on the MCCH where the information is repeated. And to indicate an SNPN only the plmn-Index can be used:

Assumption: The PLMN identity is not used in the TMGI on MCCH.

The SIB1 configuration on PCell and SCell may be different, i.e. the UE cannot rely on that the plmn-Index is the same on PCell and SCell.

It is a serious restriction when the UE cannot receive sessions on SCell that belong to an SNPN. And to reduce the MCCH size it is beneficial to use the plmn-Index:

Proposal 1: A backwards compatible ASN.1 change is acceptable in Rel-17 to enable MBS broadcast reception on SCell when plmn-Index for PLMNs/NPNs is used on MCCH.

Potential solutions to solve this problem are to include in the SCellConfig in RRCReconfiguration message (together with SCellSIB20-r17):

- Option 1: sCellSIB1-r17

- Option 2: plmn-IdentityInfoList and npn-IdentityInfoList

- Option 3: new plmn-IdentityInfoList and npn-IdentityInfoList including PLMN ID and NID info only

All these options are feasible. But the overhead is reduced greatly if only the essential information is sent:

Using the plmn-Index the UE needs to find the:

- <PLMN ID> 		for PLMNs and NPI-NPNs

- <PLMN ID, NID>	for SNPNs

There is considerable overhead in SIB1, i.e. it is not justified to sent the complete SIB1. But also the plmn-IdentityInfoList and npn-IdentityInfoList include considerable overhead per PLMN and NPN, e.g. CellIdentity (36 bits), TrackingAreaCode (24 bits) and RAN-AreaCode (8 bits):

Proposal 2: Introduce PLMN and NPN info list in SCellConfig including PLMN ID and NID info only.

A TP is provided in the Annex for discussion and agreement.

# Summary

RAN2 is kindly asked to discuss MBS broadcast on SCell using plmn-Index:

Observation: When the UE acquires the MCCH on SCell frequency and plmn-Index is used for the PLMN/NPN in the TMGI then the UE cannot receive the broadcast session on SCell.

Assumption: The PLMN identity is not used in the TMGI on MCCH.

Proposal 1: A backwards compatible ASN.1 change is acceptable in Rel-17 to enable MBS broadcast reception on SCell when plmn-Index for PLMNs/NPNs is used on MCCH.

Proposal 2: Introduce PLMN and NPN info list in SCellConfig including PLMN ID and NID info only.

# References

- R2-2303967, Discussion on the remainning MBS issues, Huawei, DISC, RAN2#121bis-e

- R2-2304327, Report of [AT121bis-e][601][MBS-R17] CP issues, Ericsson, Report, RAN2#121bis-e

# Annex: TP 38.331

CellGroupConfig ::=                        SEQUENCE {

<TEXT OMITTED>

SCellConfig ::=                     SEQUENCE {

sCellIndex                          SCellIndex,

sCellConfigCommon                   ServingCellConfigCommon                                     OPTIONAL,   -- Cond SCellAdd

sCellConfigDedicated                ServingCellConfig                                           OPTIONAL,   -- Cond SCellAddMod

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

sCellState-r16                  ENUMERATED {activated}                                          OPTIONAL,   -- Cond SCellAddSync

secondaryDRX-GroupConfig-r16    ENUMERATED {true}                                               OPTIONAL    -- Cond DRX-Config2

]],

[[

preConfGapStatus-r17             BIT STRING (SIZE (maxNrofGapId-r17))                           OPTIONAL,   -- Cond PreConfigMG

goodServingCellEvaluationBFD-r17 GoodServingCellEvaluation-r17                                  OPTIONAL,   -- Need R

sCellSIB20-r17                   SetupRelease { SCellSIB20-r17 }                                OPTIONAL    -- Need M

]],

[[

plmn-IdentityInfoList-r17        PLMN-IdentityInfoList-r17,                                     OPTIONAL    -- Need R

npn-IdentityInfoList-r17         NPN-IdentityInfoList-r17                                       OPTIONAL    -- Need R

]]

}

SCellSIB20-r17 ::= OCTET STRING (CONTAINING SystemInformation)

<TEXT OMITTED>

PLMN-IdentityInfoList-r17 ::=        SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-IdentityInfo-r17

PLMN-IdentityInfo-r17 ::=            SEQUENCE {

plmn-IdentityList-r17                SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity

}

<TEXT OMITTED>

NPN-IdentityInfoList-r17 ::=     SEQUENCE (SIZE (1..maxNPN-r16)) OF NPN-IdentityInfo-r17

NPN-IdentityInfo-r17 ::=         SEQUENCE {

npn-IdentityList-r17             SEQUENCE (SIZE (1..maxNPN-r16)) OF NPN-Identity-r17,

}

NPN-Identity-r17 ::=             CHOICE {

pni-npn-r17                      SEQUENCE {

plmn-Identity-r17                PLMN-Identity,

},

snpn-r17                         SEQUENCE {

plmn-Identity-r17                PLMN-Identity,

nid-List-r17                     SEQUENCE (SIZE (1..maxNPN-r16)) OF NID-r16

}

}