3GPP TSG-RAN2 Meeting #122	R2-2305916

Incheon, South Korea, 22 - 26 May 2023

Agenda Item:	7.11.2.1 Control plane

Source: 	Ericsson

Title:  	Multicast reception in RRC_INACTIVE

Document for:	Discussion and Decision

# Introduction

For multicast reception in RRC_INACTIVE the following topics are discussed:

- Notifications

- State transitions

- Service continuity

- PTM configuration

# Discussion

## Notifications

### Enhanced group paging

RAN2 agreed as a baseline that group paging can be used to inform Rel-18 UEs about session activation (details FFS):

As a baseline, group paging can be used to inform Rel-18 UE(s) about the session activation (Details FFS, e.g., UE behavior when receiving such group notification).

Without changes to Rel-17 group paging a Rel-18 UE that can receive multicast in RRC_INACTIVE also return to connected. But when there is congestion it is beneficial to keep Rel-18 UEs that can receive multicast in RRC_INACTIVE in RRC_INACTIVE:

Proposal 1: Introduce a bit string in the Paging message (Rel-18 extension), to indicate for each TMGI in pagingGroupList-r17 whether a Rel-18 UE that has a valid PTM configuration to receive the session in RRC_INACTIVE should not resume or not (aka “stay put” bit). When the bit is not set or when the bitstring is absent or when the UE does not have a valid PTM configuration, the Rel-18 UE resumes.

For further clarification a possible ASN.1 encoding is provided below:

Paging ::=                      SEQUENCE {

pagingRecordList                PagingRecordList                   OPTIONAL, -- Need N

lateNonCriticalExtension        OCTET STRING                       OPTIONAL,

nonCriticalExtension            Paging-v1700-IEs                   OPTIONAL

Paging-v1700-IEs ::=            SEQUENCE {

pagingRecordList-v1700          PagingRecordList-v1700             OPTIONAL, -- Need N

pagingGroupList-r17             PagingGroupList-r17                OPTIONAL, -- Need N

nonCriticalExtension            Paging-v1800-IEs                   OPTIONAL

}

PagingGroupList-r17 ::=         SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF TMGI-r17

Paging-v1800-IEs ::=            SEQUENCE {

pagingGroupList-v1800           BIT STRING (SIZE (maxNrofPageGroup-r17)), OPTIONAL, -- Need N

nonCriticalExtension            SEQUENCE {}                               OPTIONAL

}

### Start/stop monitoring G-RNTI

RAN2 agreed that the UE in RRC_INACTIVE may be notified about session activation and data-activity, e.g. to start/stop monitoring the G-RNTI of the session the UE is receiving while staying in RRC_INACTIVE:

- For one UE already in RRC_INACTIVE, it can stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon events like session deactivation/temporary no data.

But RAN2 did not agree on the delivery method of these notifications, i.e. whether to use group paging or MCCH:

- FFS which option to take: enhanced group paging or enhanced MCCH, to enable Rel-18 UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon events like session deactivation/temporary no data.

Before discussion further, first some observations:

- For the UE it is not relevant whether it should start/stop monitoring G-RNTI due to activation, data inactivity or session release, i.e. no reason to inform the UE about the “cause”.

- Multiple sessions may be mapped onto the same G-RNTI, i.e. the notification should be per TMGI.

- Start monitoring and stop monitoring cannot be decoupled, i.e. if a bit indicates that the UE shall start monitoring a G-RNTI then this implies that if the bit is not set that the UE may stop monitoring.

- Resources are wasted when start monitoring is indicated via group paging, while stop monitoring is indicated via MCCH.

In the table below notification using paging or MCCH is compared:

|  | Paging | MCCH |
| --- | --- | --- |
| Short description | - UE monitors a Paging Occasion every DRX cycle - UE receives Paging message on PDSCH if indicated on PDCCH - In pagingGroupList-r18 it is indicated per G-RNTI to start/stop monitoring | - UE monitors an MCCH occasion every Modification Period - UE receives MBSMulticastConfiguration message on PDSCH if ‘change’ is indicated on PDCCH - In MBSMulticastConfiguration message it is indicated per G-RNTI to start/stop monitoring |
| Comparison | - Triggers legacy UEs to acquire Paging PDSCH (frequently) - UE needs to be prepared to receive PDSCH in every Paging Occasion, unless PEI is configured. - Reliability may be impaired during cell re-selection and at cell border when UE misses paging, i.e. UE does not receive the data in the following “active period” - Paging message needs to be repeated more often compared to normal paging, because there is no response from the UE when the UE has received the notification. | - Does not impact legacy UEs - UE needs to be prepared to receive PDSCH in every MCCH occasion - Robust due to repetitions |

The main difference between paging and MCCH is that the MCCH is continuously repeated, i.e. by nature more robust at cell border and with cell re-selection compared to paging:

Observation 1: Notification via MCCH is more robust compared to Paging, and if the UE misses a start notification the UE does not receive the multicast data during the following “active period”.

UE vendors expressed (implicitly) concerns with introduction of a multicast MCCH for PTM configuration change because it requires the UE to wake up during MCCH occasions in addition to the Paging occasions. This concern is further discussed below.

PTM configuration change is not expected to happen often. And perhaps the MCCH could only be configured in SIB when a change is pending. But session status change ((de)activation, data-(in)activity) may happen frequently. The power consumption to monitor MCCH can be minimized when the UE only needs to wake-up and be prepared to receive MCCH PDCCH, similar as with PEI monitoring Paging:

Furthermore if a time offset is configured between PDCCH and PDSCH, the power saving is further reduced, i.e. the UE can also enter a light sleep before being prepared to receive PDSCH. The power consumption to monitor the multicast MCCH then becomes very similar to the power consumption to monitor PEI for Paging.

DCI format 4_0 is expected to be re-used for the multicast MCCH. This format has 14 reserved bits, some of which could be used to signal the session status, without increasing the DCI size. The mapping between DCI bit and G-RNTI can be signalled in RRCRelease or multicast SIB.

Thus the UE only acquires the PDSCH when the PTM configuration changes which is rarely, if at all. Only 1 bit in the DCI is needed to trigger PDSCH reception e.g. “change”. When this bit is set the UE needs to acquire the MBSMulticastConfiguration message on PDSCH to find out what has changed. But when the session status changes the UE can go back to deep sleep immediately after receiving the PDCCH:

Proposal 2: MCCH DCI is used to notify start/stop monitoring G-RNTI and a time offset can be configured between MCCH PDCCH and PDSCH.

## State transitions

### Resume threshold

RAN2 agreed that if the reception quality drops below a configured threshold the UE may resume:

- UE may trigger RRC connection resumption if the reception quality of the multicast data is below a configured threshold, FFS how to specify the threshold/reception quality.

The intention with the threshold is to guarantee some minimum quality when the UE is receiving multicast in RRC_INACTIVE. The gNB can try to release UEs that are on good coverage and stationery. But the UE may move into bad coverage or experience interference. In such case it is better that the UE returns to connected mode, where based on the UE’s HARQ feedback retransmissions can be scheduled.

The threshold should be based on existing measurements when the UE is in RRC_INACTIVE i.e. RAN4 impact would not be acceptable. This means that the measurements are limited to RSRP (indicating signal strength) and RSRQ (indicating possible interference). To reduce the impact on the UE implementation the quality triggered resume can be defined similar to the intra-frequency measurements thresholds (see SIntraSearchP and SIntraSearchQ in 38.304).

The current RAN2 agreement says that the UE “may” trigger resume when it is below the threshold. This is problematic and it is not well motivated. The UE is released to RRC_INACTIVE due to congestion, not because the UE prefers to receive multicast in RRC_INACTIVE. Thus when the UE is allowed to return to connected mode the UE is assumed to return. Furthermore the UE behavior becomes unpredictable after configuration, i.e. it is not clear if the UE will trigger resume at all due to the link quality. The gNB has control over which UEs to release to RRC_INACTIVE, and the gNB should also have control over when those UEs return:

Proposal 3: The UE resumes when the measured RSRP or RSRQ drops below the configured threshold.

Proposal 4: The threshold can be configured in MCCH or RRCRelease (when MCCH is not configured).

The possible impact on 38.331 is provided below for further information:

x.y.z Criterion for quality triggered Resume

The criterion for quality triggered Resume is fulfilled when:

-	Srxlev ≤ SResumeP, or,

-	Squal > SResumeQ, if SResumeQ is configured,

Where:

-	Srxlev = current Srxlev value of the serving cell (dB), as defined in TS 38.304 [20].

-	Squal = current Squal value of the serving cell (dB), as defined in TS 38.304 [20].

x.y.z Quality triggered Resume

1>	if qualityTriggeredResume is configured in suspendConfig or MBSMulticastConfiguration:

2>	if the quality triggered Resume criterion in x.y.z is fulfilled:

3>	initiate RRC connection resume procedure in 5.3.13.2 with resumeCause set to qualityTriggered;

SuspendConfig ::=                   SEQUENCE {

…

MBSBroadcastConfiguration-r18-IEs ::= SEQUENCE {

…

qualityTriggeredResume-r18          QualityTriggeredResume-r18   OPTIONAL        -- Need R

…

QualityTriggeredResume-r18   SEQUENCE {

s-ResumeP                      ReselectionThreshold,

s-ResumeQ                      ReselectionThresholdQ             OPTIONAL        -- Need R

}

s-ResumeP Parameter "SResumeP" in Quality triggered resume criterion in section x.y.z

s-ResumeQ Parameter "SResumeQ" in Quality triggered resume criterion in section x.y.z

### Solution with minimal network impact

Extreme congestion is expected to happen rarely, and it is beneficial to explore a minimum network implementation that still prevents service denial when there are (too) many mission critical UEs.

In our view such a minimum network solution is when the UE is provided with a PTM configuration in the RRCRelease which is only valid in the serving cell. The PTM configuration cannot be changed, i.e. MCCH is not used. This solution also relies on connected mode mobility, for which everything is in place. MCCH is also not needed for mobility. The solution does not require coordination with neighbouring cells (e.g. PTM configuration, NCL info, etc). Also note that it is possible that congestion is limited to a single cell for which such coordination is not needed.

This solution has minimal network impact, but it requires the UE to resume before cell re-selection is initiated:

Proposal 5: If a resume threshold is configured, the UE can be configured to resume before cell reselection is triggered.

### Special UE

The CN may indicate to RAN if a UE should preferably be kept in RRC_CONNECTED, e.g. team leader or a frequent talker. Unless there are other considerations, the gNB will take this information into account.

In case a special UE is released the gNB can use UE individual paging to get that UE back to RRC_CONNECTED as soon as possible:

Observation 2: gNB can use UE individual paging to get a special UE (or any other UE) back to RRC_CONNECTED before group paging is used and no further enhancements are needed.

## Power saving

### “short/long” PTM DRX

In unicast DRX, when short DRX is configured, the UE enters short DRX when the drx-InactivityTimer expires and then the drx-ShortCycleTimer is started. When there is no data activity and the drx-ShortCycleTimer expires the UE enters long DRX. Having two DRX cycles enables the NW to configure a longer long DRX cycle, and provide better power saving, while retaining responsiveness when the short DRX cycle is running.

When a session is deactivated the gNB may release the UEs to RRC_IDLE or RRC_INACTIVE, and when the session is activated again, there is group paging to get the UEs back to RRC_CONNECTED. Furthermore the gNB may release a UE to RRC_INACTIVE when the session is active and there is data-inactivity. The gNB then triggers RAN group paging when there is new multicast data.

However there is paging delay, random access and setup/resume signalling involved for these RRC state transitions. A session may be activated any time after it has been deactivated, and new multicast data may arrive at any time. It depends on the traffic characteristics and the deactivation timer that is used whether RRC state transition is beneficial or not. Furthermore a special UE is preferred to be kept in RRC_CONNECTED. And for mission critical UEs it may not be possible to release the UEs due to latency/loss requirements [1]:

Observation 3: In some cases it is not beneficial or not possible to release the UEs when a session is deactivated or when there is data-inactivity.

In case UEs are not or cannot be released when the session is deactivated or there is data-inactivity, then it is beneficial when the UEs can drop into a PTM DRX state that is more power efficient:

Proposal 6: Add another (more power efficient) PTM DRX state the UE enters when there is pro-longed data-inactivity.

Please note that the existing PTM DRX cycle is called drx-LongCyclePTM:

Proposal 7: Introduce the following PTM DRX parameters:

- drx-ShorterOnDurationTimerPTM (optional): 	the duration at the beginning of a longer DRX cycle

- drx-LongerCyclePTM (optional): 		the longer DRX cycle

- drx-LongerCycleTimerPTM (optional):	the duration the UE shall follow the long DRX cycle

## Service continuity

### NCL

RAN2 agreed that an NCL on the serving cell may be configured to avoid the latency of reading SIB/MCCH on the target cell before resume in the target cell is initiated when the session is not provided on the target cell:

- The neighbor cell list mechanism for multicast reception in RRC_INACTIVE may be configured e.g. it can be used by UE to resume RRC connection if service is not available in the re-selected cell by NCL, without reading MCCH in the re-selected cell, in some aspects similar to Rel-17 NCL mechanism in MBS broadcast.

When the NCL is not configured on the serving cell, then the UE acquires the MCCH on the target cell to determine if the session that the UE was receiving on the serving cell is provided, and the UE applies the new PTM configuration, if it has changed.

But if the UE already knows based on the NCL info on the serving cell that the session it is receiving is not supported on the target cell, the UE can trigger resume on the target cell earlier. However, in case the session is supported on the target cell but the PTM configuration is different, then the UE has to continue to use the old PTM configuration until it has acquired the new PTM configuration:

Observation 4: The NCL does not improve the service continuity when the session is provided on the neighbour cell, but the PTM configuration is different between the neighbour and serving cell.

The MCCH size would be increased considerably when PTM configuration of neighbour cells is included:

Proposal 8: The multicast NCL does not include PTM configuration info of neighbouring cells.

The latency improvement enabled by the NCL info is dependent on the configured MCCH modification period. This does not increase the monitoring frequency of stationary UEs, but requires the NW to transmit the MBSMulticastConfiguration message more often:

Observation 5: When the NCL is not configured, the service continuity can be improved by configuring a shorter repetition period during a modification period, without impacting the UE power consumption.

In our view the Rel-17 NCL mechanism can be re-used for the Rel-18 multicast NCL:

Proposal 9: The multicast NCL includes a SessionInfoList (details FFS), NeighbourCellList (re-use MBS-NeighbourCellList-r17) and mtch-NeighbourCell (bitstring).

For information:

MBS-NeighbourCellList-r17 ::=   SEQUENCE (SIZE (0..maxNeighCellMBS-r17)) OF MBS-NeighbourCell-r17

MBS-NeighbourCell-r17 ::=       SEQUENCE {

physCellId-r17                  PhysCellId,

carrierFreq-r17                 ARFCN-ValueNR                            OPTIONAL  -- Need S

}

mtch-NeighbourCell-r17      BIT STRING (SIZE(maxNeighCellMBS-r17))       OPTIONAL, -- Need S

The UE resumes in the neighbour cell before acquiring the MCCH when the NCL on the serving cell indicated that the session the UE is receiving is not supported:

Proposal 10: Based on the NCL info the UE may trigger resume in the neighbour cell before acquiring the MCCH.

The Rel-17 principles of presence and absence of NCL info are re-used:

Proposal 11: The Rel-17 principles of presence and absence of NCL info are re-used.

### Frequency prioritization

RAN2 agreed that frequency prioritization may be provided:

- Frequency prioritization may be provided to the UE for cell reselection for multicast reception in RRC_INACTIVE, detailed mechanism on how to identify the frequency info (e.g., SAI, USD, or frequency info directly provided by network) is FFS.

The Rel-17 MBS broadcast frequency prioritization is used to “start” broadcast reception, i.e. to re-select to the MBS broadcast frequency. The NCL info is used to “continue” reception.

But the Rel-18 use case is different. The UE typically starts multicast reception in RRC_CONNECTED, and the UE may be released to RRC_INACTIVE to continue reception when there is extreme congestion. It is not clear when the use case occurs when the UE starts multicast reception in RRC_INACTIVE. The UE always has to go to RRC_CONNECTED mode and join the multicast session first.

If there is a choice the UE prefers to receive multicast in RRC_CONNECTED, i.e. the UE does not prefer to re-select to a frequency where it can receive multicast in RRC_INACTIVE with reduced QoS. Multicast reception in RRC_INACTIVE for power saving reasons is not supported.

The UE does not expect to be released when the serving frequency does not enable the UE to continue multicast reception.

Congestion is not expected to happen on certain frequencies, i.e. multicast reception in RRC_INACTIVE is not expected to be configured on a specific frequency.

Congestion may occur when there is a high number of mission critical UEs in a specific geographical area. But the UE expects to be released to RRC_INACTIVE only when RRC_CONNECTED is congested on all the other frequencies, and multicast reception is configured on all frequencies. In such use case frequency prioritization is not needed.

So we think that the use case for frequency prioritization does not exist or is very rare. Furthermore the wanted behavior is not that the UE re-selects to a frequency where it can receive multicast in RRC_INACTIVE, but it is preferred to receive multicast in RRC_CONNECTED.

Finally, if multicast reception in RRC_INACTIVE is not enabled this is an indication that the gNB is not congested. If the UE re-selected to the cell when the session is active, then this triggers the UE to resume because it does not have a valid configuration of the cell. If the UE re-selected to the cell when the session is inactive, then the UE will be group paged when the session is activated.

Theoretically, there could be congestion on all frequencies, i.e. multicast reception is enabled on all frequencies, but not all sessions are supported on all frequencies. We think this is a rare case, and instead of enabling frequency prioritization the network could provide all sessions on all frequencies.

Observation 6: The use case is rare that frequency prioritization is needed for multicast reception in RRC_INACTIVE. A solution that can dynamically adapt to the area where congestion is experienced is complex because pre-configuration cannot be used. It is unwanted behavior when the UE reselects to a frequency where it can receive the session in RRC_INACTIVE instead of receiving the session in RRC_CONNECTED. And finally, there is no problem that needs to be solved, i.e. when the UE re-selects to a cell where the session is not provided in RRC_INACTIVE while the session is active the UE resumes, otherwise the UE will be group paged when the session is activated.

Proposal 12: Frequency prioritization is not supported for multicast reception in RRC_INACTIVE.

## PTM configuration

### Initial PTM configuration

RAN2 agreed that the initial PTM configuration may be provided via RRCRelease. And RAN2 agreed that the PTM configuration after cell re-selection or when it changes is provided via MCCH.

It is possible that the UE joins a session early and that the session is activated later. In such case the initial PTM configuration may not be provided in RRCRelease. The CN will trigger group paging for the UEs that are in CM-IDLE when the session is activated. However the UEs in RRC_INACTIVE, that already joined the session, could remain in RRC_INACTIVE and acquire the PTM configuration that is provided via MCCH to support mobility or change:

Proposal 13: The UE, that already joined the session, acquires the initial PTM configuration from MCCH in case it was not provided in RRCRelease.

RAN2 agreed that at least the initial PTM configuration for the serving cell can be provided in RRCRelease, FFS for other cases. We think it is more complex to coordinate the PTM configurations with neighbour cells (both intra- and inter-gNB). We prefer to keep the PTM configuration locally in the cell: only the PTM configuration of the serving cell can be provided in RRCRelease and the PTM configuration is provided via MCCH for mobility:

Proposal 14: The PTM configuration provided in RRCRelease is only for the serving cell.

### PTM configuration change

The PTM configuration is not expected to change often, if at all. If needed, the PTM configuration can be changed (in a synchronized fashion at the modification period boundary) via MCCH for UEs in RRC_INACTIVE.

RAN2 agreed that UEs in RRC_CONNECTED mode do not rely on the MCCH for the PTM configuration (change). The PTM configuration for connected mode UEs is changed via RRCReconfiguration.

This complicates a synchronized PTM configuration change for UEs in RRC_CONNECTED and RRC_INACTIVE when shared resources are used.

Furthermore in case the PTM configuration needs to be changed for a large number of UE in RRC_CONNECTED, then the gNB needs to transmit and process RRCReconfiguration and RRCReconfigurationComplete messages for perhaps thousands of UEs simultaneously. Due to the delay to reconfigure all UEs this may cause a noticeable interruption at the UE side.

Synchronization between connected and inactive PTM change can be achieved by configuring an activation time for the PTM configuration in RRCReconfiguration message that coincides with the modification period boundary on the MCCH. An activation time also enables a synchronized change for a large number of UEs in connected mode:

Proposal 15: The mrb-ActivationTime IE (OP) in MRB-ToAddMod-r17 IE indicates when the UE applies the new MRB configuration (SFN), otherwise the UE applies the configuration “now”.

The mrb-ActivationTime IE could be defined as x LSBs of the SFN.

### MCCH is absent

RAN2 agreed that, after cell re-selection, when the PTM configuration of the session the UE is receiving is not present on the MCCH, the UE initiates resume:

- When a UE reselects to a cell for which PTM configuration is not available in multicast MCCH, the UE initiates RRC resumption procedure for an active multicast session it is interested to receive or continue receiving.

Some companies indicated that absence of the MCCH should be considered a network configuration error. But the MCCH is absent when the UE moves out of the congested area, i.e. when the UE re-selects to a cell that is not congested. The NCL may not be configured on the serving cell, i.e. NCL info does not trigger resume in the neighbour cell:

Proposal 16: When a UE reselects to a cell in which the MCCH is absent and NCL info is not provided on the serving cell, the UE initiates RRC resumption procedure for an active multicast session it is interested to receive or continue receiving.

### PTM configuration structure

The PTM configuration structure should take the Rel-17 MBS broadcast configuration as a baseline and changes need to be motivated:

Proposal 17: The structure of the MBSBroadcastConfiguration message is the baseline for the multicast PTM configuration in RRC_INACTIVE (i.e. changes need to be motivated)

The source of potential complexity with the PTM configuration originates from a desire to use shared resources and to enable the UE in RRC_INACTIVE to receive HARQ retransmissions triggered by UEs in RRC_CONNECTED. Normally this is a noble desire, but in this case the complexity is not well motivated. Congestion is only expected to happen rarely. And when there is congestion there are a lot of mission critical UEs in RRC_CONNECTED that all monitor the same shared DL transmissions, which only scale with the number of active sessions. Furthermore only one UE per session has the floor, i.e. one UE per session is transmitting in the uplink. In such use case resource usage is not the main concern.

The reliability in RRC_INACTIVE can be provided via L1 repetitions or encoding as in Rel-17 MBS broadcast:

Proposal 18: Configuration of separate resources and transmission of separate PDSCH for multicast reception in RRC_INACTIVE and RRC_CONNECTED is supported.

### Dedicated vs common configuration

The PTM configuration for Rel-17 multicast is done via dedicated signalling, supporting many different configuration options to enable UE individual configuration. Dedicated signalling in some cases provides both a UE specific version and a common version of the IE, i.e. ASN.1 version.

The PTM configuration for Rel-17 broadcast is done via common signalling, i.e. the PTM configuration is cell specific and shared between the UEs.

During the Rel-17 time frame, on request from RAN3 [2], RAN2 discussed to introduce a common ASN.1 structure for the lower layer multicast configuration (for PHY/MAC/RLC) [3]. Most companies agreed that this would imply a big ASN.1 impact, and that a UE specific configuration would need to be supported (e.g. HARQ feedback configuration), and that a UE may be interested to receive multiple sessions with different configurations. A common ASN.1 structure in addition to the dedicated ASN.1 structure would then also need to be maintained in parallel in future releases, i.e. creating additional complexity:

Proposal 19: Rel-18 does not introduce a common structure for multicast configuration for UEs in RRC_INACTIVE and RRC_CONNECTED.

### RedCap UEs

The general believe is that RedCap UEs can also support MBS, therefore:

Proposal 20: A RedCap UE can indicate to support multicast reception in RRC_INACTIVE.

In Rel-17 a RedCap BWP may be configured for RedCap UE. And in Rel-18 a RedCap CFR will be introduced:

Proposal 21: A RedCap UE uses the RedCap BWP (if configured) and RedCap CFR (if configured) to receive multicast in RRC_INACTIVE.

# Summary

RAN2 is kindly asked to discuss multicast reception in RRC_INACTIVE:

## Notifications

Proposal 1: Introduce a bit string in the Paging message (Rel-18 extension), to indicate for each TMGI in pagingGroupList-r17 whether a Rel-18 UE that has a valid PTM configuration to receive the session in RRC_INACTIVE should not resume or not (aka “stay put” bit). When the bit is not set or when the bitstring is absent or when the UE does not have a valid PTM configuration, the Rel-18 UE resumes.

Observation 1: Notification via MCCH is more robust compared to Paging, and if the UE misses a start notification the UE does not receive the multicast data during the following “active period”.

Proposal 2: MCCH DCI is used to notify start/stop monitoring G-RNTI and a time offset can be configured between MCCH PDCCH and PDSCH.

## State transitions

Proposal 3: The UE resumes when the measured RSRP or RSRQ drops below the configured threshold.

Proposal 4: The threshold can be configured in MCCH or RRCRelease (when MCCH is not configured).

Proposal 5: If a resume threshold is configured, the UE can be configured to resume before cell reselection is triggered.

Observation 2: gNB can use UE individual paging to get a special UE (or any other UE) back to RRC_CONNECTED before group paging is used and no further enhancements are needed.

## Power saving

Observation 3: In some cases it is not beneficial or not possible to release the UEs when a session is deactivated or when there is data-inactivity.

Proposal 6: Add another (more power efficient) PTM DRX state the UE enters when there is pro-longed data-inactivity.

Proposal 7: Introduce the following PTM DRX parameters:

- drx-ShorterOnDurationTimerPTM (optional): 	the duration at the beginning of a longer DRX cycle

- drx-LongerCyclePTM (optional): 		the longer DRX cycle

- drx-LongerCycleTimerPTM (optional):	the duration the UE shall follow the long DRX cycle

## Service continuity

Observation 4: The NCL does not improve the service continuity when the session is provided on the neighbour cell, but the PTM configuration is different between the neighbour and serving cell.

Proposal 8: The multicast NCL does not include PTM configuration info of neighbouring cells.

Observation 5: When the NCL is not configured, the service continuity can be improved by configuring a shorter repetition period during a modification period, without impacting the UE power consumption.

Proposal 9: The multicast NCL includes a SessionInfoList (details FFS), NeighbourCellList (re-use MBS-NeighbourCellList-r17) and mtch-NeighbourCell (bitstring).

Proposal 10: Based on the NCL info the UE may trigger resume in the neighbour cell before acquiring the MCCH.

Proposal 11: The Rel-17 principles of presence and absence of NCL info are re-used.

Observation 6: The use case is rare that frequency prioritization is needed for multicast reception in RRC_INACTIVE. A solution that can dynamically adapt to the area where congestion is experienced is complex because pre-configuration cannot be used. It is unwanted behavior when the UE reselects to a frequency where it can receive the session in RRC_INACTIVE instead of receiving the session in RRC_CONNECTED. And finally, there is no problem that needs to be solved, i.e. when the UE re-selects to a cell where the session is not provided in RRC_INACTIVE while the session is active the UE resumes, otherwise the UE will be group paged when the session is activated.

Proposal 12: Frequency prioritization is not supported for multicast reception in RRC_INACTIVE.

## PTM configuration

Proposal 13: The UE, that already joined the session, acquires the initial PTM configuration from MCCH in case it was not provided in RRCRelease.

Proposal 14: The PTM configuration provided in RRCRelease is only for the serving cell.

Proposal 15: The mrb-ActivationTime IE (OP) in MRB-ToAddMod-r17 IE indicates when the UE applies the new MRB configuration (SFN), otherwise the UE applies the configuration “now”.

Proposal 16: When a UE reselects to a cell in which the MCCH is absent and NCL info is not provided on the serving cell, the UE initiates RRC resumption procedure for an active multicast session it is interested to receive or continue receiving.

Proposal 17: The structure of the MBSBroadcastConfiguration message is the baseline for the multicast PTM configuration in RRC_INACTIVE (i.e. changes need to be motivated)

Proposal 18: Configuration of separate resources and transmission of separate PDSCH for multicast reception in RRC_INACTIVE and RRC_CONNECTED is supported.

Proposal 19: Rel-18 does not introduce a common structure for multicast configuration for UEs in RRC_INACTIVE and RRC_CONNECTED.

Proposal 20: A RedCap UE can indicate to support multicast reception in RRC_INACTIVE.

Proposal 21: A RedCap UE uses the RedCap BWP (if configured) and RedCap CFR (if configured) to receive multicast in RRC_INACTIVE.

# References

- R2-2305914, Clarification for Mission Critical UEs, Ericsson, DISC, RAN2#122

- R2-2202141, LS on NR RRC to support split NR-RAN architecture for NR MBS, RAN3, LS in, To:RAN2, RAN2#117

- R2-2203780, [AT117-e][043][MBS] Invited tdocs open Issues CP, Nokia, Report, RAN2#117

- R2-2303971, RRC running CR for eMBS, Huawei, HiSilicon, draftCR 38.331	, RAN2#121bis-e