3GPP TSG-RAN WG2 Meeting #122	R2-2305921

Incheon, Korea, 22 - 26 May 2023

Agenda Item:	7.20.2

Source:	Huawei, HiSilicon

Title:	Two TAs for multi-DCI multi-TRP

Document for:	Discussion and decision

# 1. Introduction

In [1], a WID on MIMO evolution was approved. One of the objectives with RAN2 impact is the following:

Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

Both RAN1 and RAN2 have already made a number of agreements on 2 TAs for UL multi-DCI for multi-TRP operation. In this contribution, we further discuss the RAN2 impacts of 2 TAs for UL mTRP multi-DCI operation taking into account RAN1 agreements.

# 2. Discussion

In Rel-16, in multi-DCI multi-TRP operation, the UE uses the same TA for both TRPs. In Rel-17, Inter-Cell Beam Management (ICBM) was introduced in which the UE is configured with additional PCIs and transmission can switch between the serving cell TRP and a TRP associated with an additional PCI, while using the same TA for UL. Rel-18 will allow multi-DCI multi-TRP operation using TRP associated with an additional PCI as well as using 2 different TAs for different TRPs.

## 2.1 TA acquisition for 2 TAs

RAN1 agreed that 2 TA acquisition is done by CFRA triggered by PDCCH order for either serving PCI and additional PCI. Currently, parallel RACH is not supported from a MAC entity perspective for reducing the UE complexities, i.e. if a new RA is triggered, the UE either stops the ongoing RA procedure and starts a new RA procedure or continues the ongoing RA procedure as captured below [2].

NOTE 1: If a new Random Access procedure is triggered while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure (e.g. for SI request).

- This requirement should be also applicable to RA on mTRP, e.g. if both RA procedure are triggered on both serving PCI and additional PCI for the same serving cell.

In multi-DCI multi-TRP operation, parallel RACH on different TRPs is not supported.

## 2.2 TA command

On TA command for mTRP, RAN1 agreed to use absolute TA command MAC CE for mTRP for 2-step RACH on the SpCell but left the details on indicating TAG ID up to RAN2:

RAN1#110bis Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support enhancements related to indicating TAG ID via absolute TA command: - FFS: whether the indication is implicit or explicit - Detailed indication schemes are FFS - This does not preclude indication of two TAG IDs (if supported) - Note: This applies at least to MSGB in case of C-RNTI RAN1#112bis Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, how to indicate the TAG ID via absolute TA command MAC CE is left up to RAN2: - One of two TAG IDs configured in the SpCell can be indicated

Based on the RAN1 agreement, it is required to provide TAG id information in the absolute TA command MAC CE in response to MSGA transmission. As the SpCell can be configured with two TAGs at most, using one R bit of the absolute TA command MAC CE would be sufficient for indicating the TAG associated with SpCell, for instance, “0” means the first TAG configured for SpCell and “1” means the second TAG configured for SpCell.

#### 6.1.3.4a Absolute Timing Advance Command MAC CE The Absolute Timing Advance Command MAC CE is identified by MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a fixed size and consists of two octets defined as follows (Figure 6.1.3.4a-1): - Timing Advance Command: This field indicates the index value TA used to control the amount of timing adjustment that the MAC entity has to apply in TS 38.213 [6]. The size of the field is 12 bits; - R: Reserved bit, set to 0. Figure 6.1.3.4a-1: Absolute Timing Advance Command MAC CE

Use one R bit of the absolute TA command MAC CE to indicate the TAG which the TA command applies to. “0” means the first TAG configured for SpCell and “1” means the second TAG configured for SpCell.

Introducing TAG ID info in RAR (i.e. RAR for 4-step RA and successRAR/fallbackRAR for 2-step RA) is one of the alternatives (i.e. alt 1) considered by RAN1 at least for intra-cell mTRP as showed below:

RAN2#112 bis Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, down-select one of the following alternatives: - Alt 1: indicate TAG ID as part of TA command in RAR - Alt 2: indicate TAG ID as part of PDCCH order - Alt 3: divide SSBs into two groups, one for each TRP. If a SSB associated to a RACH procedure belongs to the nth group (n=1, 2), then the TA obtained via the RACH procedure corresponds to the nth TRP.

RAN2 should wait for RAN1 progress to decide whether or not to modify the RAR format for intra-cell and for inter-cell mTRP.

Wait for RAN1 to decide whether to modify the RAR format for 2-step RACH and 4-step RACH.

## 2.3 TAG

In Rel-17, a TAG contains multiple serving cells that use the same TA. RAN1 agreed to have two TAGs configured for the same serving cell and associate those two TAGs with different TCI states as shown below:

Agreement For multi-DCI based multi-TRP operation with two TAs, support configuring two TAGs belonging to a serving cell. RAN1#112 Agreement For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, support the following: Associate TAG to TCI-state - Associate TAG ID with UL/joint TCI state - For UL transmission, the TAG ID associated with the UL/joint TCI state is utilized - A baseline is UE expects that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index correspond to one TAG - Working Assumption: A UE may report that it supports that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESETPoolIndex correspond to both TAGs FFS: on how to handle association when Rel-15/16 spatial relation framework is used for - PUCCH - DG/CG Type 1/Type 2 PUSCH - AP/SP/P SRS

In current specifications, a serving cell belongs to a TAG so RAN1 agreement that two TAGs can belong to the serving cell seems not to be aligned with that. However, RAN1 also mentioned association of a TAG ID with each UL/joint TCI state. Therefore, we understand that the intention is that a serving cell can be associated with two TAGs and UL/joint TCI states of this serving cell will be associated with one of the two TAGs.

RAN2 assumes that a serving cell can be associated with two TAGs and each UL/joint TCI state of the serving cell belongs to either TAG associated with the serving cell.

Currently the UE will report UE capability on maximum TAG number in CA-ParametersNR to network as shown below [3]:

CA-ParametersNR ::= SEQUENCE { dummy ENUMERATED {supported} OPTIONAL, parallelTxSRS-PUCCH-PUSCH ENUMERATED {supported} OPTIONAL, parallelTxPRACH-SRS-PUCCH-PUSCH ENUMERATED {supported} OPTIONAL, simultaneousRxTxInterBandCA ENUMERATED {supported} OPTIONAL, simultaneousRxTxSUL ENUMERATED {supported} OPTIONAL, diffNumerologyAcrossPUCCH-Group ENUMERATED {supported} OPTIONAL, diffNumerologyWithinPUCCH-GroupSmallerSCS ENUMERATED {supported} OPTIONAL, supportedNumberTAG ENUMERATED {n2, n3, n4} OPTIONAL, ... }

The need to use different TAs for operation with different serving cells/TRPs comes from the different physical location or frequency ranges used by these serving cells/TRPs. In Rel-17, the UE can be configured with 32 serving cells but it was considered that only 4 TAGs are sufficient for all of them, i.e. it is assumed that this is sufficient for the diversity of physical location and frequency ranges. For multi-DCI multi-TRP operation, it is expected that the network deployments will not be modified so the same diversity of physical location and frequency range will exist and 4 TAGs should also be sufficient.

In Rel-18, the maximum number of TAGs is the same like in Rel-17, including when multi-DCI multi-TRP operation is used.

## 2.4 TAT expiry

According to TS 38.321[2], the TAT is maintained per TAG (e.g. sTAG or pTAG) and when the TAT expires, the UE behaviours are different between sTAG and pTAG:

## 5.2 Maintenance of Uplink Time Alignment RRC configures the following parameters for the maintenance of UL time alignment: - timeAlignmentTimer (per TAG) which controls how long the MAC entity considers the Serving Cells belonging to the associated TAG to be uplink time aligned; [skip] 1> when a timeAlignmentTimer expires: 2> if the timeAlignmentTimer is associated with the PTAG: 3> flush all HARQ buffers for all Serving Cells; 3> notify RRC to release PUCCH for all Serving Cells, if configured; 3> notify RRC to release SRS for all Serving Cells, if configured; 3> clear any configured downlink assignments and configured uplink grants; 3> clear any PUSCH resource for semi-persistent CSI reporting; 3> consider all running timeAlignmentTimers as expired; 3> maintain NTA (defined in TS 38.211 [8]) of all TAGs. 2> else if the timeAlignmentTimer is associated with an STAG, then for all Serving Cells belonging to this TAG: 3> flush all HARQ buffers; 3> notify RRC to release PUCCH, if configured; 3> notify RRC to release SRS, if configured; 3> clear any configured downlink assignments and configured uplink grants; 3> clear any PUSCH resource for semi-persistent CSI reporting; 3> maintain NTA (defined in TS 38.211 [8]) of this TAG.

RAN2 sent a LS to RAN1 asking whether UL transmission towards a TRPs are impacted when TAT for the other TRP of the same serving cell expires. For now, RAN2 should wait for RAN1 feedback.

Question 2 on operation Q2a: When the timer associated with one of the TRPs of a serving cell expires, is it according to RAN1 view that UL towards that TRP (e.g. SRS resources sets associated to that TRP) are impacted but UL towards the another TRP (e.g. SRS resources sets associated to that TRP) can remain in operation? Q2a: Is there a need from RAN1’s perspective to have two PTAGs in case of two TAGs belongs to a PCell?

However, we understand that PUSCH transmission should anyway be possible on the TRP for which the TAT is still valid. According to MAC specification, HARQ buffers for UL transmission is maintained per serving cell so there is no need to flush them if only one associated TAT expires, i.e. for multi-DCI multi-TRP operation, the network can schedule retransmission for UL data through another associated TAG whose TAT. Of course, HARQ buffers should be flushed if both TATs are expired. If this is for the PCell, HARQ buffers or all serving cells, i.e. the current procedure on HARQ buffer flushing is performed.

HARQ buffers are not flushed on a serving cell until both associated TATs expire. If this is PCell for mTRP, HARQ buffers are flushed for all serving cells if both associated TATs expire.

# 3. Conclusion

Based on the discussion in this paper, we propose the following:

- In multi-DCI multi-TRP operation, parallel RACH on different TRPs is not supported.

Use one R bit of absolute TA command MAC CE to indicate the TAG which the TA command applies to. “0” means the first TAG configured for SpCell and “1” means the second TAG configured for SpCell.

Wait for RAN1 to decide whether to modify the RAR format for both 2-step RACH and 4-step RACH defined in MAC spec.

RAN2 assumes that a serving cell can be associated with two TAGs and each UL/joint TCI state of the serving cell belongs to either TAG associated with the serving cell.

In Rel-18, the maximum number of TAGs is the same like in Rel-17, including when multi-DCI multi-TRP operation is used.

- Proposal 6: HARQ buffers are not flushed on a serving cell until both associated TATs expire. If this is PCell for mTRP, HARQ buffers are flushed for all serving cells if both associated TATs expire.

# Reference

[1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung

[2] 3GPP TS 38.321, V17.3.0

[3] 3GPP TS 38.331, V17.3.0