3GPP TSG-RAN WG2 Meeting #122	R2-2305923

Incheon, South Korea, 22– 26 May 2023

Agenda item:	7.15.3

Source:	Nokia, Nokia Shanghai Bell

Title:	On MCSt impacts on the resource selection procedure

WID/SID:	NR_SL_enh2 - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In an LS from RAN1 in R2-2304618 RAN1 has provided the following questions to RAN2 regarding the matter, and the feasibility of a set of listed solutions.

RAN1 has discussed the following approaches to implement/achieve MCSt for SL-U communication. RAN1 would like to seek RAN2’s opinion on the following questions. Approach 1: “best effort for multiple TBs” - Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) - R16/17 behavior. - Step 2: L1 report a set of candidate single-slot resource (SA) according to existing L1 resource allocation procedure - R16/17 behavior. - Step 3: Higher layer selects a set of resources either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior) to achieve MCSt. - Step 4: Repeat Step 1-3 for different TB if required. Approach 2: “guarantee MCSt for single TB and best effort for multiple TBs” - Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) + “number of slots for MCSt” which could be derived based on CAPC of the logical channel/TB or other means. - Step 2: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure (FFS: RSRP calculation / threshold may need to change) - Step 3: Higher layer selects a candidate multi-slot resource either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior). - Step 4: Repeat Step 1-3 for different TB if required. Approach 3: “guarantee MCSt for multiple TBs” - Step 1: Higher layer triggers L1 resource (re-)selection one time for one or multiple TBs with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) + “number of slots for MCSt” which could be derived based on CAPC of the multiple TBs. - Step 2: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure (FFS: RSRP calculation / threshold may need to change) - Step 3: Higher layer selects transmission resource for the one or multiple TB(s) from the reported set of candidate multi-slot resource (SA). Question 1 (for Approach 1/ Approach 2): feasibility of selecting the resource for a single TB in MAC layer (single-slot under Approach 1, multi-slot under Approach 2) with the principle of “concatenating” across separate resource selection triggers (across TBs) Question 2 (for Approach 3): feasibility of triggering the resource selection procedures for multiple SL processes at the same time Question 3 (Approach 2/ Approach 3): feasibility of providing a new parameter “number of slots for MCSt” to L1 when triggering resource (re-)selection for MCSt

This TDoc discusses these issues and proposes a way forward.

# 2	Discussion

From RAN1 discussion, we see that RAN1 has discussed three options. Below, we will discuss each of these three options, and propose a way forward, however, as some of the approaches rely on the MAC procedure to perform resource selection for multiple TBs, we propose RAN2 to discuss first (i) how to trigger the request of candidate resources from the physical layer when these are to be applied for multiple TBs by the MAC and (ii) how the SL-SCH data transmission procedure (as currently defined in 5.22.1.1 of TS 38.321) can be extended to be applicable from a single TB to be applicable across multiple TBs.

As stated in our contribution R2-2305825 the issue with requesting multiple TBs also takes presence in the SL-Coexistence work, as with a 30 kHz carrier there is no current way to split a single TB into multiple grants/resources.

Observation 1: Requesting resources for multiple TBs may also be needed in SL-Coexistence agenda item.

Proposal 1: RAN2 to discuss the feasibility for MAC of requesting and handling resources across multiple TBs before discussing the feasibility of all the approaches.

Approach 1, as depicted in Figure 1, relies on legacy behaviour between PHY and MAC where the PHY provides resources to MAC based on legacy signalling. Furthermore, RAN1 provides the possibility of offloading the behaviour of selecting consecutive slots to MAC. Two issues remain with the suggested approach; one of this is that the resource selection procedure in MAC has no concept of consecutive slots. Such issue may be handled simply by UE implementation i.e. by a note that it is up to UE implementation to only select slots which result on the MCSt behaviour. However, such selection procedure may drastically reduce the available resources upon selection, as the MAC may now not be provided with many single-slot resources that may abide to this request.

Figure 1: Approach 1: (a) Single-slot resources provided in the candidate resource for a single TB by the PHY and then the MAC identifies suitable single-slot resources that can be grouped together to create MCSt sets by applying the consecutive-slots criterion. The MAC can then select randomly between the identified MCSt sets; (b) Single-slot resources provided in separate candidate resource sets for each TB and then the MAC identifies suitable single-slot resources (from each TB candidate resource set) that can be grouped together to create MCSt sets by applying the consecutive-slots criterion.

L1 is the one responsible for selecting the resource candidate based on different criteria (prioTX, remaining PDB, LsubCH and Prsvp_TX). In order to efficiently be able to support MCSt within resource selection L1 needs to provide some sets of resources which supports the MCSt requirement.

Observation 2: Approach 1 relies entirely on MAC procedures to ensure the consecutive slot is achieved for MCSt, however this may reduce the amount of available resources to select.

Observation 3: Approach 1 may not be a feasible approach, if the PHY layers resources does not provide the concept of consecutive slots.

Proposal 2: RAN2 to agree that Approach 1 is interpreted as feasible but with little to no gain in performance.

Approach 2, as depicted in Figure 2, resolves the issue with approach 1, as L1 will supply sets of resources which can be selected with minimum change in MAC. However, RAN1 also describes in step 3 that maybe a consecutive-slot criterion is needed, which is not clear why MAC would need such a rule if the resources provided fulfill the MCSt criterion by themselves. Furthermore, the difference between Approach 1 and Approach 2 is whether multiple TBs or a single TB is requested by MAC. For MAC, the procedure in step 4 is the simplest approach in terms of multiple TBs, as the LCP procedure in MAC is defined per TB. Hence, if we want to have multiple TBs then somehow there will be the need to have some entity/procedure that coordinates the resource selection such that these TBs are in consecutive slots.

Figure 2: Approach 2: (a) Consecutive slot resources provided in the candidate resource for a single TB by the PHY and then the MAC selects randomly between the provided consecutive slot resources in the candidate resource set; (b) Single-slot resources (or consecutive slot resources) provided in separate candidate resource sets for each TB and then the MAC identifies suitable single-slot resources (or consecutive slot resources) from each TB candidate resource set that can be grouped together to create MCSt sets by applying the consecutive-slots criterion.

Observation 4: Approach 2 addresses the issue present in approach 1 by having the PHY provide to the MAC set of groups of consecutive resources, allowing the MAC apply the Rel.16/17 selection principle to select a group of consecutive slots.

Observation 5: RAN2 may need to clarify with RAN1 what the intention is with the consecutive-slots criterion since RAN1 would select one of the sets fulfilling the requirement and not have any consecutive slot issues.

Observation 6: MAC procedure does not currently support transmitting different TBs within a single resource selection procedure.

Proposal 3: RAN2 to agree that Approach 2 is feasible from RAN2 point of view at least for the single TB case.

Finally, approach 3, depicted in Figure 3, has the same advantage as approach 2, however in this case L1 depends on the resource selection procedure to also request resources for multiple TBs. This may cause a significant change to the resource selection procedure, which currently does not allow such operation. However, the approach can become feasible in case it is possible to request multiple TBs (i.e. provide the following parameters prioTX, remaining PDB, LsubCH and Prsvp_TX per TB and then the candidate resource set includes a group of resources suitable for each TB).

Figure 3: Approach 3: (a) Consecutive slot resources provided in the candidate resource for a single TB by the PHY and then the MAC selects randomly between the provided consecutive slot resources in the candidate resource set; (b) Consecutive slot resources provided in the same candidate resource set for multiple TBs and then the MAC selects randomly between the provided consecutive slot resources in the candidate resource set.

Observation 7: Approach 3 may need a significant change to the MAC specification as the resource selection procedure currently is per TB.

Proposal 4: If RAN2 agrees that if resources for multiple TBs can be requested from the PHY, then that approach 3 is feasible from RAN2 point of view.

# 3	Conclusion

We made the following observations and proposals in this document:

Observation 1: Requesting resources for multiple TBs may also be needed in SL-Coexistence agenda item.

Proposal 1: RAN2 to discuss the feasibility for MAC of requesting and handling resources across multiple TBs before discussing the feasibility of all the approaches.

Observation 2: Approach 1 relies entirely on MAC procedures to ensure the consecutive slot is achieved for MCSt, however this may reduce the amount of available resources to select.

Observation 3: Approach 1 may not be a feasible approach, if the PHY layers resources does not provide the concept of consecutive slots.

Proposal 2: RAN2 to agree that Approach 1 is interpreted as feasible but with little to no gain in performance.

Observation 4: Approach 2 addresses the issue present in approach 1 by having the PHY provide to the MAC set of groups of consecutive resources, allowing the MAC apply the Rel.16/17 selection principle to select a group of consecutive slots.

Observation 5: RAN2 may need to clarify with RAN1 what the intention is with the consecutive-slots criterion since RAN1 would select one of the sets fulfilling the requirement and not have any consecutive slot issues.

Observation 6: MAC procedure does not currently support transmitting different TBs within a single resource selection procedure.

Proposal 3: RAN2 to agree that Approach 2 is feasible from RAN2 point of view at least for the single TB case.

Observation 7: Approach 3 may need a significant change to the MAC specification as the resource selection procedure currently is per TB.

Proposal 4: If RAN2 agrees that if resources for multiple TBs can be requested from the PHY, then that approach 3 is feasible from RAN2 point of view.