3GPP RAN WG2 Meeting #122	R2-2305925
Incheon, Korea, May 22nd – 26th, 2023

Agenda Item:	7.3.2

Source:	InterDigital

Title:	Cell DTX/DRX mechanism

Document for:	Discussion, Decision

# Introduction

During the first two meetings of the R18 NR NES work item [1], a number of agreements were made, captured in the Appendix. The following FFS and open issues remaining to be discussed:

- PDCCH monitoring for retransmissions and new transmissions when cell DTX is configured and activated, including how to deal with any exceptions for SR or RACH.

- How to implement the agreement on not monitoring PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time.

- whether there is a need have DTX UE specific inactivity timer or just rely on C-DRX.

- Activation/deactivation of Cell DTX/DRX.

This contribution discussed these open issues on Cell DTX/DRX.

# Discussion

## Activation/De-activation of Cell DTX/DRX

In RAN#121bis, it was agreed that, as baseline, Cell DTX/DRX is activated/deactivated implicitly by RRC signalling, i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released. From RAN2 point of view, a majority companies see a benefit with L1 signalling for Cell DTX/DRX activation/deactivation, and an LS was sent to RAN1 accordingly [5]. Further, RAN1 had a working assumption that support of L1 signalling at least for activation and deactivation of a cell DTX and/or DRX configuration is feasible (e.g., in terms of enabling/disenabling the feature) from RAN1 perspective.

Relying on RRC signalling along to activate Cell DTX can suffer from a considerable RRC signalling processing delay and an associated ambiguity period. TS 38.331 specifies the processing delay requirements for RRC procedures. The performance requirement is expressed as the time in [ms] from the end of reception of the network -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> network response message with no access delay, which expressed as the red arrow in Figure 1.

Figure 1: Illustration of RRC procedure processing delay from TS 38.331

In the case of RRC reconfiguration, the following processing delay is specified:

| Procedure title: | Network -> UE | UE -> Network | Value [ms] |
| --- | --- | --- | --- |
| RRC reconfiguration | RRCReconfiguration | RRCReconfigurationComplete | 10 |

Such that the processing delay requirement is the time in [ms] from the end of reception of RRCReconfiguration up to when the UE shall be ready for the reception of DCI scheduling uplink grant for the transmission of RRCReconfigurationComplete with no access delay. The delay requirement is 10 ms, even in the absence of scheduling delays to receive RRCReconfigurationComplete.

Observation 1: 	TS 38.331 specifies the processing delay requirement for RRC reconfiguration as time in [ms] from the end of reception of RRCReconfiguration up to when the UE shall be ready for the reception of DCI scheduling uplink grant for the transmission of RRCReconfigurationComplete with no access delay.

This delay can therefore delay the time the gNB can assume Cell DTX status is changed by the UE up to 10 ms, or even more if time need for the UE to respond is considered. If the gNB needs to (de)-activate Cell DTX for a number of UEs, the scheduling delay can be further variable for different UEs, especially if some require retransmission of RRCReconfigurationComplete. It is therefore good to confirm the working assumption that (de)-activation of Cell DTX/DRX can be performed by group common L1 signalling.

Observation 2: 	the processing delay requirement for RRC reconfiguration is 10 ms even without the time needed to receive RRCReconfigurationComplete, which can cause considerable delays to the start the activation or deactivation of Cell DTX.

Proposal 3: 	Confirm that L1 signalling for Cell DTX/DRX can be used for activation/deactivation, from RAN2 perspective.

From a reliability perspective, it is up to the gNB whether to also use RRC signalling for some UEs (e.g. in bad coverage) in addition to common L1 signalling. In the rare case where the UE misses L1 Cell DTX de-activation signalling, the gNB should be aware as it can determine whether transmissions related to the scheduled DCIs are received or not. In case the UE misses the L1 signalling for Cell DTX activation, there is no impact on the UE as the UE can rely on C-DRX for UE power savings.

Observation 4: 	It is up to the gNB whether to use RRC or L1 signalling to (de)-activate Cell DTX for some UEs when depending on the reliability levels needed for each UE. Rare misdetection of L1 signalling for Cell DTX/DRX activation does not have any considerable negative consequences and can be rectified by the gNB.

## PDCCH monitoring during Cell DTX/DRX non-active periods

In a pre-meeting email discussion prior to RAN2#121b [3], PDCCH monitoring behaviour during the cell DTX non-active period was discussed, including the cases when the UE is in C-DRX active time or not. The discussion was also split for monitoring for new transmission (e.g. when there are no pending transmissions) and retransmissions (e.g. when drx-retransmission timers are running). In particular, the following options were discussed for the expected UE behaviour for PDCCH monitoring during Cell DTX non-active period

- Option 1: UE doesn’t monitor PDCCH for dynamic grants/assignments during Cell DTX non-active, even if the UE is in C-DRX Active time.

- Option 2: UE monitor PDCCH for dynamic grants/assignments during the UE’s C-DRX Active time per legacy behaviour, even during the Cell DTX non-active period.

The corresponding gNB scheduling behaviour during Cell DTX non-active period for option 1 is gNB does not schedule UE-specific dynamic grants/assignments during cell DTX non-active periods, even if the UE is in C-DRX Active Time. For Option 2, gNB can schedule UE-specific dynamic grants/assignments during cell DTX non-active periods, but not outside of the UE’s C-DRX Active time. It is left to gNB implementation whether to keep scheduling that UE during its (extended) C-DRX Active Time, even if outside Cell DTX/DRX active period, or to stick to the active periods of Cell DTX periodic pattern.

Option 1 was agreed for monitoring PDCCH for new transmissions with the following FFSs. The following open issues are discussed next:

- FFS how to deal with PDCCH monitoring for retransmissions.

- FFS how to deal with any exceptions (e.g. SR if agreed and RACH).

- How to implement agreed behaviour for PDCCH monitoring for new transmissions.

- FFS whether we have DTX UE specific inactivity timer.

FFS#1: PDCCH monitoring for retransmissions during Cell DTX non-active period.

Option 1 vs. Option 2 represent a trade-off between network energy savings and scheduling flexibility. Option 1 results in more energy saving for both the UE and the network, but doesn’t allow for further additional time/scheduling flexibility for scheduling retransmissions and HARQ feedback reception/transmission, e.g. while drx-InactivityTimer or drx-RetransmissionTimer is running. It is worth noting that a cell-wide Cell DTX inactivity timer associated with a given Cell DTX configuration was discussed in RAN2#121 without consensus.  Option 2 assumes that the UE monitors PDCCH according to the active time in the C-DRX cycle, per legacy assumptions, and ensures that UE QoS can still be somewhat maintained with active Cell DTX.

Given Cell DTX/DRX cycle length can be long, it is important to timely finish retransmissions rather than spread them over multiple Cell DTX/DRX on durations. Therefore, for dynamic retransmissions, it is much simpler to rely on existing C-DRX framework (i.e., monitoring PDCCH during drx-RetransmissionTimer) to allow pending transmissions to finish. Retransmissions can otherwise be delayed over long Cell DTX cycles and require designing new different schemes to determine the timing of retransmission scheduling and HARQ feedback. Delaying HARQ feedback transmission and DCIs for retransmissions can also require a large specification impact.

Proposal 4: 	For PDCCH monitoring for dynamic retransmissions during cell DTX non-active period, UE monitors PDCCH for dynamic grants/assignments for retransmissions during the UE’s C-DRX Active time per legacy behaviour, even during the Cell DTX non-active period. No specification impact is foreseen.

Accordingly, to support retransmissions during the non-active period, it should be assumed that if scheduling DCI has been received by the UE either for dynamic grant transmission or a dynamic PDSCH assignment reception, the UE should follow the NW instruction and transmit PUSCH on the dynamic grant even in Cell DRX non-active periods or receive PDSCH (e.g. containing retransmissions) even in Cell DTX non-active periods.

Proposal 5: 	UE can transmit on PUSCH dynamic grants during Cell DRX non-active periods if scheduling DCI was received by the UE, no specification impact is foreseen.

Proposal 6: 	UE can receive dynamic PDSCH assignments during Cell DTX non-active periods if scheduling DCI was received by the UE, no specification impact is foreseen.

FFS#2: how to deal with any exceptions (e.g. SR if agreed and RACH)

MAC specifications already define how and when the UE monitors PDCCH after the transmission of a scheduling request. When C-DRX is configured, MAC specifies that the Active Time includes the time while a scheduling request is sent on PUCCH and is pending. When C-DRX is not configured, MAC already specified when and how the UE monitors PDCCH during SR and RACH procedures. Therefore, no special behaviour need to be specified for PDCCH monitoring post SR transmission, even if the SR was transmitted during the Cell DTX non-active period. There is no point in allowing the UE to transmit a scheduling request if the UE cannot receive the grant timely.

Observation 5: 	TS 38.321 already specifies that the Active Time includes the time while a scheduling request is sent on PUCCH and is pending.

Similarly, for RACH, MAC specifications already specify how and when the UE monitors PDCCH during a RACH procedure. Specifically, the UE monitors PDCCH while ra-ResponseWindow or ra-ContentionResolutionTimer is running -even if C-DRX is configured-. Similarly, PDCCH monitoring for RA-BFR or CFRA is already specified. Therefore, no special behaviour needs to be specified for PDCCH monitoring during a RA procedure, even if it overlaps with a Cell DTX non-active period.

Observation 6: 	TS 38.321 already specifies how the UE monitors PDCCH during a RA procedure:

- if the ra-ResponseWindow or the ra-ContentionResolutionTimer is running, the UE monitor the PDCCH as specified in clauses 5.1.4 and 5.1.5.

- the drx Active Time includes the time while ra-ContentionResolutionTimer is running.

- PDCCH monitoring for RA-BFR or CFRA.

Proposal 7: 	No special behaviour need to be specified for PDCCH monitoring post SR transmission or during a RA procedure, even if the RA or SR procedure overlaps with a Cell DTX non-active period.

How to implement agreed behaviour to not monitor PDCCH for new transmissions during Cell DTX non-active period.

During the online in R2#121b and email discussion [3], some companies mentioned that the UE may not always be able to distinguish whether the PDCCH is for retransmission or new transmission, which is true, however the focus of the behaviour is on whether to monitor PDCCH during the cell DTX non-active period while C-DRX retransmissions timers are not running (i.e. no pending HARQ transmissions).

One way to implement the agreement is to impose a hard restriction on the C-DRX Active Time such that the UE stops the Active time during Cell DTX non-active periods. However, such hard restriction is not possible because the UE is specified to be in Active Time during an RA procedure during the non-active period or whenever an SR was transmitted and pending. Therefore, the easiest way to implement the agreement is to allow the UE to stop the drx-inactivity timer during the cell DTX non-active period if there are no pending retransmissions on any HARQ process.

Observation 7: 	A hard stop to the C-DRX Active Time during Cell DTX non-active periods impacts RA and SR procedures when an RA procedure is ongoing during the non-active period or whenever an SR was transmitted and still pending (per legacy C-DRX assumptions)

Proposal 8: 	The UE can stop drx-inactivity timer during the cell DTX non-active period if there are no pending retransmissions on any HARQ process.

FFS#3: whether we have DTX UE specific inactivity timer.

In RAN2#121 and in email discussion [4], it was discussed whether there is a need to have DTX UE specific inactivity timer, provided it was agreed that the Cell DTX pattern is periodic and configuration for cell DRX/DTX is common for Rel-18 UEs in the cell.

Given the question is about whether we have a UE specific extension, there is no need to have an additional mechanism, given we already have a UE-specific C-DRX inactivity timer. As explained in proposal 4 and 8, drx inactivity and retransmissions timers should be running per legacy assumptions, and the inactivity timer only stops when there are no pending retransmissions during the Cell DTX non-active period. The gNB can thus keep the UE in its extended C-DRX Active Time by scheduling any HARQ process.

Proposal 9: 	No need to have an additional mechanism for a DTX UE specific inactivity timer. Rely on existing UE-specific C-DRX timers (drx inactivity and retransmissions timers). No specification impact is foreseen.

# Conclusion

In this contribution, network energy saving techniques in the time domain is discussed, with focus on higher layer aspects. The following observations and proposals are provided:

Observation 1: 	TS 38.331 specifies the processing delay requirement for RRC reconfiguration as time in [ms] from the end of reception of RRCReconfiguration up to when the UE shall be ready for the reception of DCI scheduling uplink grant for the transmission of RRCReconfigurationComplete with no access delay.

Observation 2: 	the processing delay requirement for RRC reconfiguration is 10 ms even without the time needed to receive RRCReconfigurationComplete, which can cause considerable delays to the start the activation or deactivation of Cell DTX.

Proposal 3: 	Confirm that L1 signalling for Cell DTX/DRX can be used for activation/deactivation, from RAN2 perspective.

Observation 4: 	It is up to the gNB whether to use RRC or L1 signalling to (de)-activate Cell DTX for some UEs when depending on the reliability levels needed for each UE. Rare misdetection of L1 signalling for Cell DTX/DRX activation does not have any considerable negative consequences and can be rectified by the gNB.

Proposal 4: 	For PDCCH monitoring for dynamic retransmissions during cell DTX non-active period, UE monitors PDCCH for dynamic grants/assignments for retransmissions during the UE’s C-DRX Active time per legacy behaviour, even during the Cell DTX non-active period. No specification impact is foreseen.

Proposal 5: 	UE can transmit on PUSCH dynamic grants during Cell DRX non-active periods if scheduling DCI was received by the UE, no specification impact is foreseen.

Proposal 6: 	UE can receive dynamic PDSCH assignments during Cell DTX non-active periods if scheduling DCI was received by the UE, no specification impact is foreseen.

Observation 5: 	TS 38.321 already specifies that the Active Time includes the time while a scheduling request is sent on PUCCH and is pending.

Observation 6: 	TS 38.321 already specifies how the UE monitors PDCCH during a RA procedure:

- if the ra-ResponseWindow or the ra-ContentionResolutionTimer is running, the UE monitor the PDCCH as specified in clauses 5.1.4 and 5.1.5.

- the drx Active Time includes the time while ra-ContentionResolutionTimer is running.

- PDCCH monitoring for RA-BFR or CFRA.

Proposal 7: 	No special behaviour need to be specified for PDCCH monitoring post SR transmission or during a RA procedure, even if the RA or SR procedure overlaps with a Cell DTX non-active period.

Observation 7: 	A hard stop to the C-DRX Active Time during Cell DTX non-active periods impacts RA and SR procedures when an RA procedure is ongoing during the non-active period or whenever an SR was transmitted and still pending (per legacy C-DRX assumptions)

Proposal 8: 	The UE can stop drx-inactivity timer during the cell DTX non-active period if there are no pending retransmissions on any HARQ process.

Proposal 9: 	No need to have an additional mechanism for a DTX UE specific inactivity timer. Rely on existing UE-specific C-DRX timers (drx inactivity and retransmissions timers). No specification impact is foreseen.

# References

RP-223540, “New WID: Network energy savings for NR”, Huawei

TR 38.864, “Study on network energy savings for NR”, v 18.0.0

R2-2303604, “Report of [POST121][311][NES] DTX/DRX - gNB and UE behaviours”, InterDigital

R2-2302796, “Outcome of [POST121][312][NES] DTX/DRX - Configuration/ activation/ deactivation and alignment”, Huawei, HiSilicon

R2-2304568, “LS on Cell DTX/DRX activation/deactivation”, RAN2

# Annex A: Agreements on Cell DTX/DRX

- RAN2#121

Agreements

- There will be no impact to RACH, paging, and SIBs in idle/inactive for both gNB and Rel-18 and legacy UEs

- Rel-18 NES capable CONNECTED UE(s) can perform RACH and receive SIBs in non-active duration of cell DTX and/or DRX (i.e., same behavior for cell DTX and cell DRX).  No further enhancements for CBRA and CFRA will be pursued.

- Pattern configuration for cell DRX/DTX is common for Rel-18 UEs in the cell.   FFS whether we have DTX UE specific inactivity timer .  FFS on configuration signaling and stage 3.

- Confirm study item agreement that we can have separate DTX and DRX configuration.   We will focus on designing DTX/DRX for at least single configuration.  FFS whether multiple configuration of cell DTX or DRX will be supported.

- RAN2#121bis

Agreements

- A periodic cell DTX/DRX configuration is explicitly signalled to the UEs.

- A periodic cell DTX/DRX pattern is configured by UE specific RRC signalling.

- The Cell DTX/DRX configuration contains at least: periodicity, start slot/offset, on duration.

- As a baseline Cell DTX/DRX is activated/deactivated implicitly by RRC signalling, i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released.

- From RAN2 point of view, majority companies see a benefit with L1 signalling for Cell DTX/DRX activation/deactivation, send a LS to RAN1 (email 308) with our preference and ask about feasibility and design details.   Ask about feasibility and reliability of using L1 signaling.  Clarify that the question is about activation/deactivation copy the agreement from last meeting that we are focusing on single configuration.  Extract a few key benefits of dynamic signaling from email discussion and online discussions

- As baseline, UE doesn’t monitor SPS occasions during Cell DTX non-active period. As baseline, gNB is assumed to be not transmitting PDSCH to that UE on such SPS occasions during the Cell DTX non-active period

- As baseline, UE does not transmit on CG occasions during Cell DRX non-active periods

- As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, e.g. SR transmissions are dropped during the non-active period

FFS: whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to support high priority traffic

- (for the SRs that will be dropped) If SR is not to be transmitted on an PUCCH occasion during Cell DRX non-active time, the UE keep the SR pending, i.e., the UE delays the SR transmission till the Cell DRX active period without triggering RACH.  For the FFS case there may be some exceptions.

- The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time.   UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time.   FFS how to deal with any exceptions (e.g. SR if agreed and RACH).

FFS how to deal with retransmissions