3GPP RAN WG2 Meeting #122	R2-2305936

Incheon, Republic of Korea, May 22nd – 25th, 2023

Agenda Item:	7.7.4.2

Source:	InterDigital

Title:	NTN mobility enhancements for RRC_CONNECTED

Document for:	Discussion, Decision

# Introduction

Adaptations to NR mobility must address the following issues in NTN: 1) increased signalling latency due to large propagation delay; 2) frequent cell change due to satellite movement; and 3) lower robustness of measurement-based mobility due to reduced signal strength variation between cell center and cell edge and large regions of cell overlap. Rel-17 NR NTN partially addressed these issues via enhancements to conditional handover, measurement reporting, and measurement rules for cell (re)selection.

An objective for Rel-18 NR NTN is to continue enhancements for both NTN-NTN and NTN-TN mobility and service continuity. Work will consider existing methods from NR TN and Rel-17 NR NTN as baseline for further enhancements. This document focuses on NTN-NTN mobility enhancements for RRC_Connected UEs, specifically RACH-less HO, group HO, and CHO for upcoming cells.

# Rel-18 RRC_CONNECTED mobility enhancements

## RACH-less Handover

In RAN2#121bis-e, RAN2 has agreed to support RACH-less HO for the following cases [2]:

- Intra-satellite handover with the same feeder link. i.e., with same gateway/gNB;

- intra-satellite handover with different feeder links, i.e., with gateway/gNB switch, inter-satellite handover with gateway/gNB switch, and inter-satellite handover with same gateway/gNB.

Other agreements on RACH-less HO include a general procedure description and details on pre-allocated grant. Remaining open issues include the following:

- FFS whether to release UL grant if pre-allocated after RACH-less HO completion

- FFS RACH-less HO failure handling, e.g. whether UE fallback to RACH-based HO to the target cell

- FFS procedure for RACH-less HO combined with PCI unchanged or CHO if supported

Regarding the first FFS, RAN2 agreed the pre-allocated UL grant is a type-1 configured grant. Unless the CG is deactivated by the target gNB after HO completion (via RRC), the UE can maintain the CG configuration after HO completion for other purposes like data transfer. If the network is able to provide a pre-allocated grant via type-1 CG configuration, it can be assumed that there are sufficient resources available at the target gNB to support the configured grant, otherwise it could simply use a one-off dynamic grant.

Observation 1:	If the network provides a pre-allocated grant via type-1 CG configuration, it is assumed there are sufficient resources at the target to support the configured grant until explicitly deactivated.

As a baseline, the UE should therefore maintain the configuration after HO completion unless otherwise indicated by the network.

Proposal 1: 	UE maintains the type-1 CG configuration used for pre-allocated UL grant after RACH-less HO completion as baseline.

However, it is likely the CG configuration is optimized for RACH-less HO completion and may not be suitable for other purposes. It is inefficient if the network must always deactivate any pre-allocated grant after RACH-less HO complete, so RAN2 could further study optimizations to improve pre-allocated grant deactivation after RACH-less HO completion in NTN.

For the second FFS, at the time of RACH-less HO execution the UE will already have valid assistance information to access the target cell (based on RAN1 requriements from [5]), and should already have performed the necessary pre-compensation procedures to access the target cell. In an NTN system, if RACH-less HO fails the UE is already prepared to perform RACH, so it seems reasonable to allow the UE to fallback to RACH-based HO should RACH-less HO fail.

Proposal 2: 	UE falls back to RACH-based HO if RACH-less HO fails.

Regarding the third FFS on combining RACH-less HO with PCI unchanged scenario, companies are invited to refer to a companion contribution [3] on this subject.

## Group handover

In RAN2#119bis-e [4], it was agreed to continue the discussion on group handover in a subsequent meeting, as well as outlining the benefits of such a solution:

- Continue the discussion (in future meeting) on group HO / “UE specific pre-configuration of the target cell + group HO” indication in the next meeting, also on the possible real benefits

There are several scenarios in the NTN environment (e.g., feeder-link switch, quasi-earth fixed cell change) where a large number of UEs must perform mobility in a short period of time. Rel-17 introduced time and location-triggered conditional handover to partly address these scenarios, however each trigger must be simultaneously configured with an RSRP condition. Even if the cell will disappear in the near future, if the measurement condition is not met the UE will not execute CHO, and may remain connected to the cell until coverage loss. This is obviously undesirable from a service continuity perspective.

Observation 2: 	Even with time and location CHO triggers, if measurement conditions are not satisfied then the UE will remain on a cell which will shortly disappear, leading to service continuity issues.

The network may always issue a handover command to the UE, however providing HO commands for all UEs which have not yet executed CHO could be very inefficient from a signalling overhead perspective.

Observation 3: 	Issuing individual HO commands to UEs which have not fulfilled CHO triggering conditions prior to cell dissapearance is inefficient from a signalling perspective.

Group handover (i.e., providing an indication/command to a set of UEs to perform mobility) seems well suited to the NTN scenario, where many UEs may need to have mobility triggered in a short period of time. Having this option also enables greater flexibility on the NW side when configuring CHO measurement thresholds. For example, the network doesn’t have to configure a low measurement threshold (e.g., to ensure that most UEs execute prior to cell disappearance) if there would be an efficient way to trigger mobility for the remaining UEs.

In summary, there are several cases in NTN where many UEs must perform mobility in a short time. Group handover is well suited to this case, and can improve service continuity, signalling overhead, and NW flexibility.

Proposal 3: 	Group handover (i.e. an indication/command causes a group of UEs to trigger mobility) is supported in NTN.

How the UEs can be grouped can depend on several factors. For example, if the cell is an Earth moving cell, it may make sense to group UEs based on UE location. If the group HO indication is intended to trigger CHO then the UEs may be grouped based on characteristics of the triggering conditions (e.g. remaining time until CHO expiry). In either case, the network is best suited to perform the grouping, and when and how the UE is grouped should be based on NW implementation.

Proposal 4: 	How and when UEs are grouped is up to network implementation.

UEs within a specific group may then be provided with a group RNTI, which can be used to decode messages assigned for the group (e.g. measurements, HO commands, target cell configurations etc..)

Proposal 5: 	Once assigned to a group a UE is provided with a group RNTI, which is used to receive group related signalling (e.g. group HO command).

## Conditional handover configuration for upcoming NTN cells

In RAN2#119bis-e it was agreed that at least for the Earth moving cell case, the next serving cells can be largely predicted in NTN for UEs not at cell edge. A consequence of this is that for a UE in a given location, the NW can estimate the likely upcoming sequence of cells and provide a series of upcoming target cell configurations, for example, with associated time-based triggering conditions, which the UE can activate at the appropriate time/location

One objective of the Rel-18 WI is to specify CONNECTED handover enhancements for a UEs to reduce the signalling overhead. Although a target cell configuration is required for each upcoming cell, the ability to simultaneously provide multiple upcoming configurations at once increases network flexibility to load balance by freeing up resources in future cells (which would otherwise be needed to provide future CHO configurations).

Observation 4:	Providing multiple CHO configurations for upcoming cells at once allows the network greater flexibility to load balance by freeing up future resources

One potential drawback is that the target cell configuration may become invalid, e.g. based on UE movement, in which case the signalling would have been wasted. However, as per the TR UE movement accounts for only a maximum 4% variation in the HO rate, and is thus neglected based on the relative speed of satellites. This can be further reduced by limiting the maximum number of provided target configurations at one time.

Proposal 6: 	A UE can be provided with CHO configurations for multiple future NTN cells based on the predictability of NGSO satellite movement.

# Conclusion

In this contribution the following observations and proposals were made concerning RRC_CONNECTED measurement, mobility, and service continuity enhancements in Rel-18 NR NTN:

Observation 1:	If the network provides a pre-allocated grant via type-1 CG configuration, it is assumed there are sufficient resources at the target to support the configured grant until explicitly deactivated.

Observation 2: 	Even with time and location CHO triggers, if measurement conditions are not satisfied then the UE will remain on a cell which will shortly disappear, leading to service continuity issues.

Observation 3: 	Issuing individual HO commands to UEs which have not fulfilled CHO triggering conditions prior to cell dissapearance is inefficient from a signalling perspective.

Observation 4:	Providing multiple CHO configurations for upcoming cells at once allows the network greater flexibility to load balance by freeing up future resources

Proposal 1: 	UE maintains the type-1 CG configuration used for pre-allocated UL grant after RACH-less HO completion as baseline.

Proposal 2: 	UE falls back to RACH-based HO if RACH-less HO fails.

Proposal 3: 	Group handover (i.e. an indication/command causes a group of UEs to trigger mobility) is supported in NTN.

Proposal 4: 	How and when UEs are grouped is up to network implementation.

Proposal 5: 	Once assigned to a group a UE is provided with a group RNTI, which is used to receive group related signalling (e.g. group HO command).

Proposal 6: 	A UE can be provided with CHO configurations for multiple future NTN cells based on the predictability of NGSO satellite movement.

# References

RP-223534– 3GPP Work Item Description: NR NTN enhancements (Rel-18)

Draft_R2_121bise_meeting_report_v1

R2-2305937 – Satellite switching without PCI change – InterDigital

R2-2211101 – RAN2#119bis-e Meeting Report

R2-2300020 - Reply LS on RACH-less handover in NTN –  RAN1