3GPP TSG-RAN2#122	R2-2305942

Incheon, Korea, May 22-26, 2023

Agenda item:		7.3.5

Source:		Lenovo

Title:		 CHO Procedure in NES Mode

Document for:		Discussion and Decision

- Introduction

One of the Techniques to improve network energy savings is related to mobility:

From RP-230566: “Specify CHO procedure enhancement(s) in case source/target cell is in NES mode [RAN2]”

following agreements were reached in RAN2#121bis:

Agreements

-	RAN2 agree to make enhancement in CHO procedure based on that the source cell entering “NES mode”.  FFS further details

-	For source cell CHO framework, RAN2 assumes a reference scenario where the UE has already performed CHO conditions evaluation by the time the source cell starts some “NES-mode”

-	As a baseline, UE initiates CHO evaluation upon receiving the CHO configuration.  FFS what trigger is used for execution of CHO

This document looks at NES mobility procedure from source as well as target cell’s aspect.

- Discussion

The source side:

Based on the agreements from the last meeting, a source cell must send CHO to the UE early enough (as is also the main motivation generally for “conditional” handovers, allowing the RRC Connected UEs in the cell sufficient time to evaluate handover conditions contained in the CHO.

-	For source cell CHO framework, RAN2 assumes a reference scenario where the UE has already performed CHO conditions evaluation by the time the source cell starts some “NES-mode”

-	As a baseline, UE initiates CHO evaluation upon receiving the CHO configuration.  FFS what trigger is used for execution of CHO

Further, configuring relaxed measurements increases the UE’s chances of finding a target cell but, on the other hand, a target qualifying threshold(s) from relaxed measurements may not be in a radio situation/ geometry to service UE with required QoS.

This situation (early CHO/ CHO evaluation and late NES mode trigger) may lead to either a too early handover execution (due to relaxed radio conditions), thereby UE breaking connection from an otherwise very strong source cell and going to relaxed-radio-conditioned target cell. Or, to a too late handover execution (when source cell sleeps much later than the time when it sent the CHO) which may mean that the conditions once considered fulfilled are not valid anymore. Both situations leading to a bad user experience. Too late case may also arise if L1 command is used to activate/ deactivate Cell DTX/ DRX – which comes much later than measurement evaluations.

Too early case can be remedied by ensuring that UE does not start handover execution until the source cell is about to get into Cell DRX/ DTX sleep. When the source cell would start sleeping can be determined by the UE itself based on the received start offset Cell DTX/ DRX in UE dedicated RRC signalling, agreed earlier by RAN2. We see no necessity to use a L1 signal for this end.

Observation 1: The time when the source cell would start sleeping can be determined by a UE itself based on the received start offset of Cell DTX/ DRX configuration received in UE dedicated RRC signalling.

Proposal 1: If relaxed measurements are configured (and subsequently handover condition is met), UE starts handover execution only when the source cell is about to get into Cell DRX/ DTX sleep.

Proposal 2: The time when the source cell starts (or about to start) Cell DRX/ DTX sleep can be determined by the UE based on the received start offset of Cell DTX/ DRX configuration received in UE dedicated RRC signalling.

Once handover is complete, the UE is in the target cell – which would become the source cell. Therefore, we do not see the need for L1 based signaling to activate/ deactivate NES mode for executing conditional handover. The L1 based signaling to activate/ deactivate NES mode may be useful for other UEs not being handed over, but that is another discussion. So, we propose:

Proposal 3: L1 based signaling to activate/ deactivate NES mode for executing conditional handover is not considered.

To tackle, too late handovers we need to ensure that the condition evaluation (relaxed measurement event fulfillment) remain relevant (i.e., not stale) when the handover is to be executed to the corresponding target cell. This can be done using a timer that defines how long a handover condition when considered met remains valid. The timer is started when handover condition(s) is met, is stopped when handover execution is started and upon its expiry the handover condition(s) are no more considered as fulfilled and therefore UE needs to perform fresh evaluation, if required.

Proposal 4: A network configured timer is used to supervise the freshness of fulfilled handover condition(s). When timer expires, handover condition(s) are no more considered as fulfilled and therefore UE needs to perform fresh evaluation, if required.

The target side:

A handover target is a candidate cell for which all the handover condition(s) are fulfilled. UE however needs to determine if a candidate cell fulfilling handover conditions is or will be in sleep by UE’s arrival in the target (e.g., during T304 is running). If this is not done so then immediately after completion of RACH procedure either the target needs to get active to serve this UE, thus sacrificing energy saving, or UE will be released/ redirected/ handed over immediately upon arriving on the target cell (e.g., as soon as the UE transmits HO Complete on the target side in RACH Msg3), this impacting “user experience”. To avoid this, the serving cell can provide DRX/ DTX configuration for each candidate cell included in CHO. A UE determines if a candidate is using NES and if so, will it be sleeping around the time the UE arrives at the target. Another solution being proposed by some companies is that some prioritization per candidate cell can be indicated by the source cell in the CHO, but it is not clear how this ensure that the UE executes handover exactly when the target is not sleeping. Of course, it is best if the source would include only candidates that are not using NES technique, but it is unclear how far is this possible.

Proposal 5: RAN2 should discuss if it will be always possible to avoid including NES candidate cells in CHO.

Proposal 6: If NES candidate cell(s) are included in CHO, the serving cell can provide DRX/ DTX configuration for each candidate cell included in CHO.

Proposal 7: A UE determines if a candidate is sleeping around the time the UE arrives at the target.

Finally, what happens when there’s still no non-sleeping candidate/ target cell when the source cell is about to enter sleep?

- Option 1: UE selects a best amongst worst candidates

- Option 2: UE informs the serving cell about un-availability of a target cell just before the serving cell will sleep. After receiving this information from the UE, gNB can delay the state transition to sleep.

- Option 3: UE meets RLF since there’s no cell fulfilling handover conditions and source sleeps.

It may be useful to have RAN2 discussion on this, but proponent think that allowing RLF to happen is not the best choice since RAN2 has strived since release 8 to minimize RLFs, in its various studies/ work undertaken.

Observation 2: Executing handover to a sleeping cell might lead to handover failures and RLF.

Observation 3: RAN2 has strived since release 8 to minimize RLFs, in its various studies/ work undertaken. And it is sensible if we do not introduce a new reason for RLF.

Proposal 8: RAN2 kindly discuss further how to handle cases when there’s still no target when the source cell is about to enter sleep.

# Conclusion

This document discussed NES mobility procedure from source as well as target cell’s aspect. Following proposals are made:

Observation 1: The time when the source cell would start sleeping can be determined by a UE itself based on the received start offset of Cell DTX/ DRX configuration received in UE dedicated RRC signalling.

Proposal 1: If relaxed measurements are configured (and subsequently handover condition is met), UE starts handover execution only when the source cell is about to get into Cell DRX/ DTX sleep.

Proposal 2: The time when the source cell starts (or about to start) Cell DRX/ DTX sleep can be determined by the UE based on the received start offset of Cell DTX/ DRX configuration received in UE dedicated RRC signalling.

Proposal 3: L1 based signaling to activate/ deactivate NES mode for executing conditional handover is not considered.

Proposal 4: A network configured timer is used to supervise the freshness of fulfilled handover condition(s). When timer expires, handover condition(s) are no more considered as fulfilled and therefore UE needs to perform fresh evaluation, if required.

Proposal 5: RAN2 should discuss if it will be always possible to avoid including NES candidate cells in CHO.

Proposal 6: If NES candidate cell(s) are included in CHO, the serving cell can provide DRX/ DTX configuration for each candidate cell included in CHO.

Proposal 7: A UE determines if a candidate will be sleeping around the time the UE arrives at the target.

Observation 2: Executing handover to a sleeping cell might lead to handover failures and RLF.

Observation 3: RAN2 has strived since release 8 to minimize RLFs, in its various studies/ work undertaken. And it is sensible if we do not introduce a new reason for RLF.

Proposal 8: RAN2 kindly discuss further how to handle cases when there’s still no target when the source cell is about to enter sleep.

# References

[1]: R2-2304353	Summary of EMAIL DISC [121bis#xxx] on AI 7.3.5 Connected Mode Mobility Lenovo