3GPP TSG-RAN2#122	R2-2305943

Incheon, Korea, May 22-26, 2023

Agenda item:		7.4.2.3

Source:		Lenovo

Title:		 Cell Switch details

Document for:		Discussion and Decision

- Introduction

LTM based Cell Switch procedure requires early TA acquisition as well as new MAC CE based mobility procedure. This paper goes in the details of cell switch procedure and builds on RAN2 progress from the last meetings.

Following agreements were reached in RAN2#121bis:

From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order). RRC RACH configuration for early TA acquisition (e.g., including whether RAR needs to be received) is specific per target cell and is signalled separately (separate IEs) from the candidate cell configuration (the part that need to be applied at cell switch). R2 assumes that Early TA RACH option 3 (with RAR from candidate cell) is not needed in Rel-18. R2 assumes RRCReconfigurationComplete message is always sent at each LTM execution. In RACH-based LTM, the target cell is aware of the UE’s arrival based on the reception of preamble in CFRA and on the reception of Msg3/MsgA in CBRA, like the legacy HO. In RACH-less LTM, the target cell is aware of the UE’s arrival based on reception of the first UL transmission from this UE In RACH-less LTM, RRCReconfigurationComplete can be the content of the first UL MAC PDU/transmission to indicate UE arrival, i.e. no need to introduce any new signaling to indicate UE arrival (for the MCG-switch case) For RACH-based LTM, the UE considers that LTM execution procedure is successfully completed when the RACH is successfully completed. For RACH-less LTM, the UE considers that LTM execution procedure is successfully complete when the UE determines the NW has successfully received its first UL data. Following behaviors of LTM supervisor timer are agreed: - - 1: The UE starts the LTM supervisor timer, upon reception of the LTM cell switch MAC CE; - - 2: The UE stops the LTM supervisor timer, upon successful completion of LTM cell switch; - - 3: If the LTM supervisor timer for MCG expires, as baseline, the UE considers LTM failure and initiates RRC re-establishment. (SCG switch case FFS) LTM supervisor timer is RRC layer timer. At RLF or LTM execution failure (for MCG), RAN2 intend to support fast recovery to a candidate cell by LTM execution. While configured with LTM candidate cells, the UE can also execute any L3 handover command sent by the network. R2 assumes that is could be up to the network to avoid any issue due to the race condition between LTM execution and RRC Reconfiguration (e.g. L3 HO cmd), e.g. avoid sending LTM switch cmd and L3 HO cmd in the same TB.

- Preparation for Cell Switching

Both RAN1 and RAN2 have been discussing Early TA acquisition topic for a while. Once Early TA is acquired following two procedures can be used to inform UE of the TA value corresponding to a candidate cell:

- MAC RAR with Early TA is sent by the source cell (DU) to the UE

- Early TA is sent by the source cell (DU) to the UE included in the LTM i.e., no MAC RAR case.

Apart from these two there’s a third possibility where RAN2 first needs to agree if the acquired Early TA should be kept (or not) after a successful mobility. On “keeping or saving Early TA value for candidate cell(s)” may be considered a valid optimization as based on ASN.1 the TAT can be configured up to 10 sec. long. On “who” keeps the Early TA, UE, or the network, following can be considered:

If UE keeps Early TAs acquired for candidate cell(s) there’s a problem when Early TA is sent to the UE in LTM, then UE does not really know when it was acquired – unless UE implementation remembers the Msg1 transmission time in the corresponding candidate cell!

On the other hand, Early TA keeping can be managed by the network using F1 interface such that only network keeps Early TA and UE forgets about it after successful mobility. Network can send another PDCCH order upon a TAT expiry, if required. Source cell may provide the valid timing advance value(s) (Nta) of source cell as well as candidate cell(s) to target cell when receiving successful handover confirmation from it (i.e., upon UE’s arrival on the target side). Optionally, a timestamp of TA acquisition (or equivalent indicating how long the corresponding Early TAs can be considered valid) may be provided along with each TA value to help target cell determine how long the receives TA value(s) can be considered valid. This is also easier for UE implementation as it simply can release timing advance value(s) (Nta) upon successful mobility to a target cell.

Accordingly, the following proposals are made:

Validity of an early TA for a UE-candidate cell pair can be controlled using a legacy like TAT timer.

In the case of non-RAR, RAN2 to discuss which node (source cell or candidate cell) is responsible for TA maintenance.

In the case of RAR based TA acquisition, RAN2 to discuss which node (UE, source cell, candidate cell) is responsible for TA maintenance.

Source cell may transmit PDCCH order once it determines that the TA is not valid anymore. Before transmitting PDCCH order, the updated RACH resource can be obtained from candidate DU, if needed.

Source cell may provide the valid timing advance value(s) (Nta) of candidate cell(s) to target cell when receiving successful handover confirmation from it (i.e., upon UE’s arrival on the target side).

In the case of RAR based TA acquisition, RAN2 to discuss the following options for early TA value handling upon successful mobility to a target cell.

- Option 1: UE releases timing advance value(s) (Nta)

- Option 2: UE continues keeping timing advance value(s) (Nta)

Further, like normal handover procedure where once UE detaches from source cell, synchronizes to target’s DL and initiates PRACH transmission, a separate MAC entity should be used for Early TA acquisition (PRACH transmission and when configured RAR reception, Msg3 transmission etc.). There may be cases e.g., for early TA acquisition is to be started while there is an ongoing RA in serving cell. In this case we see that two RACHs may be allowed in parallel. Whether this left to UE capability or can be considered mandatory needs to be discussed at a later stage of RAN2 work. Finally, if the candidate cell (DU) does not receive preamble in the configured RO, the candidate cell determines the failure of TA acquisition and indicates this to the source cell (DU). This will enable the source cell to determine if it would like the UE to re-attempt Early TA acquisition and when so, source may request dedicated PRACH resources from the candidate cell (DU).

A separate MAC entity is used for Early TA acquisition (PRACH transmission and when configured RAR reception)

For early TA acquisition while there is an ongoing RA in serving cell, two RACHs are allowed in parallel. FFS: UE capability

If the candidate cell (DU) does not receive preamble in the configured RO, the candidate cell determines the failure of TA acquisition and indicates this to the source cell (DU). Candidate DU can provide the updated RACH resource together with failure indication.

- Mobility Execution and cell Switch command

This can be modelled similar to that in LTE. Up on receiving measurement results, and having determined a target cell, the serving cell/ source DU informs concerned target cell/ DU/ CU confirming UE mobility. Target cell may send an acknowledgement for the said Mobility-Confirmation including information about PUSCH resource used by the UE in the target cell to announce its arrival, if necessary e.g., if CG-PUSCH resources would be used. At this point (or even earlier), having determined the target cell, a cell switch command (LTM) is sent to the UE. As R2 agreed previously, LTM is conveyed in a MAC CE and contains at least one candidate configuration index, based on the index used in the first-RRC-Reconfiguration message.

The PUSCH resource(s) can also be provided using a DCI from the target cell. UE starts a T304-like timer upon reception of LTM. The T304-like timer should be generally shorter, than the T304 timer in legacy L3 based mobility since/ if the UE maintains DL Sync and a valid Early TA is available, UE should be able to quickly announce its arrival on the target cell.

Figure 2: Mobility from source to target cell under DUs in an intra-CU situation

The arrival-announcement can be done by sending a cell-switch complete message using a MAC CE, to reduce the data interruption, data can be exchanged only after the cell-switch procedure is complete. The PUSCH resource for the same can be provided to the UE by the target cell using a (first) Dynamic Grant. In an intra-CU scenario for Rel. 18, both the source and target cell (as shown in Figure 2) belong to the same centralized unit, therefore gNB can know accurately when the UE has received LTM successfully, enabling it to send the DCI for the said Dynamic Grant via the target cell/ DU. To this end, at least PDCCH-Config of the target cell, configured earlier using a first-RRC-Reconfiguration with some other information out of ServingCellConfig is applied upon reception of LTM. As the UE may need some time to process LTM, refresh L1 synchronization including frequency and/ or BWP retuning, target cell should keep sending DCI PUSCH grants on many possible DL Beams, hoping that UE appears in the target soon until T304-like timer is running. The T304-like timer is used when early TA is available. Both T304 and T304-like timers are configured to the UE in the RRC Reconfiguration and T304-like timer is shorter than t304.

A shorter T304-like timer is used to supervise the completion of LTM based cell switch procedure when Early TA for the target cell is available (either UE has it already or receives the same in LTM); legacy T304 timer is used otherwise.

The two T304 timers (short one and normal one) can be configured using RRC signalling along with the candidate cell configuration.

- Contents of Cell switch command

- RAN2 previously agreed that LTM command contains

- a) pseudo identity of a target cell

- b) Nta value of a target cell or an indication to acquire TA afresh

- In addition, a short-T304 timer (when Nta is signalled) or a normal T304 timer (when UE needs to acquire TA afresh) can be configured using RRC signalling along with the candidate cell configuration. The value of the short-T304 timer and the normal T304 timer can be same for all candidate cells configured.

- Further, from the April meeting RAN WG1 #112bis-e, RAN1 agreed the following:

From RAN1 point of view, at least the following information can be included in the cell switch command, which is conveyed by MAC CE - Information to identify the target cell(s) - The details including bit number are designed by RAN2 - TA related information (details up to the discussion in A.I. 9.10.2) - 1 joint or 1 pair of UL and DL unified TCI State index for the target Cell - Note: discussion on target SpCell is not precluded - Active DL and UL BWPs for the target cell - FFS: Triggering of aperiodic TRS transmitted from the target cell - FFS: Triggering the CSI acquisition of the target cell and reporting to the target cell - FFS: Triggering of aperiodic SRS transmission to the target cell - FFS: C-RNTI - FFS: the presence of each field (i.e. always present or configurable) Conclusion - Whether active DL and UL BWP of the target Cell/SpCell field, within the cell switch command, is always present or not is left to RAN2 decision. Working Assumption On the presence of beam indication within cell switch command, at least for scenario 2, following is supported: - A field to indicate 1 joint or 1 pair of UL and DL unified TCI State index for the target cell field is always present in the cell switch command.

- RAN1 has still some work to finalize the content of cell switch command. So, we think it is better to ask RAN1 directly on the final contents of the cell switch command rather than RAN2 concluding something from their work in progress.

Transmission of LTM to the UE from the source cell/ DU triggers UE to monitor PDCCH in the target cell for receiving PUSCH resources, enabling the UE to announce its arrival and complete cell switch procedure.

RAN2 send an LS to RAN1 requesting for final contents of LTM besides currently agreed ones (pseudo identity of a target cell and Nta value of a target cell or an indication to acquire TA afresh).

- Finally, according to one previous agreement, once LTM cell switching fails and the suitable cell is LTM cell during cell selection, UE also can perform LTM procedure. To increase the chances of mobility, LTM may contain more than one target cells that may be used for handover execution if the mobility to a previous one fails.

RAN2 to discuss if cell switching command may contain more than one target cells.

# Conclusion

This paper went in the details of cell switch procedure and builds on both RAN1 and RAN2 progress from the last meeting. Following proposals are made:

Preparation for Cell Switching:

Proposal 1: Validity of an early TA for a UE-candidate cell pair can be controlled using a legacy like TAT timer.

Proposal 2: In the case of non-RAR, RAN2 to discuss which node (source cell or candidate cell) is responsible for TA maintenance.

Proposal 3: In the case of RAR based TA acquisition, RAN2 to discuss which node (UE, source cell, candidate cell) is responsible for TA maintenance.

Proposal 4: Source cell may transmit PDCCH order once it determines that the TA is not valid anymore. Before transmitting PDCCH order, the updated RACH resource can be obtained from candidate DU, if needed.

Proposal 5: Source cell may provide the valid timing advance value(s) (Nta) of candidate cell(s) to target cell when receiving successful handover confirmation from it (i.e., upon UE’s arrival on the target side).

Proposal 6: In the case of RAR based TA acquisition, RAN2 to discuss the following options for early TA value handling upon successful mobility to a target cell.

-	Option 1: UE releases timing advance value(s) (Nta)

-	Option 2: UE continues keeping timing advance value(s) (Nta)

Proposal 7: A separate MAC entity is used for Early TA acquisition (PRACH transmission and when configured RAR reception)

Proposal 8: For early TA acquisition while there is an ongoing RA in serving cell, two RACHs are allowed in parallel. FFS: UE capability

Proposal 9: If the candidate cell (DU) does not receive preamble in the configured RO, the candidate cell determines the failure of TA acquisition and indicates this to the source cell (DU). Candidate DU can provide the updated RACH resource together with failure indication.

Mobility Execution

Proposal 10: A shorter T304-like timer is used to supervise the completion of LTM based cell switch procedure when Early TA for the target cell is available (either UE has it already or receives the same in LTM); legacy T304 timer is used otherwise.

Proposal 11: The two T304 timers (short one and normal one) can be configured using RRC signalling along with the candidate cell configuration.

- Contents of Cell switch command

Proposal 12: Transmission of LTM to the UE from the source cell/ DU triggers UE to monitor PDCCH in the target cell for receiving PUSCH resources, enabling the UE to announce its arrival and complete cell switch procedure.

Proposal 13: RAN2 send an LS to RAN1 requesting for final contents of LTM besides currently agreed ones (pseudo identity of a target cell and Nta value of a target cell or an indication to acquire TA afresh).

Proposal 14: RAN2 to discuss if cell switching command may contain more than one target cells.

# References

[1]:

Alternatively Beam signalling (TCI state) for target cell is done in LTM