3GPP TSG RAN WG2 #122                                                                         R2-2305964

Incheon, Korea, May 22-26, 2023                                            Revision of R2-2302826

Agenda Item:	7.19.3

Source:	ZTE Corporation, Sanechips

Title:	Capability definition and report for eRedCap UE

Document for:	Discussion and Decision

Introduction

In WID RP-223544[1], the objective of additional early indication for Rel-18 eRedCap UE is as below:

Complexity/cost reduction - Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - Support additional separate early indication(s) [RAN1, RAN2]

In RAN1 #111 [2], the following agreements were reached:

Agreement For the earlier RAN1 agreement achieved in RAN1#111 as following, For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. - When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied. - When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms. - FFS: value(s) of X - Otherwise, the UE behavior is up to the UE implementation. - Note: it does not mean early indication is needed - Note: it will not be used as example for unicast PDSCH For the “FFS: value(s) of X” - X = [0.5/0.25 or 1/0.5 or 2/1] ms for 15/30kHz SCS - Note: Single Value pair for X is to selected for SCSs Conclusion There is no consensus to continue discussion on “whether additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs is allowed to be configured by the SIB in the cell”. Conclusion For UE BB complexity reduction, there is no need to relax the requirements on simultaneous reception of two broadcast PDSCH transmissions for SIB1/OSI/paging/RAR. Agreement For the relaxed constraint X in the following earlier RAN1 agreement, down-select between X = 3 and X = 3.2. - UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X. - FFS: the value of X. Agreement Update the agreement for PDSCH paging with the clarification as follows: - From RAN1 perspective, for UE BB complexity reduction, for paging channel (PDSCH) to Rel-18 RedCap UEs, allow the scheduling of paging channel to be larger than 5 MHz (as in legacy operation). The scheduling of paging PDSCH is allowed to be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS. Agreement For UE BB complexity reduction, a UE is not expected to perform 2-step RACH with a MsgA PUSCH resource spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. Working Assumption - For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

In RAN2 #121 meeting [3], the following agreement was reached.

Long eDRX cycle (>10.24 s) value range of enhanced INACTIVE eDRX is same as IDLE eDRX from 20.48s to 10485.76s, i.e. hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024 RAN2 confirms the enhanced INACTIVE eDRX can be applied to all R18 UEs. FFS if it can only be supported by UEs which support R17 eDRX. We will wait for RAN1 progress to see if there is a need for a Msg1 early indication for eRedCap. The NR MIB “cellBarred” bit applies to all UEs (Normal UEs, Redcap UEs and eRedcap UEs).

In RAN2 #121bise meeting [4], the following agreement was reached.

A Rel-18 eRedCap UE should be able to indicate its support via new UE capability signaling specific to Rel-18 eRedCap

In this contribution, based on the RAN1 and RAN2 progresses, we will discuss how to define and report the UE capability for eRedCap UEs.

Discussions

eRedCap UE Capability definition

In the eRedCap WID, it has described that “aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction”, one UE capability indication (e.g. supportOfEnhanceRedCap-r18) is enough for eRedCap UEs, e.g. to indicate whether the UEs supports UE peak data rate reduction and 5 MHz BB bandwidth only for PDSCH and PUSCH). Whether finer granularity of eRedCap UE capability is necessary can wait for RAN1 decision on the eRedCap UE type definition.

Proposal 1: Use one UE capability indication (e.g. supportOfEnhanceRedCap-r18) for eRedCap UEs to indicate it’s capability of supporting UE peak data rate reduction and 5 MHz BB bandwidth only for PDSCH and PUSCH).  Whether finer granularity of eRedCap UE capability is necessary can wait for RAN1 decision on the eRedCap UE type definition.

2.2 About Msg4/MsgB with larger BB than eRedCap UE capability

In the LS from RAN1[5], RAN2 is asked to consider the UE behavior for the case when the BB complexity reduction UE detects a DCI scheduling a Msg4 PDSCH transmission with a larger bandwidth than it can receive or process.

Based on the current RAN2 specification for 4-step CBRA procedure as follows, only the Msg4 MAC PDU is successfully decoded, it will stop the ra-ContentionResolutionTimer, and decide whether the Contention Resolution is successful or not successful. Since the the BB complexity reduction UE cannot decode the Msg4 PDSCH with a larger bandwidth than it can receive or process, it will monitor the PDCCH until the ra-ContentionResolutionTimer expires. This procedure is similar as that “the Msg4 MAC PDU is successfully decoded, but the UE Contention Resolution Identity in the MAC CE does not match the CCCH SDU transmitted in Msg3”. So, it can work normally based on the current specification.

Observation 1: based on the current RAN2 specification for 4-step CBRA procedure, the BB complexity reduction UE can work when it detects a DCI scheduling a Msg4 PDSCH transmission with a larger bandwidth than it can receive or process(e.g. monitor the PDCCH until the ra-ContentionResolutionTimer expires).

Although some optimization may be possible, e.g. when the BB complexity reduction UE detects a PDCCH addressed to its TEMPORARY_C-RNTI scheduling a Msg4 PDSCH with a larger bandwidth than it can receive or process, the UE can stop the ra-ContentionResolutionTimer, consider the Contention Resolution not successful, and trigger random access process again instantly, which can save UE power and reduce the random access delay. But the gain is trivial, since if the Msg1/MSGA based early indication is used, this case can be avoid; and even Msg1/MSGA based early indication is not used, the UE behavior is as that of Contention Resolution procedure for legacy UE.

Observation 2: Some optimization may be possible for the case when BB complexity reduction UE detects a PDCCH addressed to its TEMPORARY_C-RNTI scheduling a Msg4 PDSCH with a larger bandwidth than it can receive or process, but the gain is trivial.

Thus, we suggest to no do any optimization for this case, and reply LS to RAN1 that the current RAN2 specification can work normally when the BB complexity reduction UE detects a DCI scheduling a Msg4 PDSCH with a larger bandwidth than it can receive or process.

Proposal 2a: To reply LS to RAN1 that the current RAN2 specification can work normally when the BB complexity reduction UE detects a DCI scheduling a Msg4 PDSCH with a larger bandwidth than it can receive or process.

Considering that same issue exists in 2-step CBRA procedure, and the same procedure is used in the current specification (e.g. only if a valid downlink assignment has been received on the PDCCH for the MSGB-RNTI and the received TB is successfully decoded, the UE will stop the msgB-ResponseWindow, and decide whether this Random Access procedure successfully completed or not successfully completed). Thus, RAN2 can also reply LS to RAN1 that the current RAN2 specification can work normally when the BB complexity reduction UE detects a DCI scheduling a MsgB PDSCH with a larger bandwidth than it can receive or process

Proposal 2b: To reply LS to RAN1 that the current RAN2 specification can work normally when the BB complexity reduction UE detects a DCI scheduling a MsgB PDSCH with a larger bandwidth than it can receive or process.

Conclusions

This contribution presents some of our views on early indication for Rel-18 eRedCap UEs. We have the following proposals:

Proposal 1: Use one UE capability indication (e.g. supportOfEnhanceRedCap-r18) for eRedCap UEs to indicate it’s capability of supporting UE peak data rate reduction and 5 MHz BB bandwidth only for PDSCH and PUSCH).  Whether finer granularity of eRedCap UE capability is necessary can wait for RAN1 decision on the eRedCap UE type definition.

Proposal 2a: To reply LS to RAN1 that the current RAN2 specification can work normally when the BB complexity reduction UE detects a DCI scheduling a Msg4 PDSCH with a larger bandwidth than it can receive or process.

Proposal 2b: To reply LS to RAN1 that the current RAN2 specification can work normally when the BB complexity reduction UE detects a DCI scheduling a MsgB PDSCH with a larger bandwidth than it can receive or process.

References

- RP-223544 Revised WID on Enhanced support of reduced capability NR devices.

- Draft_Minutes_report_RAN1#111_v010.

- R2-2301910 Report from eRedCap breakout session, RAN2#121, February, 2023

- Draft_R2_121bis_meeting_report_v2

- R2-2304619	LS on Msg4 PDSCH transmission to Rel-18 eRedCap UEs (R1-2304262; contact: Ericsson)