3GPP TSG-RAN WG2 Meeting #122          	          R2-2305976

# Incheon, Korea, 22 – 26 May, 2023

Title:		Discussion on the inter-node coordination issue for NR IDC

Source:	Huawei, HiSilicon

Agenda item:	7.10.2

Document for:	Discussion and Decision

# Introduction

In the past RAN 2 meeting we had made good progress and agreed the basic details for NR IDC. One big remaining issue for IDC Enhancement WI is the inter-node coordination for both FDM solution and TDM solution [1]. In this paper, we further discuss the inter-node coordination issue. Since RAN2 has agreed that no specification change for LTE, the EN-DC case is clear. Hence we only focus on the NR-DC case in this paper.

# Discussion

## 2.1 Inter-node coordination for FDM solution enhancements

Considering there are several cases for the IDC issue in case the UE is configured with MR-DC, we list the possible cases/scenarios giving the details of the Network Configuration and UE Reporting and provide analysis for each of these cases on whether coordination between MN and SN is needed.

- Case 1: Individual candidate frequencies range are affected by the IDC issue

- NW Configuration: MN configures F1 as the candidate frequency to the UE, while SN configures F2 as the candidate frequency to the UE

- UE reporting: If UE detects IDC issue on F1, UE reports the individual affected frequency F1 to the MN; If UE detects IDC issue on F2, UE reports the individual affected frequency F2 to the SN.

For such case, MN and SN can decide how to configure and how to address the IDC issue for the UE independently. There is no need for any coordination either for configuration or solution between MN and SN.

- Case 2: Combination of candidate frequency ranges due to UL CA are affected by the IDC issue

- NW Configuration: MN configures F1 and F2 as the candidate frequency to the UE, while SN configures F3 and F4 as the candidate frequency to the UE

- UE reporting: If UE detects IDC issue on UL CA combination of F1 and F2, UE reports the affected frequency combination F1+F2 to the MN; If UE detects IDC issue on UL CA combination of F3 and F4, UE reports the affected frequency combination F3+F4 to the SN.

For such case, MN and SN can also decide how to configure and how to address the IDC issue for the UE independently. There is no need for any coordination either for configuration or solution between MN and SN.

- Case 3: Combination of candidate frequency ranges due to MR-DC are affected by the IDC issue

- NW Configuration: MN configures F1 as the candidate frequency to the UE, while SN configures F2 as the candidate frequency to the UE

- UE reporting: If UE detects IDC issue on MR-DC combination of F1 and F2, then how the UE reports this to network is unclear in RAN 2 and we don’t seems to  have common understanding in RAN2 for this scenario.

For this case, we first look at what’s the procedure in EN-DC based on the legacy EN-DC procedure and the R18 FDM enhancement.

- NW configuration:

- SN forward the F2 as the candidate frequency to the MN by using the CG-Config, and SN can also configure the F2 in terms of  the centre frequency + bandwidth as the candidate frequency range to the UE directly.

- After receiving the F2 as the candidate frequency from the SN, MN can configure the F1 as a measurement object and F2 as the candidate serving frequency to the UE.

- UE reporting:

- If the UE detects the IDC issue on F2, then UE can report the affected frequency range as agreed in R18 to the SN.

- If the UE detects the IDC issue on MR-DC combination of F1 and F2, UE reports the affected frequency combination as in legacy to the MN. After that MN may forward this affected frequency range to the SN via the CG-ConfigInfo.

For the above case 3 in NR-DC, considering the IDC issue is caused by the frequency range combination instead of individual frequency range, the UE should report the affect frequency range combination to the NW. If the UE just reports individual frequency range to the corresponding node, it will be misleading and may cause the NW to take the wrong action. So we think the simplest way is to follow the same procedure as in EN-DC. The specification impact would include:

- To add the candidateServingFreqRangeListNR in the CG-Config

- To add the affectedCarrierFreqRangCombInfoListMRDC in the MRDC-AssistanceInfo of CG-ConfigInfo

- For the UE reporting procedure, if the UE detect the IDC issue on MR-DC frequency range combination, the UE reports the affected frequency range combination to the MN.

After MN receives the affected frequency range combination from the UE for NR-DC, it is up to the MN to decide how to address the IDC issue for the UE by using the FDM solution as below. No further coordination between MN and SN for addressing the IDC issue is needed.

- If the MN decide to address the IDC problem by itself e.g. by switching the BWP, it does not forward any info to SN;

- If the MN decide to ask the SN to address the IDC problem caused by the MR-DC frequency combination, then MN forwards affected frequency combination List to SN for it to resolve the IDC problem. Then it is up to the SN to decide how to apply FDM solution.

Based on the above discussion, we propose that:

Proposal 1: For the case of IDC issue caused by NR-DC frequency range combination, the same procedure as for EN-DC is supported, specifically:

- To add the candidateServingFreqRangeListNR in the CG-Config

- To add the affectedCarrierFreqRangeCombInfoListMRDC in the MRDC-AssistanceInfo of CG-ConfigInfo

- For the UE reporting procedure, if the UE detect the IDC issue on MR-DC frequency range combination, the UE reports the affected frequency range combination to the MN.

## 2.2 Inter-node coordination for TDM solution

Similar to the analysis for the FDM enhancement, we also provide analysis on whether inter-node coordination for TDM solution is needed on case by case basis.

- Case 1: Individual candidate frequencies range are affected by the IDC issue

- NW Configuration: MN configures F1 as the candidate frequency and TDM to the UE, while SN configures F2 as the candidate frequency and TDM to the UE

- UE reporting: If UE detects IDC issue on F1, UE reports the individual affected frequency F1 and TDM assistance information to the MN; If UE detects IDC issue on F2, UE reports the individual affected frequency F2 and the corresponding TDM assistance information to the SN.

For such case, MN and SN can decide whether to apply the TDM solution to help the UE to address the IDC issue, and if so, they can configure the proper DRX/periodic pattern for the UE based on the UE’s TDM assistance information independently. So, there is no need for any coordination either for configuration or solution between MN and SN.

- Case 2: Combination of candidate frequency ranges due to UL CA are affected by the IDC issue

- NW Configuration: MN configures F1 and F2 as the candidate frequency and TDM to the UE, while SN configures F3 and F4 as the candidate frequency and TDM to the UE

- UE reporting: If UE detects IDC issue on UL CA combination of F1 and F2, UE reports the affected frequency combination F1+F2 to the MN; If UE detects IDC issue on UL CA combination of F3 and F4, UE reports the affected frequency combination F3+F4 to the SN.

According to the current running CR [2], UE can also report the TDM assistance information for the NR UL CA case. If majority companies support that TDM solution based on UE’s TDM assistance information can also be applied to NR UL CA case, we think the MN or SN can configure  proper DRX for the UE based on the UE’s TDM assistance information independently as for Case 1, i.e. there is also no need for any coordination either for configuration or solution between MN and SN.

- Case 3: Combination of candidate frequency ranges due to MR DC are affected by the IDC issue

- NW Configuration: MN configures F1 as the candidate frequency and TDM to the UE, while SN configures F2 as the candidate frequency and TDM to the UE

- UE reporting: If UE detects IDC issue on MR-DC combination of F1 and F2, then how the UE reports the TDM assistance information seems to be unclear in RAN 2 and we do not have common understanding for this scenario in RAN2.

According to the current running CR [2], UE also reports the TDM assistance information for the NR-DC case. If majority companies support that TDM solution based on UE’s TDM assistance information can also be applied to NR DC case, we need to discuss the details of the procedure for this case.

Since the IDC issue is caused by the simultaneous transmission on the MCG UL carrier and the SCG UL carrier, there is no IDC issue for the non-3GPP reception as long as there is no simultaneous transmission from the 3GPP side. So, only one node needs to configure proper DRX to the UE based on the TDM assistance information. During the inactive time of the DRX configured by this node, it will not schedule the UE’s UL transmission. Then no matter whether the other node schedules the UE’s UL transmission or not, there will be no IDC issue.

Based on the analysis, we think the UE only needs to report the TDM assistance information to the MN in case of IMD issue are detected for the NR-DC. After receiving the UE’s TDM assistance information together with the affected frequency range combination, it is up to the MN to decide how to address the IDC issue by using TDM solution for the UE as described below.

- If the MN decides to address the IDC problem caused by the MR-DC frequency combination by itself and apply TDM solution, it can (re)configure the DRX for the UE without sending any informing to the SN.

- If the MN knows that the SN support the TDM solution for IDC and decides to ask the SN to address the IDC problem caused by the MR-DC frequency combination, then MN forwards affected frequency combination List as well as the TDM assistance information to SN. Then it is up to SN to decide whether to address the IDC issue for the UE by using FDM solution (e.g.by switching BWP) or by using TDM solution (i.e. configure proper DRX for the UE).

Based on the above discussion, we propose that:

Proposal 2: For the case of IDC issue caused by NR-DC frequency range combination,

- If the SN enables TDM solution for the UE, it indicates it to the MN via the CG-Config;

- The UE only reports the TDM assistance information to the MN;

- The MN can forward the IDC TDM assistance information reported by the UE to the SN via the CG-ConfigInfo.

## 2.3 Common signaling flow for FDM and TDM for NR DC

From the discussion in section 2.1 and 2.2, we  can observe that there are commonalities between the two procedures a common signaling flow can be applied to the FDM and TDM to address the IMD issue caused by NR-DC. The overall signaling flow is shown in Figure 1.

Figure 1 Common signaling flow for inter node co-ordination to address the IMD issue for NR-DC

The key steps for common signaling flow for both FDM and TDM between MN and SN are listed below

- When the SN configures the UE with the information of Candidate SN Serving Frequency Range List and enables the TDM reporting for the UE, SN also send the information of Candidate SN Serving Frequency Range List and the indication that TDM reporting is enabled for UE to MN.

- If the UE detects IDC problem is due to combination of the frequencies configured by MN and SN, UE reports the affected frequency combination to the MN along with the TDM Assistance Information (when TDM reporting is also enabled)

- If the MN decide to address the IDC problem by itself by applying FDM solution (e.g. by deactivating a SCell or by switching the BWP) or by applying TDM Solution (i.e. configuring appropriate DRX for the UE) , it does not forward any info to SN

- If the MN decide to ask the SN to address the IDC problem caused by the MR-DC frequency combination then MN forwards affected frequency combination List to SN and the TDM Assistance information received from the UE for SN. SN may apply FDM or TDM solution to resolve the IDC problem.

Hence we propose the following.

Proposal 3: For the case of IDC issue caused by NR-DC frequency range combination common/generalised signalling flow for enhanced FDM and TDM solution is adopted to address the IMD issue in R18 which is characterised as follows

- When the SN configures the UE with the information of Candidate SN Serving Frequency Range List and enables the TDM reporting for the UE, SN also send the information of Candidate SN Serving Frequency Range List and the indication that TDM reporting is enabled for UE to MN.

- If the UE detects IDC problem is due to combination of the frequencies configured by MN and SN, UE reports the affected frequency combination to the MN along with the TDM Assistance Information (when TDM reporting is also enabled)

- If the MN decide to address the IDC problem by itself by applying FDM solution (e.g. by deactivating a SCell or by switching the BWP) or by applying TDM Solution (i.e. configuring appropriate DRX for the UE) , it does not forward any info to SN

- If the MN decide to ask the SN to address the IDC problem caused by the MR-DC frequency combination then MN forwards affected frequency combination List to SN and the TDM Assistance information received from the UE for SN. SN may apply FDM or TDM solution to resolve the IDC problem.

# Conclusions

One big remaining issue in IDC WI is the inter-node coordination for both FDM solution and TDM solution. In this paper, we further analyze the inter-node coordination issue for various scenarios and then propose the specification impacts for FDM and TDM solutions separately. We then propose the common signaling flow combining the FDM and TDM Solution signalling for addressing the IMD issue for NR-DC.

Inter-node coordination for FDM solution enhancements

Proposal 1: For the case of IDC issue caused by NR-DC frequency range combination, the same procedure as for EN-DC is supported, specifically:

- To add the candidateServingFreqRangeListNR in the CG-Config

- To add the affectedCarrierFreqRangeCombInfoListMRDC in the MRDC-AssistanceInfo of CG-ConfigInfo

- For the UE reporting procedure, if the UE detect the IDC issue on MR-DC frequency range combination, the UE reports the affected frequency range combination to the MN.

Inter-node coordination for TDM solution

Proposal 2: For the case of IDC issue caused by NR-DC frequency range combination,

- If the SN enables TDM solution for the UE, it indicates it to the MN via the CG-Config;

- The UE only reports the TDM assistance information to the MN;

- The MN can forward the IDC TDM assistance information reported by the UE to the SN via the CG-ConfigInfo.

Common signaling flow for FDM and TDM for NR DC

Proposal 3: For the case of IDC issue caused by NR-DC frequency range combination common/generalised signalling flow for enhanced FDM and TDM solution is adopted to address the IMD issue in R18 which is characterised as follows

- When the SN configures the UE with the information of Candidate SN Serving Frequency Range List and enables the TDM reporting for the UE, SN also send the information of Candidate SN Serving Frequency Range List and the indication that TDM reporting is enabled for UE to MN.

- If the UE detects IDC problem is due to combination of the frequencies configured by MN and SN, UE reports the affected frequency combination to the MN along with the TDM Assistance Information (when TDM reporting is also enabled)

- If the MN decide to address the IDC problem by itself by applying FDM solution (e.g. by deactivating a SCell or by switching the BWP) or by applying TDM Solution (i.e. configuring appropriate DRX for the UE) , it does not forward any info to SN

- If the MN decide to ask the SN to address the IDC problem caused by the MR-DC frequency combination then MN forwards affected frequency combination List to SN and the TDM Assistance information received from the UE for SN. SN may apply FDM or TDM solution to resolve the IDC problem.

# References

- R2-2305580 [Post121][655][IDC] Discussion on Leftover issues for IDC

- R2-2302980	38.331 Running CR for Rel-18 IDC UE capabilities