3GPP TSG-RAN WG2 Meeting #122	R2-2305979

Incheon, Korea, 22th – 26th May, 2023

Agenda Item:	7.9.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on service continuity

Document for:	Discussion and Decision

# Introduction

In RAN2 #121bis meeting, we have reached the following agreements for Rel-18 service continuity enhancements part.

Agreements for stage-2 spec: The following sub-clause can be made under “16.12 Sidelink Relay” for intra/inter-gNB i2i on TS 38.300. - A new sub-clause “16.12.6.x Switching from indirect to indirect path” under “16.12.6 Service Continuity for L2 U2N relay”. Agreements for uplink/downlink lossless issue: For uplink lossless data delivery for path switch, continue considering solutions U3 and U5 from R2-2304305. Other solutions are not pursued. For downlink lossless data delivery for path switch, Solution-D4 is taken as the baseline solution and keep Solution-D3/D5 on the table for further decision at the next meeting.

Besides, RAN1 has sent out the Reply LS on the comparison of SL-RSRP and SD-RSRP measurements [1], which suggests RAN2 to decide whether to use the direct comparison of SL-RSRP and SD-RSRP measurements

In this paper, we discuss the comparison of SL-RSRP and SD-RSRP measurements, the solutions for avoiding UL and DL data loss during path switch, and the stage-2 procedures to be included in TS 38.300 taking the above agreements into account.

# Discussion

## 2.1 New measurement event

In previous meetings, some companies wanted to introduce event Z2 to help remote UE find a proper target relay UE in indirect-to-indirect path switch procedure.

- Event Z2: Candidate L2 U2N Relay UE becomes an offset better than serving L2 U2N Relay UE

However, the opponents thought that  this measurement event is not feasible as the direct comparison of SL-RSRP and SD-RSRP measurements is unfair due to the power control mechanism applied on PC5 unicast links.

In last meeting, RAN1 and RAN4 have sent out the Reply LS to RAN2 about this issue. In the LS from RAN1, RAN1 indicated the aspects that will affect the direct comparison of SL-RSRP and SD-RSRP measurements and let RAN2 to decide whether to use the direct comparison of SL-RSRP and SD-RSRP. In the LS from RAN4, RAN4 has similar views as RAN1, i.e., only if the difference caused by transmission power control mechanism can be properly handled, SL-RSRP and SD-RSRP can be used to trigger a measurement report. In the following, we will further analyse whether the difference caused by transmission power control mechanism can be handled. Based on RAN 1 answer , the following four issues have been indicated.

Issue 1: Unicast is subject to transmit power control based on SL pathloss if parameter sl-P0-PSSCH-PSCCH is provided, while broadcast is not subject to this power control mechanism.

Issue 2: Due to CBR-based power control, if configured, maximum transmit power may depend on the priority of the transmission. The priority for discovery messages may be different from the priority used for the transmissions over which SL-RSRP is measured.

Issue 3: Transmission of discovery messages can take place in dedicated discovery pools; these may be configured differently, e.g. with respect to power control settings, from the pool(s) on which SL-RSRP is measured.

Issue 4: Note: Sidelink carrier aggregation, for which an objective was added to Rel-18 NR_SL_enh2 at RAN#99, may additionally impact SL transmit power.

Note: RAN1 has not started work on SL carrier aggregation. Unicast is subject to transmit power control based

According to the TS 38.213 section 16.2.1, the power for sidelink data or discovery transmission (i.e. PSSCH transmission) is decided according the following procedures:

### 16.2.1 PSSCH A UE determines a power PPSSCH(i) for a PSSCH transmission on a resource pool in symbols where a corresponding PSCCH is not transmitted in PSCCH-PSSCH transmission occasion i on active SL BWP b of carrier f as: PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm] where - PCMAX is defined in [8-1, TS 38.101-1] - PMAX,CBR is determined by a value of sl-MaxTxPower based on a priority level of the PSSCH transmission and a CBR range that includes a CBR measured in slot i-N [6, TS 38.214]; if sl-MaxTxPower is not provided, then PMAX,CBR=PCMAX; - if dl-P0-PSSCH-PSCCH is provided - PPSSCH,D(i)=PO,D+10log102μ⋅MRBPSSCHi+αD⋅PLD [dBm] - else - PPSSCH,D(i)=minPCMAX,PMAX,CBR [dBm] where - PO,D is a value of dl-P0-PSSCH-PSCCH-r17 if using the parameter is supported by the UE and the parameter is provided; else dl-P0-PSSCH-PSCCH-r16 if provided - αD is a value of dl-Alpha-PSSCH-PSCCH, if provided; else, αD=1 - PLD=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - MRBPSSCH(i) is a number of resource blocks for the PSSCH transmission occasion i and μ is a SCS configuration - if sl-P0-PSSCH-PSCCH is provided and if a SCI format scheduling the PSSCH transmission includes a cast type indicator field indicating unicast or is SCI format 2-C - PPSSCH,SL(i)=PO,SL+10log102μ⋅MRBPSSCHi+αSL⋅PLSL [dBm] - else - PPSSCH,SL(i)=minPCMAX,PPSSCH,D(i) [dBm] where - PO,SL is a value of sl-P0-PSSCH-PSCCH-r17, if using the parameter is supported by the UE and the parameter is provided; else sl-P0-PSSCH-PSCCH-r16 if provided - αSL is a value of sl-Alpha-PSSCH-PSCCH, if provided; else, αSL=1 - PLSL= referenceSignalPower – higher layer filtered RSRP, where - referenceSignalPower is obtained from a PSSCH transmit power per RE summed over the antenna ports of the UE, higher layer filtered across PSSCH transmission occasions using a filter configuration provided by sl-FilterCoefficient, and - higher layer filtered RSRP is a RSRP, as defined in [7, TS 38.215], that is reported to the UE from a UE receiving the PSCCH-PSSCH transmission and is obtained from a PSSCH DM-RS using a filter configuration provided by sl-FilterCoefficient - MRBPSSCH(i) is a number of resource blocks for PSCCH-PSSCH transmission occasion i and μ is a SCS configuration The UE splits the power PPSSCH(i) equally across the antenna ports on which the UE transmits the PSSCH with non-zero power. A UE determines a power PPSSCH2(i) for a PSSCH transmission on a resource pool in the symbols where a corresponding PSCCH is transmitted in PSCCH-PSSCH transmission occasion i on active SL BWP b of carrier f as PPSSCH2(i)=10log10MRBPSSCHi-MRBPSCCHiMRBPSSCHi+PPSSCH(i) [dBm] where MRBPSCCH(i) is a number of resource blocks for the corresponding PSCCH transmission in PSCCH-PSSCH transmission occasion i. The UE splits the power PPSSCH2(i) equally across the antenna ports on which the UE transmits the PSSCH with non-zero power.

For the transmission power differences between the data transmission and the discovery message transmission of the source relay UE, we can note that the transmit power is the minimum values among PCMAX,  PMAX,CBR,  minPPSSCH,Di,PPSSCH,SL(i).

- Issue 1: Impact of SL path loss on data transmission

The SL pathloss only affects the power control of the unicast transmission. When the distance between the remote UE and the source relay UE is very large, the sidelink path loss is very large, which results in the large value of PPSSCH,SL(i). Since the final power is the minimum value among PCMAX,  PMAX,CBR,  minPPSSCH,Di,PPSSCH,SL(i), the impact of SL pathloss on the data transmission power will be very small, which means the SL pathloss will not impact the transmission power difference between the data and discovery message.

Observation 1: Impact of SL path loss can be neglected when there is a large distance between remote UE and serving relay UE

- Issue 2: Impact of priority level and CBR on discovery and data on transmission

For higher priority level transmission, the UE can choose corresponding sidelink TX parameters according to the CBR value.  Hence for the transmission of the discovery message and data, the possible power difference is introduced by the transmission priority, where the discovery message priority level is 1 (highest) as defined in TS 38.321 and the priority of the data communication may be lower than 1. When the data transmission priority level is 1 which is same as the discovery message, there is no impact from the priority level. Even when the data transmission priority level is not equal to 1, as the data priority and the CBR can be known by the gNB, the gNB can estimate the difference of PMAX,CBR of the discovery message and the data of the serving relay UE, which is a fixed value.

Observation 2: The impact of the priority of data and discovery message can be estimated by gNB.

- Issue 3: Impact of the resource pool configuration on discovery and data on transmission

Considering the configurations of the resource pool for discovery and data are also known by the gNB, the gNB can also estimate the difference according to the configurations. In Rel-17 SL relay, the remote UE can either use SD-RSRP or SL-RSRP to trigger the event X1 and event X2, where gNB only configures one threshold. This indicates that the gNB can take into consideration the pool configurations.

- Issue 4: Impact of the sidelink carrier aggregation

According to the LS from RAN1, the sidelink CA discussion has not started in RAN1 and the impact of the power control is not clear. However, the gNB at least can know whether the CA is configured or not. Therefore, even the sidelink CA has impact on the power control, the impact can be neglected via the gNB implementation, e.g., only configure the event Z2 when no sidelink CA is configured.

Observation 3: The impact of the pool configuration and sidelink CA configuration of data and discovery message can be avoided by gNB.

When comparing the serving SL-RSRP and the candidate SD-RSRP, there may be two scenarios having different SL pathloss which may satisfy the event Z2.

- The remote UE is far away from the serving relay UE and there is a candidate relay UE which is an offset better than the serving relay UE, i.e. there is a bad serving relay UE and a good candidate relay UE scenario. This is the motivation of introducing this event, and the triggering of the event Z2 is to help choose a better relay for service continuity. Considering that when different link measurement types of the serving relay UE, i.e., SL-RSRP and SD-RSRP, are used for the measurement event Z2, as the difference of SL-RSRP and SD-RSRP of serving relay UE can be estimated as illustrated in observations 1 through 3, gNB can configure an appropriate offset for comparison of different measurement types.

- The remote UE is close to the serving relay UE and there is a candidate relay UE which is an offset better than the serving relay UE, i.e., both serving relay UE and candidate relay UEs are good, but candidate is better. This is a scenario that may result in a useless report to the network and subsequent signalling. For this situation, considering that when different link measurement types of the serving relay UE, i.e., SL-RSRP and SD-RSRP,  are used for the measurement event Z2, as the difference of SL-RSRP and SD-RSRP of serving relay UE can be estimated by considering the impact of pathloss, gNB can configure an appropriate offset for comparison of different measurement types.

Based on the above analysis we think that comparison of SL-RSRP and SD-RSRP is feasible for the measurement event Z2 as the gNB can configure appropriate offset for such comparison through implementation.

Proposal 1: Comparison of SL-RSRP and SD-RSRP for the measurement event Z2 can be supported as the gNB can configure appropriate offset for such comparison through implementation.

## 2.2 Service continuity procedures

In the following subsection, we describe the stage-2 procedures for:

- inter-gNB indirect-to-direct path switching,

- inter-gNB direct-to-indirect path switching,

- intra-gNB indirect-to-indirect path switching, and

- inter-gNB indirect-to-indirect path switching.

Which can be included in the stage-2 specification.

## 2.2.1 Inter-gNB indirect-to-direct path switching

Figure 1 shows the procedure for the inter-gNB indirect-to-direct path switching case which is extended from Rel-17 intra-gNB indirect-to-direct path switching procedure as defined in clause 16.12.6.1 in TS 38.300 [2].

Remote UE

Relay UE

Source gNB

9. sidelink release

Target gNB

3. HO request

4. HO ACK

6. RA to target gNB

8. RRC Reconfiguration

UL and DL data

5. RRC Reconfiguration message

1. Measurement configuration and reporting

2. HO decision

7. RRC Reconfiguration complete message

Figure 1.  Inter-gNB indirect-to-direct path switching procedure

Step 1. Remote UE triggers the measurements report when the sidelink measurement quantity between Relay UE and Remote UE indicates deteriorated link quality. The triggering Event X1 and Event X2 can be reused in the measurement configuration. The Uu measurements of neighbour cells can be included in the measurements report.

- Event X1: Serving L2 U2N Relay UE becomes worse than threshold1 and NR Cell becomes better than threshold2

- Event X2: Serving L2 U2N Relay UE becomes worse than threshold

Step 2. Source gNB decides to switch the Remote UE onto direct Uu path in the target gNB.

Step 3. Source gNB sends the HANDOVER REQUEST message to target gNB over the Xn interface/NG interface, which contains target cell ID, Remote UE’s UE context, and other necessary information. (Here we take Xn-based path switching as an example).

Step 4. Target gNB determines RRC configuration for the Remote UE at the target cell, and sends it via HANDOVER REQUEST ACKNOWLEDGE message to the source gNB.

Step 5. Source gNB sends the RRC reconfiguration to Remote UE. The Remote UE stops UP and CP transmission via the Relay UE after reception of the RRC Reconfiguration message with the path switch configuration.

Step 6. The UE (Remote UE in previous steps) synchronizes with the target gNB and performs Random Access.

Step 7. The UE sends the RRC Reconfiguration complete message to target gNB via the direct path. From this step, the UE uses the RRC connection via the direct path to the gNB.

Step 8. Source gNB sends RRC Reconfiguration message to the Relay UE to reconfigure the connection between the L2 U2N Relay UE and the gNB.

Step 9. Sidelink between Remote UE and Relay UE is released, which can be triggered by either Relay UE or Remote UE.

In the above procedures, we can see the HO preparation between source gNB and target gNB in step 3 and step 4 are needed to enable inter-gNB path switching, other than that the whole procedure of intra-gNB path switching is applicable to inter-gNB path case. The information/configuration contained in the messages in step 3 and step 4 are the same as those used for a normal UE’s handover.

Proposal 2a: In inter-gNB indirect-to-direct path switching, the HO preparation between source gNB and target gNB is performed in the same way as legacy inter-gNB HO, without additional RAN2/RAN3 spec impact.

Proposal 2b: Agree the above procedure in Figure 1 as a baseline for inter-gNB indirect-to-direct path switching procedure which can be contained in the clause 16.12.6.1 of TS 38.300.

## 2.2.2 Inter-gNB direct-to-indirect path switching

Remote UE

Target Relay UE

Source gNB

8. sidelink establishment if not exist

1. U2N Relay discovery

7. RRC Reconfiguration for Remote UE

Target gNB

3. HO decision

2. Measurement configuration and reporting

4. HO request

5. HO ACK

9. RRC Reconfiguration complete message

6. RRC Reconfiguration message

UL and DL data

Figure 2 shows the inter-gNB direct-to-indirect path switching procedure, which is extended from Rel-17 procedure for L2 U2N Remote UE switching to indirect path as defined in clause 16.12.6.2 in TS 38.300 [2].

Figure 2.  Inter-gNB direct-to-indirect path switching procedure

Steps 1 and 2: Remote UE reports one or more candidate Relay UE(s) to the source gNB, which is discovered in step 1. The triggering Event Y1 and Event Y2 can be reused in the measurement configuration. The reported results includes at least the Relay UE ID (source Layer-2 ID of Relay UE), Relay UE’s serving cell ID, and sidelink measurement quantity (SD-RSRP derived by measuring the discovery message broadcasted by the Relay UE).

- Event Y1: PCell becomes worse than absolute threshold1 AND candidate L2 U2N Relay UE becomes better than another absolute threshold2;

- Event Y2: Candidate L2 U2N Relay UE becomes better than absolute threshold;

Step 3: The source gNB decides to switch the UE to an indirect path in the target cell of the target gNB.

Step 4: To perform HO preparation, source gNB sends the HANDOVER REQUEST message to target gNB via Xn interface. (Here we take Xn-based path switching as an example). A list of candidate relay UEs belonging to the same target cell is included in the HANDOVER REQUEST message.

Step 5. Target gNB determines RRC configuration for the Remote UE at the target cell, and sends it via HANDOVER REQUEST ACK to the source gNB.

Step 6. Source gNB sends the RRC configuration to Remote UE.

Step 7. Target gNB sends the relaying reconfigurations to the Relay UE.

Step 8. Remote UE establishes the sidelink RRC connection with the target Relay UE if it does not exist.

Step 9. Remote UE sends the RRC complete message to gNB via the Relay UE.

Proposal 3: Agree the above procedure in Figure 2 as a baseline for Inter-gNB direct-to-indirect path switching which can be contained in the clause 16.12.6.2 of TS 38.300.

Regarding the measurement configuration, it would be beneficial if gNB can indicate the Allowed-List and Block-List to restrict the Relay UE’s serving cell. Then Remote UE would not report the Relay UE that is camped on the Block-List cells or contained in the Block-List for Relay UE, which can avoid unnecessary reporting and reduce signalling overhead.

Proposal 4: Introduce the Allowed-List/Block-List to restrict candidate relay UE’s serving cell.

## 2.2.3 Intra-gNB indirect-to-indirect path switching

The indirect-to-indirect path switch type has not been discussed in R17 SL relay, but some legacy procedures and principle can be reused as well with some modification. Combining the procedure of direct to indirect path switching with the one for indirect to direct path switching, Figure 3 shows the intra-gNB indirect-to-indirect path switching procedure. Before the path switching, the Remote UE is connected with the same gNB via a Relay UE. After the path switching, the Remote UE established unicast link with a new Relay UE (i.e. target Relay UE), and the unicast between the Remote UE and the old Relay UE is be released (as in step 9).  Meanwhile, the gNB sends RRC message to the old Relay UE to release/reconfigure the relaying configuration associated the Remote UE (as in step8). These procedures are similar to the indirect-to-direct path switching case in R17.

Remote UE

Relay UE

gNB

Target Relay UE

3. HO decision

1. U2N Relay discovery

9. sidelink release

8. RRC Reconfiguration

5. RRC Reconfiguration for Remote UE

2. Measurement configuration and reporting, contain the target Relay UE info

UL and DL data

4. RRC Reconfiguration message

6. sidelink establishment if not exist

7. RRC Reconfiguration complete message

Figure 3.  Intra-gNB indirect-to-indirect path switching procedure

Step 1 and Step 2. Remote UE reports one or multiple candidate Relay UE(s), when the sidelink measurement quantity between Relay UE and Remote UE shows deteriorated link quality. The triggering Event Z1 and Event Z2 (if agreed) can be used in the measurement configuration. The reporting results includes at least the Relay UE ID (source Layer-2 ID of Relay UE), Relay UE’s serving cell ID, and sidelink measurement quantity (SD-RSRP derived by measuring the discovery message broadcasted by the Relay UE).

- Event Z1: Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2

- Event Z2 (if agreed): Candidate L2 U2N Relay UE becomes an offset better than the serving L2 U2N Relay UE

Step 3. The gNB decides to switch the Remote UE to a target Relay UE in the target cell of the same gNB based on the reporting results.

Step 4. The gNB sends an RRC configuration message to the Remote UE. The RRC message include at least L2 U2N Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic and the associated end-to-end radio bearer(s).

Step 5. The gNB sends an RRC configuration message to the target Relay UE, which includes at least the Remote UE's L2 ID and local ID, Uu and PC5 Relay RLC channel configuration for relay traffic and bearer mapping configuration.

Step 6. Remote UE establishes the sidelink RRC connection with the target Relay UE if not exist.

Step 7. Remote UE sends the RRC complete message to gNB via the target Relay UE.

Step 8. The gNB sends an RRC message to the source Relay UE to reconfigure the connection between the source Relay UE and the gNB.

Step 9. Sidelink between Remote UE and the source Relay UE is released.

Proposal 5: Agree the above procedure in Figure 3 as a baseline for intra-gNB indirect-to-indirect path switching which can be contained in the new sub-clause “16.12.6.x Switching from indirect to indirect path” of TS 38.300.

## 2.2.4 Inter-gNB indirect-to-indirect path switching

Figure 4 shows the inter-gNB indirect-to-indirect path switching procedure, which can reuse the procedures for intra-gNB indirect-to-indirect case as a baseline.

Remote UE

Relay UE

Source gNB

Target Relay UE

3. HO decision

1. U2N Relay discovery

7. RRC Reconfiguration for Remote UE

Target gNB

4. HO request

5. HO ACK

11. sidelink release

10. RRC Reconfiguration

UL and DL data

2. Measurement configuration and reporting, contain the target Relay UE info

6. RRC Reconfiguration message

8. sidelink establishment if not exist

9. RRC Reconfiguration complete message

Figure 4.  Inter-gNB indirect-to-indirect path switching procedure

Rel-18 inter-gNB indirect-to-indirect path switching can be supported on top of Rel-18 intra-gNB path switching from indirect to indirect path, the HO preparation between source gNB and target gNB can be performed in the same way as Rel-18 inter-gNB path switching from direct to indirect path, without additional RAN2/RAN3 spec impact. Hence we can consider the above procedure as the baseline for inter-gNB indirect-to-indirect path switching.

Proposal 6: Agree the above procedure in Figure 4 as a baseline for inter-gNB indirect-to-indirect path switching which can be contained in the new sub-clause “16.12.6.x Switching from indirect to indirect path” of TS 38.300.

  - Data Delivery for Inter-gNB cases

In RAN2 121bis meeting, we have reached the following agreements for the DL and UL lossless data delivery in inter-gNB path switching. In the following, we further discuss pros and cons of each solution.

    - 2.3.1 UL data delivery

For uplink lossless data delivery for path switch, according to the agreement, we continue considering solutions U3 and U5.  Other solutions are not pursued.

Solution- U3: Remote UE’s PDCP retransmission based on DL PDCP Status Report from target gNB

Solution-U5: Source Relay UE continues to transmit UL data to source gNB and gNB forwards to the target gNB

For solution - U3:

According to the behaviour during the handover, the remote UE’s PDCP entity need to be re-established when the remote UE receive the handover command which contains the indication of reestablishPDCP. During the PDCP reestablishment procedure, the transmit PDCP entity will determine the first SDU to retransmit based on the lower layer ACK status. Considering the DL PDCP Status Report is received when the remote UE preform random access with the target cell, it is not possible for remote UE to determine the retransmission SDU based on the received  PDCP Status Report during the PDCP reestablishment procedure.

Consider the behaviour of receiving the legacy PDCP Status Report is to discard the PDCP SDUs confirmed by the report, it cannot trigger the retransmission. Therefore, there are two options to trigger the retransmission:

Option 1: A new DL PDCP Status Report can be introduced, e.g. a new type of DL PDCP Status Report can trigger the remote UE’s PDCP entity to retransmit the missing SDU.

Option 2: Legacy DL PDCP Status Report is used but add new behaviour to the remote UE, e.g., the first DL PDCP Status Report received by the remote UE after PDCP reestablishment can trigger the remote UE to retransmit the missing SDU.

Considering option 1 is easy to be controlled by the gNB, we prefer option 1, but we are fine for the option 2.

For solution - U5:

Since source gNB sends the SN STATUS TRANSFER message to the target gNB to convey the uplink PDCP SN receiver status and also the HFN for the UL for security synchronization, this means when the gNB start to forward the uplink SDUs, the status needs to be frozen. If the source gNB froze the receiving status first with fixed HFN, then the SDUs received later on from the relay may cause the HFN synchronization issues. To avoid HFN synchronization issues, the source gNB needs to wait for the transmission of all the PDUs from the relay UE which are successfully received from the remote UE and then freeze the receiver status. Consider the source gNB doesn’t know whether the relay UE has transmitted all the PDUs of remote UE, it will be very difficult to decide when to freeze the receiver status.

Observation 4: Solution-U5 may cause HFN synchronization issues if forwarding occurs after receiver status is frozen in the source gNB. It is difficult for the source gNB decide when to freeze the receiver status.

Proposal 7: Solution- U3 is pursued to avoid UL data loss. FFS the following options for Solution-U3:

Option 1: A new DL PDCP Status Report can be introduced, e.g. a new type of DL PDCP Status Report can trigger the remote UE’s PDCP entity to retransmit the missing SDU.

Option 2: Legacy DL PDCP Status Report is used but a  new behaviour to the remote UE is added in the specification, e.g., the first DL PDCP Status Report received by the remote UE after PDCP reestablishment can trigger the remote UE to retransmit the missing SDU.

    - 2.3.2 DL data delivery

For downlink lossless data delivery for path switch, Solution-D4 is taken as the baseline solution and keep Solution-D3/D5 on the table for further decision at the next meeting.

Solution-D4: Enhanced Data forwarding from source gNB to target gNB per target gNB request (legacy PDCP status report based)

Solution-D3: A PDCP status report sent from Remote UE to the source gNB

Solution-D5: Proactive Data forwarding from source gNB to target gNB

For solution – D4:

According to the UL PDCP status report from the remote UE, the target gNB can obtain the receiving status of the remote UE. Based on the information of the unacknowledged SDUs, there are two options to enhance the data forwarding:

Option 1: complementary data forwarding, i.e., keep legacy data forwarding procedure and request a secondary data forwarding to forward the missing data.

Option 2: late data forwarding, i.e., when the path is an indirect path in the source cell, the source gNB wait for the target gNB request to perform data forwarding.

Note that target gNB doesn’t need to wait for the complementary data forwarding for data transmission. Therefore, we prefer option 1.

For solution – D3:

If Remote can successfully provide the receiving status to the source gNB for data forwarding via PDCP status report in the source gNB, the legacy data forwarding will not be affected. However, in legacy procedure, after receiving the path switch command, the remote UE will stop the data transmission with the relay UE as the link quality is already deteriorating and try to establish the new connection in the direct link.  Transmitting the PDCP status report after receiving the path switch command will have the risk that the gNB will not receive the PDCP status report in most of the cases.

Observation 5: Solution -D3 will not work due to the deteriorating link quality between the remote UE and the relay UE.

For solution – D5:

Solution-D4 enable the  source gNB forward redundant DL packet to the target gNB. One may argue that choosing which packets to forward can be based on the source gNB implementation.  This requires the source gNB to forward PDCP SDUs that has already been acknowledged by the lower layer to the target gNB in the path switch scenarios. Considering that the source gNB cannot accurately know the data stuck in the relay UE without any enhancement between the relay UE and source gNB, the data lossless data transfer cannot be guaranteed.

Observation 6: Solution -D5 cannot guarantee the lossless data transfer during the path switch.

Proposal 8: Solution- D4 is pursued to avoid DL data loss via a complementary data forwarding based on the target gNB request.

# Conclusion

Based on above discussion, we have the following proposal.

New measurement event

Observation 1: Impact of SL path loss can be neglected when there is a large distance between remote UE and serving relay UE

Observation 2: The impact of the priority of data and discovery message can be estimated by gNB.

Observation 3: The impact of the pool configuration and sidelink CA configuration of data and discovery message can be avoided by gNB.

Proposal 1: Comparison of SL-RSRP and SD-RSRP for the measurement event Z2 can be supported as the gNB can configure appropriate offset for such comparison through implementation.

Service Continuity Procedures

Inter-gNB indirect-to-direct path switching

Proposal 2a: In inter-gNB indirect-to-direct path switching, the HO preparation between source gNB and target gNB is performed in the same way as legacy inter-gNB HO, without additional RAN2/RAN3 spec impact.

Proposal 2b: Agree the above procedure in Figure 1 as a baseline for inter-gNB indirect-to-direct path switching procedure which can be contained in the clause 16.12.6.1 of TS 38.300.

Inter-gNB direct-to-indirect path switching

Proposal 3: Agree the above procedure in Figure 2 as a baseline for Inter-gNB direct-to-indirect path switching which can be contained in the clause 16.12.6.2 of TS 38.300.

Proposal 4: Introduce the Allowed-List/Block-List to restrict candidate relay UE’s serving cell.

Intra-gNB indirect-to-indirect path switching

Proposal 5: Agree the above procedure in Figure 3 as a baseline for intra-gNB indirect-to-indirect path switching which can be contained in the new sub-clause “16.12.6.x Switching from indirect to indirect path” of TS 38.300.

Inter-gNB indirect-to-indirect path switching

Proposal 6: Agree the above procedure in Figure 4 as a baseline for inter-gNB indirect-to-indirect path switching which can be contained in the new sub-clause “16.12.6.x Switching from indirect to indirect path” of TS 38.300.

Data Delivery for Inter-gNB cases

UL data delivery

Observation 4: Solution-U5 may cause HFN synchronization issues if forwarding occurs after receiver status frozen in the source gNB. It is difficult for the source gNB decide when to  freeze the receiver status.

Proposal 7: Solution- U3 is pursued to avoid UL data loss. FFS the following options for Solution-U3:

Option 1: A new DL PDCP Status Report can be introduced, e.g. a new type of DL PDCP Status Report can trigger the remote UE’s PDCP entity to retransmit the missing SDU.

Option 2: Legacy DL PDCP Status Report is used but a  new behaviour to the remote UE is added in the specification , e.g., the first DL PDCP Status Report received by the remote UE after PDCP reestablishment can trigger the remote UE to retransmit the missing SDU.

DL data delivery

Observation 5: Solution -D3 will not work due to the deteriorating link quality between the remote UE and the relay UE.

Observation 6: Solution -D5 cannot guarantee the lossless data transfer during the path switch.

Proposal 8: Solution- D4 is pursued to avoid DL data loss via a complementary data forwarding based on the target gNB request.

# Reference

[1] R1-2304210, Draft Reply LS on comparison of SL-RSRP and SD-RSRP measurements, Nokia

[2] 3GPP TS 38.300, V17.4.0, NR; Overall description; Stage 2

[3] RP-221262, Revised WID on NR sidelink relay enhancements, LG Electronics