3GPP TSG-RAN WG2 #122	R2-2305988

Incheon, May 22nd – May 26th, 2023

Agenda Item:	8.13.3

Source:	Ericsson

Title:	MDT enhancements

Document for:	Discussion, Decision

# 1	Introduction

In RAN2-121 meeting, the following were agreed.

Agreements For solution 2:

1	Extend the LTE LoggedMeasurementConfiguration to include Logged MDT type indication information

2	NR signaling is needed to inform the gNB that signaling based MDT is configured by E-UTRA.

3	Try to reuse R17 NR signaling by the UE to inform gNB whether signaling based MDT is configured even when it is configured by E-UTRA.

=>	FFS in RAN3 the details and the Need of differentiating the RAT type. Further discuss whether priority handling for signalling logged MDT configuration between different RAT types is needed or not.

In this contribution, we shall highlight some concerns regarding the agreed solution and propose way forward to mitigate the concerns.

# 2	Discussion

## 2.1 Protection of Signalling-Based MDT configuration in inter-RAT scenario

Indicating the LTE signalling based MDT when connecting to NR

When the UE connects to the NR cell (coming from LTE network), as part of procedural text the UE checks the RAT information before forwarding the signalling based logged MDT protection flag to NR i.e., as of now only NR signalling based logged MDT report is allowed to be flagged to the NR nodes. This is shown in the following excerpt from TS38.331 section 5.3.3.4.

2> if the sigLoggedMeasType in VarLogMeasReport is included: 3> if T330 timer is running and the logged measurements configuration is for NR: 4> set sigLogMeasConfigAvailable to true in the RRCSetupComplete message; 3> else: 4> if the UE has logged measurements available for NR: 5> set sigLogMeasConfigAvailable to false in the RRCSetupComplete message;

In Rel-17, UE checks if the signaling based configuration is for NR before setting signaling based logged MDT availability flag in the RRC complete message.

Considering the current agreements in the meeting RAN2#121, UE shall inform NR gNB regarding the availability of LTE signalling based MDT configuration in RRC complete messages; thus, this restriction should be removed.

2> if the sigLoggedMeasType in VarLogMeasReport is included: 3> if T330 timer is running and the logged measurements configuration is for NR: 4> set sigLogMeasConfigAvailable to true in the RRCSetupComplete message; 3> else: 4> if the UE has logged measurements available for NR: 5> set sigLogMeasConfigAvailable to false in the RRCSetupComplete message;

In NR, UE reports availability of signalling based logged MDT configuration without checking the RAT information.

Some proposals regarding separate flags for NR and LTE signalling based logged MDT configuration availability has been brought forward. In our opinion, since network should not overwrite signalling based logged MDT configuration, differentiating if the configuration is for LTE or NR is not important to the network. Thus, the same flag of NR can be re-used for LTE, as suggested in the agreements from RAN2#121.

RAN2 confirms that sigLogMeasConfigAvailable can be re-used for to indicate the availability of the LTE signalling based logged MDT in NR.

LTE report collection in NR node

If the T330 timer for the LTE signalling based MDT configuration finishes, NR node still can’t configure it with a new configuration before the reports are fetched, which can take up to 48 hrs.

NR node needs to wait 48hrs after the T330 timer finishes for the LTE signalling based MDT configuration. This leads to long delay for NR to collect its own MDT measurements.

An alternative to this is, NR can collect LTE signalling based MDT report after the T330 timer has expired and forward the report to TCE. This reduces probability of losing the LTE report and provides NR more flexibility to configure the UE with its own configuration. Some concerns regarding the impact of the reporting LTE logged MDT report to NR have been raised.

To analyse, we point out some excerpt from TS36.331.

LogMeasReport-r10 ::=				SEQUENCE {

absoluteTimeStamp-r10				AbsoluteTimeInfo-r10,

traceReference-r10					TraceReference-r10,

traceRecordingSessionRef-r10		OCTET STRING (SIZE (2)),

tce-Id-r10							OCTET STRING (SIZE (1)),

logMeasInfoList-r10					LogMeasInfoList-r10,

logMeasAvailable-r10				ENUMERATED {true}				OPTIONAL,

...,

[[	logMeasAvailableBT-r15			ENUMERATED {true}				OPTIONAL,

logMeasAvailableWLAN-r15		ENUMERATED {true}				OPTIONAL

]]

}

As seen from the excerpt, UE includes TCE ID, Trace reference and trace reconrding session reference to identify proper TCE and Trace for the RAN to forward the collected logged MDT report.

UE includes TCE ID in the logged MDT report to enable network to identify the TCE where the collected logged MDT report should be forwarded.

However, from 32.422 the following definition of TCE ID can be found-

### 5.10.11	Trace Collection Entity (TCE) Id

This is a mandatory parameter for Logged MDT which defines the TCE Id which is sent to the UE. The UE returns it to the network together with the logged data. The network has a configured mapping of the TCE Id and the destination to which trace records shall be transferred.

The destination is determined considering the PLMN (delivered as part of the Trace Reference) and the IP address of TCE for the file-based trace reporting, or URI of the TCE or MnS trace reporting MnS consumer for the streaming trace reporting. The mapping needs to be unique within the PLMN. The size of the parameter is one byte.

For further details on the procedure see clause 4.1.1.2a for UTRAN, clauses 4.1.1.6a and 4.1.1.6b for E-UTRAN and clause 4.1.1.9.2 for NR.

The highlighted parts of the definition shows that for a given PLMN, the network has a unique mapping between TCE ID and destination where the trace records will be transferred.

The mapping between TCE ID and destination where the trace records will be transferred is unique within a PLMN.

Based on the analysis, to avoid underutilizing NR MDT framework we propose the following.

LTE MDT measurements can be fetched by NR nodes, to avoid underutilizing NR MDT mechanism.

# Conclusion

In the previous sections we made the following observations:

Observation 1	In Rel-17, UE checks if the signaling based configuration is for NR before setting signaling based logged MDT availability flag in the RRC complete message.

Observation 2	NR node needs to wait 48hrs after the T330 timer finishes for the LTE signalling based MDT configuration. This leads to long delay for NR to collect its own MDT measurements.

Observation 3	UE includes TCE ID in the logged MDT report to enable network to identify the TCE where the collected logged MDT report should be forwarded.

Observation 4	The mapping between TCE ID and destination where the trace records will be transferred is unique within a PLMN.

Based on the discussion in the previous sections we propose the following:

Proposal 1	In NR, UE reports availability of signalling based logged MDT configuration without checking the RAT information.

Proposal 2	RAN2 confirms that sigLogMeasConfigAvailable can be re-used for to indicate the availability of the LTE signalling based logged MDT in NR.

Proposal 3	LTE MDT measurements can be fetched by NR nodes, to avoid underutilizing NR MDT mechanism.

# References

R2-2209023 [AT117e][801] Total RAN delay calculation, 3GPP