3GPP TSG-RAN WG2 #122	R2-2305991

Incheon, May 22nd – 26th, 2023

Agenda Item:	7.13.8

Source:	Ericsson

Title:	RAN observability issues for DRBs with stringent QoS requirements

Document for:	Discussion, Decision

# 1	Introduction

In this contribution, we address RAN observability issues for DRBs with stringent QoS requirements e.g., URLLC type of services, considering the issues existing in the current specified solution for average delay measurement at the UE.

# 2	Discussion

URLLC types of services are one of key use cases specified to be provided via NR network that require stringent QoS requirements as specified in TS 23.501. Such stringent QoS requirements for the URLLC type of services shall be provisioned by the RAN nodes particularly over the radio interface that maybe impaired by the interference, mobility as well as coverage issues. This requires an accurate observability mechanism in RAN node.

Provisioning URLLC type of services requires fulfilling stringent QoS requirements.

Stringent QoS requirements of URLLC services need to be fulfilled by NR RAN node and requires a high-resolution observability.

However, the current solution specified for delay measurement for the UP PDCP in the TS 38.331 provides a very coarse measurement period corresponding to the report interval (minimum value 120ms). This does not provide enough accuracy for pinpointing impairments on the stringent QoS requirements for the URLLC services.

ul-DelayValueConfig

If the field is present, the UE shall perform the actual UL PDCP Packet Average Delay measurement per DRB as specified in TS 38.314 [53] and the UE shall ignore the fields reportQuantityCell and maxReportCells. The applicable values for the corresponding reportInterval are (one of the) {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1,min6, min12, min30}. The reportInterval indicates the periodicity for performing and reporting of UL PDCP Packet Average Delay per DRB measurement as specified in TS 38.314 [53].

The current measurement period for the delay measurement performed by the UE is inherited from the report interval configuration (the minimum configurable value is 120ms).

As highlighted above the minimum measurement window (i.e., periodicity for performing and reporting of UL PDCP Packet Average Delay) is 120ms. According to the PDCP TS 38.323 the maximum size of a PDCP SDU can be up to 9000 bytes.

The maximum supported size of a PDCP SDU is 9000 bytes. The maximum supported size of a PDCP Control PDU is 9000 bytes.

This means during a minimum measurement time window (120ms), it is plausible that UE transmits many PDCP SDUs. According to the current average delay measurement the UE needs to measure the average delay for thousands of PDCP SDUs and report one single measurement. Although such an averaging (e.g., average delay of 1000 PDCP SDUs in 120ms) could suffice traditional services like MBB in LTE network, in NR and in particular for URLLC types of services would not provide enough observability in RAN node to take counter actions when delay in some packets leads to sever deterioration of the QoS. The report interval in the RRC spec is originally designed for providing RRM measurements e.g., for mobility purpose etc., which is not suitable in terms of measurement time scale for the URLLC QoS observability purpose at RAN.

Unlike traditional types of services e.g., MBB which have more relaxed QoS requirements, the new emerging types of services in NR e.g., the URLLC types of services requires stringent QoS requirements and for such services the tail of the delay distribution is a more crucial measurement, reflecting how much delay might have been induced by the PDCP SDUs that arrive late, affecting even the PDCP SDUs that met the QoS requirements.

The current report interval of the UL PDCP average delay (inherited from LTE) might be enough for the traditional services like MBB but not sufficient for URLLC observability.

Therefore, for a more accurate observability at RAN we would need to enhance the UL PDCP Packet Average Delay with a finer granularity of measurement periods for those DRB types with stringent QoS requirements.

RAN2 to enhance the UL PDCP Packet Average Delay measurements with a shorter measurement interval.

As previously mentioned, in legacy configuration, the measurement period corresponds to the report interval, i.e. the UL PDCP Packet Average Delay is averaged over a measurement window which is configured through the reportInterval. Hence, introducing a shorter measurement period, as proposed in Proposal 1, would imply a shorter report interval which in turn may cause additional overhead on the radio signaling.

A shorter measurement period for the average delay measurements, as proposed in Proposal 1, would imply more signaling overhead over the air interface.

Therefore, we would like to request RAN2 to discuss how to reduce the overhead of signaling average delay measurements with a shorter measurement period over the air interface.

RAN2 discuss how to reduce the overhead of the signaling of average delay measurements with shorter measurement period, over the air interface.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Provisioning URLLC type of services requires fulfilling stringent QoS requirements.

Observation 2	Stringent QoS requirements of URLLC services need to be fulfilled by NR RAN node and requires a high-resolution observability.

Observation 3	The current measurement period for the delay measurement performed by the UE is inherited from the report interval configuration (the minimum configurable value is 120ms).

Observation 4	The current report interval of the UL PDCP average delay (inherited from LTE) might be enough for the traditional services like MBB but not sufficient for URLLC observability.

Observation 5	A shorter measurement period for the average delay measurements, as proposed in Proposal 1, would imply more signaling overhead over the air interface.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to enhance the UL PDCP Packet Average Delay measurements with a shorter measurement interval.

Proposal 2	RAN2 discuss how to reduce the overhead of the signaling of average delay measurements with shorter measurement period, over the air interface.