3GPP TSG RAN WG2 Meeting #122                        R2- 2305992

May 22nd – May 26th, 2023

Source:	Sharp

Title:	[IoT NTN Enh] GNSS operation enhancements

Agenda Item:	7.6.2.2

Document for:	Discussion and Decision

# Introduction

One of the objectives in Release 18 WID states:

- Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed.

In RAN2#121-bis-e meeting, numerous companies discussed whether IoT NTN devices should move to idle state if GNSS acquisition fails while in GNSS gap measurement [1]. Furthermore, in RAN1, the UE behaviour of GNSS re-acquisition failure was also discussed based on a previous agreement in RAN1#111 [2]:

Agreements:

For GNSS measurement in RRC connected, if eNB aperiodically triggers connected UE to make GNSS measurement, UE can re-acquire GNSS position fix with a gap

•	FFS details of gap configuration

The UE may re-acquire GNSS autonomously (when configured by the network) if UE does not receive eNB trigger to make GNSS measurement

•	FFS based on configured timing

This paper discusses further considerations on the behavior of IoT NTN devices and GNSS acquisition failures.

# Discussion

## 2.1    GNSS and IoT NTN

In previous RAN2 meeting, one of the email discussions focused on behaviour of UE if GNSS acquisition fails during GNSS measurement gap [1].  Majority of the companies agreed that the UE should move to idle mode if the UE GNSS acquisition fails while other companies stated that the UE perform another GNSS acquisition while still in GNSS measurement gap.

RAN1 is also discussing the use of autonomous GNSS re-acquisition.  While no agreements have been made with autonomous GNSS re-acquisition, the behaviour of UE after GNSS re-acquisition should be discussed, specifically if the GNSS re-acquisition fails.

The primary purpose of GNSS acquisition is to maintain connection mode between NTN satellites and IoT NTN devices.  One method is to use GNSS gap measurement configured by the network.  Another possible method is UE autonomously re-acquire GNSS measurement also configured by the network.

If autonomous measurement is implemented, the UE may fail to obtain GNSS values.  Furthermore, if the GNSS validity timer also expired, then attempting multiple GNSS acquisitions may still not result in a successful GNSS measurement.

Observation 1: For IoT NTN devices, UE may need to obtain GNSS values but at times, the acquisition may fail.

Battery conservation is vital for any IoT devices.  Power consumption is higher in connected state versus idle mode. In addition, GPS also utilizes high battery power. If the autonomous GNSS measurement fails and the GNSS validity duration already expired, then the UE should move to idle state instead of remaining in connected mode and attempt additional GPS fixes.

Proposal 1: If UE autonomous GNSS acquisition (as configured by the network) fails and the GNSS validity timer expired, the UE should move to idle mode.

# Conclusions

Observation 1: For IoT NTN devices, UE may need to obtain GNSS values but at times, the acquisition may fail.

Proposal 1: If UE autonomous GNSS acquisition (as configured by the network) fails and the GNSS validity timer expired, the UE should move to idle mode.

# References

RAN2#121-bis-e R2-2304244

Draft Report of 3GPP TSG RAN WG1 #111