3GPP TSG-RAN WG2 Meeting #122	R2-2306004

# Incheon, South Korea, 22nd – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1624 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on which CSI-RS resources in IAB restricted beam MAC CEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS 38.321 clause 6.1.3.61 and 6.1.3.62, these two MAC CEs should indicate a reference CSI-RS resource ID in order for the UE to identify which CSI-RS resources set to use in order to perform certain actions. Since there are basically three types of CSI-RS resources set with three different resource IDs, it would be good to clarify that the CSI-RS resouce ID to which one the MAC CEs is referring is the NZP-CSI-RS-ResourceId. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 6.1.3.61 and 6.1.3.62 - Clarified that the recommended/restricted beams for IAB-MT/IAB-DU is 8 Impact Analysis Impacted 5G architecture options: IAB Impacted functionality: Recommended/restricted beams for IAB-MT/IAB-DU Inter-operability: 1. If the network is implemented according to the CR and the UE is not, there is no inter-operability issue. 2. If the UE is implemented according to the CR and the network is not, the UE may not understand to which set of CSI-RS resources the MAC CE is referring to. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not approved, the UE may not understand to which set of CSI-RS resources the MAC CE is referring to. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.61, 6.1.3.62 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 6.1.3.61	Child IAB-DU Restricted Beam Indication MAC CE

The Child IAB-DU Restricted Beam Indication MAC CE is identified by MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size with following fields (Figure 6.1.3.61-1):

-	Resource Configuration ID: This fields indicates the RRC configuration which applies to this MAC CE and corresponds to iab-ResourceConfigID parameter in IAB-ResourceConfig as specified in TS 38.331 [5]. The length of the field is 16 bits;

-	Number of child IAB-DU beams ID: This field indicates the number of restricted child IAB-DU beams included in the MAC CE. The length of the field is 8 bits;

-	IAB-DU restricted beams type IDi: This field determines the type of information used as an indication of a set of child IAB-DU's restricted beams, indicated with the Child IAB-DU Resource set IDi field. IAB-DU restricted beams type ID0 refers to the first restricted beam within the resource set, IAB-DU restricted beams type ID1 to the second one and so on. If the value of the field is 00, SSB index is used (contained in the 6 rightmost bits of the Child IAB-DU Resource set IDi field, with the two leftmost bits being set to zero). If the value of the field is 01, SSB index (contained in the 6 rightmost bits of the Child IAB-DU Resource set IDi field) and STC index (contained in the two leftmost bits of the Child IAB-DU Resource set IDi field) are both used. If the value of the field is set to 11, CSI-RS index, which is identified by NZP-CSI-RS-ResourceId as specified in TS 38.331 [5], is used (comprising all 8 bits of the Child IAB-DU Resource set IDi field). The length of the field is 2 bits;

-	Ci1: This field indicates whether the octet containing the Number of cell configurations IDi field is included or not. C01 refers to the first restricted beam within the resource set, C11 to the second one and so on. The field is set to 0 to indicate the octet is not included (and subsequently, that no cell information is included for the relevant restricted beam). The field is set to 1 to indicate that Number of cell configurations IDi field is included. The length of the field is 1 bits;

-	Ci2: This field indicates whether the child IAB-DU beam indicated with the Child IAB-DU Resource set IDi field is associated with one or more uplink or downlink IAB-MT beams, indicated with the IAB-MT Resource set IDij fields. C02 refers to the first restricted beam within the resource set, C12 to the second one and so on. The field is set to 1 to indicate that at least one IAB-MT Resource set IDij field is included (and that Number of associated IAB-MT beams IDi is included), and it is set to 0 to indicate that no IAB-MT Resource set IDij field is present (and that the Number of associated IAB-MT beams IDi is absent). The length of the field is 1 bit;

-	R: Reserved bit, set to 0;

-	Child IAB-DU Resource set IDi: an indication of the child IAB-DU's beam in the direction of which simultaneous operation is restricted. The length of the field is 8 bits;

-	Number of cell configurations IDi: This field indicates the number of cell configurations associated with the beam restriction indicated in Child IAB-DU Resource set IDi. Number of cell configurations ID0 refers to the first restricted beam within the resource set, Number of cell configurations ID1 to the second one and so on. The length of the field is 8 bits;

-	Cell info type IDij: indicates the type of information included in the Cell info IDij field. If the value of the field is set to zero, the combination of {MT CC, DU cell} is included in the Cell info IDij field, by including the IAB-DU cell in the first nine bits of the field, and the IAB-MT Serving Cell index into the next 5 bits of the field,. If the value of the field is set to one, only DU cell is included in the Cell info IDij field, by including the IAB-DU cell in the first nine bits of the field, and by setting the remaining five bits to zero. The length of the field is 1 bit;

-	Mij: This field indicates whether the octet containing multiplexing mode info (containing the Multiplexing mode info IDij field) is included or not. Mi0 refers to the first combination of {MT CC, DU cell} associated with Child IAB-DU Resource set IDi, Mi1 to the second one and so on. The field is set to 1 to indicate that multiplexing mode info is included, and it is set to 0 to indicate that multiplexing mode info is not present. The length of the field is 1 bit;

-	Cell info IDij: indicates the cell configuration associated with beam restriction indicated in Child IAB-DU Resource set IDi. Cell info IDi0 field refers to the first cell configuration associated with the beam restriction indicated in Child IAB-DU Resource set IDi, Cell info IDi1 field refers to the second cell configuration associated with the beam restriction indicated in Child IAB-DU Resource set IDi, and so on. The length of the field is 15 bits;

-	Multiplexing mode info IDij: The two rightmost bits indicate which of the four multiplexing modes defined in TS 38.473 [27] is applicable. The third rightmost bit of this field indicates whether multiplexing restrictions mode information contained in the two rightmost bits of the field are applicable to non-overlapping frequency resources. This bit is set to 1 when multiplexing mode information contained in the two rightmost bits of the Multiplexing mode info IDij field is applicable to non-overlapping frequency resources. This field is set to 0 when multiplexing mode information contained in the two rightmost bits of the Multiplexing mode info IDij field is not applicable to non-overlapping frequency resources. The remaining 5 bits of this field are set to zero. The length of the field is 8 bits;

-	Number of associated IAB-MT beams IDi: This field indicates the number of IAB-MT beams associated with the beam indicated in Child IAB-DU Resource set IDi. Number of associated IAB-MT beams ID0 refers to the first restricted beam within the resource set, Number of associated IAB-MT beams ID1 to the second one and so on. The length of the field is 8 bits;

-	Associated IAB-MT's beams type IDij: This field determines the type of information used as an indication of an uplink or downlink IAB-MT beam associated with the child IAB-DU's restricted beams, indicated with the IAB-MT Resource set IDij. If the value of the field is 00, the information included in the IAB-MT Resource set IDij field is the TCI state ID (contained in the 7 rightmost bits of the field). If the value of the field is 01, the information included in the IAB-MT Resource set IDij field is the SSB ID (contained in the 6 rightmost bits of the field). If the value of the field is 10, the information included in the IAB-MT Resource set IDij field is the CSI-RS index (comprising all 8 bits), which is identified by NZP-CSI-RS-ResourceId as specified in TS 38.331 [5]. If the value of the field is 11, the information included in the IAB-MT Resource set IDij field is the SRI (contained in the 4 rightmost bits of the field). The length of the field is 2 bits;

-	IAB-MT Resource set IDij: an indication of the IAB-MTs downlink or uplink beams which are associated with the child IAB-DU restricted beam indicated in Child IAB-DU Resource set IDi field. IAB-MT Resource set IDi0 field refers to the first IAB-MT's beam associated with the child IAB-DU's restricted beam indicated in Child IAB-DU Resource set IDi field, IAB-MT Resource set IDi1 field refers to the second IAB-MT's beam associated with the child IAB-DU's restricted beam indicated in Child IAB-DU Resource set IDi field, and so on. The length of the field is 8 bits.

Figure 6.1.3.61-1: Child IAB-DU Restricted Beam Indication MAC CE

#### 6.1.3.62	IAB-MT Recommended Beam Indication MAC CE

The IAB-MT Recommended Beam Indication MAC CE is identified by MAC subheader with eLCID as specified in Table 6.2.1-2b. It has a variable size with following fields (Figure 6.1.3.62-1):

-	Resource Configuration ID: This field indicates the RRC configuration which applies to this MAC CE and corresponds to iab-ResourceConfigID parameter in IAB-ResourceConfig as specified in TS 38.331 [5]. The length of the field is 16 bits;

-	Number of IAB-MT beams ID: This field indicates the number of recommended IAB-MT beams included in the MAC CE. The length of the field is 8 bits;

-	IAB-MT beam type IDi: This field determines the type of information used as an indication of an uplink or downlink IAB-MT beam, indicated with the IAB-MT Resource set IDi. If the value of the field is 00, the information included in the IAB-MT Resource set IDi field is the TCI state ID (contained in the 7 rightmost bits of the field). If the value of the field is 01, the information included in the IAB-MT Resource set IDi field is the SSB ID (contained in the 6 rightmost bits of the field). If the value of the field is 10, the information included in the IAB-MT Resource set IDi field is the CSI-RS index (comprising all 8 bits), which is identified by NZP-CSI-RS-ResourceId as specified in TS 38.331 [5]. If the value of the field is 11, the information included in the IAB-MT Resource set IDi field is the SRI (contained in the 4 rightmost bits of the field). The length of the field is 2 bits;

-	IAB-MT Resource set IDi: an indication of the IAB-MTs recommended downlink or uplink beams. IAB-MT Resource set ID0 field refers to the first IAB-MT's beam, IAB-MT Resource set ID1 field refers to the second IAB-MT's beam, and so on. The length of the field is 8 bits;

-	Ci1: This field indicates whether the octet containing the Number of cell configurations IDi field is included or not. C01 refers to the first recommended beam within the resource set, C11 to the second one and so on. The field is set to 0 to indicate the octet is not included (and subsequently, that no cell information is included for the relevant recommended beam). The field is set to 1 to indicate that Number of cell configurations IDi field is included. The length of the field is 1 bits;

-	R: Reserved bit, set to 0;

-	Number of cell configurations IDi: This field indicates the number of cell configurations associated with the beam indicated in IAB-MT Resource set IDi. Number of cell configurations ID0 refers to the first recommended beam within the resource set, Number of cell configurations ID1 to the second one and so on. The length of the field is 8 bits;

-	Cell info type IDij: indicates the type of information included in the Cell info IDij field. If the value of the field is set to zero, the combination of {MT CC, DU cell} is included in the Cell info IDij field, by including the IAB-DU cell in the first nine bits of the field, and the IAB-MT Serving Cell index into the next 5 bits of the field. If the value of the field is set to one, only MT CC information is included in the Cell info IDij field, by including the IAB-MT Serving Cell index in the first 5 bits of the field, and by setting the remaining nine bits to zero. The length of the field is 1 bit;

-	Mij: This field indicates whether the octet containing multiplexing mode info (containing the Multiplexing mode info IDij field) is included or not. Mi0 refers to the first combination of {MT CC, DU cell} associated with IAB-MT Resource set IDi, Mi1 to the second one and so on. The field is set to 1 to indicate that multiplexing mode info is included, and it is set to 0 to indicate that multiplexing mode info is not present. The length of the field is 1 bit;

-	Cell info IDij: indicates the cell configuration associated with beam recommendation indicated in IAB-MT Resource set IDi. Cell info IDi0 field refers to the first cell configuration associated with the beam recommendation indicated in IAB-MT Resource set IDi, Cell info IDi1 field refers to the second cell configuration associated with the beam recommendation indicated in IAB-MT Resource set IDi, and so on. The length of the field is 14 bits.

-	Multiplexing mode info IDij: The two rightmost bits indicate which of the four multiplexing modes defined in TS 38.473 [27] is applicable. The third rightmost bit of this field indicates whether multiplexing restrictions mode information contained in the two rightmost bits of the field are applicable to non-overlapping frequency resources. This bit is set to 1 when multiplexing mode information contained in the two rightmost bits of the Multiplexing mode info IDij field is applicable to non-overlapping frequency resources. This field is set to 0 when multiplexing mode information contained in the two rightmost bits of the Multiplexing mode info IDij field is not applicable to non-overlapping frequency resources. The remaining 5 bits of this field are set to zero. The length of the field is 8 bits.

Figure 6.1.3.62-1: IAB-MT Recommended Beam Indication MAC CE

END OF CHANGES