3GPP TSG-RAN WG2 Meeting #122	R2-2306020

Incheon, South Korea, 22nd – 26th May 2023

Agenda Item:	7.2.2

Source:	Ericsson

Title:	Sidelink positioning

Document for:	Discussion and Decision

# 1	Introduction

The work items for SL in RAN2 include

  - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2].

  - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]:

    - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)).

    - Specify the protocol and procedures for SL positioning between UEs and LMF.

In last meeting, RAN2 progressed with below agreements;

Agreement:

LS on group positioning is postponed for a reply from next meeting.

WA: RAN2 understand that group positioning is to acquire location estimates of multiple target UEs (absolute positioning) or multiple UE pairs (Ranging/relative positioning) per LCS request, in line with the guidance already received from SA2.

WA: At least part of the group management for group positioning is performed at upper/application layer.

Agreement:

Sidelink Positioning Architecture in Figure 1-4 on R2-2304301 is taken as baseline in TS 38.305 for further discussion.

RAN2 understanding is that there is no impact to LTE specs from this objective.

Agreement:

Anchor UE and target UE roles can be shown in the sidelink positioning procedures in stage 2.  Server UE can be further discussed at least for the case that the server UE is separate from the target and anchor.

Agreement:

Discovery procedure is included in the sidelink positioning procedure at least for out of coverage scenario.

Agreement:

Anchor UE selection can be included in the sidelink positioning procedures at least for out of coverage scenario.

Agreements:

RAN2 confirms that discovery messages will be used to carry information for targeted discovery and candidate selection of SL positioning UEs, including at least the indication of anchor UE, target UE. and server UE roles.  FFS how much information is indicated about anchor UEs (e.g., knowledge of location).

The UE role information is indicated in the discovery SLPP metafield.  FFS if this applies to both discovery modes and which messages.

Agreements:

R2 agree that for session-based SLPP, a SLPP session is used among UEs in PC5-only case in order to obtain location related measurements/location estimates, to transfer assistance data, or to exchange of capabilities.

RAN2 agree that for session-based SLPP, a single SLPP session is created to support a single location request at least in case of a single target UE; FFS how sessions work if there are multiple target UEs in a single location request.

TP in R2-2304005 is postponed.

RAN2 agree that, for session-based SLPP, SLPP transactions are indicated at the SLPP protocol level with a transaction ID in order to associate messages with one another (e.g., request and response)”

RAN2 agree that for session-based SLPP, messages within a transaction are linked by a common transaction identifier.

In this paper, we discuss the 1) SL positioning architecture 2) SLPP procedures for OoC 3) location server in IC 4)SL positioning parameter for discovery 5) multiple target UE positioning 6) potential signalling procedures for LPP extension to Hybrid Positioning (Joint PC5-Uu scenario) 7) hybrid positioning measurement report 8) Partial Coverage 9) LMF to UE protocol Discussion 10) SL ranging measurement when sync source changes 11) Overhearing

# 2	Discussion

## 2.1 SL positioning architecture

Figure 1: Figure 1-4 on R2-2304301

In the previous meeting, the Figure 1-4 from R2-2304301 was agreed to be taken as baseline for further discussion. There are two connections marked as FFS. We consider the connection marked as “FFS LTE-Uu” not needed, as the scope of SL positioning is NR. We do not see that even the LTE transport is in the scope. The other FFS link should also be removed as there is no strong motivation to keep it. Since, there is already NR PC5 connection between UE A and UE C, and between UE A and UE B. Hence, there is further no need to keep between UE C and UE D.

Figure 2 UE Positioning Overall Architecture applicable to NG-RAN

RAN2 to consider modification of Figure 2 on top of the baseline architecture figure.

## 2.2 SLPP procedures in OoC

Figure 3 SLPP procedure in OoC. UE1 and UE2 can be target UE, anchor UE, or SL positioning UE server. Target UE or anchor UE can act as SL Positioning Server UE, and the procedures involving SL positioning UE server are marked in dash lines.

- Target UE discovers anchor UE or SL positioning UE server.

- SL positioning UE server request capabilities from target UE and/or anchor UE, and target UE and/or anchor UE send capability to SL positioning UE server.

- Anchor UE(s) is selected.

Note: it is FFS whether SL positioning UE server and/or target UE decide the anchor UE

- SL positioning UE server and target UE and/or anchor UE exchange assistance data including the positioning method to use and the SL-PRS config to measure.

- Target UE and/or anchor UE performs the Ranging/SL Positioning procedures.

- Target UE and/or anchor UE sends the measurement/positioning information to SL positioning UE server.

- SL positioning UE server calculates the Ranging/SL positioning results based on the received ranging/SL positioning measurements and location information received from target UE and/or anchor UE sends.

Take the procedure in Figure 3 as baseline SLPP procedures.

## 2.3 In-Coverage (IC) Scenario

In IC scenario, target UE an anchor UE(s) are all in network coverage. LMF establishes SLPP session with target UE via SLPP or extended LPP for capability exchange, AD distribution, obtaining measurement, and ranging/SL positioning calculation, LMF can also establishes SLPP session with anchor UE that are PRU to perform these procedures. For non-PRU anchor UEs, target UE can forward their messages to LMF and the message from LMF to them. Thus in IC, there is no need to include SL positioning UE server.

SL positioning UE server is not involved for in-coverage SL positioning, and no LMF to SL positioning UE server interaction to be supported in Rel-18

## 2.4 Sidelink positioning parameters in discovery signalling

In the last meeting, RAN2 has agreed that discovery messages will be used to carry information for targeted discovery and candidate selection of SL positioning UEs, including at least the indication of anchor UE, target UE and server UE roles. Besides the agreed UE roles, for Hybrid Positioning (Uu+SL) we see the need that Anchor/Reference UE broadcast whether it is PRU capable or not. In this way, the Target UE should be able to identify a PRU UE during discovery procedure and inform this to LMF to initiate hybrid Positioning (Uu+SL). This discovery procedure can be initiated also upon LMF request.

Anchor/Reference UE indicate whether it is PRU capable or not in the SL positioning discovery message.

Besides indicating UE roles in the SL positioning discovery message, anchor UE indicating its supported QoS and target UE indicating the required QoS is beneficial to save latency and resources. Moreover, indicating the SL positioning applies in both discovery mode.

Target UE may include QoS requirement for SL positioning in discovery message, and anchor UE may include supported QoS for SL positioning in discovery message.

Sidelink positioning parameters can be indicated in both discovery modes.

## 2.5 Multiple target UEs positioning

From last meeting, RAN2 has come to the working assumption is group positioning is to acquire location estimates of multiple target UEs (absolute positioning) or multiple UE pairs (Ranging/relative positioning) per LCS request. The next questions would be how AMF deals with the LCS request. If AMF forward the LCS request transparently to LMF or AMF can divide the LCS request to multiple requests for each target UE to LMF, and the latter has no impact for LMF or LPP session. For either way, the multiple UE IDs in the LCS request should be provided to AMF and LMF.

For multiple target UEs positioning, the multiple target UE IDs in LCS request should be provided to AMF and LMF.

## 2.6 Hybrid Positioning with LPP Extension

There are different use cases which require LMF to perform hybrid positioning such as request from external client to be able to provide the range between two UEs or improve the UE positioning accuracy with assistance from another UE such as reference UE/PRU. For ranging determination, to improve the reliability and accuracy of range estimation; LMF can perform hybrid positioning such as to obtain the absolute UE location and also range estimation between UEs.

For SL ranging/positioning, signalling procedures including capability exchange, reference/anchor UE selection, assistance data transfer, ranging/sidelink positioning measurement, report and calculation are needed. For IC scenario, LMF can collect and provide the above information to calculate the requested ranging/positioning results as in LPP. Such signalling procedures are similar to the LPP procedures, it is natural to extend LPP with signalling procedures for SL ranging/positioning.

LPP session should be extended such that multiple UEs can join the positioning session with LMF and allow LMF to perform hybrid Positioning. LPP with signalling procedures for SL ranging/positioning which is used at Uu interface between LMF and UE should also support SL UE to UE operations.

There can be below two Options on how LPP session can be extended to accommodate SL operations.

Option1: A common LPP session to have multiple target devices. Transaction ID range to different target devices are set to different range in order not to collide.

Option2: Multiple LPP sessions each with one individual device are used to support a single ranging/SL positioning request. The association between SL session and Uu session has to be maintained by LMF.

Option 2 has less impact to the legacy LPP.

Figure 4 Example extended LPP session procedure with Option 1.

Step 1. A Uu LPP positioning request with session ID X is either sent from LMF to target UE.

Step 2. Target UE reports Uu measurement to LMF with session ID X.

Step 3. Based on the Uu measurement from target UE in step 2, LMF decide if SL measurement is needed. If not needed, LMF calculates target UE’s position. If needed, continue to below steps.

Step 4. LMF sends request to target UE to initiate SL procedure & provide AD(potential anchor UEs) with session ID X and Transaction ID k1 in range T1

Step 5. Target UE discover anchor UEs/PRUs.

Step 6. Target UE report discovered anchor UE(s) ID(s) to LMF using Uu LPP with session ID X and Transaction ID k1 in range T1.

Step 7. Along with AMF, LMF resolve SUPI of the anchor UEs from their App/L2 ID

Step 8. LMF sets up Uu LPP Positioning Sessions and provide SL AD with anchor UEs with Session ID X and Transaction IDs in ranges T2, T3 respectively.

Step 9. LMF requests the target UE to perform Uu and/or SL measurement with session ID X and Transaction ID k2 in range T1.

Step 10. Target UE responds to LMF with Uu and/or SL measurement with session ID X and Transaction ID k2 in range T1

Step 11. LMF requests the anchor UEs to perform Uu and/or SL measurement with session ID X and Transaction ID m in range T2 and Transaction ID n in range T3

Step 12. Anchor UEs responds to LMF with Uu and/or SL measurement with session ID X and Transaction ID m in range T2 and Transaction ID n in range T3

Step 13. LMF computes target UE Location using Uu & SL Measurements with Session ID X

Figure 5 Example extended LPP session procedure with Option 2.

Step 1. A Uu LPP positioning request with session ID X is either sent from LMF to target UE.

Step 2. Target UE reports Uu measurement to LMF with session ID X.

Step 3. Based on the Uu measurement from target UE in step 2, LMF decide if SL measurement is needed. If not needed, LMF calculates target UE’s position. If needed, continue to below steps.

Step 4. LMF sends request to target UE to initiate SL procedure & provide AD(potential anchor UEs)

Step 5. Target UE discover anchor UEs.

Step 6. Target UE report discovered anchor UE(s) ID(s) to LMF using Uu LPP.

Step 7. Along with AMF, LMF resolve SUPI of the anchor UEs from their App/L2 ID

Step 8. LMF sets up Uu LPP Positioning Session with session IDs Y, Z with anchor UEs, Provide SL AD with Session IDs Y, Z.

Step 9. LMF requests the target UE to perform Uu and/or SL measurement with session ID X and transaction ID k.

Step 10. Target UE responds to LMF with Uu and/or SL measurement with session ID X and transaction ID k

Step 11. LMF requests the anchor UEs to perform Uu and/or SL measurement with session IDs Y, Z and transaction ID k

Step 12. Anchor UEs responds to LMF with Uu and/or SL measurement with session IDs Y, Z and transaction ID k

Step 13. LMF computes target UE Location using Uu & SL Measurements with Session IDs X, Y, Z and transaction ID k

In above examples, apart from Uu operation, there has to be sidelink operations between UEs and also one of the UEs can be a reference UE/PRU. We see the steps needed are:

- LMF should be able to establish LPP session with UEs and configure SLPP configuration between UEs (considering they are in same cell or area); Or

- A target UE should be able to convey to LMF that it has discovered a PRU,

- The LMF should be able to resolve SL L2/Application ID of PRU to SUPI and establish LPP session for Uu measurements

- The LMF should be able to setup Uu positioning with target UE and PRU,

- The LMF should be able to provide SL AD or request to gNB to allocate resource and provide configuration for SL-PRS configuration

- The LMF should be able to obtain both SL measurements and Uu measurements from multiple UEs.

LPP session between UE and NW is extended to support UE to UE SL Operations and to allow LMF to execute hybrid Positioning procedures by obtaining Uu and SL measurements

The target UE shall discover a reference UE/PRU and report to LMF and LMF setups LPP positioning session with the reference UE/PRU and SL session between reference and target UE.

LMF sets up LPP positioning session with multiple UEs and SL session among multiple UEs and obtains both Uu and SL measurements. Downselect from option 1 “same LPP session with multiple UEs but using different transaction ID ranges” and option 2 “multiple sessions with each UE but using same transaction ID”.

Send LS to SA2 on how PRU SL ID can be resolved to SUPI so that LMF can initiate positioning towards such UE.

To implement SL ranging/positioning, discovery procedure is needed to establish PC5 path, thus LMF should be able to request ProSe Capability from UE. For SL ranging/positioning, the UE capability to transmit SL-PRS, measure and calculate for each detailed SL ranging/positioning method such as RTT, SL-TDOA should also be defined and supported by LPP.

- LPP should support to request and provide 5G ProSe Capability.

- The PC5 Capability for Ranging/SL positioning should be defined, and LPP should support to request and provide PC5 Capability for Ranging/SL positioning.

## 2.7 Hybrid positioning measurement alignment in time domain

Hybrid positioning collects both Uu and SL positioning measurement to determine and/or improve UE’s positioning/ranging result. However, Uu and SL measurement periods cannot always be aligned. Furthermore, the availability of the SL PRS for the SL positioning measurements depends on the SL resource pool configuration e.g. slots which are configured for the SL. Therefore, there is no guarantee that the Uu and SL positioning measurements can always be completed by the UE during the same time or even close in time. They will most likely be misaligned. When the target UE moves, misalignment between Uu and SL positioning measurements will cause degradation to the positioning accuracy, or even fail to compute positioning results with hybrid measurements.

When the target UE has non-zero velocity, misalignment between Uu and SL positioning measurements will cause degradation to the positioning accuracy, or even fail to compute positioning results with hybrid measurements.

To better align the Uu and SL measurement in the time domain, NW may instruct UE by sending assistance data to UE of the Uu and SL reference signals that are close in time domain and/or request UE to only measure Uu and SL reference signals within a certain time interval.

NW provides configuration to UE for the Uu and SL reference signals that are close in time domain and request UE to measure Uu and SL reference signals within a certain time interval.

## 2.8 Partial Coverage

Partial coverage is a way to provide the out of coverage UE (remote UE) cellular coverage by means of a relay UE (U2N relay). It is possible that anchor UE, reference UE etc. can also act as relay UE. As provided in the below Figure, one option is to use Layer 2 U2N relay where remote UE can be reached by the LMF using LPP.

Figure 6 LPP Protocol Stack with L2 NW-to-UE relay for PC

Partial coverage is supported by means of L2 U2N Relay where the LPP protocol terminates/originates at remote UE and LMF.

## 2.9 LMF to UE protocol support for Sidelink

The TR 38.859 contains the below Options for protocol support for Sidelink between LMF and UE

-	Extension of LPP, whereby new signaling is to be defined to support hybrid Uu and PC5 based positioning, i.e., extend the existing LPP to support sidelink based positioning between UE and LMF

-	Enhancement of LPP whereby SLPP signaling can be transported within LPP transparently, i.e., use the newly defined SLPP to support sidelink based positioning and use the existing LPP to support Uu based positioning; and the SLPP is carried as a container in LPP

-	Use of SLPP between the UE and the LMF

Our view is that we should follow the approach of 1st Option; i.e Extension of LPP to develop the ASN.1 for now. However, at a later stage depending upon how ASN.1 appears a decision can be made to use container or direct SLPP. Since it is easier to extend the LPP and then considering this baseline evaluate different options. The example IE of LPP extension can be found in Subsection Appendix.

The baseline for developing ASN.1 to support SL operations between LMF and UE is by means of extension of LPP. The decision whether to use container solution or direct SLPP between UE and LMF is taken after evaluating the baseline extension.

## 2.10 SL Ranging measurements when Synch Source or Coverage changes

For SL positioning/ranging measurements, accurate synchronisation between UEs is essential. The sidelink synchronization reference source can change for different reasons, e.g. due to UE mobility, channel conditions etc., while UE is performing SL positioning measurements which may lead inaccurate measurements. How would UE cope when the synch source during positioning measurement changes? Additionally, another question would be: UEs may be (pre)configured resources to perform SL ranging measurements in different coverage scenario (IC, PC, OOC). A group of UEs (at least 2 UEs; e.g. car platooning) may perform ranging continuously; however in the coverage crossing area (IC to OOC or vice versa) where UEs may have performed the measurement in one coverage but now happen to be in different coverage; would the ongoing measurements be valid? The impact on change of coverage and synchronization reference source while performing the SL positioning measurements needs to be investigated.

The Synchronization and measurement validity are RAN4 related question which should be provided by RAN4. Depending upon the outcome of the results, the SL procedure would have impact, such as whether to continue the SL procedure or restart the procedure.

Send LS to RAN4 requesting how would the ongoing SL measurements be impacted when the synch source changes and/or coverage status changes.

## 2.11 Overhearing

As part of the Rel-18 work item on sidelink (SL) positioning, a new SL positioning reference signal (SL-PRS) will be specified to be used for measurements between UEs. PRU is UE with known location. It would benefit if PRU is also involved in the ranging/SL positioning process to provide extra measurement by overhearing the SL-PRS by the target UE(s). For example, it can improve the accuracy if the target UE performing ranging are in NLOS condition, and can validate the positioning result by the overhearing measurement as PRU’s location is known.

Figure 7 and Figure 8 show the mechanism of PRU overhear SL-PRS requested by LMF and target UE respectively.  From Figure 7 when the PRU is requested by LMF to overhear, it causes no extra overhead to the target UEs.

UE1

UE2

PRU(s)

LMF

3. LPP: Request to PRU            to listen/ overhear UE(s) within coverage

4. LPP: Provide an ACK/ NACK

7. LPP: Report the result

6. SL-PRS measurements

1. UEs reserve SL-PRS

2. LMF request and UEs in coverage provide SL-PRS configuration

5. LPP: Forward the SL-PRS configurations to overhear

Figure 7 LMF requests PRU to overhear SL-PRS

Step 1: UEs performing SL positioning/ranging reserve SL-PRS by either scheme.

Step 2: LMF requests for UEs that are in coverage to send their SL-PRS configuration, and UEs provide LMF the SL-PRS configuration.

Step 3: LMF request PRU to measure SL-PRS transmitted by the UEs via LPP.

Step 4: PRU reply to the request of LMF via LPP.

Step 5: LMF forward the SL-PRS configuration to PRU.

Step 6: PRU measures SL-PRS.

Step 7: PRU send the measurement results to LMF.

UE1

UE2

Listening UEs

LMF

9a. LPP: Report the result

8. SL-PRS measurements

1. UEs reserve SL-PRS

2. LMF sends AD about potential Listening UEs (PRUs)

3. PC5: UEs discover listening UEs (PRU(s))

5. PC5: Provide an ACK/ NACK

4. PC5: Request to UE to listen/overhear SL-PRS and send SL-PRS configurations

7. Establish LPP session

6. LPP: Forward the ID(s) of PRU(s)

9b. PC5: Report the result

10. LPP: Forward the measurements from PRU

Figure 8 target UE requests PRU to overhear SL-PRS.

Step 1: UEs performing SL positioning/ranging reserve SL-PRS by either scheme.

Step 2: Optionally, LMF may send potential PRUs to the target UEs.

Step 3: Target UEs discover the PRUs if they have not done it from previous SL communication or SL positioning.

Step 4: Target UEs request PRU to measure/overhear SL-PRS and send the SL-PRS configuration via PC5.

Step 5: UE/PRU reply to the request from the target UE.

If the ranging/SL positioning result is to be collected by LMF then

Step 6: Target UE forward the L2 ID or Application ID to LMF.

Step 7: LMF retrieve the SUPI of PRU with the help of AMF. LMF establish a LPP session with PRU.

Step 8: UE/PRU measures SL-PRS.

Step 9: UE/PRU send the measurement results to LMF and/or target UE based on the request received from LMF or target UE.

Step 10. If 9b is performed, target UE may forward the measurements to LMF and the information about PRU including PRU’s location. In this case, Step 6 and 7 are not required.

UE overhearing SL-PRS benefit the ranging/SL positioning accuracy causing no or little overhead to the UEs.

UE overhearing SL-PRS is supported.

# 3	Conclusion

Observation 1	LPP session should be extended such that multiple UEs can join the positioning session with LMF and allow LMF to perform hybrid Positioning. LPP with signalling procedures for SL ranging/positioning which is used at Uu interface between LMF and UE should also support SL UE to UE operations.

Observation 2	When the target UE has non-zero velocity, misalignment between Uu and SL positioning measurements will cause degradation to the positioning accuracy, or even fail to compute positioning results with hybrid measurements.

Observation 3	UE overhearing SL-PRS benefit the ranging/SL positioning accuracy causing no or little overhead to the UEs.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to consider modification of Figure 2 on top of the baseline architecture figure.

Proposal 2	Take the procedure in Figure 3 as baseline SLPP procedures.

Proposal 3	SL positioning UE server is not involved for in-coverage SL positioning, and no LMF to SL positioning UE server interaction to be supported in Rel-18

Proposal 4	Anchor/Reference UE indicate whether it is PRU capable or not in the SL positioning discovery message.

Proposal 5	Target UE may include QoS requirement for SL positioning in discovery message, and anchor UE may include supported QoS for SL positioning in discovery message.

Proposal 6	Sidelink positioning parameters can be indicated in both discovery modes.

Proposal 7	For multiple target UEs positioning, the multiple target UE IDs in LCS request should be provided to AMF and LMF.

Proposal 8	LPP session between UE and NW is extended to support UE to UE SL Operations and to allow LMF to execute hybrid Positioning procedures by obtaining Uu and SL measurements

Proposal 9	The target UE shall discover a reference UE/PRU and report to LMF and LMF setups LPP positioning session with the reference UE/PRU and SL session between reference and target UE.

Proposal 10	LMF sets up LPP positioning session with multiple UEs and SL session among multiple UEs and obtains both Uu and SL measurements. Downselect from option 1 “same LPP session with multiple UEs but using different transaction ID ranges” and option 2 “multiple sessions with each UE but using same transaction ID”.

Proposal 11	Send LS to SA2 on how PRU SL ID can be resolved to SUPI so that LMF can initiate positioning towards such UE.

Proposal 14	NW provides configuration to UE for the Uu and SL reference signals that are close in time domain and request UE to measure Uu and SL reference signals within a certain time interval.

Proposal 15	Partial coverage is supported by means of L2 U2N Relay where the LPP protocol terminates/originates at remote UE and LMF.

Proposal 16	The baseline for developing ASN.1 to support SL operations between LMF and UE is by means of extension of LPP. The decision whether to use container solution or direct SLPP between UE and LMF is taken after evaluating the baseline extension.

Proposal 17	Send LS to RAN4 requesting how would the ongoing SL measurements be impacted when the synch source changes and/or coverage status changes.

Proposal 18	UE overhearing SL-PRS is supported.

# Appendix Example LPP Extension for SL

### –	RequestCapabilities

The RequestCapabilities message body in a LPP message is used by the location server to request the target device capability information for LPP and the supported individual positioning methods.

-- ASN1START

RequestCapabilities ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

requestCapabilities-r9		RequestCapabilities-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

RequestCapabilities-r9-IEs ::= SEQUENCE {

commonIEsRequestCapabilities		CommonIEsRequestCapabilities		OPTIONAL,	-- Need ON

a-gnss-RequestCapabilities			A-GNSS-RequestCapabilities			OPTIONAL,	-- Need ON

otdoa-RequestCapabilities			OTDOA-RequestCapabilities			OPTIONAL,	-- Need ON

ecid-RequestCapabilities			ECID-RequestCapabilities			OPTIONAL,	-- Need ON

epdu-RequestCapabilities			EPDU-Sequence						OPTIONAL,	-- Need ON

...,

[[	sensor-RequestCapabilities-r13	Sensor-RequestCapabilities-r13		OPTIONAL,	-- Need ON

tbs-RequestCapabilities-r13		TBS-RequestCapabilities-r13			OPTIONAL,	-- Need ON

wlan-RequestCapabilities-r13	WLAN-RequestCapabilities-r13		OPTIONAL,	-- Need ON

bt-RequestCapabilities-r13		BT-RequestCapabilities-r13			OPTIONAL	-- Need ON

]],

[[	nr-ECID-RequestCapabilities-r16	NR-ECID-RequestCapabilities-r16		OPTIONAL,	-- Need ON

nr-Multi-RTT-RequestCapabilities-r16

NR-Multi-RTT-RequestCapabilities-r16

OPTIONAL,	-- Need ON

nr-DL-AoD-RequestCapabilities-r16

NR-DL-AoD-RequestCapabilities-r16	OPTIONAL,	-- Need ON

nr-DL-TDOA-RequestCapabilities-r16

NR-DL-TDOA-RequestCapabilities-r16	OPTIONAL,	-- Need ON

nr-UL-RequestCapabilities-r16	NR-UL-RequestCapabilities-r16		OPTIONAL	-- Need ON

]],

[[	sl-RTT-RequestCapabilities-r18	SL-RTT-RequestCapabilities-r18		OPTIONAL	-- Need ON

sl-TDOA-RequestCapabilities-r18	SL-TDOA-RequestCapabilities-r18		OPTIONAL	-- Need ON

sl-AOA-RequestCapabilities-r18	SL-AOA-RequestCapabilities-r18		OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

#### –	SL-RTT-RequestCapabilities

The IE SL-RTT-RequestCapabilities is used by the location server to request the capability of the target device to support SL-RTT and to request SL-RTT Capabilities from a target device.

-- ASN1START

SL-RTT-RequestCapabilities-r18 ::= SEQUENCE {

...

}

-- ASN1STOP

#### –	SL-TDOA-RequestCapabilities

The IE SL-TDOA-RequestCapabilities is used by the location server to request the capability of the target device to support SL-TDOA and to request SL-TDOA Capabilities from a target device.

-- ASN1START

SL-TDOA-RequestCapabilities-r18 ::= SEQUENCE {

...

}

-- ASN1STOP

#### –	SL-AoA-RequestCapabilities

The IE SL-AoA-RequestCapabilities is used by the location server to request the capability of the target device to support SL-AoA and to request SL-AoA Capabilities from a target device.

-- ASN1START

SL-AoA-RequestCapabilities-r18 ::= SEQUENCE {

...

}

-- ASN1STOP

### –	ProvideCapabilities

The ProvideCapabilities message body in a LPP message indicates the LPP capabilities of the target device to the location server.

-- ASN1START

ProvideCapabilities ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

provideCapabilities-r9		ProvideCapabilities-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

ProvideCapabilities-r9-IEs ::= SEQUENCE {

commonIEsProvideCapabilities		CommonIEsProvideCapabilities			OPTIONAL,

a-gnss-ProvideCapabilities			A-GNSS-ProvideCapabilities				OPTIONAL,

otdoa-ProvideCapabilities			OTDOA-ProvideCapabilities				OPTIONAL,

ecid-ProvideCapabilities			ECID-ProvideCapabilities				OPTIONAL,

epdu-ProvideCapabilities			EPDU-Sequence							OPTIONAL,

...,

[[	sensor-ProvideCapabilities-r13	Sensor-ProvideCapabilities-r13			OPTIONAL,

tbs-ProvideCapabilities-r13		TBS-ProvideCapabilities-r13				OPTIONAL,

wlan-ProvideCapabilities-r13	WLAN-ProvideCapabilities-r13			OPTIONAL,

bt-ProvideCapabilities-r13		BT-ProvideCapabilities-r13				OPTIONAL

]],

[[	nr-ECID-ProvideCapabilities-r16	NR-ECID-ProvideCapabilities-r16			OPTIONAL,

nr-Multi-RTT-ProvideCapabilities-r16

NR-Multi-RTT-ProvideCapabilities-r16	OPTIONAL,

nr-DL-AoD-ProvideCapabilities-r16

NR-DL-AoD-ProvideCapabilities-r16		OPTIONAL,

nr-DL-TDOA-ProvideCapabilities-r16

NR-DL-TDOA-ProvideCapabilities-r16		OPTIONAL,

nr-UL-ProvideCapabilities-r16	NR-UL-ProvideCapabilities-r16			OPTIONAL

]],

[[	sl-RTT-ProvideCapabilities-r18	SL-RTT-ProvideCapabilities-r18		OPTIONAL,

sl-TDOA-ProvideCapabilities-r18	SL-TDOA-ProvideCapabilities-r18		OPTIONAL,

sl-AoA-ProvideCapabilities-r18	SL-AoA-ProvideCapabilities-r18		OPTIONAL

]]

}

-- ASN1STOP

#### –	SL-RTT-ProvideCapabilities

The IE SL-RTT-ProvideCapabilities is used by the target device to indicate its capability to support SL-RTT and to provide its SL-RTT for positioning capabilities to the location server.

-- ASN1START

SL-RTT-ProvideCapabilities-r18 ::= SEQUENCE {

…

}

-- ASN1STOP

#### –	SL-TDOA-ProvideCapabilities

The IE SL-TDOA-ProvideCapabilities is used by the target device to indicate its capability to support SL-TDOA and to provide its SL-TDOA for positioning capabilities to the location server.

-- ASN1START

SL-TDOA-ProvideCapabilities-r18 ::= SEQUENCE {

…

}

-- ASN1STOP

#### –	SL-AoA-ProvideCapabilities

The IE SL-AoA-ProvideCapabilities is used by the target device to indicate its capability to support SL-AoA and to provide its SL-AoA for positioning capabilities to the location server.

-- ASN1START

SL-AoA-ProvideCapabilities-r18 ::= SEQUENCE {

…

}

-- ASN1STOP

AD Request and Provide

### –	RequestAssistanceData

The RequestAssistanceData message body in a LPP message is used by the target device to request assistance data from the location server.

-- ASN1START

RequestAssistanceData ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

requestAssistanceData-r9	RequestAssistanceData-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

RequestAssistanceData-r9-IEs ::= SEQUENCE {

commonIEsRequestAssistanceData		CommonIEsRequestAssistanceData		OPTIONAL,

a-gnss-RequestAssistanceData		A-GNSS-RequestAssistanceData		OPTIONAL,

otdoa-RequestAssistanceData			OTDOA-RequestAssistanceData			OPTIONAL,

epdu-RequestAssistanceData			EPDU-Sequence						OPTIONAL,

...,

[[	sensor-RequestAssistanceData-r14

Sensor-RequestAssistanceData-r14	OPTIONAL,

tbs-RequestAssistanceData-r14	TBS-RequestAssistanceData-r14		OPTIONAL,

wlan-RequestAssistanceData-r14	WLAN-RequestAssistanceData-r14		OPTIONAL

]],

[[	nr-Multi-RTT-RequestAssistanceData-r16	NR-Multi-RTT-RequestAssistanceData-r16	OPTIONAL,

nr-DL-AoD-RequestAssistanceData-r16		NR-DL-AoD-RequestAssistanceData-r16		OPTIONAL,

nr-DL-TDOA-RequestAssistanceData-r16	NR-DL-TDOA-RequestAssistanceData-r16	OPTIONAL

]]，

[[	sl-RTT-RequestAssistanceData-r18		SL-RTT-RequestAssistanceData-r18		OPTIONAL,

sl-TDOA-RequestAssistanceData-r18		SL-TDOA-RequestAssistanceData-r18		OPTIONAL,

sl-AoA-RequestAssistanceData-r18		SL-AoA-RequestAssistanceData-r18		OPTIONAL,

]]

}

-- ASN1STOP

#### –	SL-RTT-RequestAssistanceData

The IE SL-RTT-RequestAssistanceData is used by the device to request assistance data from a location server.

-- ASN1START

SL-RTT-RequestAssistanceData-r18 ::= SEQUENCE {

…

}

-- ASN1STOP

#### –	SL-RTT-RequestAssistanceData

The IE SL-TDOA-RequestAssistanceData is used by the device to request assistance data from a location server.

-- ASN1START

SL-TDOA-RequestAssistanceData-r18 ::= SEQUENCE {

…

}

-- ASN1STOP

#### –	SL-AoA-RequestAssistanceData

The IE SL-AoA-RequestAssistanceData is used by the device to request assistance data from a location server.

-- ASN1START

SL-AoA-RequestAssistanceData-r18 ::= SEQUENCE {

…

}

-- ASN1STOP

### –	ProvideAssistanceData

The ProvideAssistanceData message body in a LPP message is used by the location server to provide assistance data to the target device either in response to a request from the target device or in an unsolicited manner.

-- ASN1START

ProvideAssistanceData ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

provideAssistanceData-r9	ProvideAssistanceData-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

ProvideAssistanceData-r9-IEs ::= SEQUENCE {

commonIEsProvideAssistanceData		CommonIEsProvideAssistanceData		OPTIONAL,	-- Need ON

a-gnss-ProvideAssistanceData		A-GNSS-ProvideAssistanceData		OPTIONAL,	-- Need ON

otdoa-ProvideAssistanceData			OTDOA-ProvideAssistanceData			OPTIONAL,	-- Need ON

epdu-Provide-Assistance-Data		EPDU-Sequence						OPTIONAL,	-- Need ON

...,

[[

sensor-ProvideAssistanceData-r14	Sensor-ProvideAssistanceData-r14	OPTIONAL,	-- Need ON

tbs-ProvideAssistanceData-r14		TBS-ProvideAssistanceData-r14		OPTIONAL,	-- Need ON

wlan-ProvideAssistanceData-r14		WLAN-ProvideAssistanceData-r14		OPTIONAL	-- Need ON

]],

[[	nr-Multi-RTT-ProvideAssistanceData-r16

NR-Multi-RTT-ProvideAssistanceData-r16

OPTIONAL,	-- Need ON

nr-DL-AoD-ProvideAssistanceData-r16

NR-DL-AoD-ProvideAssistanceData-r16	OPTIONAL,	-- Need ON

nr-DL-TDOA-ProvideAssistanceData-r16

NR-DL-TDOA-ProvideAssistanceData-r16

OPTIONAL	-- Need ON

]],

[[	sl-RTT-ProvideAssistanceData-r18

SL-RTT-ProvideAssistanceData-r18	OPTIONAL,	-- Need ON

sl-TDOA-ProvideAssistanceData-r18

SL-TDOA-ProvideAssistanceData-r18	OPTIONAL,	-- Need ON

sl-AoA-ProvideAssistanceData-r18

SL-AoA-ProvideAssistanceData-r18	OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

#### –	SL-RTT-ProvideAssistanceData

The IE SL-RTT-ProvideAssistanceData is used by the location server to provide assistance data to enable SL-RTT.

-- ASN1START

SL-RTT-ProvideAssistanceData-r18 ::= SEQUENCE {

…

}

-- ASN1STOP

#### –	SL-TDOA-ProvideAssistanceData

The IE SL-TDOA-ProvideAssistanceData is used by the location server to provide assistance data to enable SL-TDOA.

-- ASN1START

SL-TDOA-ProvideAssistanceData-r18 ::= SEQUENCE {

…

}

-- ASN1STOP

#### –	SL-AoA-ProvideAssistanceData

The IE SL-AoA-ProvideAssistanceData is used by the location server to provide assistance data to enable SL-AoA.

-- ASN1START

SL-AoA-ProvideAssistanceData-r18 ::= SEQUENCE {

…

}

-- ASN1STOP

Location Information Request and Provide

### –	RequestLocationInformation

The RequestLocationInformation message body in a LPP message is used by the location server to request positioning measurements or a position estimate from the target device.

-- ASN1START

RequestLocationInformation ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

requestLocationInformation-r9	RequestLocationInformation-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

RequestLocationInformation-r9-IEs ::= SEQUENCE {

commonIEsRequestLocationInformation

CommonIEsRequestLocationInformation	OPTIONAL,	-- Need ON

a-gnss-RequestLocationInformation	A-GNSS-RequestLocationInformation	OPTIONAL,	-- Need ON

otdoa-RequestLocationInformation	OTDOA-RequestLocationInformation	OPTIONAL,	-- Need ON

ecid-RequestLocationInformation		ECID-RequestLocationInformation		OPTIONAL,	-- Need ON

epdu-RequestLocationInformation		EPDU-Sequence						OPTIONAL,	-- Need ON

...,

[[

sensor-RequestLocationInformation-r13

Sensor-RequestLocationInformation-r13

OPTIONAL,	-- Need ON

tbs-RequestLocationInformation-r13	TBS-RequestLocationInformation-r13	OPTIONAL,	-- Need ON

wlan-RequestLocationInformation-r13	WLAN-RequestLocationInformation-r13	OPTIONAL,	-- Need ON

bt-RequestLocationInformation-r13	BT-RequestLocationInformation-r13	OPTIONAL	-- Need ON

]],

[[	nr-ECID-RequestLocationInformation-r16

NR-ECID-RequestLocationInformation-r16

OPTIONAL,	-- Need ON

nr-Multi-RTT-RequestLocationInformation-r16

NR-Multi-RTT-RequestLocationInformation-r16

OPTIONAL,	-- Need ON

nr-DL-AoD-RequestLocationInformation-r16

NR-DL-AoD-RequestLocationInformation-r16

OPTIONAL,	-- Need ON

nr-DL-TDOA-RequestLocationInformation-r16

NR-DL-TDOA-RequestLocationInformation-r16

OPTIONAL	-- Need ON

]],

[[	sl-RTT-RequestLocationInformation-r18

SL-RTT-RequestLocationInformation-r18

OPTIONAL,	-- Need ON

sl-TDOA-RequestLocationInformation-r18

SL-TDOA-RequestLocationInformation-r18

OPTIONAL,	-- Need ON

sl-AoA-RequestLocationInformation-r18

SL-AoA-RequestLocationInformation-r18

OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

#### –	SL-RTT-RequestLocationInformation

The IE SL-RTT-RequestLocationInformation is used by the location server to request SL-RTT location measurements from a device.

-- ASN1START

SL-RTT-RequestLocationInformation-r18 ::= SEQUENCE {

…

}

-- ASN1STOP

#### –	SL-TDOA-RequestLocationInformation

The IE SL-TDOA-RequestLocationInformation is used by the location server to request SL-TDOA location measurements from a device.

-- ASN1START

SL-TDOA-RequestLocationInformation-r18 ::= SEQUENCE {

…

}

-- ASN1STOP

#### –	SL-AoA-RequestLocationInformation

The IE SL-AoA-RequestLocationInformation is used by the location server to request SL-AoA location measurements from a device.

-- ASN1START

SL-AoA-RequestLocationInformation-r18 ::= SEQUENCE {

…

}

-- ASN1STOP

### –	ProvideLocationInformation

The ProvideLocationInformation message body in a LPP message is used by the target device to provide positioning measurements or position estimates to the location server.

-- ASN1START

ProvideLocationInformation ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

provideLocationInformation-r9	ProvideLocationInformation-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

ProvideLocationInformation-r9-IEs ::= SEQUENCE {

commonIEsProvideLocationInformation

CommonIEsProvideLocationInformation	OPTIONAL,

a-gnss-ProvideLocationInformation	A-GNSS-ProvideLocationInformation	OPTIONAL,

otdoa-ProvideLocationInformation	OTDOA-ProvideLocationInformation	OPTIONAL,

ecid-ProvideLocationInformation		ECID-ProvideLocationInformation		OPTIONAL,

epdu-ProvideLocationInformation		EPDU-Sequence						OPTIONAL,

...,

[[

sensor-ProvideLocationInformation-r13

Sensor-ProvideLocationInformation-r13

OPTIONAL,

tbs-ProvideLocationInformation-r13	TBS-ProvideLocationInformation-r13	OPTIONAL,

wlan-ProvideLocationInformation-r13	WLAN-ProvideLocationInformation-r13	OPTIONAL,

bt-ProvideLocationInformation-r13	BT-ProvideLocationInformation-r13	OPTIONAL

]],

[[	nr-ECID-ProvideLocationInformation-r16

NR-ECID-ProvideLocationInformation-r16		OPTIONAL,

nr-Multi-RTT-ProvideLocationInformation-r16

NR-Multi-RTT-ProvideLocationInformation-r16 OPTIONAL,

nr-DL-AoD-ProvideLocationInformation-r16

NR-DL-AoD-ProvideLocationInformation-r16	OPTIONAL,

nr-DL-TDOA-ProvideLocationInformation-r16

NR-DL-TDOA-ProvideLocationInformation-r16	OPTIONAL

]],

[[	sl-RTT-ProvideLocationInformation-r18

SL-RTT-ProvideLocationInformation-r18

OPTIONAL,	-- Need ON

sl-TDOA-ProvideLocationInformation-r18

SL-TDOA-ProvideLocationInformation-r18

OPTIONAL,	-- Need ON

sl-AoA-ProvideLocationInformation-r18

SL-AoA-ProvideLocationInformation-r18

OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

#### –	SL-RTT-ProvideLocationInformation

The IE SL-RTT-ProvideLocationInformation is used by the target device to provide SL-RTT location measurements to the location server. It may also be used to provide SL-RTT positioning specific error reason.

-- ASN1START

SL-RTT-ProvideLocationInformation-r18 ::= SEQUENCE {

…

}

-- ASN1STOP

#### –	SL-TDOA-ProvideLocationInformation

The IE SL-TDOA-ProvideLocationInformation is used by the target device to provide SL-TDOA location measurements to the location server. It may also be used to provide SL-TDOA positioning specific error reason.

-- ASN1START

SL-TDOA-ProvideLocationInformation-r18 ::= SEQUENCE {

…

}

-- ASN1STOP

#### –	SL-RTT-ProvideLocationInformation

The IE SL-AoA-ProvideLocationInformation is used by the target device to provide SL-AoA location measurements to the location server. It may also be used to provide SL-AoA positioning specific error reason.

-- ASN1START

SL-AoA-ProvideLocationInformation-r18 ::= SEQUENCE {

…

}

-- ASN1STOP

# 4	References

- [1] TR 23.700