3GPP TSG-RAN WG2 #122	Tdoc R2-2306036

Incheon Korea, May 2023

Agenda Item:	7.20.2

Source:	Ericsson

Title:	On 2TA operation

Document for:	Discussion

# 1	LS on Multi-DCI Multi-TRP with two TAs

RAN2 has received LS in R1-2213004 “LS on Multi-DCI Multi-TRP with two Tas” with the below content:

# 1.  Overall Description

In Rel-18 discussions on MIMO Evolution for Downlink and Uplink, RAN1 has agreed to support multi-DCI multi-TRP operation with two TAs.  The following has been agreed/concluded in RAN1:

Agreement

For multi-DCI based multi-TRP operation with two TAs, support configuring two TAGs belonging to a serving cell.

Conclusion

For multi-DCI based Multi-TRP operation with two TA enhancement, it is up to RAN2 to decide whether there is a need to enhance CBRA procedure to support per TRP UE-initiated RACH procedure.

2.  Actions

To RAN WG2

ACTION: RAN1 would like to kindly ask RAN2 to take the above agreements/conclusions into account in their future work related to two TAs for multi-DCI multi-TRP operation, and notify RAN1 of any RAN2 decisions on per TRP UE-initiated RACH procedure that require RAN1 involvement.

RAN2 has also received LS in R1-2302226 which contains RAN1 agreements so far from the different MIMO RAN1 agenda items, eUTCI, 2TA, CSI, DMRS, SRS, STxMP, 8Tx UL. In this paper we discuss the 2TA aspects.

In the second LS RAN1 informs e.g. on the following agreements related to 2TA:

Agreement

For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, support the following:

Associate TAG to TCI-state

- Associate TAG ID with UL/joint TCI state

- For UL transmission, the TAG ID associated with the UL/joint TCI state is utilized

- A baseline is UE expects that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index correspond to one TAG

- Working Assumption: A UE may report that it supports that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESETPoolIndex correspond to both TAGs

FFS: on how to handle association when Rel-15/16 spatial relation framework is used for

- PUCCH

- DG/CG Type 1/Type 2 PUSCH

- AP/SP/P SRS

Agreement

Confirm the following working assumption:

For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, one additional PRACH configuration is supported for each configured additional PCI

- the additional PRACH configuration is used in a RACH procedure triggered by a PDCCH order for the corresponding configured additional PCI

Agreement

For multi-DCI multi-TRP operation with two TAs in a CC, two DL reference timings are supported where each DL reference timing is associated with one TAG

- baseline assumption is that the Rx timing difference between the two DL reference timings is no larger than CP length

- as an optional UE capability, Rx timing difference between the two DL reference timings can be assumed to be larger than CP length

  - FFS: the maximum Rx timing difference (could be up to RAN4)

  - Other than UE capability details and relevant configuration, no additional RAN1 specification enhancement specific for this case is expected

Agreement

For multi-DCI multi-TRP operation with two TAs in a CC, two DL reference timings are supported where each DL reference timing is associated with one TAG

- baseline assumption is that the Rx timing difference between the two DL reference timings is no larger than CP length

- as an optional UE capability, Rx timing difference between the two DL reference timings can be assumed to be larger than CP length

  - FFS: the maximum Rx timing difference (could be up to RAN4)

  - Other than UE capability details and relevant configuration, no additional RAN1 specification enhancement specific for this case is expected

RAN2 has discussed the 2TA aspects including per TRP UE initiated RACH procedure (CBRA), configuring more than one TAG per serving cell based on RAN1 input and RAN2 contributions.

As for whether CBRA is to be supported for Initial TA acquisition per TAG/TRP (when UE is configured with 2TA operation), RAN2 agreed the following:

From RAN2 perspective, per TRP UE-initiated RACH procedure is not supported.

# 2	TA groups

As for configuring more than one TAG per serving cell aspects related to grouping and related operation for 2TAs, RAN2 send some questions to RAN1.

There is general understanding in RAN2 that, in the functionality RAN1 is designing, two time-alignment timers are used for one serving cell. One timer is associated to UL towards one TRP and the other timer is associated to UL towards the other TRP.

In TS 38.300, the following is specified for current TAG operation:

****************************************************************************************************************

### 9.2.9	Timing Advance

In RRC_CONNECTED, the gNB is responsible for maintaining the timing advance to keep the L1 synchronised. Serving cells having UL to which the same timing advance applies and using the same timing reference cell are grouped in a TAG. Each TAG contains at least one serving cell with configured uplink, and the mapping of each serving cell to a TAG is configured by RRC.

For the primary TAG the UE uses the PCell as timing reference. In a secondary TAG, the UE may use any of the activated SCells of this TAG as a timing reference cell, but should not change it unless necessary.

Timing advance updates are signalled by the gNB to the UE via MAC CE commands. Such commands restart a TAG-specific timer which indicates whether the L1 can be synchronised or not: when the timer is running, the L1 is considered synchronised, otherwise, the L1 is considered non-synchronised (in which case uplink transmission can only take place on PRACH).

****************************************************************************************************************

For the 2TA operation, RAN1 has agreed the following reference timing functionality:

Agreement

For multi-DCI multi-TRP operation with two TAs in a CC, two DL reference timings are supported where each DL reference timing is associated with one TAG

- baseline assumption is that the Rx timing difference between the two DL reference timings is no larger than CP length

- as an optional UE capability, Rx timing difference between the two DL reference timings can be assumed to be larger than CP length

  - FFS: the maximum Rx timing difference (could be up to RAN4)

  - Other than UE capability details and relevant configuration, no additional RAN1 specification enhancement specific for this case is expected

“RAN2 impact: none”

“RAN4 impact: max Rx timing difference”

If PCell is configured with two TA operation RAN2 should discuss whether there is the 2 PTAGs considered to be configured for the UE or if the other TAG is STAG. Current procedures in TS 38.321 for PTAG are as follows:

1>	when a timeAlignmentTimer expires:

2>	if the timeAlignmentTimer is associated with the PTAG:

3>	flush all HARQ buffers for all Serving Cells;

3>	notify RRC to release PUCCH for all Serving Cells, if configured;

3>	notify RRC to release SRS for all Serving Cells, if configured;

3>	clear any configured downlink assignments and configured uplink grants;

3>	clear any PUSCH resource for semi-persistent CSI reporting;

3>	consider all running timeAlignmentTimers as expired;

3>	maintain NTA (defined in TS 38.211 [8]) of all TAGs.

Options to support are as follows:

- UE can be configured with two PTAGs and if one of the associated timer expires, UE follows the already specified procedures of one PTAG.

- UE can only be configured with one PTAG and the second TAG is redefined to be STAG. This means the definition of STAG and PTAG need to be refined as the groups consider now TRPs and not serving cells.

Option 1 does not allow UE to continue operation if it still has one timer associated to the PTAG running which likely is not according to the functionality envisioned by RAN1. Then again, current definition and procedures of PTAG can be reused. For option 2, RAN2 needs to discuss whether there is hierarchy between the PTAG and STAG timers both associated to PCell, or whether PTAG procedures are applied only when both timers expire.

RAN2 to discuss how to apply the 2TA functionality to PCell.

Similar discussion can be done for STAG. Current procedures in TS 38.321 for STAG are as follows:

2>	else if the timeAlignmentTimer is associated with an STAG, then for all Serving Cells belonging to this TAG:

3>	flush all HARQ buffers;

3>	notify RRC to release PUCCH, if configured;

3>	notify RRC to release SRS, if configured;

3>	clear any configured downlink assignments and configured uplink grants;

3>	clear any PUSCH resource for semi-persistent CSI reporting;

3>	maintain NTA (defined in TS 38.211 [8]) of this TAG.

Options to support are as follows:

- UE can be configured with two STAGs per serving cell and if one of the associated timer expires, UE follows the already specified procedures for on STAG.

- UE can be configured with two STAGs per serving cell and only if both timers associated to STAGs expire, UE follows the already specified procedures.

Option 1 does not allow UE to continue operation if it still has one timer associated to the STAG running which likely is not according to the functionality envisioned by RAN1. For Option 2, the procedures need to be redefined such that only the UL/DL operation related to the associated TRP are impacted. Eventually, RAN2 needs to discuss per UL/DL operation but this paper focuses on more stage-2 level aspects as instructed in the agenda.

RAN2 to discuss how to apply the 2TA functionality to the sCells.

RAN2 asked RAN1 in the LS whether there are any restrictions to configured serving cell/TRPs for the TAG. While waiting for the response, RAN2 can discuss the operation from RAN2 perspective. If UE has a serving cell that is associated to two different TAGs via its TRPs, RAN2 needs to discuss the following cases. What happens to the other serving cells of a TAG, if the TAG contains a TRP of a serving cell that has both timers expired, while the other serving cells still have one timer running for one TRP since they would belong to another TAG via those TRPs? Similar question arises if UE can be configured with TAGs that contain both serving cells with 1 and 2 TA operation.

RAN2 to discuss if from RAN2 perspective UE can be configured serving cells that are associated to different TAGs.

RAN2 to discuss if from RAN2 perspective UE can be configured with TAGs that contain both serving cells with 1 and 2 TA operation.

# 2	PRACH configuration “per additional PCI”

RAN1 had the following agreement:

Agreement

Confirm the following working assumption:

For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, one additional PRACH configuration is supported for each configured additional PCI

- the additional PRACH configuration is used in a RACH procedure triggered by a PDCCH order for the corresponding configured additional PCI

Note that RAN1 supports only CFRA for 2TA operation and it is up to RAN2 to decide on CBRA. Hence this discussion is related to CFRA and it is assumed RAN2 can decide later whether CBRA needs to be supported or not.

In TS 38.331, PRACH configuration is given to the UE in IE The CellGroupConfig, which is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels with associated RLC entities and of a primary cell (SpCell) and one or more secondary cells (SCells).

Within that IE the PRACH configuration is given with IE ReconfigurationWithSync:

ReconfigurationWithSync ::=         SEQUENCE {

spCellConfigCommon                  ServingCellConfigCommon                                     OPTIONAL,   -- Need M

newUE-Identity                      RNTI-Value,

t304                                ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

rach-ConfigDedicated                CHOICE {

uplink                              RACH-ConfigDedicated,

supplementaryUplink                 RACH-ConfigDedicated

}                                                                                               OPTIONAL,   -- Need N

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

daps-UplinkPowerConfig-r16      DAPS-UplinkPowerConfig-r16                                      OPTIONAL    -- Need N

]],

[[

sl-PathSwitchConfig-r17         SL-PathSwitchConfig-r17                                         OPTIONAL    -- Cond DirectToIndirect-PathSwitch

]]

}

#### –	RACH-ConfigDedicated

The IE RACH-ConfigDedicated is used to specify the dedicated random access parameters.

RACH-ConfigDedicated information element

-- ASN1START

-- TAG-RACH-CONFIGDEDICATED-START

RACH-ConfigDedicated ::=        SEQUENCE {

cfra                            CFRA                                                                    OPTIONAL, -- Need S

ra-Prioritization               RA-Prioritization                                                       OPTIONAL, -- Need N

...,

[[

ra-PrioritizationTwoStep-r16    RA-Prioritization                                                       OPTIONAL, -- Need N

cfra-TwoStep-r16                CFRA-TwoStep-r16                                                        OPTIONAL  -- Need S

]]

}

CFRA ::=                    SEQUENCE {

occasions                       SEQUENCE {

rach-ConfigGeneric              RACH-ConfigGeneric,

ssb-perRACH-Occasion            ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

OPTIONAL  -- Cond Mandatory

}                                                                                                       OPTIONAL, -- Need S

resources                       CHOICE {

ssb                             SEQUENCE {

ssb-ResourceList                SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource,

ra-ssb-OccasionMaskIndex        INTEGER (0..15)

},

csirs                           SEQUENCE {

csirs-ResourceList              SEQUENCE (SIZE(1..maxRA-CSIRS-Resources)) OF CFRA-CSIRS-Resource,

rsrp-ThresholdCSI-RS            RSRP-Range

}

},

...,

[[

totalNumberOfRA-Preambles INTEGER (1..63)                                                             OPTIONAL -- Cond Occasions

]]

}

CFRA-TwoStep-r16 ::=                    SEQUENCE {

occasionsTwoStepRA-r16                  SEQUENCE {

rach-ConfigGenericTwoStepRA-r16         RACH-ConfigGenericTwoStepRA-r16,

ssb-PerRACH-OccasionTwoStepRA-r16       ENUMERATED {oneEighth, oneFourth, oneHalf, one,

two, four, eight, sixteen}

}                                                                                                     OPTIONAL, -- Need S

msgA-CFRA-PUSCH-r16                     MsgA-PUSCH-Resource-r16,

msgA-TransMax-r16                       ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}    OPTIONAL, -- Need S

resourcesTwoStep-r16                    SEQUENCE {

ssb-ResourceList                        SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource,

ra-ssb-OccasionMaskIndex                INTEGER (0..15)

},

...

}

CFRA-SSB-Resource ::=           SEQUENCE {

ssb                             SSB-Index,

ra-PreambleIndex                INTEGER (0..63),

...,

[[

msgA-PUSCH-Resource-Index-r16   INTEGER (0..3071)     OPTIONAL  -- Cond 2StepCFRA

]]

}

CFRA-CSIRS-Resource ::=         SEQUENCE {

csi-RS                          CSI-RS-Index,

ra-OccasionList                 SEQUENCE (SIZE(1..maxRA-OccasionsPerCSIRS)) OF INTEGER (0..maxRA-Occasions-1),

ra-PreambleIndex                INTEGER (0..63),

...

}

-- TAG-RACH-CONFIGDEDICATED-STOP

-- ASN1STOP

There is also related RRC procedure specified in Clause 5.3.5.8.3 which states that at T304 expiry (Reconfiguration with sync Failure) or T420 expiry (Path switch failure) UE release preambles provided in rach-ConfigDedicated if configured and so on.

Currently, rach-ConfigDedicated is given per cell group and RAN1 is asking to have 7 additional RACH configurations per each serving cell which is configured with intercell mTRP to potentially to be used by the other TRP. RAN2 should discuss and clarify from RAN1, what exactly needs to be added for RACH configuration for the 2TA operation. It is also possible, that the added RACH configuration could be something similar to IE BeamFailureRecoveryConfig.

RAN2 to clarify what exactly needs to be different per RACH configuration per additional PCI.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to discuss how to apply the 2TA functionality to PCell.

Proposal 2	RAN2 to discuss how to apply the 2TA functionality to the sCells.

Proposal 3	RAN2 to discuss if from RAN2 perspective UE can be configured serving cells that are associated to different TAGs.

Proposal 4	RAN2 to discuss if from RAN2 perspective UE can be configured with TAGs that contain both serving cells with 1 and 2 TA operation.

Proposal 5	RAN2 to clarify what exactly needs to be different per RACH configuration per additional PCI.